3GPP TSG RAN WG1 Meeting #106-e	                               R1-2106958

e-Meeting, August 16th-27th , 2021

Source:	CATT

Title:	Enhancements for PUCCH formats for up to 71GHz operation

Agenda Item:	8.2.3

Document for:	Discussion

- Introduction

In RAN#90e meeting, the “Enhancements for PUCCH formats 0/1/4” was approved. The objective of the work item is presented in [1]. The objective in [1] is:

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

In RAN1#104bis-e meeting [2], the following agreements were made:

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions.

Agreement:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfil the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

  - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

Agreement:

For UCI of enhanced PF4, support pre-DFT block wise spreading using OCCs of length 2 and 4 only, as in Rel-15/16.

Agreement:

For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

Agreement:

For UCI of enhanced PF4, support pre-DFT block wise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e).

Agreement:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

R1-2104060	[DRAFT] LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band

R1-2104061	LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band

Agreement:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

In RAN1#105-e meeting [2], the following agreements were made:

Agreement:

- For 120 kHz SCS:

  - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration

  - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following:

    - PUCCH resources before dedicated PUCCH resource configuration

    - PUCCH resources after dedicated PUCCH resource configuration

  - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details

- Notes:

  - Alt-1 = all REs within each RB are mapped

  - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping)

  - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106

- Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

In this contribution, we study different design options corresponding to the objective of the work item and we propose some ways to move forward.

- Discussion

Number of RBs for Enhanced PF0/1/4

In RAN1#104bis-e meeting, the following agreements were made [2]:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

In [8], the following answer is provided:

- RAN4 can confirm that the current regulatory limits, i.e. max EIRP and max TRP, are higher than the above values. We further note that for the 52.6 to 71 GHz frequency range, regulations in some regions also specify a maximum spectral power density (EIRP).

- While power class performance is TBD, an FWA UE is expected to yield the highest minimum peak EIRP, and it may be specified around 25 dBm or higher. However, further study is needed to confirm this and provide an exact minimum peak EIRP value. A power ranging from minimum peak EIRP to below the regulatory maximum EIRP limit, is technically valid for the UE to transmit out.

For additional context, different radiated output powers in 38.101-2 are summarized in the table below. Please note that there is no conducted output power requirements defined for FR2 in RAN4 specifications.

Table 1. FR2 minimum peak EIRP requirements

| Power class | Max TRP [dBm] | FR2 band | Min peak EIRP [dBm] | Max EIRP [dBm] |
| --- | --- | --- | --- | --- |
| Power class 1 Fixed wireless access UE | 35 | n257/n258/n261 | 40.0 | 55 |
|  |  | n260 | 38.0 |  |
| Power class 2 Vehicular UE | 23 | n257/n258/n261 | 29.0 | 43 |
| Power class 3 Handheld UE | 23 | n257/n258/n261 | 22.4 | 43 |
|  |  | n260 | 20.6 |  |
|  |  | n259 | 18.7 |  |
|  |  | n262 | 16.0 |  |
| Power class 4 High-power non-handheld UE | 23 | n257/n258/n261 | 34.0 | 43 |
|  |  | n260 | 31.0 |  |
| Power class 5 Fixed wireless access UE | 23 | n257 | 30.0 | 43 |
|  |  | n258 | 30.4 |  |

TRP: Total Radiated Power

EIRP: Effective Isotropic Radiated Power

Considering RAN4 is in the early stages of our discussions, this is the information we can provide at this time. Further guidance will be provided as power class discussions progress in RAN4.

As discussed in RAN1#104bis-e meeting, the maximum PUCCH bandwidth (in MHz) for the US region before the UE limits kick in is given by:

BWMHZ= min10UE_EIRP-TxBF-710，10UE_P-CM-710

The above equation is a function of UE_EIRP, UE_P, TxBF, and the cubic metric (CM).

In R1-2104061 (LS to RAN4), RAN1 had assumed the following UE hardware limits:

- Maximum UE Conducted Power = 21 dBm

- Maximum UE EIRP = 25 dBm

Based on RAN4’s reply in [8] that “the current regulatory limits, i.e. max EIRP and max TRP, are higher than the above values”, we can study different values of these parameters to see how it affects the number of RBs. The following additional set of values is to be considered in addition to the already agreed evaluation assumptions in Table 2 and 3 in Section 2.3 of [5]:

- UE_EIRP = 40 dBm

- UE_P = 25 dBm

- TxBF = 0 dBi

N_RB is derived simply by converting the maximum PUCCH bandwidth from the above equation to number of RBs for 120, 480, and 960 kHz SCS. The maximum values for the configured number of RBs with the additional set of values are:

- CM = 0 dB

  - 44 RBs for 120 kHz SCS

  - 11 RBs for 480 kHz SCS

  - 6 RBs for 960 kHz SCS

- CM = 2 dB

  - 28 RBs for 120 kHz SCS

  - 7 RBs for 480 kHz SCS

  - 4 RBs for 960 kHz SCS

It can be seen that for different UE power classes, the maximum configurable number of RBs are different. Base on RAN4’s answer [8] “A power ranging from minimum peak EIRP to below the regulatory maximum EIRP limit, is technically valid for the UE to transmit out.”, the maximum values for enhanced PF0/1/4 may be configured as two sets. The smaller set can be used to reduce the bandwidth, while the larger set can be used to improve the coverage.

For different UE power classes, the maximum configurable numbers of RBs are different.

Two sets of maximum values for enhanced PF0/1/4 can be used. The smaller set can be used to reduce the bandwidth, while the larger set can be used to improve the coverage.

Sequence Construction for Enhanced PF0/1

In RAN1#104e meeting, the following agreements were made [3]:

- For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives:

  - Alt-1: A single sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that uses Interlace PUCCH-PUSCH is not configured.

  - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction:

    - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that uses Interlace PUCCH-PUSCH is configured

- At least the following aspects should be considered in the study

  - Coverage (maximum isotropic loss (MIL)), including

    - Required SNR to fulfill PUCCH detection criterion

    - PAPR/CM as a function of N_RB

  - Specification impact

From the above agreement, there are two options to expand the length of the sequence for PUCCH Format 0/1 in frequency domain. One is “a single sequence of length equal to the total number of mapped REs of the PUCCH resource is used”, which is named as “long sequences” in the later sections. The other is “a single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used” and “considered for PAPR/CM reduction”, which is named as “repetitive sequences with low PAPR” in the later sections.

Long sequences

To PUCCH format 0/1, the Zadoff-Chu sequences of length 36 or above can be used by truncation and expansion to align with the length of multiple PRBs similar to that used for SRSs.

For MZC≥3NscRB, the base sequence  is given by [4]

where

The length  is given by the largest prime number such that.

If the long sequence is introduced to enhance PUCCH format0/1, new PUCCH formats are also needed.

The Zadoff-Chu sequences of length 36 or above in NR can be used by truncation or expansion to align with the length of multiple PRBs.

Repetitive sequences with low PAPR

If repetitions of the sequences used for PUCCH format 0/1 are introduced to meet the channel occupancy requirement, the PAPR is expected to increase with the number of repetition increase.

The PAPR results from different number of sequences repetition are shown in Figure1. From Figure 1, the PAPR increases in proportion to the increase of the number of repetitions. The increased PAPR for the higher number of sequence repetitions would reduce the maximum Tx power, which reflect in the reduced the coverage of PUCCH.   Thus, the PAPR reduction is essential for the sequence repetition for PUCCH format 0/1.

Figure 1: PAPR for Repetitive Sequences of different RBs

If sequence repetition is used for PUCCH format 0/1, PAPR increases as the number of repetition increases.

The method to reduce the PAPR should be supported if repetitive sequences are adopted.

CM of different sequences

To compare the two options, the results of CM and BLER are shown in Figure 2~6. For repetitive sequence, the method to reduce the PAPR is same as used in interlace.

Figure 2: CM for Different Sequences of 3 RBs

Figure 3: CM for Different Sequences of 6 RBs

Figure 4: CM for Different Sequences of 12 RBs

Figure 5: CM for Different Sequences of 24 RBs

Figure 6: CM for Different Sequences of 36 RBs

From Figure 2, 3, 4 and 6, the CM of the repetitive sequence with low PAPR is larger than that of the Zadoff-Chu sequence when the length of sequence is 3, 6, 24 and 36. From Figure 5, the CM of the repetitive sequence with low PAPR is almost same as that of the Zadoff-Chu sequence when the length of sequence is 12. It can be found that with different number of RBs, the CM of the Zadoff-Chu sequence doesn’t change as much as the repetitive sequence with low PAPR, which is beneficial for design of RF unit.

As described in 2.1, the number of RBs for Enhanced PF0/1/4 may range from 1 to 44 for 120 kHz SCS. In this case, the long sequences are preferred to keep similar CM for sequences with different lengths.

With different number of RBs, the CM of the Zadoff-Chu sequence doesn’t change as much as the repetitive sequence with low PAPR.

For enhanced PUCCH format 0/1 sequence, Alt1 (long sequences) is preferred to keep similar CM for sequences with different lengths.

RB granularity

In RAN1#104bis-e meeting, the following agreements were made [2]:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfil the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

  - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

The number of PRBs for enhanced (multi-RB) PF 0/1/4 should be a range rather than a definite value. However, when the maximum value for the configured number of RBs is larger than 16, it is unnecessary to support all the integer values in the range [1 .. max(NRB)].

On one hand, the performance is similar when the number of RB is close. On the other hand, more bits are needed when the configured number of RBs is larger than 16, which is 4 bits for PUCCH format2/3 in R16.

There is a solution that the RB granularity is configurable. When the RB granularity is set to 1, it is equivalent to Alt-1; and in other cases, it is equivalent to Alt-2. The advantages of variable RB granularity are as follows:

- Support both Alt-1 and Alt-2 through different configurations

- The RB granularity can be adjusted flexibly according to the maximum value for the configured number of RBs. So the number of bits used to indicate the number of RBs configuration can be limited to 4 bits.

- The RB granularity can be adjusted flexibly according to the number of UEs in system. So the gNB could adjust the number of matched UEs (UEs with the same RBs configuration).

It should be clarified that Alt-1 and Alt-2 are for discussing level of granularity supported. For both alternatives, the RB granularity is configurable.

The configurable RB granularity is preferred for the configuration of the number of RBs.

PUCCH Resource Sets Configuration

As mentioned in 2.1, we can find that the number of required RBs is different for different power classes even under the same PSD limit, because of the different EIPR limits. Furthermore, even if the power classes are same, the number of required RB will be different when the UE antenna gain is different.

Besides the power class, because of the power control, the Tx power for different UE is different. If the same number of RBs is used for different UEs, it will result in a waste of frequency resource. When there are lots of users in the cell, it will lead to frequency domain resources shortage. For UEs, longer sequences detection will consume larger power; therefore, it is beneficial for UE power saving to use different numbers of RBs for different Tx powers.

Therefore, the number of PRBs or enhanced (multi-RB) PF 0/1/4 may be different for different UEs, and can be varied with time for a UE. Accordingly, the gNB needs to inform the values of PUCCH RBs before the UE transmits PUCCH.

The number of RBs for PUCCH format0/1/4 can be cell specific or UE specific configured.

In the following, we discussed how the UE of different states will know the RB number of the PUCCH.

RRC connected UE

- RRC configuration

For RRC connected UEs, one solution is the gNB could associate the PUCCH Resource ID with the number of RBs similar to that of PUCCH format 2/3 in [6]. When the UE obtains the PUCCH Resource ID, it will obtain the number of RBs.

PUCCH-ResourceId ::=                    INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (n1,n2,..,nn),

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (n1,n2,..,nn),

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (n1,n2,..,nn),

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

}

The gNB can calculate the number of RBs through the information UE reported such as PHR. To help the gNB to decide the number of RB for PUCCH, the reference number of RB for PUCCH could be reported by UE through RRC.

- DCI information

As an alternative, the gNB could inform the number of RBs for PUCCH through DCI. This method needs a new field in the DCI format for PUCCH.

The bit width of this new field is decided by the maximum value for the configured number of RBs and the RB granularity. If the RB granularity is variable, the bit width could be same for all SCSs.

For RRC connected UEs, the gNB could use RRC configuration or DCI to indicate UE the configured number of RBs.

Initial access process

As mentioned above, because the SCS, PSD limit and EIPR limits may be different, the supported number of PUCCH RBs may be different for different deployment scenario. Then, during the initial access process, the gNB needs to indicate the PUCCH format to the UE with the configured number of RBs for PUCCH format0/1/4 RBs which are used for HARQ of MSG4 during the initial access process.

The configuration of the RB number for the PUCCH could be informed by SIB1. There are two methods for the gNB to inform the number of RBs for PUCCH.

Option1: The gNB uses SIB1 to configure an index of PUCCH resource sets and the index associate the PUCCH resource sets index with the number of RBs similar to the table in [7].

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | Number of RBs | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 1 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 1 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 12 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 1 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 1 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 12 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 12 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 1 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 1 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 12 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 12 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 1 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 1 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 12 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 12 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 | 1 |  | {0, 3, 6, 9} |

Option2: The available number of RBs for PUCCH can be informed through broadcast directly, e.g., SIB1.

If needed, the UE could report its information in MSG3 to help the gNB to obtain the number of RBs, such as the reference number of RBs for PUCCH, PHR and so on.

The gNB needs to indicate the UE with the configured number of RBs for PUCCH format0/1/4 during the initial access process.

- Conclusions

This contribution discussed the PUCCH enhancements for the support of NR operation up to 71 GHz. We have the following observations and proposals,

- Observation 1: For different UE power classes, the maximum configurable numbers of RBs are different.

- Observation 2: The Zadoff-Chu sequences of length 36 or above in NR can be used by truncation or expansion to align with the length of multiple PRBs.

- Observation 3: If sequence repetition is used for PUCCH format 0/1, PAPR increases as the number of repetition increases.

- Observation 4: With different number of RBs, the CM of the Zadoff-Chu sequence doesn’t change as much as the repetitive sequence with low PAPR.

- Observation 5: It should be clarified that Alt-1 and Alt-2 are for discussing level of granularity supported. For both alternatives, the RB granularity is configurable.

- Proposal 1: Two sets of maximum values for enhanced PF0/1/4 can be used. The smaller set can be used to reduce the bandwidth, while the larger set can be used to improve the coverage.

- Proposal 2: The method to reduce the PAPR should be supported if repetitive sequences are adopted.

- Proposal 3: For enhanced PUCCH format 0/1 sequence, Alt1 (long sequences) is preferred to keep similar CM for sequences with different lengths.

- Proposal 4: The configurable RB granularity is preferred for the configuration of the number of RBs.

- Proposal 5: The number of RBs for PUCCH format0/1/4 can be cell specific or UE specific configured.

- Proposal 6: For RRC connected UEs, the gNB could use RRC configuration or DCI to indicate UE the configured number of RBs.

- Proposal 7: The gNB needs to indicate the UE with the configured number of RBs for PUCCH format0/1/4 during the initial access process.

- References

RP-202925, “Revised WID: on Extending current NR operation to 71 GHz”

3GPP TSG RAN WG1, “RAN1 Chairman’s Notes”, Meeting #104bis-e, e-Meeting, April 12th – 20th, 2021”

3GPP TSG RAN WG1, “RAN1 Chairman’s Notes”, Meeting #104-e, e-Meeting, January 25th – February 5th, 2021”

R1-2102127, “FL Summary 4 for Enhancements for PUCCH formats 0/1/4”, Meeting #104-e, e-Meeting, January 25th – February 5th, 2021”

3GPP TS 38.211, “NR; Physical channels and modulation”

3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

3GPP TS 38.213, “NR; Physical layer procedures for control”

R4-2107950, “LS reply on maximum UE EIRP and conducted power”