3GPP TSG RAN WG1 #106-e	      	R1-2106959

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Beam management for new SCSs for up to 71GHz operation

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

The work item on supporting NR from 52.6 GHz to 71 GHz was update and approved in RAN#90-e meeting [1]. Objects of WI are listed as following:

- Physical layer aspects including [RAN1]: - In addition to 120 kHz SCS, specify new SCS, 480 kHz and 960 kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Time line related aspects adapted to 480 kHz and 960 kHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120 kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240 kHz, 480 kHz, 960 kHz) for SSB, and additional SCS (480 kHz, 960 kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480 kHz, 960 kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

In this document, we focus on the issues of beam management from 52.6GHz to 71 GHz for new SCSs, which includes BFR mechanism enhancements, beam switching related parameters in beam management procedure and beam switching gap handling for signals/channels, multiple beam indications for multi-PUSCH/ PDSCH scheduling and enhancements to beam management for random access procedure.

- Discussion

  - Study of Beam Management and BFR mechanism enhancements

- Aperiodic CSI-RS for Beam Management and BFR

In R16 BFR mechanism, UE measures periodic CSI-RS and SSB for beam failure detection and candidate beam selection. If the LBT mechanism is introduced for NR operation in unlicensed spectrum, gNB may fail to transmit periodic CSI-RS due to the LBT failure. If gNB did not transmit periodic CSI-RS, the RSRP of the beam measured by UE is likely to be lower than the threshold, and the beam would be regarded as not available to UE. If the gNB fails to transmit the periodic CSI-RS for multiple beams for a period of time due to LBT failure, all the candidate beams of UE measurement may below the threshold. UE will immediately trigger a beam failure event and indicate to higher layer. UE can be configured in triggering the beam failure recovery procedure when one or two times the beam failure instance indication has been received from lower layers. In this case, the UE will enter the wrong beam failure recovery procedure which leads to radio resources waste and the transmission delay. If the UE knows the LBT failure caused by the miss detection of the non-transmitted periodic CSI-RS, UE can disable the triggering of the beam failure instance. UE cannot detect no CSI-RS transmission from LBT failure or beam misalignment unless UE perform hypothesis testing of energy detection of CSI-RS transmission at every instance.

On the other hand, the gNB has full knowledge of no CSI-RS transmission because of LBT failure. gNB could trigger the transmission of aperiodic CSI-RS and indicate to the UE at the next available instance. UE could use the A-CSI-RS to make up the measurement for beam management and perform DL/UL beam correspondence due to LBT failure of periodic CSI-RS transmission.

Observation 1: When UE cannot measure the periodic CSI-RS at the scheduled transmission instance for beam management due to LBT failure, gNB could transmit aperiodic CSI-RS and indicate to the UE as the alternative measurement.

Proposal 1: Aperiodic CSI-RS could be used as the alternative solution of missed L1 RSRP measurement of periodic CSI-RS due to LBT failure with little specification change.

- Increased number of RSs

The maximum number of SSBs supported by a cell is 64 for FR2 or frequency range above 7 GHz.  The maximum number of RSs configured for beam management is 10. From [1], up to 64 SSB beams for licensed and unlicensed spectrum are supported for NR operation in 52.6GHz to 71GHz. The propagation loss would be much higher when NR operates in 52.6 GHz – 71 GHz. In order to accommodate the propagation loss in high frequency band, the beam width would be formed to sharply narrow. When the beam width becomes narrower, the coverage area of each beam would be smaller. The number of beams for UE to monitor and to maintain/manage needs to increase to support UE mobility moving between beams. The beam management and CSI-RS configuration for beam management needs to be enhanced for NR operation with narrower beam in 52.6 – 71 GHz. Furthermore, the number of bits in the TCI (transmission configuration indication) bit domain in DCI for beam indication may need to be increased. The beam management frame work should be reused for NR operation in 52.6-71 GHz.

Proposal 2: The beam management frame work should be reused for NR operation in 52.6-71 GHz.

  - Beam switching related parameters

Agreement: Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH Agreement: For timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Following candidate values of FR2 are reused for 120 kHz: - timeDurationForQCL: 14 and 28 symbols - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols - beamReportTiming: 14, 28 and 56 symbols - For 480 kHz - Support at least the candidate values for 120 kHz scaled by 4x - FFS: Support for additional candidate value(s) - For 960 kHz - Support at least the candidate values for 120 kHz scaled by 8x - FFS: Support for additional candidate values(s) - FFS: UE capability signalling details - Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16).

According to the agreement in RAN1#104bis-e, beam switching related parameters may consider about the following aspects:

- Additional beam switching timing delay d

When DCI schedule the aperiodic CSI-RS, the subcarrier spacing of PDCCH carrying the triggering DCI is different from aperiodic CSI-RS, the additional beam switching time delay d are introduced to lengthen the beamSwitchTiming reported by UE, the unit of d is OFDM symbol which result in the shorten of d as SCS increasing, hence the value of delay d should increase when SCS of PDCCH/CSI-RS is 480KHz or 960KHz. The value in specification (TS 38.214) may change as below:

Table 1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |
| 3 | 26 |
| 5 | 48 |
| 6 | 90 |

- maxNumberRxTxBeamSwitchDL

maxNumberRxTxBeamSwitchDL defines the number of Tx and Rx beam changes UE can perform on this band within a slot. UE shall report one value per subcarrier spacing supported by the UE. When SCS is 480KHz or 960KHz, the duration of each OFDM symbol would be shorter which may not be reasonable for UE to perform beam switching as much as 14 times within a slot, Moreover, the minimum available value can be smaller in order to limit the number of times of beam switching indicated by the gNB to UE within a slot. The value in specification (TS 38.214) may change as below:

maxNumberRxTxBeamSwitchDL         SEQUENCE {

scs-15kHz  				    ENUMERATED { n4, n7, n14 }          		 OPTIONAL,

scs-30kHz   				   ENUMERATED { n4, n7, n14 }	OPTIONAL,

scs-60kHz   			   ENUMERATED { n4, n7, n14 }          		 OPTIONAL,

scs-120kHz      			ENUMERATED { n4, n7, n14 }          		 OPTIONAL,

scs-240kHz      			ENUMERATED { n4, n7, n14 }          		 OPTIONAL,

scs-480kHz     				ENUMERATED { n2, n4, n7 }  				OPTIONAL,

scs-960kHz     				ENUMERATED { n2, n4, n7 }     			OPTIONAL

}

Observation 2: When SCS is 480KHz or 960KHz, the duration of each OFDM symbol would be shorter.  UE may not support performing beam switching as much as 14 times within a slot.

Proposal 3: For SCS 480/960 kHz, the minimum and maximum available value of maxNumberRxTxBeamSwitchDL should be reduced.

  - QCL assumption of the UE For multi-PDSCH scheduling with a single DCI

Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL.

In NR Rel-15/R16, the QCL assumption of UE receiving a DCI with indication in switching the beam for the PDSCH reception and processing can mainly summarize into following instances.

When the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the QCL assumption of the UE is the same as CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot.

When the offset K0 between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are QCL’d (quasi co-located) with the RS(s) used for channel tracking with the same TCI state.

When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH.

When the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to perform independent reception of PDSCH and PDCCH associated with that CORESET with DL/UL correspondence is based on the PDCCH QCL assumption.

If a DCI schedules multi-PDSCH is supported, the QCL assumptions of scheduled PDSCHs can be discussed under two scenarios according to the agreement above from RAN1#104-e.

- Scenario one

A DCI schedules multiple PDSCHs, all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL.

Because of the UE does not have enough time to apply the beam indication according to DCI, all the PDSCH(s) should QCL’d with the first PDSCH scheduled by DCI except for the PDSCH overlapping with the PDCCH.

- Scenario two

A DCI schedules multiple PDSCHs, some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL and some have scheduling offset equal to or greater than timeDurationForQCL.

For the part of PDSCH(s) have scheduling offset less than timeDurationForQCL, the PDSCHs should QCL’d with the first PDSCH scheduled by DCI except for the PDSCH(s) overlap with the PDCCH as the UE do not have enough time to apply the beam indication according to DCI.

For the part of PDSCH(s) have scheduling offset equal to or greater than timeDurationForQCL, when the PDSCH(s) QCL’d with the RS in the TCI state indicated by the DCI, UE may obtain the reception gain on the corresponding indicated beam. Alternatively, when the PDSCH(s) QCL’d with the PDSCH(s) which have scheduling offset less than timeDurationForQCL, the DMRS may benefit from the joint channel estimation as the PDSCHs are contiguous.

Both options can be supported for multi-PDSCH scheduled by single DCI, and UE could choose one of them according to different scheduling conditions. For instance, if the resource reservation is available for DMRS joint channel estimation, e.g. the PDSCHs are time continuous with same position in the frequency domain, then the QCL assumption for the PDSCH(s) can be same as PDSCH(s) which have scheduling offset less than timeDurationForQCL. Otherwise, if the resource reservation is not available for DMRS joint channel estimation, e.g., the scheduled PDSCHs are not time continuous or with overlapping PDCCH, then the PDSCH(s) can QCL’d with the RS in the TCI state indicated by the DCI.

Observation 3: In some scenarios, PDSCH(s) QCL’d with the RS in the TCI state indicated by the DCI can achieve reception gain; while in other scenarios PDSCH(s) QCL’d with the PDSCH(s) which have scheduling offset less than timeDurationForQCL can achieve reception gain.

Proposal 4: When some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL and some have scheduling offset equal to or greater than timeDurationForQCL, both options below should be supported for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL. FFS conditions to determine the QCL assumption.

- The scheduled PDSCHs quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state in DCI.

- The scheduled PDSCHs quasi co-located with the RS(s) based on the activated TCI states in the first slot with the scheduled PDSCH.

  - Beam switching gap

The normal CP length is approximately 146ns for 480 KHz SCS, which is long enough for beam switching time typically at about 100 ns. When SCS is 960 KHz, the CP length is about 73ns and is not sufficiently for the time of beam switching. So the additional beam switching gap is needed to introduce for 480/960 kHz SCS. There are two following possible solutions shown in Figure 1:

- Avoided by gNB scheduling

The first potential solution is the gNB implementation to avoid scheduling any transmission during the transition of beam switching.

In Rel-16, beam switching may occur in certain scenarios that the scheduling offset between DCI and scheduled PDSCH is equal to or greater than the threshold timeDurationForQCL. Then, for 480/960 kHz SCS, beam switching may occur in certain scenarios that the scheduling offset between DCI and scheduled PDSCH is equal to or greater than the threshold timeDurationForQCL+1. One symbol should be reserved for beam switching.

- Define a gap symbol in PDSCH resources

The second alternative is to define a gap symbol before the first symbol of PDSCH for beam switching. The original first symbol of PDSCH is used for beam switching, and the original second symbol of PDSCH is redefined as the starting symbol.

For the second solution, the threshold is still timeDurationForQCL, because the symbol used for beam switching has included in the PDSCH resources.

Figure 1: Beam switching gap solutions

Observation 4: In order to guarantee the reception performance of PDSCH, the additional beam switching gap need to be reserved before the PDSCH.

Proposal 5: When the additional beam switching gap is introduced, QCL assumption needs to be investigated.

  - Multiple beam indications for multi-PUSCH/PDSCH scheduling

Agreement: For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI - FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

If multi-PUSCH/PDSCH scheduling is supported, the PDSCHs scheduled by a DCI may transmit on one   or multiple beams in different scenarios. When PDSCHs scheduled to transmit on the same beam, the existing TCI state field in DCI for the beam indicator could be reused. However, there might be more than one qualified TCI state for the narrow band operation above 52.6GHz, and with similar beam qualities. gNB may obtain diversity gain by scheduling multiple PDSCHs through PDSCH in different TCI states at different time. In particular for NR operation in unlicensed band, the interference for different beams might be different. Transmitting on different beams might reduce the possibility of PDSCHs transmission failure.

Another issue is for multi-TRP scenarios, single DCI scheduled multiple PDSCHs may need to be transmitted from different TRPs, which PDSCH would have different TCI states (different beams), as in Figure 2. In this situation, gNB could schedule the PDSCHs of different TRPs with different TCI states for PDSCH transmission. The indication of multiple beam direction needs to incorporate multiple TCI values in a DCI for PDSCH transmissions. However, each TCI will bring at least 3 bits’ overhead to the DCI, and when the number of scheduled PDSCH(s) is large, the overhead increase could be a serious problem.

Figure 2: Multi-TRP scenarios

Proposal 6: If single DCI schedule multi-PUSCH/PDSCH is supported, multiple beam indications of PDSCH with different TCI states need to be investigated.

Proposal 7: If separate TCI state for each scheduled PDSCH introduced for multi-PDSCH scheduling with a single DCI, the solution for overhead reduction need to be investigated.

  - Beam management enhancement for random access procedure

During the RACH procedure, the same beam is assumed by UE corresponding to the received beam from SSB selected by UE before transmit RACH Msg1. When UE transmit RACH Msg1, the UL beam direction is derived based on the beam correspondence to the SS/PBCH block beam. When UE receive the RACH Msg2, the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block that UE used for RACH association. The same DL/UL beam correspondence mechanism is assumed by UE for the operation in unlicensed spectrum. If the beam needs to be dynamically adapted during the RACH procedure due to UE mobility in moving to other cell portion or LBT failure on the beam for the operation in unlicensed spectrum, UE can restart the PRACH procedure from RACH Msg1.

For above 52.6GHz, narrow beam width beam formed operation would be expected. The beam adaptation of SSB detection and RACH procedure may be dynamically changed the DL/UL beam correspondence. Additional enhancement of beam adaptation for initial access needs to be studied.

If the beam becomes narrower, the corresponding beam may likely misalign or vary in time during the initial access procedure due to the UE mobility or blocking of line of sight signals. The static DL/UL beam correspondence assumed by UE during random access procedure might not work properly. The initial random access procedure may need to dynamically adapt the UL beam among different RACH transmissions. The dynamic beam adaptation for RACH Msg3 and response to the Msg4 transmission needs further investigation.

Figure 3: Procedure of Msg3/Msg4 beam re-selecting

Proposal 8: In initial access, the beam adaptation for Msg3 and Msg4 transmission can be adapted based on the beam measurement report from UE.

- Conclusion

Observation 1: When UE cannot measure the periodic CSI-RS at the scheduled transmission instance for beam management due to LBT failure, gNB could transmit aperiodic CSI-RS and indicate to the UE as the alternative measurement.

Observation 2: When SCS is 480KHz or 960KHz, the duration of each OFDM symbol would be shorter.  UE may not support performing beam switching as much as 14 times within a slot.

Observation 3: In some scenarios, PDSCH(s) QCL’d with the RS in the TCI state indicated by the DCI can achieve reception gain; while in other scenarios PDSCH(s) QCL’d with the PDSCH(s) which have scheduling offset less than timeDurationForQCL can achieve reception gain.

Observation 4: In order to guarantee the reception performance of PDSCH, the additional beam switching gap need to be reserved before the PDSCH.

Proposal 1: Aperiodic CSI-RS could be used as the alternative solution of missed L1 RSRP measurement of periodic CSI-RS due to LBT failure with little specification change.

Proposal 2: The beam management frame work should be reused for NR operation in 52.6-71 GHz.

Proposal 3: For SCS 480/960 kHz, the minimum and maximum available value of maxNumberRxTxBeamSwitchDL should be reduced.

Proposal 4: When some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL and some have scheduling offset equal to or greater than timeDurationForQCL, both options below should be supported for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL. FFS conditions to determine the QCL assumption.

- The scheduled PDSCHs quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state in DCI.

- The scheduled PDSCHs quasi co-located with the RS(s) based on the activated TCI states in the first slot with the scheduled PDSCH.

Proposal 5: When the additional beam switching gap is introduced, QCL assumption needs to be investigated.

Proposal 6: If single DCI schedule multi-PUSCH/PDSCH is supported, multiple beam indications of PDSCH with different TCI states need to be investigated.

Proposal 7: If separate TCI state for each scheduled PDSCH introduced for multi-PDSCH scheduling with a single DCI, the solution for overhead reduction need to be investigated.

Proposal 8: In initial access, the beam adaptation for Msg3 and Msg4 transmission can be adapted based on the beam measurement report from UE.

- Reference

RP-202925, “Extending current NR operation to 71 GHz”, CMCC

3GPP TS 38.214, “Physical layer procedures for data (Release 16)”, V16.2.0, Jun. 2020.