3GPP TSG RAN WG1 #106-e            	   	                        R1-2106971

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on mitigating UE and gNB Rx/Tx timing errors

Agenda item:	8.5.1

Document for:	Discussion and Decision

Introduction

At RAN1#104b-e and RAN1#105-e meetings, a lot of agreements were achieved related to accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors [1][2].

This contribution further discusses mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors for UL and DL+UL positioning methods, based on our previous contribution in RAN1#105-e meeting [3]. Section 2 discusses the mitigation of UE and gNB Rx/Tx timing errors. Section 3 discusses the configuration of measurement time windows. Section 4 summarizes the proposals with conclusions.

Mitigation of UE/gNB Rx/Tx timing errors

In this section, we analyse the mitigation methods based on measurements and compensations of Rx/Tx timing delays, for UL-TDOA and DL+UL positioning methods.

UL positioning methods (UL-TDOA)

The following agreement was achieved in RAN1#105-e related to mitigating UE Tx timing errors for UL TDOA [1]:

Agreement: - For mitigating UE Tx timing errors for UL TDOA, support one of the following options: - Option 1: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs. - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs - Option 2: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs. - Support the serving gNB to forward the association information provided by the UE to the LMF - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs - FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

As shown in the above agreement, there are two options on how the UE provides the association information of SRS-Pos with Tx TEGs to the LMF for mitigating UE Tx timing errors for UL TDOA:

- Option 1: Subject to UE’s capability, support a UE providing the association information of SRS-Pos with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

- Option 2: Subject to UE’s capability, support a UE providing the association information of SRS-Pos with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

In our point of view, since there are LPP signalling between UE and LMF, and LMF is responsible for the calculation of UE position, it is reasonable for UE to provide the association information of SRS-Pos with Tx TEGs directly to the LMF, so that LMF can use such association information to calculate the UE position. In addition, such direct provision to LMF can reuse current LPP signallings and introduce less standardization impacts. Therefore, we support Option 1 as the method of UE providing the association information to LMF.

Proposal 1: For mitigating UE Tx timing errors for UL TDOA, support  the following option1:

- Option 1: Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

About the issue of whether to support LMF to forward the association information of UL SRS resources for positioning with Tx TEGs provided by the UE to the serving and neighboring gNBs, the key is whether the serving and neighboring gNBs need the association information. In our point of view, since LMF is responsible for the calculation of UE position, the serving and neighboring gNBs don’t need such association information, and the gNBs will report the associated SRS-Pos resource ID/resource set ID of the RTOA measurement to LMF, then LMF will know the Tx TEGs of the RTOA measurement after UE providing the association information of SRS-Pos with Tx TEGs to the LMF.

Proposal 2: Don’t support LMF to forward the association information of UL SRS resources for positioning with Tx TEGs provided by the UE to the serving and neighboring gNBs.

DL+UL positioning methods

The following agreement was achieved in RAN1#105-e related to mitigating UE/TRP Tx/Rx timing errors in DL+UL positioning [1]:

Agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

As shown in the above agreement, there are two options on whether to report UE RxTx TEG ID for mitigating UE Tx/Rx timing errors for DL+UL positioning:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

In our point of view, UE should report UE Rx TEG ID and UE Tx TEG ID to LMF, then LMF can know the combination of {UE Rx TEG ID, UE Tx TEG ID} to mitigating UE Tx/Rx timing errors for DL+UL positioning. And UE don’t need to additionally report UE RxTx TEG ID to LMF.

About the issue of how to let LMF to know whether two pairs of {UE Rx TEG ID, UE Tx TEG ID} have the same Rx+Tx timing errors or are within the same range of Rx+Tx timing errors, UE can provide the mapping information of {UE Rx TEG ID, UE Tx TEG ID} pairs to the range of Rx+Tx timing errors, so LMF can determine which UE Rx-Tx time difference measurements have the same Rx+Tx timing errors or are within the same range of Rx+Tx timing errors based on the mapping information.

To explain the mapping information of {UE Rx TEG ID, UE Tx TEG ID} pairs to the range of Rx+Tx timing errors, assuming a UE has two Rx TEG IDs (i.e., UE Rx TEG ID #1 and #2) and two Tx TEG IDs (i.e., UE Tx TEG ID #1 and #2). As shown in Table 1, if the two pairs of {UE Rx TEG #1, UE Tx TEG #2} and {UE Rx TEG #2, UE Tx TEG #1} are within the same range of Rx+Tx timing errors, then the two pairs will belong to the same range of Rx+Tx timing errors, so the two pairs will have the same range index of Rx+Tx timing errors. Therefore, if UE Rx-Tx time difference measurement #1 is associated with {UE Rx TEG #1, UE Tx TEG #2} and UE Rx-Tx time difference measurement #2 is associated with {UE Rx TEG #2, UE Tx TEG #1}, then UE Rx-Tx time difference measurement #1and #2 will have the same range index of Rx+Tx timing errors and be within the same range of Rx+Tx timing errors.

Table 1: Mapping informaiton of {UE Rx TEG ID, UE Tx TEG ID} pairs to the range of Rx+Tx timing errors

| Range index of Rx+Tx timing errors | {UE Rx TEG ID, UE Tx TEG ID} pairs |
| --- | --- |
| Range Index #1 | {UE Rx TEG #1, UE Tx TEG #1} |
| Range Index #2 | {UE Rx TEG #1, UE Tx TEG #2};{UE Rx TEG #2, UE Tx TEG #1} |
| Range Index #3 | {UE Rx TEG #2, UE Tx TEG #2} |

The  information provided by UE to LMF for mitigating UE Tx/Rx timing errors for DL+UL positioning as follows:

- UE reports the association information of UE Rx TEG ID with DL PRS resources to LMF

- UE reports the association information of UE Tx TEG  ID with SRS-Pos resources to LMF

- UE provides the mapping information of {UE Rx TEG ID, UE Tx TEG ID} pairs to the range of Rx+Tx timing errors

Based on the above discussion, we have the following proposal:

Proposal 3: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, the following option 2 in the previous agreement of RAN1#105-e:

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

About the issue of the associated information of a UE Tx TEG ID, there are three alternatives in the previous agreement of RAN1#105-e for further down-selection:

- Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

- Alt. 2: the Tx timing of the Rx-Tx measurement

- Alt. 3: one or more UL SRS resources for positioning

We prefer Alt.3, i.e., a Tx TEG ID is associated with one or more UL SRS resources for positioning. In fact, we prefer UE Tx TEG ID to be decoupled with a particular UE Rx-Tx time difference measurement to avoid the issue of potential mismatch between UE and gNB Rx-Tx time difference measurements, i.e., the UE indicates the UE Rx-Tx time difference measurement associated with one UE TX TEG ID, but the gNB only receives the SRS-Pos associated with other UE Tx TEG ID and use this SRS-Pos to derive the gNB Rx-Tx time difference measurement. Therefore, LMF will use incorrect UE Tx TEG ID to mitigate the UE Tx/Rx timing errors for Multi-RTT positioning.

In order to explain the issue of  potential mismatch above, let us consider the following example:

According to the definition of UE Rx-Tx time difference measurement in Rel-16 [4], The UE Rx-Tx time difference is defined as TUE-RX – TUE-TX, TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point(TP), defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP, as shown in Table 2 below.

Table 2: Definition of UE Rx-Tx time difference measurement in Rel-16

| Definition of UE Rx – Tx time difference | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |

If there are two SRS-Pos resources with different Tx TEGs are transmitted in the same UL subframe (subframe #j or other subframes), i.e., SRS-Pos#1 with Tx TEG #1 and SRS-Pos #2 with Tx TEG #2. UE calculates UE Rx-Tx time difference measurement #1 based on SRS-Pos #1, but gNB cannot receive SRS-Pos#1 and can receive SRS-Pos #2, therefore, gNB will calculate gNB Rx-Tx time difference measurement #2 based on SRS-Pos #2. According to the definition of UE Rx-Tx time difference measurement in Rel-16, the UE Rx-Tx time difference measurement #1 associated with SRS-Pos #1 and the UE Rx-Tx time difference measurement #2 associated with SRS-Pos #2 have the same TUE-TX.

If Alt.1 is adopted and the UE Tx TEG ID is associated with an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement, a Rx-Tx time difference measurement is not independent of UE Tx TEG. In this case, some clarifications are needed on the definition of the UE Rx-Tx time difference measurements when TEG is introduced. And the issue of a potential mismatch between UE and gNB Rx-Tx time difference measurements should be considered and solved. Moreover, Alt.2 has the similar issue of potential mismatch as Alt.1, since Alt.2 also coupled UE Tx TEG ID with Tx timing of the Rx-Tx measurement.

On the other hand, if Alt.2 is adopted and the UE Tx TEG ID is associated with one or more UL SRS resources for positioning and decoupled with the Tx timing of the Rx-Tx measurement, then UE can provide a separate report of the association of the Tx TEG ID with the SRS-Pos resources. And the Tx time of UE Rx-Tx time difference measurement is determined by the closest UL subframe #j (as Rel-16 definition), but not by UL Tx time of the SRS-Pos resources. With Alt.2, the reporting of UE Rx-Tx time difference measurement is decoupled from the UE Tx TEG, UE derives Tx timing for the UE Rx-Tx time difference measurement report based on the expected UL Tx time. And the real UL Tx timing errors for transmission of the SRS-Pos resource will be provided by the UE through Tx TEG information. In this way, LMF can determine the UL Tx timing errors based on the received SRS-Pos resource ID to solve the issue of potential mismatch between UE and gNB Rx-Tx time difference measurements mentioned above.

In addition, if the UE does not know the Tx time difference between the UL SRS resources (or Tx TEGs), there is no point to support Option 1 and 2, since UE does not know the difference the Tx timings of the Rx-Tx measurement. On the other hand, if the UE knows the Tx time difference between the UL SRS resources,  then UE can simply report the Tx time difference of the UL SRS resources (or Tx TEGs) to LMF together with associaltion of the Tx TEG ID with UL SRS resources.

Based on the above discussion, we have the following proposals:

Proposal 4: A Tx TEG ID should be associated with one or more UL SRS resources for positioning and decoupled with the Tx timing of the Rx-Tx measurement.

Proposal 5: No need to change the definition of UE Rx-Tx time difference measurement provided in Rel-16.

Configuration of Measurement Time Windows

In this section, we discuss the configuration of measurement time windows. The following agreement was made in RAN1#104-e related to the measurement time windows [2]:

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

For DL, UL and DL+UL positioning methods, UE reports one or more measurement instances in a single measurement report to LMF for UE-assisted positioning, and/or TRP reports one or more measurement instances in a single measurement report to LMF. The measurement instances should be within a configured measurement time window, and each measurement instance is reported with its own timestamp.

The measurement time window can help LMF to eliminate the influence of timing errors of TRPs and UE. The purposes for introducing measurement time window are as follows:

- Limit the measurement behaviour of UE or TRP, and only DL-PRS/ SRS-Pos resources within the measurement time window will be measured.

- Limit the measurement time of each measurement instance, and support the measurement instance which only corresponds to one DL-PRS/SRS-Pos occasion for one-shot measurement.

- Facilitate the timestamps matching among various measurement instances, e.g., among UE Rx-Tx time difference measurement instances and gNB Rx-Tx time difference measurement instances for multi-RTT positioning method.

- Indicate whether the measurement instances are measured within the same measurement time window.

- Help LMF to track and mitigate the timing error drift over time.

As mentioned above, there are many purposes for introducing measurement time windows, and different purposes may need different configurations of measurement time windows. Therefore, the measurement time windows should be configurable to support various purposes, in which the UE or TRP measurement instances are measured.

Proposal 6: The configurable measurement time windows should be supported, in which the UE or TRP measurement instances are obtained.

For UE and TRP, the measurement time windows need to be configured independently in order to measure the DL-PRS/SRS-Pos resources in the windows. The configuration parameters of UE measurement time windows and TRP measurement time windows can be the same or different to support different purposes. As shown in Figure 1 and Figure 2 below, the UE measurement time windows and TRP measurement time windows are configured in the same manner.

UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources. TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources. And UE or TRP is not expected to report the measurement instances corresponding to a measurement time window from DL-PRS or SRS-Pos outside of the measurement time window.

Proposal 7: UE measurement time windows and TRP measurement time windows can be configured independently. They can be configured to be the same or different.

- UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources.

- TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources.

Proposal 8:  UE (or TRP) is not expected to measure DL-PRS (or SRS-Pos) outside of the measurement time window.

Two configuration methods will be discussed in the following: one indirectly configures the length of the measurement time window, and the other directly configures the length of the measurement time window.

Configuration method 1: Indirectly configure the length of the measurement time window

In this section, the configuration method 1 will be discussed, which indirectly configure the length of the measurement time window.

In order to configure the UE/TRP measurement time window, the following parameters for UE/TRP measurement time window should be configured and indicated via LPP and NRPPa signalling:

- The periodicity of UE/TRP measurement time window.

- The start time of UE/TRP measurement time window.

- The number of UE/TRP measurement instances included in the UE/TRP measurement time window.

- The number of instances of DL-PRS/SRS-Pos resource set or DL-PRS/SRS-Pos occasions contained by each UE/TRP measurement instance.

Figure 1: Configuration method 1 of UE measurement time windows (UE MTWs)

Figure 2: Configuration method 1 of TRP measurement time windows (TRP MTWs)

As shown in Figure 1, UE is configured with two measurement time windows, i.e., UE MTW#1 and MTW#2.

For UE MTW#1, its start time is T1, and only one measurement instance (MI#1) is configured and it contains the measurement results from one instance of DL-PRS resource set (R1). And the measurement report MR#1 contains MI#1, whose timestamp is t1.

For UE MTW#2, its start time is T2, and there are two measurement instances (MI#2 and MI#3) being configured. MI#2 contains the measurement results from two instances of DL-PRS resource sets (R3&R4). MI#3 contains the measurement results from one instance of DL-PRS resource sets (R5). And the measurement report MR#2 contains MI#2 and MI#3, whose timestamp is t2 and t3, respectively.

Based on the above discussion, we have the following proposal:

Proposal 9: (Configuration method 1): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- J: The number of UE measurement instances included in the UE measurement time window.

- Ni: The number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- K: The number of TRP measurement instances included in the TRP measurement time window.

- Mi: The number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance.

When UE or TRP reports a measurement instance, it also reports the time stamp of the measurement instance. If the measurement instance contains more than one DL-PRS or SRS-Pos resource set, the timestamp of the measurement instance should correspond to the reception time between the first and last DL-PRS resource set (or the first and last SRS-Pos resource set) that are used to determining the measurement instance. For example, as shown in Figure 2, measurement instance MI#2 contains two SRS-Pos resource sets (R3 and R4); the timestamp of MI#2 should be one certain time t2 between R3 and R4.

About the start time of each UE/TRP measurement time window and timestamp of each measurement instance, it can be defined with an absolute time (UTC time) or a relative time (SFN + slot index).

Proposal 10: When UE reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last DL-PRS resource sets that are used to determining the measurement instance.

Proposal 11: When TRP reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last SRS-Pos resource sets that are used to determining the measurement instance.

With the above parameters of UE/TRP measurement time window, the length of UE/TRP measurement time window can be defined as follows,

Proposal 12: For configuration method 1 and the periodic DL-PRS, the length of UE measurement time window MTWUE can be defined as:

MTWUE=PDDL-PRS×i=1JPNPi

        - PDDL-PRS is the periodicity of DL-PRS resource set;

        - JP is the number of UE measurement instances included in the UE measurement time window, JP≥1;

        - NPi is the number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance，NPi≥1.

Proposal 13: For configuration method 1 and the periodic/semi-persistent SRS-Pos, the length of TRP measurement time window MTWTRP can be defined as:

MTWTRP=PDSRS-Pos×i=1KPMPi

        - PDSRS-Pos is the periodicity of SRS-Pos resource set;

        - KP is the number of TRP measurement instances included in the TRP measurement time window, KP≥1;

        - MPi is the number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance，MPi≥1.

The number of instances of the DL-PRS resource set or SRS-Pos resource set contained by one UE/TRP measurement instance (i.e., Ni and Mi) should be configurable.

About whether Ni or Mi can be configured with 1, in order to support the measurement instance which only corresponds to one DL-PRS/SRS-Pos occasion for one-shot measurement, each UE or TRP measurement instance can be configured to only contain one instance of DL-PRS resource set or SRS-Pos resource set. Therefore, Ni or Mi can be configured with 1. As shown in Figure 2, measurement instance MI#1 only contains one instance of SRS-Pos resource set (R1) with the timestamp t1, and measurement report MR#1 only include the measurement results from one instance of SRS-Pos resource set (R1). The TRP measurement report MR#1 can be seen as one-shot measurement without the combination cross more than one instance of SRS-Pos resource sets.

Configuration method 2: Directly configure the length of the measurement time window

In this section, the configuration method 2 will be discussed, which directly configure the length of the measurement time window.

In order to configure the UE/TRP measurement time window, the following parameters for UE/TRP measurement time window should be configured and indicated via LPP and NRPPa signalling:

- The periodicity of UE/TRP measurement time window.

- The start time of UE/TRP measurement time window.

- The length of UE/TRP measurement time window.

Figure 3: Configuration method 2 of UE measurement time windows (UE MTWs)

Figure 4: Configuration method 2 of TRP measurement time windows (TRP MTWs)

As shown in Figure 3, UE is configured with two measurement time windows, i.e., UE MTW#1 and MTW#2. For UE MTW#1, its start time is T1, with the length L1. For UE MTW#2, its start time is T2, with the length L2.

Based on the above discussion, we have the following proposal:

Proposal 14: (Configuration method 2): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- L1: The length of UE measurement time window.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- L2: The length of TRP measurement time window.

Comparison between Configuration method 1 and Configuration method 2

In section 3.1 and section 3.2, we discussed Configuration method 1 and Configuration method 2 for the measurement time window. Compared with Configuration method 1, the merit of Configuration method 2 is reducing the complexity and overhead of configuration signalling for the measurement time window. And a lot of details of the configuration of the measurement time window will be up to UE/TRP implementation.

However, Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2. For example, Configuration method 1 can indicate or configure the number of UE/TRP measurement instances included in the UE/TRP measurement time window, as well as the number of instances of DL-PRS/SRS-Pos resource set contained by each UE/TRP measurement instance. It will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE. Therefore, Configuration method 1 is preferred to configure the measurement time window.

Observation 1: Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2.

Proposal 15: Configuration method 1 should be adopted to configure the measurement time window, since it will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE.

Conclusions

In this contribution, we discuss accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors, and give the following observations and proposals:

Proposal 1: For mitigating UE Tx timing errors for UL TDOA, support  the following option1:

- Option 1: Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

Proposal 2: Don’t support LMF to forward the association information of UL SRS resources for positioning with Tx TEGs provided by the UE to the serving and neighboring gNBs.

Proposal 3: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, the following option 2 in the previous agreement of RAN1#105-e:

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

Proposal 4: A Tx TEG ID should be associated with one or more UL SRS resources for positioning and decoupled with the Tx timing of the Rx-Tx measurement.

Proposal 5: No need to change the definition of UE Rx-Tx time difference measurement provided in Rel-16.

Proposal 6: The configurable measurement time windows should be supported, in which the UE or TRP measurement instances are obtained.

Proposal 7: UE measurement time windows and TRP measurement time windows can be configured independently. They can be configured to be the same or different.

- UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources.

- TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources.

Proposal 8:  UE (or TRP) is not expected to measure DL-PRS (or SRS-Pos) outside of the measurement time window.

Proposal 9: (Configuration method 1): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- J: The number of UE measurement instances included in the UE measurement time window.

- Ni: The number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- K: The number of TRP measurement instances included in the TRP measurement time window.

- Mi: The number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance.

Proposal 10: When UE reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last DL-PRS resource sets that are used to determining the measurement instance.

Proposal 11: When TRP reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one certain reception time between the first and last SRS-Pos resource sets that are used to determining the measurement instance.

Proposal 12: For configuration method 1 and the periodic DL-PRS, the length of UE measurement time window MTWUE can be defined as:

MTWUE=PDDL-PRS×i=1JPNPi

        - PDDL-PRS is the periodicity of DL-PRS resource set;

        - JP is the number of UE measurement instances included in the UE measurement time window, JP≥1;

        - NPi is the number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance，NPi≥1.

Proposal 13: For configuration method 1 and the periodic/semi-persistent SRS-Pos, the length of TRP measurement time window MTWTRP can be defined as:

MTWTRP=PDSRS-Pos×i=1KPMPi

        - PDSRS-Pos is the periodicity of SRS-Pos resource set;

        - KP is the number of TRP measurement instances included in the TRP measurement time window, KP≥1;

        - MPi is the number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance，MPi≥1.

Proposal 14: (Configuration method 2): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- L1: The length of UE measurement time window.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- L2: The length of TRP measurement time window.

Observation 1: Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2.

Proposal 15: Configuration method 1 should be adopted to configure the measurement time window, since it will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #105-e.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e.

- R1-2104520, Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, CATT, RAN1 #105-e.

- 3GPP TS 38.215, “NR; Physical layer measurements”, v16.4.0, 2020-12.