3GPP TSG RAN WG1 #106-e                                            R1-2106980

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on other aspects related to complexity reduction

Agenda Item:	8.6.1.4

Document for:	Discussion and Decision

Introduction

According to the WID, RedCap UE shall support reduced maximum number of DL MIMO layers and relaxed maximum modulation order [1]. In RAN1#105-e, the following agreements and conclusions were reached [2]:

Conclusion: - For a RedCap UE, when motivated by reduced max number of DL MIMO layers modifications to CSI measurement and/or reporting mechanisms are not pursued in Rel-17. Agreements: - For a RedCap UE, 64QAM MCS tables (Table 5.1.3.1-1 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-1 in TS 38.214 for UL w/ transform precoding respectively) are the “default” ones and are mandatory. - The following is optionally supported by RedCap UEs: - 256QAM MCS tables (Table 5.1.3.1-2 in TS 38.214 for DL and UL OFDM) - 64QAM low SE MCS tables (Table 5.1.3.1-3 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) Agreements: - For a RedCap UE, “CQI table 1” (Table 5.2.2.1-2 in TS 38.214), that corresponds to MCS Table 5.1.3.1-1 in TS 38.214, is mandatory. - The following is optionally supported by a RedCap UE: - “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) that corresponds to MCS Table 5.1.3.1-2 in TS 38.214 (256QAM MCS table) - “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) that corresponds to MCS Table 5.1.3.1-3 in TS 38.214 (64QAM low SE MCS table) Agreement: - Both 256QAM MCS table for PDSCH and “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) are supported by a RedCap UE indicating support of 256QAM for PDSCH. Agreement: - For a RedCap UE, support of 64QAM low SE MCS table for PDSCH and support of “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) are not coupled and capability of each can be reported independent of the other. Agreement: - For a RedCap UE, support of 64QAM low SE MCS table for PDSCH (Table 5.1.3.1-3 in TS 38.214) and support of 64QAM low SE MCS tables for PUSCH (Table 5.1.3.1-3 in TS 38.214 for UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) are not coupled and capability of each can be reported independent of the other.

In this contribution, we provide our views on the DL MIMO layers and relaxed MCS and CQI for RedCap UE, based on the agreements so far.

Discussion

Remaining issues on reduced number of DL MIMO layers

It was agreed in the WID that for a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. And, for a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported [1]. Thus, the maximum number of DL MIMO layers are clear, i.e., maximum 2 layers for Rx=2 UE, maximum 1 layer for Rx=1 UE, respectively.

Observation 1: The maximum number of DL MIMO layers for RedCap UE is already clear.

It was concluded that modifications to CSI measurement and/or reporting mechanisms are not pursued due to reduction of the maximum number of DL MIMO layers. Also, whether related DCI fields will be modified or not is under discussion in agenda 8.6.1.2. To be specific, we believe that modification on DCI fields for reduced number of Rx branches is not needed. It can be seen that the main issues related to the reduced maximum number of DL MIMO layers are also clear.

However, current TS/TR assumes that the UE equips with at least 4 Rx branches in FR1. For example, in TS 38.306, there is no definition of a UE with Rx=2 or Rx=1, as shown in Table 1 [3]:

Table 1 Definitions for parameters in TS 38.306.

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

In TR 38.822, similar description can be found in MIMO-specific parameter, as shown in Table 2 [4]:

Table 2 Layer-1 feature list in TR 38.822.

| 2-3 | PDSCH MIMO layers | Supported maximal number of MIMO layers | 2-1 | maxNumberMIMO-LayersPDSCH | FeatureSetDownlinkPerCC | n/a | n/a |  | For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signalling. It is not expected that there is a signalling change (i.e. signalling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In our view, RedCap UE type is a special UE type in NR, and RedCap UE shall support NR features by default. But due to the difference between RedCap UE and normal UE in physical layer, some features may not be supported by RedCap UE naturally, or some parameters may become improper for RedCap UE. Current specifications need to be updated to reflect the change from normal UE to RedCap UE. In a later phase, the parameters in RAN1/RAN2 specifications shall be checked term by term to accommodate the RedCap UE.

Observation 2: Parameters in RAN1/RAN2 specifications shall be checked term by term to accommodate the RedCap UE in a later phase.

Remaining issues on MCS and CQI

In RAN1#105e, it was agreed that the default MCS/CQI table for normal UE is also the default MCS/CQI table for RedCap UE. The 256QAM MCS/CQI table is optional (instead of mandatory) for RedCap UE. The RedCap UE also optionally supports SE MCS/CQI table [2]. These agreements accord with the WID that 256QAM is optional (instead of mandatory) for RedCap UE in DL.

Another issue is whether the RedCap UE can optionally support 256QAM in UL. Though there is no relaxation of modulation order in UL in the WID, the WID does not preclude the support of 256QAM in UL, either. In general, the features that optional for normal UE can be optionally supported by RedCap UE, unless strictly forbidden due to the physical layer restriction defined in the WID.

Specifically, our views on the pros and cons of supporting 256QAM in UL are summarized as follows:

Table 3 Pros and cons of supporting 256QAM in UL.

| Pros | Cons |
| --- | --- |
| - Improve the peak data rate of the RedCap UE. This increase the chance to meet the extreme high data rate requirement (50Mbps in UL) from some wearable service in TDD band. - Improve the spectrum efficiency of the network. For the RedCap UEs with good channel condition, using 256QAM in UL is possible, which provides around 33% SE gain compared to 64QAM. | - Increase the cost in ADC/DAC, UL processing block, power amplifier and transceiver, due to the higher requirement for these blocks. Note that during the SI, there is no evaluation or conclusion on the cost increase from 64QAM to 256QAM in UL for RedCap UE [5]. |

From the above analyze, it seems no harm to support 256QAM in UL as an optional feature for RedCap UE. The UE vendors are still able to not implement 256QAM if they think it improper, because it is just optional. And this does not increase the network complexity since 256QAM is a traditional feature for normal UE.

We have the following proposal:

Proposal 1: The RedCap UE can optionally support 256QAM in UL.

Conclusion

In this contribution, we provide our view on remaining issues on DL MIMO layers and MCS/CQI tables for RedCap UE. The observations and proposal are summarized as follows:

Observation 1: The maximum number of DL MIMO layers for RedCap UE is already clear.

Observation 2: Parameters in RAN1/RAN2 specifications shall be checked term by term to accommodate the RedCap UE in a later phase.

Proposal 1: The RedCap UE can optionally support 256QAM in UL.

Reference

- RP-211574, Revised WID on support of reduced capability NR devices, Ericsson, RAN#92-e, e-Meeting, June 14th – 18th, 2021.

- Chairman's Notes RAN1#105-e, 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- 3GPP TS 38.306, User Equipment (UE) radio access capabilities, V16.5.0.

- 3GPP TR 38.822, User Equipment (UE) feature list, V16.0.0.

- 3GPP TR 38.875, Study on support of reduced capability NR devices, V17.0.0.