3GPP TSG RAN WG1 #106-e	R1- 2107059

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.5.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Views on enhancing DL AoD

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#90 a new WI was approved and updated at RAN#91 on enhancements in Rel-17 to positioning [1]. This contribution discusses our views on the enhancements related to DL AoD. Our companion contributions discuss our others views [2-6]. The objective from the WID is to

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

# Discussion

## Additional measurements for DL-AoD

At RAN1#104-e and RAN1 #105-e there was discussion about introducing additional measurements to increase positioning accuracy of DL-AoD method. The following decision was made at RAN1#104-e:

Agreement:

- For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path

  - Option 1: Information corresponds to PRS-RSRP of the first arriving path

  - Option 2: Information corresponds to the angle of departure of the first arriving path

  - Option 3: Information corresponds to the arrival time of the first path

  - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path

  - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path

- FFS: Reporting of additional path to the first arriving path.

- FFS: Measurement definition details

- FFS: additional assistance data to support these enhancements

- FFS: how the “first path” is selected among PRS resources in a PRS resource set

- Note 1: Supporting multiple options as well as none of the options above is not precluded.

At RAN1 #105e meeting, support of the above option 1 was agreed as follows:

Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path

In general, the accuracy of downlink (DL) positioning may be degraded in the presence of multipath propagation. The strongest multipath component may not necessarily correspond to the shortest/first arriving path relative to a surrounding transmitting cell - this happens, e.g. when the line-of-sight (LoS) direction is obstructed. Thereofore, additional power measurement of the first path may not bring additional benefit. The first path of a NLOS beam may be still a NLOS path with high power. However, by comparing arrival time of different beams (associated with different PRS occasions) can identify which beam arrival first, although the beam may having lower power or RSRP value.

Observation 1: In the presence of NLoS, the strongest multipath component in terms of PRS-RSRP may not necessarily correspond to the first arrival/shortest path.

Regarding the reporting of additional path, the accuracy of the positioning may theoretically be increased by having measurements from several spatial components/beams, in practice DL TX beams that are close to each other in the spatial domain generally contribute very similar information for positioning. For example, DL PRS resources corresponding to adjacent DL TX beams and selected only based on RSRP measurements may be spatially correlated, and thus may contribute only marginal improvements for the positioning accuracy.

Figure 1 shows an example of reporting adddtional paths associated with arrival time information. UE is configured with multiple PRS resources and reporting configuration with ranking of reported PRS resources according to time of arrival (e.g. report N- different DL PRS resources with shortest arrival times or relative time difference of arrival information between reported resources). When the network has aforementioned arrival timing information available, it can enhance the reliability of estimation related to UE location (showed as shaded area around the UE).

Observation 2: To enhance positioning accuracy in the presence of different NLoS, it is beneficial to report multiple PRS resources each associated with time of arrival information.

Proposal 1: For DL-AoD support reporting of multiple PRS resources per PRS resource set, with each resource being associated with time of arrival information.

Figure 1. Example of reporting of multiple additional paths with arrival of time information.

## Other DL-AoD Enhancements

In RAN1 #104e, the following agreement was made, but we could not reach a consensus to support one option in RAN1 #105e.

Agreement:

For UE-assisted DL AOD, select one of the following options for reporting of RSRP measurements per TRP

- Option 1: Up to 8 measurements in a measurement report (as in release 16)

- Option 2: Up to 8 measurements in a measurement report, for the same Rx beam index

- Option 3: Up to N>=8 measurements

  - Note: Multiple measurements corresponding to different Rx Beam index may be  reported for a given PRS resource.

  - FFS: value for N.

In our view, options 1 and 2 do not allow enough flexibility in terms of how the UE performs the PRS measurements. In addition, for the improvement of positioning accuracy by the high resolution of the beam grid, option 3 is necessary. Option 2 links the measurements to the UE RX beam indices, and such, the measurement report size is linked to the UE beamforming capabilities, without necessarily bringing additional benefits, since some of the UE RX beams may not be relevant to the positioning session. In our view, the UE measurement process should be transparent to the network. While these options currently are for RSRP measurements we think that this potential enhancement should also apply to any additional measurements for DL-AoD (e.g., first arrival path).

Proposal 2: Support “Option 3: Up to N>8 measurements” as candidate enhancement. FFS value of N.

At RAN1 #105e the following were agreed:

Agreement:

For the beam/antenna information to be optionally provided to the LMF by the gnodeB, select one or more of the following:

- Option 1: the gNB reports the antenna configuration including at least the following parameter:

  - the number of antenna elements (vertical and horizontal)

  - antenna spacing dh and dv

  - FFS: For DFT-based beams, precoder information for each PRS resource

    - Check whether the already reported boresight directions are sufficient, or whether more information is needed

  - FFS: Antenna Element pattern Information

    - FFS: Details

  - FFS: If additional information about panel/orientation is needed

- Option 2: the gNB reports a mapping of angle and beam gains for each of the PRS resources.

  - FFS: representation of the mapping (e.g. parametric function approximating the beam response, or gain/angle table, beamwidth, intersection point of multiple beams (angle, RSRP)intersection point)

- Other options are not precluded

In either option, the gNB beam/antenna information can optionally be provided to the UE by the LMF for UE-based DL-AoD

For the enhancement of providing additional beam/antenna information to the LMF it should be left to the TRP how much additional information to provide. The specification should support additional details but not mandate that the TRP report specific information (outside of the boresight direction supported in Rel-16). In addition, providing additional antenna information could be related to AoA based positioning since AoA measurement is associated with a specific antenna elements and/or a group of antenna elements. It might be necessary to provide geometry information of antenna but providing precoding information may be an unnecessary disclosure of detail of the gNB implementation.

Observation 3: Providing precoding information may be an unnecessary disclosure of details of the gNB implementation.

It is useful for the LMF to have a better understanding of the beam shape and polarization and to quantize the pattern for multiple directions. The boresight direction can be assumed to be the maximum power and then the relative power for multiple additional directions can be optionally reported by the TRP.

Proposal 3: Support TRPs to optionally report multiple directions per DL PRS resource with each direction being associated with a power value relative to the boresight power for that resource.

Proposal 4: Include additional assistance data for UE based positioning, including TRP polarization and geometry.

For the improvement of DL-AoD positioning, various options were discussed at RAN1 #105e and the following agreement was made.

Agreement:

For UE-assisted DL-AOD positioning method, select one or more of the following to enhance the signaling to the UE for the purpose of PRS resource(s) measurement and reporting:

- Option 1: the LMF explicitly identify adjacent beams in the assistance data (AD)

- Option 2: the LMF send the beam information in the AD with an order of priority of PRS resources.

- Option 3: the LMF includes boresight direction information for each PRS resource in the assistance data.

- Option 4: the LMF send the beam information in the AD with indicated subset of PRS resources.

- FFS: Detailed signaling and procedure

- FFS: How to define adjacent beams

Concerning adjacent beam reporting, the PRS-RSRP reporting flexibility of a UE is already quite high in Rel-16 and in Rel-17 may be further enhanced. If a UE has determined that a given PRS resource is not going to be reported for DL-AoD that means that the UE with capability to report 8 PRS-RSRP measurements has determined that 8 other DL PRS resources are better than the given PRS resource. The PRS resource then seems like it will not be useful at the LMF for AoD calculation. In fact if we look at Figure 1 the UE may determine to report beam 8 and 9 if using adjacent beams but beam 1 (or beam 5 in the case of obstructed LoS) would be better beams to report as they carry more information. It is not useful in our view for the network to constrain the UE reporting to ensure that particular adjacent beams are reported.

Observation 4: Reporting adjacent beams may be worse than letting the UE determine the best beams to report.

Proposal 5: Do not support enhancements for adjacent beam reporting (i.e., do not support option 1).

In case the LMF provides the UE of the beam direction for each PRS resource, the gNB needs to strictly follow the beam direction when it is transmitting each PRS resource but it is difficult for the gNB to always guarantee the exact beam direction due to errors such as the beam offset error, which is discussed in the last section in more details. This option 3 has been already supported for UE-based positioning, but the benefit is unclear to expand this feature to UE-assisted positioning and option 3 should be jointly discussed with beam offset error compensation.

The major benefit of option 2 may be assisting the UE to determine priority on PRS measurement and processing. The priority may be different for each UE depending on its location, so UE-specific configuration is necessary which leads to more network signaling burden. In addition, the priority on the PRS resources may not be a fixed value. It may be changed according to the UE's location so the update is unavoidable. The expected benefit of option 4 may be also similar with option 2 such that the UE could exclude a part of PRS resources for measurement. A trade-off additional siganlling burden and benefit is not clear, and configuring priority and resrtricting PRS measurement may require priori information on the UE’s location. In the absence of initial location information about the target positioning UE, two-stage based DL-AoD is much useful by association with two PRS resource sets. Thus, we prefer to jointly discuss the listed option with two-stage beam sweeping, and the following section elaborates on two-stage beam sweeping in more details.

## Two stage beam sweeping

At RAN1#105-e the following agreement was reached:

Agreement:

- For both UE-based and UE-assisted DL methods, at least for two-stage PRS beam sweeping, study further at least the following:

  - Enhancements in the association between resources belonging to two DL PRS resource sets of the same TRP

- Companies are encouraged to evaluate whether other potential enhancements in this subagenda or other subagendas (e.g. additional beam information, on-demand PRS framework) could be used to enable this feature (potentially by implementation).

- Note: Two-stage PRS beam sweeping corresponds to different DL PRS resource sets

In the Rel-16 specifications there is support for up to 2 DL PRS resource sets per TRP. In some ways this already enables the two stage beam-sweeping by allowing for one DL PRS resource set to be narrow beam and one DL PRS resource set to be wide beam. However, what is lacking in the current specification is some association information between the DL PRS resources within the two sects (e.g., DL PRS resource X and Y from set 2 are nested in DL PRS resource Z from set 1).

Observation 5: The current specification has insufficient support for two stage beam sweeping.

Proposal 6: Support association between resources belonging to two DL PRS resource sets (at the same TRP) to facilitate support of two stage beam sweeping.

According to current Rel-16 DL-AoD positioning measurement and report behavior, a UE shall measure all configured PRSs (probably with up to 2 resource sets) and report the highest RSRP (or multiple highest RSRP) and associated PRS ID(s). In two-stage beam sweeping, the first stage PRSs are used to identify preferred second stage PRSs to be measured. Reporting a PRS resource ID and RSRP measurement from the first stage resource set (or wide-beam resource set) is undesired and unnecessary for the final positioning estimation.

Proposal 7: In two-stage beam sweeping DL-AoD, the positioning report should be generated only based on the second-stage PRSs.

One potential further enhancement for two-stage PRS beam sweeping is to reduce transmission overhead for PRSs, especially the second stage PRSs. It worths to study on-demand PRS (muting) framework for two-stage PRS and in particular look at ways to reduce the network overhead to minimize unnecessary PRS transmissions.

Proposal 8: Support and study on-demand PRS framework for two-stage PRS beam sweeping.

## Expected Angle of Departure

Regarding the support of an expected uncertainty window for better AoD measurements, the necessity was identified at RAN1 #104bis-e meeting and the following agreement was made. It has been intensitvely discussed but RAN1 could not reach a consensus. From the FL summary [Ref#, R1-2106255], the consensus showed the clear necessity of either or both of the option1 and option 2 for DL-AoD measurements, so at least option 3 could be excluded.

Agreement:

For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options:

- Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

  - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP]

- Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

  - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

- Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced.

- FFS: details of signaling

- FFS: Applicability of this agreement to other Positioning methods

Indication of expected AoA and uncertainty to the UE can help UE to decide Rx beamforming in which direction it should receive DL PRS from TRPs. This information is more useful in DL-AoD method, as it can filter out NLOS paths with high RSRP value.

Proposal 9: Support Option 2 - Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE.

In the case of UE-based mode we also see that there can be some benefit for the LMF to tell the UE the expected DL-AoD (e.g., approximately what the UE should compute as the DL-AoD). Then the UE may be able to determine NLOS status, or other larger error, if the calculated DL-AoD is far from the expected value.

Proposal 10: For UE-based mode, support option 1: indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE.

Proposal 11: Support of indication of expected AoD/ZoD value and uncertainty (of the expected AoD/ZoD value) range(s) is signaled by the LMF to gNBs/TRPs in on-demand PRS framework.

See our companion paper [6] for more discussion of this issue related to on-demand PRS.

## Beam Offset Errors

One of the main objectives of the Rel-17 work on positioning enhancements is to improve the accuracy of the RAT-dependent positioning methods. One technique that was agreed to be improved is downlink angle of departure (DL AoD). DL AoD-based positioning is sensitive to orientation uncertainties of the TRP beams. Specifically, each TRP introduces orientation errors in their beamed transmission, i.e. each beam is sent under an angle characterized by an error as depicted in Figure 3. For example, since beam alignment process works as selection of the best beam from a finite set of configurations, the TRP may provide limited beam resolution and each beam is characterized by an orientation uncertainty.

Then, when an LMF computes the location estimate of the UE using the DL-AoD method, these beam orientation errors affect the accuracy of the position estimation. If until recently these errors could be absorbed by the overall location error (i.e. Rel-16 allows errors in the meter range), in Rel-17 they are not negligible anymore. Consequently, the location management function or location server needs to evaluate and potentially compensate for these errors, e.g. estimate and cancel them or trigger compensation methods at the TRP, in order to obtain accurate UE position estimates. In Rel-16, the TRP signals the LMF with the azimuth and elevation angles per DL PRS resource. However, the exact method for the TRP to determine these azimuth and elevation angles is left undefined. As the DL PRS beam configuration should remain flexible and can change over time it is also possible that the beam orientation errors vary over time.

Figure 3: Orientation errors for NR positioning.      		  Figure 4: Example geometry of beam orientation.

To further illustrate the problem, assume the scenario depicted in the Figure 4. The TRP is located at known position [a, b] and the UE at unknown location [x,y]. The distance between the TRP and the UE is d. The ideal AoD at the gNB is g, i.e. if the beam would be perfectly tuned to the AoD g. Due to the beam offset error e however, in reality, the AoD is (g+e). The LMF computes the estimated positions x, y using the distance d and the ideal AoD, i.e. what the gNB believes the AoD should be, as x, y =[a-dsing, b-dcosg]. However, the actual position is x, y=a-dsin(g+e),   b-dcos(g+e)  since the actual AoD is in fact (g+e). The Euclidian distance between the actual and the estimated location, i.e. E=x-x 2+y-y 2  measures the position error due to the beam offset error e and it is E=d21-cose. An example of how the beam error e impacts the position error is given in Figure 5, for a distance d = 30 m.

Figure 5: Positioning error as a function of beam offset.

As Figure 5 shows, even if the beam offset is the sole error source, a small beam offset or orientation error would cause 10-40 cm of positioning error. This clearly is not acceptable for meeting high accuracy use cases which are proned to errors stemming not only for beam offsets.

Observation 6: DL-AoD cannot meet the positioning requirements without correcting for beam orientation errors.

Proposal 12: RAN1 to study beam orientation errors and potential correction mechanisms in order to improve the positioning accuracy achievable with DL-AoD. Including:

- UE-based positioning: the beam offset (BO) could be signaled to the UE, as either an indicator, e.g. low/medium/high, each specifying an error range or as a specific value computed by the network

- UE-assisted positioning: LMF should be aware of the BO and compensate it when computing the position estimate.

- Signaling aspects:

  - LMF signals to TRPs that a BO beam re-tuning is needed. The BO correction may be explicitly signalled to the TRP by the LMF; alternatively, the LMF may send a Boolean indication that a BO recomputation and adjustement is needed.

  - UE measurement reports to facilitate BO identification and potential correction.

During RAN1#104-e the following agreement was reach on reference devices:

Agreement:

- Study specification impact for enabling a reference device with known location to support the following functionalities:

  - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF;

  - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF;

  - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations;

  - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information

  - FFS: The device with the known location being a UE and/or a gNB

  - FFS: Precision to which location of reference device is known

- Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI

In addition to the above functionalities, a reference device should also support measurements to enable the LMF to detect and correct for beam offset errors for one or more beamed PRS of one or more TRPs. Specifically, the network may designate one or more reference devices to perform measuremets of the beamed PRS from one or more TRPs and use the reported measurements to compute TRP beam offsets. In addition to the beam orientation/position measurements, the reference device may be configured to reports its own location acquired via non-RAT based methods. e.g. via GNSS or fixed location. To enable the beam correction via a reference device, the LMF should request the designated reference device to measure beamed PRS of selected TRPs and report at least the following:

- Own non-RAT based location estimation and optionally a level of trust of said estimate, e.g. uncertainty.

- Beamed PRS measurements such as, TOA, RSTD, RSRP, for one or more detected multipath components, and if specified LOS related parameters .

The LMF may use the above reports to estimate a BO value per TRP beam, or to detect whether there is a BO as observed by the reference device(s). The estimated BO per beam, or the BO indication per beam may then be sent to the TRPs, for the latter to apply the corrections. In addition or alternatively, the LMF may use the estimated BO to correct the position estimates of other UEs being localized via the problematic beams. For the particular case of UE-based positioning, the BO per beam may be explicitly signalled via the LPP assistance data.At RAN1#105 it was determined to use positioning reference unit (PRU) as a placeholder for reference device discussion.

Proposal 13: RAN1 to specify support for enabling a PRU to support configuration by the network to help with beam offset estimation, among other parameters. In particular, RAN1 should investigate methods and signaling required to enable the selected reference device to ability of reference device to determine beam offset errors are present.

# Conclusion

We made the following observations and proposals in this paper:

Observation 1: In the presence of NLoS, the strongest multipath component in terms of PRS-RSRP may not necessarily correspond to the first arrival/shortest path.

Observation 2: To enhance positioning accuracy in the presence of different NLoS, it is beneficial to report multiple PRS resources each associated with time of arrival information.

Proposal 1: For DL-AoD support reporting of multiple PRS resources per PRS resource set, with each resource being associated with time of arrival information.

Proposal 2: Support “Option 3: Up to N>8 measurements” as candidate enhancement. FFS value of N.

Observation 3: Providing precoding information may be an unnecessary disclosure of detail of the gNB implementation.

Proposal 3: Support TRPs to optionally report multiple directions per DL PRS resource with each direction being associated with a power value relative to the boresight power for that resource.

Proposal 4: Include additional assistance data for UE based positioning, including TRP polarization and geometry.

Observation 4: Reporting adjacent beams may be worse than letting the UE determine the best beams to report.

Proposal 5: Do not support enhancements for adjacent beam reporting (i.e., do not support option 1).

Observation 5: The current specification has insufficient support for two stage beam sweeping.

Proposal 6: Support association between resources belonging to two DL PRS resource sets (at the same TRP) to facilitate support of two stage beam sweeping.

Proposal 7: In two-stage beam sweeping DL-AoD, the positioning report should be generated only based on the second-stage PRSs.

Proposal 8: Support and study on-demand PRS framework for two-stage PRS beam sweeping.

Proposal 9: Support Option 2 - Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE.

Proposal 10: For UE-based mode, support option 1: indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE.

Proposal 11: Support of indication of expected AoD/ZoD value and uncertainty (of the expected AoD/ZoD value) range(s) is signaled by the LMF to gNBs/TRPs in on-demand PRS framework.

Observation 6: DL-AoD cannot meet the positioning requirements without correcting for beam orientation errors.

Proposal 12: RAN1 to study beam orientation errors and potential correction mechanisms in order to improve the positioning accuracy achievable with DL-AoD. Including:

- UE-based positioning: the beam offset (BO) could be signaled to the UE, as either an indicator, e.g. low/medium/high, each specifying an error range or as a specific value computed by the network

- UE-assisted positioning: LMF should be aware of the BO and compensate it when computing the position estimate.

- Signaling aspects:

  - LMF signals to TRPs that a BO beam re-tuning is needed. The BO correction may be explicitly signalled to the TRP by the LMF; alternatively, the LMF may send a Boolean indication that a BO recomputation and adjustement is needed.

  - UE measurement reports to facilitate BO identification and potential correction.

Proposal 13: RAN1 to specify support for enabling a PRU to support configuration by the network to help with beam offset estimation, among other parameters. In particular, RAN1 should investigate methods and signaling required to enable the selected reference device to ability of reference device to determine beam offset errors are present.

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

[2] R1-2107057, Views on on mitigating UE and gNB Rx/Tx timing errors, Nokia, Nokia Shanghai Bell.

[3] R1-2107058, Views on enhancing UL AoA, Nokia, Nokia Shanghai Bell.

[4] R1-2107060, Views on PHY Latency Reductions, Nokia, Nokia Shanghai Bell.

[5] R1-2107061, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

[6] R1-2107062, Additional views on Inactive Mode Positioning and on-demand PRS, Nokia, Nokia Shanghai Bell.