3GPP TSG RAN WG1 Meeting #106-e	R1-2107082

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.4

Source:	Futurewei

Title:	Enhancement to support HST-SFN deployment scenario

Document for:	Discussion/Decision

# Introduction

In 3GPP RAN Meeting #86, a new work item (WI) on Further enhancements on MIMO for NR (NR_FeMIMO, see RP-193133) was approved. Among the multiple objectives in the WI, the following is concerned with multi-TRP HST-SFN enhancements:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

…

  - Enhancement to support HST-SFN deployment scenario:

    - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission

    - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

Up to 3GPP RAN1 Meeting #105-e, agreements were achieved for M-TRP HST-SFN enhancements and some are captured below:

Agreement

Scheme 1 is supported in Rel-17

- TRS is transmitted in TRP-specific / non-SFN manner

- DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner

- FFS other details

Agreement

When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations

- Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI state can be associated with {delay spread}  and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties

- Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD)

- Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting

- Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation.

- This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

Agreement

For scheme 1 and SFN transmission of PDCCH support Variant E for QCL assumption in TCI state when TRS is used as source RS

Agreement

Two TCI states are supported for scheme 1 in FR2

Agreement

Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both:

- UL RS based Doppler estimation by gNB

  - FFS: Details including UL RS enhancement

- DL RS based Doppler feedback by UE

  - FFS: Details

  - FFS: Whether UE capability needs to be introduced

- Whether to support one or both will be decided later

Agreement

Scheme 1 for PDSCH is identified by

- New RRC parameter and the number of TCI states indicated by DCI

  - FFS RRC configuration details, e.g., per BWP or per CC

  - FFS whether or not restriction to a single CDM group for DM-RS is also supported

Agreement

Confirm the following working assumption from RAN1#104b-e:

All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1.

Agreement

UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with scheme 1 PDSCH by RRC , but not capable to support dynamic switching between scheme 1 and single-TRP by TCI state field in DCI Format 1_1/1_2

Agreement

For specification based TRP-based frequency offset pre-compensation scheme

- Support dynamic (DCI -based) switching with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - This feature is UE optional

  - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with TRP-based frequency PDSCH by RRC , but not capable to support dynamic switching between TRP-based frequency and single-TRP by TCI state field in DCI Format 1_1/1_2

- Support semi-static (RRC based) switching with Rel-16 schemes 1a, 2a, 2b, 3, 4

- Support semi-static (RRC based) switching with Rel-17 scheme 1 (PDSCH)

Agreement

Enhanced MAC CE signaling is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Working Assumption

For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) are supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states.

- FFS: Additional support of Variant B

Agreement

- For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type

  - FFS rule or signalling to determine which TCI state with dropped QCL parameters

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.

  - FFS whether this restriction is per UE or per CC

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs.

  - FFS whether this restriction is per UE or per CC

In this contribution, HST-SFN enhancement as part of multi-TRP support, is discussed in detail.

# Discussion

## Schemes and options

The following discussions are related to details of UE-based Scheme 1 and gNB-based pre-compensation scheme, with the agreed QCL Variant E for Scheme 1, defined as “both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)”, and candidate QCL Variants A/B/C/E for gNB-based pre-compensation scheme.

For Scheme 1 with PDCCH, the TRSs are TRP-specific / non-SFN and separate, but the PDCCH DMRS forms SFN. Each PDCCH candidate and its DMRS are associated with two TCI states simultaneously.

For Scheme 1 with PDSCH, the TRSs are TRP-specific / non-SFN and separate, but the PDSCH DMRS forms SFN. Each PDSCH and its DMRS ports are associated with two TCI states simultaneously. A PDSCH DMRS port is configured/activated to be associated with 2 TRSs at the same time.

With gNB pre-compensation, the SFN DMRSs are pre-compensated, which may be QCLed to an optionally transmitted second set of TRSs in Step 3, or QCLed to the TRP-specific TRSs, that is, several options exist.

In more detail, the PDSCH is configured/activated with only one set of DMRS ports, and hence one antenna port(s) field is configured in the DCI scheduling the PDSCH, and also one group of antenna port(s) indicated by the DCI or other signaling. In addition, each DMRS ports is configured/activated to be associated with 2 TRS at the same time, i.e., 2 TCI states at the same time, e.g., two TCI fields are configured in the DCI, two TCI states are activated by MAC, etc.

The SFN DMRS needs to be associated with multiple TCI state indices, each TCI state index specifies a QCL (generally Type A) relation to one of the two TRSs. Based on our understanding, for DMRS SFN (or PDCCH/PDSCH/TRS SFN) formed by two TRPs, the Doppler shifts for the transmissions from the two TRPs should not be significantly different, otherwise the UE processing complexity will be high. Thus UE assumption and minimum UE behavior (if any) need to be specified. For example, in Scheme 1 without pre-compensation, the UE needs to assume the channel on a DMRS port is a composite channel, a superposition of individual channels associated with the TRSs and hence Variant E is adopted. However, some further details and UE assumptions/behavior are needed.

We use Figure 1 as an illustration for the general setup for different options.

Figure 1 Illustration of SFN scheme and channel properties

As shown in the figure, the TRPs transmit two TRSs to the UE, based which the UE can acquire the individual channel responses as follows:

- TRS0: channel 0 which has cluster(s) 0 with average delay 0, delay spread 0, Doppler spread 0, and Doppler shift 0.

- TRS1: channel 1 which has cluster(s) 1 with average delay 1, delay spread 1, Doppler spread 1, and Doppler shift 1.

The UE then needs to derive the properties for the SFN channel for the SFNed DMRS. Clearly, in the composite channel, both clusters (or taps / paths / multipath components) are present. Depending on the relation between the two TRSs and the two TRPs, there could be at least the following options:

### Option 1 TRS/DMRS design (i.e., UE-based solution Scheme 1)

Option 1 TRS/DMRS design: no pre-compensation for TRS Doppler shifts or DMRS Doppler shifts, with TRP-specific TRS (TRP 0 transmits TRS 0, and TRP 1 transmits TRS 1, no pre-compensation) and SFN DMRS using different Doppler shifts (TRP 0 transmits DMRS 0, and TRP 1 transmits DMRS 1, no pre-compensation)

For this option, the TRPs are synchronized, and they transmit in a synchronized fashion without pre-compensation for Doppler shifts for all of their signals/channels, including the TRP-specific TRSs, SFN DMRS, and PDCCH/PDSCH. This is the UE-based solution Scheme 1. Then UE sees different average delays, delay spreads, Doppler spreads, and Doppler shifts for different TRSs, i.e., the TRSs are TRP-specific. Based on the TRSs, a UE can estimate TRP-specific average delays, delay spreads, Doppler spreads, and Doppler shifts. The SFN DMRS is from both TRPs without pre-compensation and hence with different Doppler shifts, so the UE uses the channel properties estimated from TRSs to adjust the Dopper shifts and then processes the DMRS.

Each TRP-specific TRS can be QCLed (Type C, and for FR2, also Type D) to the corresponding TRP-specific SSB. This TRS corresponds to the first set of TRS in the agreement. No second set of TRS is needed.

For the QCL of SFN DMRS to the TRSs:

- Delay profile

The average delay and delay spread of the DMRS have to come from the superposition from the two TRPs, i.e., from both TRSs.

- Doppler profile

The Doppler properties are different for the TRSs and the DMRS. The UE estimates the Doppler shifts from the TRSs and then use the estimates to process the DMRS. The UE complexity would be high.

Variant E is adopted for this option. Different from Variant E for other options below, Variant E for this option requires the UE to know that the DMRS is not pre-compensated, which needs to be signaled to the UE from the gNB if more than one Variant E option is supported.

This option does not require the second set of TRSs or UE Dopper sounding/feedback based on the first set of TRS, but the UE DMRS processing complexity is high.

### Option 2 TRS/DMRS design

Option 2 TRS/DMRS design: with TRP-specific TRSs (TRP 0 transmits TRS 0, and optionally TRP 1 transmits TRS 1, no pre-compensation), the second set of TRSs in Step 3 (pre-compensated for Doppler shift(s)), and SFN DMRS (same pre-compensation as the second set of TRSs)

For this option, the TRPs are synchronized, and they transmit in a synchronized fashion, without pre-compensation for Doppler shifts for the first set of TRP-specific TRSs, and then with sufficient pre-compensation for Doppler shift(s) for the second set of TRSs and SFN DMRS. The pre-compensation may be based on UE signals or feedback sent in the UL, and the network may adjust the frequency for one or both of the second set of TRSs so that the UE will experience the same Doppler shift on the TRSs. The TRSs may form a SFN if the network chooses to send the TRSs on the same REs, but the network can alternatively choose to send the TRSs on different REs.

The first set of TRSs can be QCLed (Type C) to one or more of the SSBs, and the UE’s UL sounding should be “QCLed” to the TRS 0 of the first set of TRSs, or similarly, the UE generates Doppler feedback based on TRS 0. Note that TRS 1 of the first set of TRSs is not necessary (or TRS 1 is sent but TRS 0 is optional).

The second set of TRSs can be QCLed (Type C) to one or more of the SSBs. The UE does not need to know the relation between the second set of TRSs and the first set of TRSs. The pre-compensation of the second set of TRSs may be done based on any of the following, and can be transparent to the UE:

- The Doppler shift may be adjusted based on TRS 0, i.e., the network pre-compensates TRP 1 to match TRP 0; OR

- The Doppler shift may be adjusted based on TRS 1 i.e., the network pre-compensates TRP 0 to match TRP 1; OR

- The Doppler shift may be adjusted based on both TRSs, i.e., the network pre-compensates both TRPs to the average of the TRPs’ frequencies; OR

- The Doppler shift may be adjusted based on gNB preference, i.e., the network pre-compensates both TRPs to a frequency determined by the gNB.

For the QCL of SFN DMRS to the TRSs, generally only the second set of TRSs is useful and the first set is not used:

- Delay profile

The channels for the second set of TRSs have different average delays and delay spreads, so both TRSs have to be used for acquiring DMRS average delay and delay spread. That is, DMRS delay profile is from both TRSs unless the second set forms TRS SFN.

- Doppler profile

The TRS channels and the DMRS channels have the same Doppler (due to pre-compensation). Doppler profile may be derived from both TRSs or only one of them. So there can be a few alternatives. If the Doppler profile is from only one of them, this is Variant A, but to specify only one of them for Doppler requires more signaling and does not lead to any performance benefit. Alternatively, both TRSs could be used, and Variant E could be used. Furthermore, if the second set of TRSs forms TRS SFN, then QCL Type A can be used.

Therefore, this option can be supported by Variant E (or less preferably, Variant A) if the second set of TRSs are on different resources, or more likely, supported by QCL Type A if the second set of TRSs form SFN, which can be implemented as standard transparent SFN for TRSs, DMRSs, and PDCCH/PDSCH. The corresponding UE behavior for processing the second set of TRSs and the SFN DMRS is simple. The main standard impact is to support the QCL-like relation between TRS 0 (or TRS 1) and SRS.

However, this Variant E is different from Variant E for Option 1, as Option 1 DMRSs are not pre-compensated but Option 2 DMRSs are pre-compensated to the average Doppler shift of TRS 0 and TRS 1. gNB signaling to UE to differentiate the options are needed if both Option 1 and Option 2 are supported.

### Option 3 TRS/DMRS design

Option 3 TRS/DMRS design: TRS 0 transmitted by both TRP 0 (without pre-compensation) and TRP 1 (with pre-compensation), and optionally TRS 1 transmitted by TRP 1 (without pre-compensation). SFN DMRS with the same transmission scheme as TRS 0

For this option, the TRPs are synchronized, and they transmit in a synchronized fashion. TRS 0 is sent by both TRP 0 and TRP 1, without pre-compensation by TRP 0 but with sufficient pre-compensation for Doppler shift by TRP 1 so that TRS 0 forms a SFN by TRP 0 and TRP 1. TRS 1 is an optional TRS, which may be sent only by TRP 1 without pre-compensation. In other words, after the network receives the UE UL signals based on only TRS 0 or only TRS 1, it adjusts TRP 1’s frequency to align with TRP 0’s frequency, and hence then UE sees one Doppler shift for TRS 0, and optionally another Doppler shift for TRS 1.

The TRSs can be QCLed to one or more of the SSBs. TRS 0 is part of the first set of TRSs, but can also be directly used by the UE as the second set of TRSs since TRS 0 forms SFN and DMRS is transmitted the same way as TRS 0.

For the QCL of SFN DMRS to the TRSs:

- Doppler profile

The Doppler properties have to come from TRS 0. If both TRS 0 and TRS 1 are transmitted, it requires the network to indicate TRS 0 to the UE for obtaining the Doppler properties.

- Delay profile

For average delay and delay spread of the DMRS, they are the same as those of TRS 0, but the cluster(s) of TRS 1 is also included in TRS 0, so the cases are:

- The average delay and delay spread may be from TRS 0, which can be supported by legacy QCL Type A to TRS 0; OR

- The average delay and delay spread may be from the superposition of both TRSs, which can be supported by Variant A.

Therefore, this option can be supported by legacy QCL Type A or Variant A. We focus on supporting this option with QCL Type A to TRS 0, which simplifies UE behavior. Also since TRS 1 without pre-compensation is optional, the RS overhead can be reduced. Overall, this option has low RS overhead and low UE complexity, and the only standard impact is to support the QCL-like relation between TRS 0 and SRS.

### Option 4 TRS/DMRS design

Option 4 TRS/DMRS design: no pre-compensation for TRS Doppler shifts, with TRP-specific TRS (TRP 0 transmits TRS 0, and TRP 1 transmits TRS 1); with SFN DMRS with pre-compensation. No second set TRSs

For this option, the TRPs are synchronized, and they transmit in a synchronized fashion, without pre-compensation for Doppler shifts for the first set of TRP-specific TRSs. Then UE sees different average delays, delay spreads, Doppler spreads, and Doppler shifts for different TRSs, i.e., the TRSs are TRP-specific. Based on the TRSs, a UE can estimate TRP-specific average delays, delay spreads, Doppler spreads, and Doppler shifts. Then the UE sends signals in the UL, and the network may adjust the frequency for one or both of the DMRSs so that the UE will experience the same Doppler shift on the DMRSs, i.e., the DMRSs from the TRPs form SFN with the same Doppler shift.

The TRSs can be QCLed to one or more of the SSBs. No second set of TRSs is transmitted. This is similar to Option 2 except that Option 2 has the second set of TRSs. There are several alternatives for the gNB to transmit the DMRSs:

- Alt. 1: The Doppler shift may be adjusted based on TRS 0, i.e., the network pre-compensates TRP 1’s DMRS transmission to match TRP 0; OR

- Alt. 2: The Doppler shift may be adjusted based on TRS 1 i.e., the network pre-compensates TRP 0’s DMRS transmission to match TRP 1; OR

- Alt. 3: The Doppler shift may be adjusted based on both TRSs, i.e., the network pre-compensates both TRPs’ DMRSs to the average of the TRPs’ frequencies.

For the QCL of SFN DMRS to the TRSs:

- Doppler profile

For Alt. 1, the Doppler properties have to come from TRS 0 and this requires the network to indicate TRS 0.

For Alt. 2, the Doppler properties have to come from TRS 1 and this requires the network to indicate TRS 1.

For Alt. 3, the Doppler properties have to come from TRS 0 and TRS 1 and this requires the network to indicate both TRSs.

- Delay profile

The channels for the TRSs have different average delays and delay spreads, so both TRSs have to be used for acquiring DMRS average delay and delay spread. That is, DMRS delay profile is from both TRSs.

Variant E or Variant A can be used to support this option. More specifically, this option with Alt. 1 or Alt. 2 can be supported by Variant A, and a dynamic switching to change the anchor TRP / frequency can also be supported. In addition, this option with Alt. 3 can be supported by Variant E. However, this Variant E is different from Variant E for Option 1, as Option 1 DMRSs are not pre-compensated but Option 4 DMRSs are pre-compensated to the average Doppler shift of TRS 0 and TRS 1. gNB signaling to UE to differentiate the options are needed if both Option 1 and Option 4 are supported.

This option does not require the second set of TRSs, but the UE needs to perform sounding or Doppler feedback based on the first set of TRS. the UE DMRS processing complexity is lower than Option 1 but higher than Option 2 or Option 3.

- A variation of this option is that a SFN TRS sent from both TRPs is also transmitted, and this SFN TRS can be used for delay profile estimation. Then the Doppler profile estimation is either from TRS 0 or TRS 1, and Variant B can be adopted. The motivation for this variation seems to be that the dynamic switching of the anchoring frequency between TRS 0 and TRS 1 can be done more easily, especially when the UE passes the middle point of the two TRPs. However, the UE needs to track all 3 TRSs at the same time and some companies have concerns on this. Since the dynamic switching can be supported by other options, it seems this variation is not needed.

- Yet another variation of this option is that the first set of TRSs are not on orthogonal resources but on overlapping resources so that a legacy SFN with different Doppler frequencies is formed for the first set. This SFN cannot be used for Doppler estimation but can be used for delay profile estimation. Then for Doppler estimation, a second TRS by TRP 0 also transmitted. For this implementation, Variant A or Variant B may be adopted. However, as Variant A is already agreed to be supported, it is unclear why Variant B needs to be further considered. Also this scheme requires 2 sets of TRSs and UL signal, though it has similar UE complexity as Option 2 and Option 3. Therefore, it seems this scheme may not be useful and Variant B is not necessary.

## Summary for all TRS/DMRS QCL/TCI design options

Based on the analysis, different QCL Types and Variants are needed for different options. Even with the same QCL Variant E, the UE behaviors are still different depending on the options, which also requires gNB signaling to UE. Further discussion is needed for the options and UE behaviors. A summary of the options and pros/cons is provided below.

Table 1 Summary of options and pros/cons (Pc: pre-compensation; Df: Doppler frequency)

We have the following proposals.

Proposal 1: Further discuss / clarify TRS/DMRS transmission options: with or without the second set of TRSs, with or without Doppler shift pre-compensation for TRS(s), and with or without Doppler shift pre-compensation for the SFN DMRS.

Proposal 2: Support Variant E

- For Variant E, define gNB signaling to UE whether DMRS pre-compensation is performed, and define different UE behaviors accordingly.

## UE behavior and UE assumptions

RAN1 has decided to signal the existing QCL types to UE but UE may drop certain QCL parameters from the indicated QCL type to support some QCL variants. First we note that Variant E does not require such additional behavior/rule/signaling, which can be part of the advantage of Variant E. For the agreed Variant A, the Doppler parameters need to be dropped from QCL type A (i.e., the network indicates QCL type A, but the Doppler parameters are not used by the UE). The simplest way to do so is to have a one-bit flag (or more bits are needed if more QCL variants are to be supported), which tells the UE whether the Doppler parameters are to be dropped or not. This flag may be joint with the indication of the anchor TRP/TRS, i.e., when indicating the anchor TRS, the dropped parameter is also indicated. It may be possible to interpret that a non-anchor (non-pre-compensated) TRS or a non-pre-compensated SFN TRS needs to apply the dropping of Doppler parameters. The detail depends on which SFN schemes are to be supported and whether other QCL variants are supported.

For the UE assumption on no different SFN schemes, though more flexibility can be allowed by restriction only on the carrier level, it is unclear whether there is any performance advantage with the additional flexibility. To reduce UE/gNB complexity, it is preferred to have the restriction per UE and not per carrier.

Proposal 3: For UE behavior/assumption:

- Revisit the rule/signaling to determine dropped QCL parameters after finalizing the supported schemes and QCL variants;

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH, and the restriction applies per UE;

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs, and the restriction applies per UE.

## UE-assisted TRP-based pre-compensation

In order for the network to apply Doppler shift pre-compensation value for a TRP before transmitting the SFN PDSCH, the UE may need to transmit UL signal to each TRP, and the UL signal may be a SRS based on which the TRP can estimate the Doppler shift (Option 1) or may carry the Doppler information that the UE experiences for that TRP in DL (Option 2). Option 1 may be supported by existing mechanisms. Alternatively, the UE may need to send Doppler feedback to the gNB based on the TRS in DL. This simplifies gNB implementation, but requires more UE processing and standard support of Doppler feedback. Therefore, there can be some tradeoff between the UE complexity and gNB complexity. To reduce UE complexity for estimating Doppler shifts, it is preferred to simplify the UE behavior via transmitting SRS towards the TRPs and rely on the TRPs to estimate the Doppler shifts. The current standards can already support this behavior, and if additional requirements are identified to further enhance SRS transmissions, RAN1 can consider to support new SRS designs, but so far there does not seem to be a strong justification for this purpose.

Proposal 4: Support SRS for Doppler estimation for UE-assisted TRP-based pre-compensation.

## Scheme differentiation

An HST-capable UE needs to operate under different environments, such as on a HST operating on a SFN scheme (Scheme 1, Scheme 2 if supported), or off a HST operating on a single-TRP mode or multi-TRP mode. The schemes need to be differentiated and indicated to the UE to avoid ambiguity and errors. At least the following potential schemes should be differentiated:

- Scheme 1 with PDSCH

This can be configured to the UE via the new RRC configuration parameter as agreed in the last meeting and further by the number of TCI states indicated by DCI for fast switching purpose.

To tell this scheme apart from other schemes in a signalling to UE, the new RRC configuration parameter can configure Scheme 1 to be supported, and further for dynamic switching, Scheme 1 is identified as the PDSCH is configured/activated with only one set of DMRS ports and there is only 1 antenna port(s) field in DCI and 1 group of antenna port(s) indicated by DCI. Each port is associated with 2 TCI states (e.g., two TCI fields in DCI, or one TCI field but each TCI codepoint is linked to two TCI states according to MAC command, etc.), and the number of ports corresponds to the number of layers/streams (i.e., rank) of the transmission. That is, at least different DCI/MAC designs with different fields should be provided, or an operation type flag may be signalled in the DCI or MAC or RRC signalling. To distinguish Scheme 1 from other schemes, different DCI types may be used (to accommodate two TCI fields in a DCI, for example), or alternatively a new field for the number of streams/layers (or equivalently, the number of DMRS ports per PDSCH stream/layer) is added to the DCIs, or alternatively each TCI codepoint in the DCI is linked to two TCI states according to MAC command.

More generally, detailed analysis can show that various schemes can be differentiated based on a number of parameters, such as the number of TRSs configured, the number of TCI states / QCL configured / activated for each TRS, the number of DMRS ports per PDSCH layer, the number of PDSCH layers, etc. To summarize, the PDSCH cases are listed below with different parameters.

Table 1 PDSCH transmission scheme differentiation based on parameters

|  | # TRP(s) | # TRS(s) or SSB(s) | # TCI states/QCL per TRS (1 TRS links to how many SSB) | # TCI states/QCL per DMRS port (1 DMRS port links to how many TRS/SSB) | # DMRS port(s) per PDSCH layer | # total DMRS port(s) | # PDSCH layer(s) | Interpretation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1 | 1 | 1 | 1 | 1 | L | L | L layer S-TRP transmission |
| Case 2 | 2 | 1 | 2 (for TRS) or N/A (for SSB) | 1 | 1 | L | L | L layer SFN (SSB/TRS/DMRS/PDSCH SFN, standard transparent) |
| Case 3 | 2 | 2 | 1 | 1 (TRS1 or 2) | 1 | (L1, L2), L1=L2 * | L1 = L2 * | M-TRP FDM/TDM R16 Scheme 2/3/4 |
| Case 4 | 2 | 2 | 1 | 2 (TRS1 & 2) | 1 | L | L | M-TRP SFN Scheme 1 (DMRS/PDSCH SFN) |
| Case 5 | 2 | 2 | 1 | 1 (TRS1 or 2) | 2 ** | 2L ** | L | M-TRP SFN Scheme 2 (PDSCH SFN) |
| Case 6 | 2 | 2 | 1 | 1 (TRS1 or 2) | 1 | L1 or L2, not both * | L1 or L2 * | M-TRP gNB selection scheme |
| Case 7 | 2 | 2 | 1 | 1 (TRS1 or 2) | 1 | (L1, L2) * | L1+L2 * | M-TRP SDM R16 Scheme 1a |

* L1: # of DMRS port(s) QCLed to TRS1 (or SSB1)

* L2: # of DMRS port(s) QCLed to TRS2 (or SSB2)

** 2 or 2L: half of it is QCLed to TRS1 and the other half TRS2

Case 2 includes the standard-transparent SFN scheme and TRS-SFN scheme.

We have the following proposals:

Proposal 5: The PDSCH SFN Scheme 1 and other relevant schemes (if supported) can be differentiated via RRC parameters and dynamic parameters such as the numbers of TRSs, TCI states, DMRS ports per PDSCH layer, and PDSCH layers.

# Conclusion

In this contribution, we discussed Rel-17 SFN-HST enhancements. The following are proposed:

Proposal 1: Further discuss / clarify TRS/DMRS transmission options: with or without the second set of TRSs, with or without Doppler shift pre-compensation for TRS(s), and with or without Doppler shift pre-compensation for the SFN DMRS.

Proposal 2: Support Variant E

- For Variant E, define gNB signaling to UE whether DMRS pre-compensation is performed, and define different UE behaviors accordingly.

Proposal 3: For UE behavior/assumption:

- Revisit the rule/signaling to determine dropped QCL parameters after finalizing the supported schemes and QCL variants;

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH, and the restriction applies per UE;

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs, and the restriction applies per UE.

Proposal 4: Support SRS for Doppler estimation for UE-assisted TRP-based pre-compensation.

Proposal 5: The PDSCH SFN Scheme 1 and other relevant schemes (if supported) can be differentiated via RRC parameters and dynamic parameters such as the numbers of TRSs, TCI states, DMRS ports per PDSCH layer, and PDSCH layers.