3GPP TSG RAN WG1 Meeting #106-e	R1-2107087

e-Meeting, Aug 16th – 27th, 2021

Agenda Item:	8.14.2

Source:	Futurewei

Title:	XR evaluation methodology

Document for:	Discussion and decision

# Introduction

In RAN1#105-e meeting the discussion of coverage evaluation methodologies started. The following note was made in [1] because of the discussion

For companies to further study and if necessary, discuss in RAN1#106-e

(Coverage evaluation methodology) For XR/CG in DL or UL, coverage is defined to be the A-percentile point in CDF of Coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa)

- A = [5], other value can also be reported

- FFS: Value of B, e.g. B = 1, capacity, etc.

- Note: Coupling gain for coverage evaluation is defined as the ratio of received and transmitted power measured in dB, and includes antenna gains, path loss, shadowing, indoor- or body loss, etc. Example of coupling gain can refer to TR 37.910.

An alternate method could be to use the “traditional” method such as what is used in the CE study/work item.

In this contribution we analyze the two coverage evaluation methodology techniques that are listed in the Chairman’s notes and present results/analysis for both. The first considered methodology is based on link-level simulations and using the link budget template from the CE study/work item. The two channels analyzed are the PDSCH and PUSCH. The simulation assumptions and results are presented in Section 2. The second evaluation methodology considered is based on the system-level simulation analysis as described in the note above. The simulation assumptions and results are presented in Section 3. Discussion and analysis comparing the two evaluation methodologies is presented in Section 4. It is concluded based on the analysis that link-level based methodology is the right approach to capture coupling loss.

# Coverage Evaluation Methodology: Link-Level

To evaluate coverage for XR, link-level simulations were performed to obtain the required SNR for the PDSCH and PUSCH channels under performance target. The FR1 with the carrier frequency of 4 GHz has been considered.

As an initial step towards making progress, we have performed the link-level simulations based on assumptions listed in Table1 and Table2 for PDSCH and Table1 and Table3 for PUSCH. Table 1 is a common table considering general assumptions while Table2 and Table3 list the specific channel assumptions.

## PDSCH

### Simulation Assumptions

This section includes the simulation assumptions used for Dense Urban FR1 scenario which follows the agreements made in meeting RAN1 #104-e and RAN1 #104-bis-e and can be found in [2] and [3]. The corresponding results are presented in Section 2.2. Table 1 summarizes the Link-level simulation assumptions while Table2 summarizes more specific channel parameters the downlink channel.

Table 1: Link-level Simulation Assumptions

| Scenario | Dense Urban FR1 |
| --- | --- |
| Carrier Frequency | 4 GHz |
| BW | 100 MHz |
| SCS | 30 kHz |
| Frame Structure | DDDUU |
| gNB antenna configuration | 32 TxRUs (8,2,2,1,1;8,2) |
| UE antenna configuration | (Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), |
| Channel Model | TDL-C, NLOS |
| UE antenna correlation | Low |
| UE velocity | 3km/hr |
| Channel estimation | practical |

Table 2: PDSCH Channel Specific Assumptions

| Target Rate | 30 Mbps |
| --- | --- |
| # of RBs | 275 |
| PDSCH symbols | 12 OFDM symbols |
| DMRS | Type 1, 2 DMRS symbol no multiplexing with data |
| MCS index | 22 for 1 layer, 13 for 2 layers |
| Number of transmissions (initial + retransmissions) | 2 |
| Diversity scheme | PRB bundle size 2 |
| Rx Combining | MMSE-IRC |
| Number of PDSCH layers | 1, 2 |

In RAN1#104-e [2], the following agreement was made

Agreement:

Parameters of Truncated Gaussian distribution for packet size of DL video stream in case of single stream evaluation (note: these parameter values are those before the truncation):

- [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size

- Other values that can be used for evaluation: [STD, Max, Min] = [4, 112, 88] % of Mean for single eye buffer, [3, 109, 91] % of Mean for dual eye buffer

- FFS: Whether and how to evaluate single eye and dual eye buffer

- Note: Companies report the values used in their simulation results.

- Note: There is no consensus that the [10.5, 150, 50]% of mean packet size is the best set of parameters

With the assumption of 60 fps, we may choose the TBS such that the largest size packet may be transmitted, the maximum packet size is calculated as

Max size= 1.5 x mean = 1.5x 30 Mbps x 16.667 ms/frame =750 kbits/frame

The assumption made here is that the packet in the DL video stream having maximum packet size need to be transmitted within 10 msec packet delay budget (PDB). With the assumption of 30 kHz SCS, the 10 msec PDB is equivalent to 20 slots. Therefore, in our simulations, the packet has been divided into 6 transport blocks. The number of transmissions was set to 2. All transport blocks need to be transmitted successfully for packet to be considered as successfully transmitted. Figure 1 is an illustrative figure of the explanation provided here.

Figure 1: DDDUU configuration with the assumption of a packet split into 6 transport blocks allowing for a second retransmission within the 10 msec PDB (20 slots)

### Simulation Results

Figure 2 shows the PER versus SNR performance for one layer and two layers of PDSCH transmissions.

Figure 2: PDSCH performance for 4 GHz assuming layer 1 transmission

The required SNR is ~-3.37 dB for the case of one layer transmission assuming a PER target of 0.001. A 1.5 dB improvement is reported with 2 layers of transmission as shown in the Figure 2. Further link-level assumptions were made in Table 3 and the required SNR was used in CE template excel sheet that allows calculation of maximum coupling loss. The excel sheet is provided in the Appendix.

Table 3: Link Budget Assumptions

| Downlink Power Spectrum Density dBm/MHz | 36 |
| --- | --- |
| Receiver Interference Density dBm/Hz | -999 |
| Receiver implementation margin dB | 2 |
| Receiver noise figure dB | 9 |
| Occupied Channel Bandwidth for data channel Hz | 275*30kHz*12 |

The MCL is then defined as and presented in Table4:

Total transmit power - Receiver sensitivity + gNB antenna gain (component 2)  [4]

Observation 1: Under the simulation assumptions made in Section 2.1, an SNR of -3.37dB is required for target performance of 0.001 PER for 1-layer PDSCH transmission. For the case of two downlink layers, the performance is enhanced by 1.5dB.

Observation 2: The MCL for single layer transmission PDSCH is -142.37dB while the MCL for the case of two layers PDSCH transmission is -143.8dB.

Table 4: PDSCH MCL results

| Number of layers results | SNR for target performance 10^-3 in dB | MCL dB |
| --- | --- | --- |
| 1 layer | -3.37 | -142.37 |
| 2 layers | -4.80 | -143.80 |

## PUSCH

### Simulation Assumptions

This section includes the simulation assumptions used for PUSCH channel. The corresponding results are presented in Section 2.2.2. Table1 from Section 2.2.1 which summarizes the Link-level simulation assumptions still apply while Table3 summarizes specific simulation assumptions for the PUSCH channel.

According to the agreement below from RAN1#105-e. Two configurations are provided for the UL in terms of requirements, one to model VR/CG Pose/control with lower rates and less frequent interarrival packets and second model for the AR with single stream and more frequent traffic.

Agreement

When companies are submitting evaluation results to RAN1, it is recommended to submit results at least the following parameters in the below table.

- Note 1: This is only intended to have more results from more companies at least for the corresponding configuration. RAN1 agreements regarding baseline vs. optional for simulation scenarios, configurations, parameters, remain the same.

- Note 2: Companies are encouraged to submit results for other baseline/optional configurations as much as they can.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

In this contribution we focus on the higher rate configuration i.e. the model in the last row of the table in the Agreement. More precisely on the UL AR Option 1 with data rate of 10 Mbps, packet arrival rate of 60 fps and PDB of 30 msec.

Table 5: PUSCH specific parameters

| Target Rate | 10 Mbps | 0.2 Mbps |
| --- | --- | --- |
| # of RBs | 75 for 1 layer and 76 for 2 layers | 5 RBs |
| PUSCH symbols | 14 including DMRS | 14 including DMRS |
| MCS index | 22 for 1 layer and 13 for 2 layers | 8 for 1 layer |
| Number of transmissions (initial + retransmissions) | 2 | 3 |
| Rx Combining | MMSE-IRC | MMSE-IRC |

In RAN1#104b-e [2], the following agreement was made for AR in UL

Agreement:

For evaluations of AR in UL:

- Option 1 (Baseline for power and capacity evaluations): Two streams as defined below

  - Stream 1: pose/control

    - Traffic model and QoS parameters are same as for pose/control for UL CG/VR.

  - Stream 2: A stream aggregating streams of scene, video, data, and audio.

    - Packet size: Truncated Gaussian distribution with the parameter values same as for DL

    - Periodicity: 60 fps

      - Jitter (optional): same model as for DL

    - Data rate: 10 Mbps (baseline), 20 Mbps (optional)

    - PDB: [60] ms (baseline), [10/15] ms (optional)

- Option 2 (Optional for power evaluation and baseline for capacity evaluation): Single stream as defined below

  - Packet size: Truncated Gaussian distribution with the parameter values same as for DL

  - Periodicity: 60 fps

    - Jitter (optional): same model as for DL

  - Data rate: 10 Mbps (baseline), 20 Mbps (optional)

  - PDB: [60] ms (baseline), [10/15] ms (optional)

With the assumption of 60 fps. We may choose the TBS such that the largest size packet may be transmitted

Max 1.5 x mean = 1.5x 10 Mbps x 16.667 ms/frame =250 kbits/frame

The maximum packet size that needs to be transmitted should be within the 30 msec packet delay budget PDB. However, the interarrival packet is 16.67 msec is less than the PDB therefore a successful packet needs to be transmitted before a new packet arrives. In our simulations, the packet is divided into 6 transport blocks. The number of retransmissions was set 2. All transport blocks needed to be transmitted successfully for a packet to be considered successfully transmitted. An illustrative figure for this explanation is shown in Figure 3.

Figure 3: DDDUU configuration with the assumption of a packet split into 6 transport blocks allowing for a second retransmission with packet interarrival of 16.67msec.

### Simulation Results

Figure 4: PUSCH performance for 4 GHz assuming layer 1 transmission for 10 Mbps and 0.2Mbps target rates

For uplink traffic of 10 Mbps, the required SNR is 1.25 dB (1.10 dB) for the case of one (two) layer transmission assuming a PER target of 0.001. For uplink traffic of 0.2 Mbps, the required SNR is 0.0 dB. The CE template excel sheet allows calculation of maximum coupling loss and was used to calculate the maximum coupling loss. The link budget analysis using the excel sheet is provided in the Appendix. MCL results are reported in Table 6.

Observation 3: Under the simulation assumptions made in Section 2.1, an SNR of 1.10dB is required for target performance of 0.001 PER for 2-layer PUSCH transmission for uplink traffic of 10 Mbps and an SNR of 0.0 dB is required for uplink traffic of 0.2 Mbps.

Observation 4: For the case of 10 Mbps data rates, the MCL for single layer transmission PUSCH is 114.44dB. The MCL for two layers of transmission is 114.59. For the case of 0.2 Mbps data rates the MCL 127.45 assuming CP-OFDM.

Table 6: PUSCH MCL results

| Number of layers results | Target rates | SNR for target performance 10^-3 in dB | MCL dB |
| --- | --- | --- | --- |
| 1 layer | 10Mbps | 1.25 | 114.44 |
| 2 layer | 10Mbps | 1.10 | 114.59 |
| 1 layer (CP-OFDM) | 0.2Mbps | 0 | 127.45 |
| 1 layer (DFT-S-OFDM) | 0.2Mbps | 1.75 | 125.70 |

# Coverage Evaluation Methodology: System-Level

In this discussion, the coupling loss is evaluated based on system-level simulations. An example of a potential definition of coupling gain over the S Tx antenna ports and U Rx antenna ports is expressed as in Figure 5

Figure 5: Coupling Gain

Two assumptions are being discussed as the candidate number of UEs per cell denoted by B. The case of B=1 UE/cell and case of B=capacity UEs/cell. Neither of these two values are technically correct for the following reasons. The former assumption falls back to simply CDF of coupling loss of the assumed deployment and thus is not informative. The latter assumption with number of UEs/cell equal to the capacity, almost 90% of the UEs are satisfactory UEs. Unsatisfactory UEs could be a result of interference and/or coverage issue and therefore it is not clear on how the performance interaction between capacity and coverage would have affected these UEs.

In the following we provide the results (CDF) and report the 5%-tile coupling gain for each of the cases in the downlink. Our simulation setup matches the simulation assumptions in [5] for the FR1 DU and FR1 UMa deployment scenarios. Therefore, details on system level assumptions are therefore omitted in this contribution.

### DU Deployment Scenario

In this section we consider the DU deployment for the two cases,

Case B=1: For the case of one UE per cell, all UEs are satisfied. The 95%-tile CL would be limited by the considered deployment scenario’s ISD. Figure 5 (left) shows CDF result where the 5%-tile CL is reported in Table 8.

Case B=capacity: As reported in our contribution in [5], we include the capacities of the XR system in Table 7 for the case of DU. The reported capacities correspond to the baseline case of ZF precoder (if a BiT based precoder is assumed the capacity increases due to the mitigation of intercell and intracell interference however this is out of scope of this contribution). It is noted that the capacity is based on the baseline assumption of 90% satisfactory UEs with PER of 10^-2 (99% satisfactory transmitted packets with 10 msec). For the DU scenario, the capacity is approximated by 9 (closest integer) to be able to plot the CDF of coupling loss. Moreover, we plot the CDF considering two cases 1) satisfied and unsatisfied UEs and 2) only satisfied UEs. Figure 5 shows that the CDF curves for the two cases present little difference at the 5%-tile. The corresponding 5%-ile results are reported in Table 9.

Table 7: XR System Capacity from [5]

| Scenario | Capacity |
| --- | --- |
| FR1 DU | 8.7 |
| FR1 UMa | 6.1 |

Figure 5: Coupling Loss CDF for DU deployment B=1 and B=capacity.

Table 8: Coupling Loss at the 5%-tile for DU

| Scenario | Coupling Loss |
| --- | --- |
| B=1 | -107 |
| B=capacity, (satisfactory UEs only) | -108 |
| B=capacity, (unsatisfactory+satisfactory UEs only) | -108 |

Note that to obtain the left figure for B=1, the results have been averaged over 100 drops. All UEs were satisfied in this case for each drop. For the right figure with B=capacity, the results been obtained while averaging over 3 drops each having 9 UEs/cell.

### UMa Deployment Scenario

Assumption of B=1: For the case of one UE per cell, all UEs are satisfied. The 95%-tile CL would be limited by the considered deployment scenario’s ISD. Figure 6 (left) shows CDF result where the 5%-tile CL is reported in Table 9.

Assumption of B=capacity: For the UMa scenario, the capacity reported is 6.1 in [5]. We plot the CDF for this case in Figure 6 on the right.

Similar to the observation made for DU deployment, we compare the CDF considering satisfied UEs only to the case that considers all UEs both satisfied and unsatisfied UE. Figure 6 (right) shows that the CDF considering both cases present 2dB differences. The corresponding 5%-ile results are reported in Table 10. The cdf of unsatisfactory UEs is also provided in yellow. It can be concluded that coupling loss may not be only reason why these UEs are unsatisfactory – therefore relying on this methodology may not be appropriate in assessing coupling loss.

It is noted that to obtain the left figure for B=1, the results have been averaged over 100 drops. All UEs were satisfied in this case. For the right figure with B=capacity, the results been obtained while averaging over 3 drops each drop having 6 UEs per cell.

Figure 6: Coupling Loss CDF for UMa deployment B=1 and B=capacity.

Table 9: Coupling Loss at the 5%-tile for UMa

| Scenario | Coupling Loss dB |
| --- | --- |
| B=1 | -130 |
| B=capacity, (satisfactory UEs only) | -128 |
| B=capacity, (unsatisfactory+satisfactory UEs only) | -130 |

Observation 5: With the assumption of B=capacity UEs/cell, the 5%-tile coupling loss obtained from considering satisfied UEs only compared to the 5%-tile coupling loss obtained from considering all UEs

- 2 dB difference for DU scenario

- almost equal for UMa

Regarding obtaining coupling loss CDF for uplink channel, it is not clear what the value of B should be which is similar problem to that discussed in the downlink. Moreover, in case the value of B to be considered is equal to the capacity, it is further not clear whether the UL and DL joint capacity should be considered or not.

Observation 6: For uplink channel coupling loss/gain results, it is not clear what the value of B should be. In the case of B=capacity, it is further not clear whether capacity is obtained from joint UL and DL simulations or based on uplink simulations only.

# Analysis of the Proposed Methodologies

There are several issues that need to be addressed regarding the system-level simulation-based methodology, in the following we summarize the main points.

Issue#1: The case with B=1

With the assumption of having 1 UE per cell the CDF of coupling gain is equivalent to simply the CDF of coupling loss of the scenario. In this case, most likely all UEs are satisfied, and the 95%tile CL would be limited by the considered ISD (pathloss, etc..) in the deployment scenario. Thus, this would not be that meaningful or informative.

Issue#2: The case with B=capacity

In this case, roughly 10% of the UE are unsatisfactory. However, it is not clear whether the unsatisfactory UEs have not met the satisfactory constraints due to being interference limited or coverage limited. Moreover, the case of B=capacity may be difficult to align between companies as different companies have reported different capacity values.

Observation 7: The assumption of having 1 UE per cell may not provide meaningful results as it is equivalent to a simple CDF of coupling loss.

Observation 8: With the assumption of having number of UEs per cell > 1 and specifically equal to capacity, interference affects the system capacity and thus it is not clear how one may analyze the effects of coverage separately

Observation 9: Capacity results have not been aligned between companies thus the use of B=capacity may be problematic.

Observation 10: The system level approach does not provide information on which channel or data stream is the bottleneck for coverage.

Issue#3: Common baseline

If this coverage methodology is to be adopted, we need a single baseline value for B, so that the baseline coverage results are comparable among companies. However, currently it is unclear what a technically correct value of B should be. Moreover, even if B=capacity, it is difficult to align the value of B between companies as it has been shown that a wide range of capacity values is possible for same deployment scenario.

Unless the previous observations have been addressed, in our views the proposed evaluation methodology may not yield accurate coverage results. On the other hand, with more work required to prove the accuracy of this SLS model, a further delay in the study progress may be incurred.

Observation 11: Progress should not be pursued at cost of having accurate results. Moreover, resolving the issues mentioned above may cause a delay in SI progress as a lot of studies needs to be done for proving accuracy.

An alternate method to the system-level based coverage evaluation methodology is the link-level based evaluation methodology which may be more accurate in capturing the maximum coupling loss. Moreover, a template link budget is also available. The group would only need to decide on remaining evaluation assumptions such as number of transmissions, packet size and transport block sizes, number of RBs, MCS levels etc. based on the agreed traffic models. Such parameters can be worked out as was presented in Section 2. One other aspect that needs to be taken care of is the assumption of the interference from link-level simulations. In such as case, accounting of interference may be done by assuming a certain value for receiver interference density.

Observation 12: Link-level based coverage analysis is mature and more accurate and does not have the issues listed above as has been followed by CE and RedCap SIs in Rel-17.

Observation 13: The group may discuss the suitable parameters for link-level based coverage evaluations:

- Size of packet: maximum size frame size according to the adopted statistical model

- Number of Retransmission with the agreed TDD configuration and PDB

- Receiver interference density to model the interference aspect

Based on the above analysis and observations, we have the following proposal:

Proposal 1: The link-level based evaluation methodology is adopted as evaluation methodology for XR coverage analysis.

# Conclusion

In this contribution, we present our views on coverage evaluation methodology. The following observations and conclusions have been made.

Observation 1: Under the simulation assumptions made in Section 2.1, an SNR of -3.37dB is required for target performance of 0.001 PER for 1-layer PDSCH transmission. For the case of two downlink layers, the performance is enhanced by 1.5dB.

Observation 2: The MCL for single layer transmission PDSCH is -142.37dB while the MCL for the case of two layers PDSCH transmission is -143.8dB.

Observation 3: Under the simulation assumptions made in Section 2.1, an SNR of 1.10dB is required for target performance of 0.001 PER for 2-layer PUSCH transmission for uplink traffic of 10 Mbps and an SNR of 0.0 dB is required for uplink traffic of 0.2 Mbps.

Observation 4: For the case of 10 Mbps data rates, the MCL for single layer transmission PUSCH is 114.44dB. The MCL for two layers of transmission is 114.59. For the case of 0.2 Mbps data, rates the MCL 127.45 assuming CP-OFDM

Observation 5: With the assumption of B=capacity UEs/cell, the 5%-tile coupling loss obtained from considering satisfied UEs only compared to the 5%-tile coupling loss obtained from considering all UEs

- 2 dB difference for DU scenario

- almost equal for UMa

Regarding obtaining coupling loss CDF for uplink channel, it is not clear what the value of B should be which is similar problem to that discussed in the downlink. Moreover, in case the value of B to be considered is equal to the capacity, it is further not clear whether the UL and DL joint capacity should be considered or not.

Observation 6: For uplink channel coupling loss/gain results, it is not clear what the value of B should be. In the case of B=capacity, it is further not clear whether capacity is obtained from joint UL and DL simulations or based on uplink simulations only.

Observation 7: The assumption of having 1 UE per cell may not provide meaningful results as it is equivalent to a simple CDF of coupling loss.

Observation 8: With the assumption of having number of UEs per cell > 1 and specifically equal to capacity, interference affects the system capacity and thus it is not clear how one may analyze the effects of coverage separately

Observation 9: Capacity results have not been aligned between companies thus the use of B=capacity may be problematic.

Observation 10: The system level approach does not provide information on which channel or data stream is the bottleneck for coverage.

Observation 11: Progress should not be pursued at cost of having accurate results. Moreover, resolving the issues mentioned above may cause a delay in SI progress as a lot of studies needs to be done for proving accuracy.

Observation 12: Link-level based coverage analysis is mature and more accurate and does not have the issues listed above as has been followed by CE and RedCap SIs in Rel-17.

Observation 13: The group may discuss the suitable parameters for link-level based coverage evaluations:

- Size of packet: maximum size frame size according to the adopted statistical model

- Number of Retransmission with the agreed TDD configuration and PDB

- Receiver interference density to model the interference aspect

Based on the above analysis and observations, we have the following proposal:

Proposal 1: The link-level based evaluation methodology is adopted as evaluation methodology for XR coverage analysis.

# References

3GPP RAN1, RAN1#105-e, Chairman Notes.

3GPP RAN1, RAN1#104-e, Chairman Notes.

3GPP RAN1, RAN1#104-bis-e, Chairman Notes.

3GPP RAN1, RAN1#102-e, Chairman Notes

R1-2107088, XR Initial Performance Evaluations

# Appendix

PDSCH link budget for one layer

The link level budget for the case of two layers transmission for PDSCH with SNR required is obtained by using Coverage Enhancement link budget analysis in Rel-17

| System configuration | Ref UE (NLoS O-to-I) |
| --- | --- |
| Carrier frequency (GHz) | 4.00 |
| Total carrier bandwidth (MHz) | 100.00 |
| Transmission bit rate for control channel (bit/s) | - |
| Transmission bit rate for data channel (bit/s) | 30000000.00 |
| Target packet error rate for the required SNR in item (19a) for control channel | - |
| Target packet error rate for the required SNR in item (19b) for data channel | 10^-3 |
| Pathloss model | TDL-C, NLOS |
| UE speed (km/h) | 3.00 |
| Transmitter |  |
| (1) Number of transmit antennas. | 32.00 |
| (2a) # of gNB TXRUs | 32.00 |
| (2b) Number of transmit chains | 32.00 |
| (3a) Downlink Power Spectrum Density (dBm/MHz) | 36.00 |
| (3b) Total transmit power for carrier bandwidth (dBm) | 56.00 |
| (3bis) Transmit power for occupied channel bandwidth for control channel (17a) or data channel (17b) | 55.96 |
| (4) Transmitter antenna gain (dB) at antenna gain component 3 & antenna gain component 4 = (4a) +10*log10( (1) / (2a) ) - (4b) (dB) for downlink, and = (4a) +10*log10( (1) / (2b) ) - (4b) (dB) for uplink | 8.00 |
| (4a) Transmitter antenna element gain (dBi) | 8.00 |
| (4b) Transmitter antenna gain correction factor at antenna gain component 3 & antenna gain component 4 (dB) Note: delta2 for downlink and delta3 for uplink | 0.00 |
| (5) Transmitter antenna gain (dB) at antenna gain component 2 Note: void (=zero) for uplink | 0.00 |
| (6) Control channel power boosting gain (dB) | 0.00 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.00 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 3.00 |
| (9a) Control channel EIRP = (3bis) + (4) + (5) + (6) – (8) dBm | - |
| (9b) Data channel EIRP = (3bis) + (4) + (5) – (7) – (8) dBm | 60.96 |
| Receiver |  |
| (10) Number of receive antennas | 4.00 |
| (10bis) Number of receive chains | 4.00 |
| (11) Receiver antenna gain (dB) at antenna gain component 3 & antenna gain component 4 = (11a) +10*log10( (10) / (10bis) ) - (11b) (dB) for downlink; and = (11a) +10*log10( (10) / (2a) ) - (11b) (dB) for uplink | 0.00 |
| (11a) Receiver antenna element gain (dBi) | 0.00 |
| (11b) Receiver antena gain correction factor at antenna gain component 3 & antenna gain component 4 (dB) Note: delta2 for uplink, and delta3 for downlink | 0.00 |
| (11bis) Receiver antenna gain (dB) at antenna gain component 2 Note: void (=zero) for downlink | 0.00 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 1.00 |
| (13) Receiver noise figure (dB) | 9.00 |
| (14) Thermal noise density (dBm/Hz) | -174.00 |
| (15a) Receiver interference density for control channel (dBm/Hz) | - |
| (15b) Receiver interference density for data channel (dBm/Hz) | -999.00 |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz | - |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz | -165.00 |
| (17a) Occupied channel bandwidth for control channel (Hz) | - |
| (17b) Occupied channel bandwidth for data channel (Hz) | 99000000.00 |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm | - |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm | -85.04 |
| (19a) Required SNR for the control channel (dB) | - |
| (19b) Required SNR for the data channel (dB) | -3.37dB |
| (20) Receiver implementation margin (dB) | 2.00 |
| (21a) H-ARQ gain for control channel (dB) Note: Only applicable if HARQ is not considered in LLS | - |
| (21b) H-ARQ gain for data channel (dB) Note: Only applicable if HARQ is not considered in LLS | 0.00 |
| (22a) Receiver sensitivity for control channel = (18a) + (19a) + (20) – (21a) dBm | - |
| (22b) Receiver sensitivity for data channel = (18b) + (19b) + (20) – (21b) dBm | -87.84 |
| (23a) Hardware link budget for control channel (MIL) = (9a) + (11) + (11bis) − (12) − (22a) dB | - |
| (23b) Hardware link budget for data channel (MIL) = (9b) + (11) + (11bis) − (12) − (22b) dB | 147.80 |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.00 |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | - |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 |
| (26) BS selection/macro-diversity gain (dB) | 0.00 |
| (27) Penetration margin (dB) | 26.25 |
| (28) Other gains (dB) (if any please specify) | 0.00 |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) dB | - |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) dB | 117.07 |
| (40a) MCL for control channel = (3bis) + (6) − (22a) + (5) + (11bis) dB | - |
| (40b) MCL for data channel = (3bis) − (7) − (22b) + (5) + (11bis) dB | -142.37 |

PUSCH link budget for one layer

| System configuration | Ref UE (NLoS O-to-I) |
| --- | --- |
| Carrier frequency (GHz) | 4.00 |
| Total carrier bandwidth (MHz) | 100.00 |
| Transmission bit rate for control channel (bit/s) | - |
| Transmission bit rate for data channel (bit/s) | 10000000.00 |
| Target packet error rate for the required SNR in item (19a) for control channel | - |
| Target packet error rate for the required SNR in item (19b) for data channel | 10^-3% |
| Pathloss model | TDL-C, NLOS |
| UE speed (km/h) | 3.00 |
| Transmitter |  |
| (1) Number of transmit antennas. | 4.00 |
| (2a) # of gNB TXRUs | 32.00 |
| (2b) Number of transmit chains | 4.00 |
| (3a) Downlink Power Spectrum Density (dBm/MHz) | - |
| (3b) Total transmit power for carrier bandwidth (dBm) | 23.00 |
| (3bis) Transmit power for occupied channel bandwidth for control channel (17a) or data channel (17b) | 23.00 |
| (4) Transmitter antenna gain (dB) at antenna gain component 3 & antenna gain component 4 = (4a) +10*log10( (1) / (2a) ) - (4b) (dB) for downlink, and = (4a) +10*log10( (1) / (2b) ) - (4b) (dB) for uplink | 0.00 |
| (4a) Transmitter antenna element gain (dBi) | 0.00 |
| (4b) Transmitter antenna gain correction factor at antenna gain component 3 & antenna gain component 4 (dB) Note: delta2 for downlink and delta3 for uplink | 0.00 |
| (5) Transmitter antenna gain (dB) at antenna gain component 2 Note: void (=zero) for uplink | 0.00 |
| (6) Control channel power boosting gain (dB) | 0.00 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0.00 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1.00 |
| (9a) Control channel EIRP = (3bis) + (4) + (5) + (6) – (8) dBm | - |
| (9b) Data channel EIRP = (3bis) + (4) + (5) – (7) – (8) dBm | 22.00 |
| Receiver |  |
| (10) Number of receive antennas | 32.00 |
| (10bis) Number of receive chains | 32.00 |
| (11) Receiver antenna gain (dB) at antenna gain component 3 & antenna gain component 4 = (11a) +10*log10( (10) / (10bis) ) - (11b) (dB) for downlink; and = (11a) +10*log10( (10) / (2a) ) - (11b) (dB) for uplink | 8.00 |
| (11a) Receiver antenna element gain (dBi) | 8.00 |
| (11b) Receiver antena gain correction factor at antenna gain component 3 & antenna gain component 4 (dB) Note: delta2 for uplink, and delta3 for downlink | 0.00 |
| (11bis) Receiver antenna gain (dB) at antenna gain component 2 Note: void (=zero) for downlink | 0.00 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3.00 |
| (13) Receiver noise figure (dB) | 5.00 |
| (14) Thermal noise density (dBm/Hz) | -174.00 |
| (15a) Receiver interference density for control channel (dBm/Hz) | - |
| (15b) Receiver interference density for data channel (dBm/Hz) | -999.00 |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz | - |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz | -169.00 |
| (17a) Occupied channel bandwidth for control channel (Hz) | - |
| (17b) Occupied channel bandwidth for data channel (Hz) | 27000000.00 |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm | - |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm | -94.69 |
| (19a) Required SNR for the control channel (dB) | - |
| (19b) Required SNR for the data channel (dB) | 1.25 |
| (20) Receiver implementation margin (dB) | 2.00 |
| (21a) H-ARQ gain for control channel (dB) Note: Only applicable if HARQ is not considered in LLS | - |
| (21b) H-ARQ gain for data channel (dB) Note: Only applicable if HARQ is not considered in LLS | 0.00 |
| (22a) Receiver sensitivity for control channel = (18a) + (19a) + (20) – (21a) dBm | - |
| (22b) Receiver sensitivity for data channel = (18b) + (19b) + (20) – (21b) dBm | -91.44 |
| (23a) Hardware link budget for control channel (MIL) = (9a) + (11) + (11bis) − (12) − (22a) dB | - |
| (23b) Hardware link budget for data channel (MIL) = (9b) + (11) + (11bis) − (12) − (22b) dB | 118.44 |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) | 7.00 |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | - |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 |
| (26) BS selection/macro-diversity gain (dB) | 0.00 |
| (27) Penetration margin (dB) | 26.25 |
| (28) Other gains (dB) (if any please specify) | 0.00 |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) dB | - |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) dB | 87.71 |
| (40a) MCL for control channel = (3bis) + (6) − (22a) + (5) + (11bis) dB | - |
| (40b) MCL for data channel = (3bis) − (7) − (22b) + (5) + (11bis) dB | 114.44 |