3GPP TSG RAN WG1 #106-e		R1-2107118

e-Meeting, August 16th – 27th, 2021

Source:	Panasonic

Title: 	Discussion on PUCCH enhancement for NR coverage enhancement

Agenda Item:		8.8.2

Document for:	Discussion

Introduction

A work items on NR coverage enhancement was approved [1]. One of objectives of this work item is PUCCH enhancements such as

- Specify signalling mechanism to support dynamic PUCCH repetition factor indication

- Specify mechanism to support DMRS bundling across PUCCH repetitions

This document provides our view on PUCCH enhancement.

Discussion

Dynamic PUCCH repetition factor indication

In RAN1#105e, following working assumption was made.

Working assumption: (RAN1#105e)

- In Rel.17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

  - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel.16 PUCCH resource indication mechanism based on “PUCCH resource indicator (PRI) field and starting CCE index (when applicable based on Rel.16 specification) of DCI to indicate a PUCCH resource and its associated repetition factor.

    - FFS: RRC signaling enhancement details

One of remaining issues is RRC signaling enhancement details. The IE PUCCH-Config is used to configure UE-specific PUCCH parameters. PUCCH-Config information element in Rel.16 is described in Appendix A. In Rel.16, repetition factor (nrofSlots) is included in PUCCH-FormatConfig field, which are parameters common for all PUCCH resources of each PUCCH format. In order to enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource, potential enhancement would be to include repetition factor in PUCCH-Resource field as follows.

PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, intraSlotFrequencyHopping ENUMERATED { enabled } OPTIONAL, -- Need R secondHopPRB PRB-Id OPTIONAL, -- Need R nrofSlots-r17 ENUMERATED {n2,n4,n8} format CHOICE { format0 PUCCH-format0, format1 PUCCH-format1, format2 PUCCH-format2, format3 PUCCH-format3, format4 PUCCH-format4 } }

In Rel.17 IIoT/URLLC WI, there is following agreements.

Agreement:

- Support sub-slot-based PUCCH repetition for HARQ-ACK based on the Rel.16 PUCCH procedure for slot-based PUCCH applied to sub-slot-based PUCCH.

  - Note: The intention is to take the Rel.16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary.

  - FFS whether or not there is any restriction for the applicability of sub-slot-based PUCCH repetition for HARQ-ACK

  - Dynamic repetition indication is supported also for sub-slot-based PUCCH in Rel.17.

    - FFS: If the method to be specified in CovEnh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed.

Agreement:

- Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot-based PUCCH repetition.

  - FFS: Support for slot-based PUCCH repetition

According to above agreements, dynamic repetition factor indication is also applied to short PUCCH formats. Therefore, the parameter for the dynamic repetition factor indication (e.g., nrofSlots-r17) is needed for all PUCCH formats. If the value range of the repetition factor is same among PUCCH formats, e.g., {2, 4, 8}, including repetition factor for Rel.17 (nrofSlots-r17) in PUCCH-Resource field is sufficient. If the value range of the repetition factor is different among PUCCH formats, e.g., {2, 4, 8} for PUCCH format 1/3/4, and {2, 3, 4, 7} for PUCCH format 0/2, repetition factor for Rel.17 (nrofSlots-r17) should be included in formatX description as follows.

PUCCH-format3 ::= SEQUENCE { nrofPRBs INTEGER (1..16), nrofSymbols INTEGER (4..14), startingSymbolIndex INTEGER(0..10) nrofSlots-r17 ENUMERATED {n2,n4,n8} }

In our view, in spite of the actual performance difference among PUCCH formats, different UCCH formats without repetitions are used in normal coverage (i.e., no coverage enhancement) conditions. For coverage enhancement conditions, the similar amount of the repetition can be applied regardless of PUCCH formats. Therefore, common range for repetition factor can be sufficient.

Proposal 1: Common parameter (e.g., nrofSlots-r17) among PUCCH formats is introduced for dynamic repetition factor indication.

Observation 1: Repetition factor is included in PUCCH-Resource description. The detailed RRC structure is up to RAN2.

On dynamic repetition factor indication, there was discussion on whether dynamic PUCCH repetition factor indication should be applied to semi-static PUCCH. On whether dynamic PUCCH repetition factor indication is applied to P/SP-CSI, in our view, periodic is purely semi-static configuration, and therefore, to support dynamic indication is difficult. The repetition factor for periodic CSI would be set considering the maximum payload size of CSI reporting. For SP CSI, to introduce PRI like indication to activation DCI is one of possibility if the motivation to introduce dynamic indication is clarified.

On SPS HARQ-ACK, since SPS activation is via activation DCI, similar to SP CSI, if the motivation to introduce dynamic indication is clarified, to introduce PRI like indication to activation DCI is one of possibility.

On SR, as SR needs to be sent before the channel status may be known, we don’t see the need to have dynamic PUCCH repetition factor indication to SR.

Proposal 2: At least for periodic CSI and SR, there is no need to have dynamic PUCCH repetition factor indication.

Support DMRS bundling across PUCCH repetitions

A UE shall not change a transmission power over a period to allow a joint channel estimation at gNB. This condition shall be satisfied even if a change of pathloss measurements or the reception of TPC command. The period to keep the transmission power was agreed to call as time domain window in the context of joint channel estimation for PUSCH. The same mechanism to support DMRS bundling across repetitions discussed in PUSCH enhancement should be applied for PUCCH enhancement.

For dynamically scheduled PUCCH transmission, the length of time domain window needs to be known by UE. For RRC configured PUCCH transmission, the length of time domain window also needs to be known semi-statically by RRC. The length of time domain window could be determined implicitly by the repetition factor, hopping pattern, how long continuous transmission continues, or the UE capability. Alternatively, it may be signalled explicitly. In addition to PUCCH repetition factor indication, enabling or disabling DMRS bundling can be indicated as an additional parameter in the PUCCH resource set and PUCCH resource indicator field can be reused. If the length of time domain window needs to be explicit, it can be also indicated in PUCCH resource indicator field. In order to allow the flexibility, extending the PUCCH resource indicator field would be one possibility.

Proposal 3: PUCCH resource indicator field can be used to inform UE that enabling/disabling of DMRS bundling. Consider extending PUCCH resource indicator field for further flexibility. If the length of time domain window needs explicit indication, it also can be indicated in PUCCH resource indicator field.

A combination of inter-slot frequency hopping with inter-slot bundling for enabling joint channel estimation is also considered. In order to allow joint channel estimation, PRB position should be the same during at least the period of joint channel estimation. In eMTC. the period of joint channel estimation and the period of inter-slot frequency hopping are cell level configuration. However, in NR, it would be difficult to use cell level configuration as more flexibility would be required. To allow joint channel estimation, the time domain window should be used for the same frequency allocation in inter-slot frequency hopping procedure. To keep a reasonable flexibility, one or more lengths of time domain windows are configured to be jointly used with inter-slot frequency hopping, wherein each of the one or more lengths of time domain windows and length of inter-slot frequency hopping are the same or different. Within each of the one or more lengths of time domain windows, UE maintains the required conditions for enabling joint channel estimation. Above discussed the case of frequency hopping. The same can be applied for achieving spatial diversity, where precoder cycling is performed every cycling period.

Proposal 4: One or more lengths of time domain windows are configured to be jointly used with inter-slot frequency hopping / precoder cycling.

- Each of the one or more lengths of time domain windows is used for the same frequency allocation in inter-slot frequency hopping procedure.

- Each of the one or more lengths of time domain windows is used for the same precoder allocation in precoder cycling procedure.

Proposal 5: Each of the one or more lengths of time domain windows and a length of inter-slot frequency hopping/precoder cycling period can be the same or different.

Link-level simulation

We evaluated the BLER performance of PUCCH repetition. The detailed parameters for link level simulation are listed in the Appendix B.

Figure 1 shows the BLER performance of PUCCH repetition with and without inter-slot frequency hopping. Joint channel estimation with enabled and disabled inter-slot frequency hopping are evaluated. When the inter-slot frequency hopping is enabled, the length of time domain window for joint channel estimation and the period of inter-slot frequency hopping is same and set to a half of the number of repetitions. When the inter-slot frequency hopping is disabled, the length of time domain window for joint channel estimation is same as the number of repetitions. It was observed that the joint channel estimation with inter-slot frequency hopping can provide an improvement of gain of 4 ~5 dB, as compared with that of the joint channel estimation without inter-slot frequency hopping, even when the length of time domain window is reduced to a half compared with the case without hopping.

Observation 1: Joint channel estimation with inter-slot frequency hopping provides an improvement of gain of 4 ~5 dB, as compared with that of joint channel estimation without inter-slot frequency hopping and doubling a length of time domain window.

Figure 2 shows the BLER performance of PUCCH repetition with both joint channel estimation and inter-slot frequency hopping in condition of residual frequency offset. A uniform distribution with +/- 0.1 ppm is assumed for residual frequency offset/error. It was observed that in the condition of +/- 0.1 ppm residual frequency offset/error, the performance loss dues to residual frequency offset/error can be negligible at least when performing 2, 4, and 8 repetition with joint channel estimation and inter-slot frequency hopping.

Observation 2: The performance loss due to frequency offset/error of +/- 0.1 ppm can be negligible at least when performing 2, 4, and 8 repetitions with joint channel estimation and inter-slot frequency hopping.

(a) 11-bit UCI payload                                              (b) 22-bit UCI payload

Fig.1: Link-level simulation results for inter-slot frequency hopping with joint channel estimation

(a) 11-bit UCI payload                                              (b) 22-bit UCI payload

Fig.2: Impact of residual frequency error

Conclusion

In this contribution, we provide our view on PUCCH enhancement. We made following proposals and observations.

Proposal 1: Common parameter (e.g., nrofSlots-r17) among PUCCH formats is introduced for dynamic repetition factor indication.

Observation 1: Repetition factor is included in PUCCH-Resource description. The detailed RRC structure is up to RAN2.

Proposal 2: At least for periodic CSI and SR, there is no need to have dynamic PUCCH repetition factor indication.

Proposal 3: PUCCH resource indicator field can be used to inform UE that enabling/disabling of DMRS bundling. Consider extending PUCCH resource indicator field for further flexibility. If the length of time domain window needs explicit indication, it also can be indicated in PUCCH resource indicator field.

Proposal 4: One or more lengths of time domain windows are configured to be jointly used with inter-slot frequency hopping / precoder cycling.

- Each of the one or more lengths of time domain windows is used for the same frequency allocation in inter-slot frequency hopping procedure.

- Each of the one or more lengths of time domain windows is used for the same precoder allocation in precoder cycling procedure.

Proposal 5: Each of the one or more lengths of time domain windows and a length of inter-slot frequency hopping/precoder cycling period can be the same or different.

Observation 1: Joint channel estimation with inter-slot frequency hopping provides an improvement of gain of 4 ~5 dB, as compared with that of joint channel estimation without inter-slot frequency hopping and doubling a length of time domain window.

Observation 2: The performance loss due to frequency offset/error of +/- 0.1 ppm can be negligible at least when performing 2, 4, and 8 repetitions with joint channel estimation and inter-slot frequency hopping.

Reference

[1] RP-202928, “New WID on NR coverage enhancement,” China Telecom, RAN#90e, December 2020.

- Appendix A: PUCCH-Config information element in Rel.16

-- ASN1START -- TAG-PUCCH-CONFIG-START PUCCH-Config ::= SEQUENCE { resourceSetToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet OPTIONAL, -- Need N resourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource OPTIONAL, -- Need N resourceToReleaseList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId OPTIONAL, -- Need N format1 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format2 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format3 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format4 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig OPTIONAL, -- Need N schedulingRequestResourceToReleaseList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId OPTIONAL, -- Need N multi-CSI-PUCCH-ResourceList SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId OPTIONAL, -- Need M dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) OPTIONAL, -- Need M spatialRelationInfoToAddModList SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo OPTIONAL, -- Need N spatialRelationInfoToReleaseList SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId OPTIONAL, -- Need N pucch-PowerControl PUCCH-PowerControl OPTIONAL, -- Need M ..., [[ resourceToAddModListExt-r16 SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceExt-r16 OPTIONAL, -- Need N dl-DataToUL-ACK-r16 SetupRelease { DL-DataToUL-ACK-r16 } OPTIONAL, -- Need M ul-AccessConfigListDCI-1-1-r16 SetupRelease { UL-AccessConfigListDCI-1-1-r16 } OPTIONAL, -- Need M subslotLengthForPUCCH-r16 CHOICE { normalCP-r16 ENUMERATED {n2,n7}, extendedCP-r16 ENUMERATED {n2,n6} } OPTIONAL, -- Need R dl-DataToUL-ACK-DCI-1-2-r16 SetupRelease { DL-DataToUL-ACK-DCI-1-2-r16} OPTIONAL, -- Need M numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2-r16 INTEGER (0..3) OPTIONAL, -- Need R dmrs-UplinkTransformPrecodingPUCCH-r16 ENUMERATED {enabled} OPTIONAL, -- Cond PI2-BPSK spatialRelationInfoToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfo OPTIONAL, -- Need N spatialRelationInfoToReleaseListSizeExt-v1610 SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfoId OPTIONAL, -- Need N spatialRelationInfoToAddModListExt-v1610 SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoExt-r16 OPTIONAL, -- Need N spatialRelationInfoToReleaseListExt-v1610 SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoId-r16 OPTIONAL, -- Need N resourceGroupToAddModList-r16 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroup-r16 OPTIONAL, -- Need N resourceGroupToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroupId-r16 OPTIONAL, -- Need N sps-PUCCH-AN-List-r16 SetupRelease { SPS-PUCCH-AN-List-r16 } OPTIONAL, -- Need M schedulingRequestResourceToAddModListExt-v1610 SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1610 OPTIONAL -- Need N ]] } PUCCH-FormatConfig ::= SEQUENCE { interslotFrequencyHopping ENUMERATED {enabled} OPTIONAL, -- Need R additionalDMRS ENUMERATED {true} OPTIONAL, -- Need R maxCodeRate PUCCH-MaxCodeRate OPTIONAL, -- Need R nrofSlots ENUMERATED {n2,n4,n8} OPTIONAL, -- Need S pi2BPSK ENUMERATED {enabled} OPTIONAL, -- Need R simultaneousHARQ-ACK-CSI ENUMERATED {true} OPTIONAL -- Need R } PUCCH-MaxCodeRate ::= ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80} -- A set with one or more PUCCH resources PUCCH-ResourceSet ::= SEQUENCE { pucch-ResourceSetId PUCCH-ResourceSetId, resourceList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId, maxPayloadSize INTEGER (4..256) OPTIONAL -- Need R } PUCCH-ResourceSetId ::= INTEGER (0..maxNrofPUCCH-ResourceSets-1) PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, intraSlotFrequencyHopping ENUMERATED { enabled } OPTIONAL, -- Need R secondHopPRB PRB-Id OPTIONAL, -- Need R format CHOICE { format0 PUCCH-format0, format1 PUCCH-format1, format2 PUCCH-format2, format3 PUCCH-format3, format4 PUCCH-format4 } } PUCCH-ResourceExt-r16 ::= SEQUENCE { interlaceAllocation-r16 SEQUENCE { rb-SetIndex INTEGER (0..4), interlace0 CHOICE { scs15 INTEGER (0..9), scs30 INTEGER (0..4) } } OPTIONAL, --Need R formatExt-v1610 CHOICE { interlace1-v1610 INTEGER (0..9), occ-v1610 SEQUENCE { occ-Length-v1610 ENUMERATED {n2,n4} OPTIONAL, -- Need M occ-Index-v1610 ENUMERATED {n0,n1,n2,n3} OPTIONAL -- Need M } } OPTIONAL, -- Need R ... } PUCCH-ResourceId ::= INTEGER (0..maxNrofPUCCH-Resources-1) PUCCH-format0 ::= SEQUENCE { initialCyclicShift INTEGER(0..11), nrofSymbols INTEGER (1..2), startingSymbolIndex INTEGER(0..13) } PUCCH-format1 ::= SEQUENCE { initialCyclicShift INTEGER(0..11), nrofSymbols INTEGER (4..14), startingSymbolIndex INTEGER(0..10), timeDomainOCC INTEGER(0..6) } PUCCH-format2 ::= SEQUENCE { nrofPRBs INTEGER (1..16), nrofSymbols INTEGER (1..2), startingSymbolIndex INTEGER(0..13) } PUCCH-format3 ::= SEQUENCE { nrofPRBs INTEGER (1..16), nrofSymbols INTEGER (4..14), startingSymbolIndex INTEGER(0..10) } PUCCH-format4 ::= SEQUENCE { nrofSymbols INTEGER (4..14), occ-Length ENUMERATED {n2,n4}, occ-Index ENUMERATED {n0,n1,n2,n3}, startingSymbolIndex INTEGER(0..10) } PUCCH-ResourceGroup-r16 ::= SEQUENCE { pucch-ResourceGroupId-r16 PUCCH-ResourceGroupId-r16, resourcePerGroupList-r16 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerGroup-r16)) OF PUCCH-ResourceId } PUCCH-ResourceGroupId-r16 ::= INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16) DL-DataToUL-ACK-r16 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15) DL-DataToUL-ACK-DCI-1-2-r16 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) UL-AccessConfigListDCI-1-1-r16 ::= SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) -- TAG-PUCCH-CONFIG-STOP -- ASN1STOP

- Appendix A: Simulation parameters

| General simulation parameters |  |
| --- | --- |
| Scenario | FR1, Rural |
| Frequency | 700 MHz (FDD) |
| System bandwidth | 20 MHz |
| SCS | 15 kHz |
| gNB modelling in LLS | 2 gNB receive chains |
| Channel model | TDL-C for NLOS |
| Delay spread | 300 ns |
| UE velocity | 3 km/h |
| Residual frequency error | Ideal or 0.1 ppm (uniform distribution) |
| PUCCH evaluation parameters |  |
| Number of UE transmit chains | 1 |
| PUCCH format | PUCCH format 3 |
| PRBs | 1 PRB |
| UCI payload size | 11, 22 bits |
| Duration | 14 OFDM symbols |
| Intra-slot frequency hopping | Disabled |
| Inter-slot frequency hopping | Enabled or Disabled |
| DMRS configuration | 2 DMRS symbols per hop |
| Repetition | 2, 4, 8 repetitions |

- Appendix B: Agreements in previous meetings

RAN1#104e

Agreements:

- Down selection from the following two options to support dynamic PUCCH repetition factor indication.

  - Option 1 (without DCI enhancement): Enhance RRC signalling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

    - FFS: Details, e.g., via reusing the “PUCCH resource indicator” field (without increasing the number of bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc.

    - FFS: RRC signalling enhancement details

  - Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI.

    - E.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication

    - FFS: Whether there is a need for RRC update

Agreements:

- Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

  - FFS: Details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel.16

  - Strive for common design for PUSCH / PUCCH with DMRS bundling as much as possible

Conclusion:

- For the study of enhancing inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, at least the following aspects can be considered.

  - Performance trade-off between maximizing the number of consecutive UL slots in one frequency hop (to achieve more DMRS bundling gain) and maximizing the number of hops (to achieve more diversity gain).

    - Note: The maximum number of frequency hopping positions is still 2 as in Rel.15/16.

  - Interaction between hopping boundary determination and TDD configuration

Conclusion:

- For the simulations to study the enhancement of inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, simulation assumptions in TR38.830 are reused as a starting point.

- Note: Additional simulation scenarios / assumptions are not precluded.

Agreements:

- Subject to prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

  - FFS: The configuration is per UE or per PUCCH resource.

  - FFS: Whether additional dynamic signalling is needed to enable/disable PUCCH repetitions with DMRS bundling

  - FFS: Necessity of additional signalling / configuration of DMRS bundling duration / window and associated size

Conclusion:

- In Rel.17, deprioritize the study of DMRS pattern / location / granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of DMRS pattern / location / granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3.

RAN1#105e

Working assumption:

- In Rel.17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

  - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel.16 PUCCH resource indication mechanism based on “PUCCH resource indicator (PRI) field and starting CCE index (when applicable based on Rel.16 specification) of DCI to indicate a PUCCH resource and its associated repetition factor.

    - FFS: RRC signaling enhancement details

Conclusion:

- For PUCCH repetitions, the following use cases are deprioritized in RAN1 work on PUCCH DMRS bundling.

  - Use case 1: Back-to-back PUCCH repetitions within one slot

  - Use case 2: non-back-to-back PUCCH repetitions within one slot

    - Use case 2a: No uplink transmission in the middle of two PUCCH repetitions

    - Use case 2b: Other uplink transmissions in the middle of two PUCCH repetitions

Agreement:

- For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements.

  - Strive for common design of the time domain window for PUSCH / PUCCH with DMRS bundling as much as possible.