3GPP TSG RAN WG1 #106-e												  R1-2107147

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.1.3

Source:	NEC

Title:               Discussion on SRS enhancement

Document for:	Discussion and Decision

- Introduction

In RAN1#104b-e meeting [1], agreements for SRS enhancements were achieved as:

Agreement

A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. Adopt at least one of the following options for the reference slot.

- Opt. 1: Reference slot is the slot with the triggering DCI.

- Opt. 2: Reference slot is the slot indicated by the legacy triggering offset.

- FFS the detailed definition of “available slot” considering UE processing complexity and timeline to determine available slot, potential co-existence with collision handling, etc., e.g.,

  - Based on only RRC configuration, “available slot” is the slot satisfying: there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set

- FFS explicit or implicit indication of t

- FFS whether updating candidate triggering offsets in MAC CE may be beneficial

Agreement

For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values

- N_symbol = 8, R = {1, 2, 4, 8}

- N_symbol = 12, R = {1, 2, [3], 4, 6, 12}

- FFS the following configurations

  - N_symbol = 10, R = {1, 2, 5, 10}

  - N_symbol = 14, R = {1, 2, 7, 14}

- FFS options to reduce SRS BW for R>1

Agreement

On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values

- 1T6R: N_max = 3

- 1T8R: N_max = 4

- 2T6R: N_max = 3

- 2T8R: N_max = 4

- [4T8R: N_max = 2]

- The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS.

- FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed

Agreement

For RB-level partial frequency sounding (RPFS) in Rel-17

- The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1}

  - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern

- Support to determine PF and Noffset at least via RRC configuration per SRS resource.

  - FFS whether to introduce DCI and/or MAC CE in addition

Working Assumption

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Agreement

On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e

- Alt 1: All the non-zero integer values <= N_max are supported for N

- Alt 2: Support N=N_max only

- Alt 3: Support specific N values <= N_max

- FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot

- FFS: whether different alternatives may be selected for different xTyR configuration

Agreement

Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Agreement

- Up to 4 “t” values can be configured per SRS resource set.

Agreement

- For RPFS in Rel-17, support PF = {2, 4}.

- FFS  3, 8, 12, 16 or fractional numbers

- Support at least one of the following alternatives (to be decided in RAN1#105-e)

  - Alt 1: 1PFmSRS, BSRS is an integer value

  - Alt 2: 1PFmSRS, BSRS is an integer value with minimum value 4

  - Alt 3:  is a multiple of 4

  - Alt 4: Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2

Agreement

On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2

Agreement

For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e)

- Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence

- Alt 2: Truncate from legacy length-12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS

Agreement

For antenna switching, support one of the following

- Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS

- Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set

  - Note: the two SP-SRS resource sets are not activated at the same time.

- FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed

- FFS whether configurations on SRS repetitions have impact

- FFS relevant UE capability design

In this contribution, we provided our views on the enhancements on SRS.

- Discussion

Triggering offset

It was agreed that aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, and t is dynamically indicated via DCI for flexibility. One remaining issue for aperiodic SRS triggering is definition of the reference slot, which has been discussed for several meetings.

Regarding the flexibility, two options for the reference slot were listed as slot with the triggering DCI (Opt.1) or slot indicated by the legacy triggering offset (Opt.2). Considering the tradeoff between flexibility and overhead, reference slot to be slot indicated by the legacy triggering offset (Opt.2) is better. As legacy configured triggering offset has already considered the SRS triggering and scheduling requirement, the target to introduce more flexible triggering offset is to encounter dynamic changing of slot format and less opportunity for SRS triggering, so only adjusting the transmission by several slots is sufficient. So we propose that:

Proposal 1: For AP SRS triggering, support Opt.2 (i.e. Reference slot is the slot indicated by the legacy triggering offset).

Coverage/capacity enhancement

For SRS coverage and capacity enhancement, partial frequency sounding with reduced number of RBs was agreed, and regarding the use case, at least it can be applied to frequency hopping.

And considering non-frequency hopping, it should also be supported to increase coverage and capacity, otherwise the scheme is incomplete. While the structure for SRS sequence design should be studied considering PAPR. As property of ZC sequence with a single long sequence is broken, in this case, a separate sequence for each contiguous RBs can be applied.

Proposal 2: For partial frequency sounding with reduced number of RBs, it should be applied to both frequency hopping and non-frequency hopping. And sequence structure in case of non-frequency hopping can be studied considering PAPR issue.

Another remaining issue for partial frequency sounding is the value of 1PFmSRS, BSRS based on the partial factor, and four alternatives were listed in RAN1#104b-e.

Regarding Alt.3 (1PFmSRS, BSRS is a multiple of 4), in our opinion, this restriction will lead the feature of partial frequency sounding useless. As in current spec, value of SRS subband is a multiple of 4, and current frequency hopping framework already supports the reduction value of SRS transmission subband, example as shown in Table. 1.

Table. 1. Values of SRS subbands

| 0 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 2 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| 3 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |
| 4 | 16 | 1 | 8 | 2 | 4 | 2 | 4 | 1 |

As shown in Table. 1, if  = 3 or 4, the SRS bandwidth is 16, and based on Alt.3, if PF=2, then the partial subband is 8, which can be supported with configuration  = 4 and , similarly if PF=2, then the partial subband is 4, which can be supported with configuration of  = 3 and . And when  = 2, the SRS bandwidth is 12, based on Alt.3, partial frequency sounding is not supported. Based on this, there is no new thing based on partial frequency sounding. So we don’t think this alternative is a right way to go.

Regarding Alt 1 and Alt 2, which are similar that 1PFmSRS, BSRS can be an integer value, while the issue is multiplexing and capacity. For example, if the value of partial subband is 5 or 6, the nested framework of SRS is broken, and the new SRS can not be orthogonal with legacy UEs, which is not good for multiplexing and capacity enhancement.

Based on the discussion, we think Alt 4 (Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2) is a good method, which can support enhancements and also be friendly to legacy UEs.

Proposal 3: Regarding the value of 1PFmSRS, BSRS, support Alt 4 (Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2).

While not only the value of 1PFmSRS, BSRS, but also the starting position of the partial subband will impact the multiplexing between UEs. For example, if the SRS bandwidth is 12, and PF = 2. Based on current agreement, the starting position of the second partial subband within 12 RBs is Noffset=6, while the offset is not aligned with the unit of 4. This misalignment may cause non-orthogonal between non-partial sounding UEs, and also cause non-orthogonal between partial sounding UEs with different positions of 12 RBs for SRS bandwidth. As even the partial subband for two UEs are both 6RBs, but the different starting positions will cause partial overlapping of the partial subbands, while based on current framework, if there is only full overlapping or non-overlapping between subbands with same value, and multiplexing can be naturally supported.

Figure. 1. Misaligned starting position of partial subbands

To make full use of the partial frequency sounding, we think the starting position for the partial subband should still be aligned with a multiple of 4, otherwise, capacity will be reduced.

Proposal 4: The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs should be aligned with a multiple of 4.

Another issue is the maximum number of cyclic shift, in current spec, the value depends on the value of transmission comb (KTC). Actually, the value should depend on the length of the SRS sequence, and the interval between different cyclic shift should be integer number of samples, based on which orthogonality can be guaranteed. So we think the maximum number of cyclic shifts should be based on the SRS sequence length, for example, a function of KTC and PF. And for Comb-8, the SRS sequence length is a multiple of 6, so the maximum number of CSs should be 6.

Proposal 5: The maximum number of cyclic shifts should be based on the SRS sequence length, for example, a function of KTC and PF. And for Comb-8, the maximum number of CSs is 6.

In addition, regarding the sequence generation for partial frequency sounding SRS, ZC sequence should be maintained, otherwise PAPR property will be broken.

Proposal 6: For RPFS SRS in Rel-17, support Alt 1: Generate length -12PFmSRS, BSRSComb ZC sequence.

- Conclusion

In this contribution, we provided our views on SRS enhancement, and we proposed that:

Proposal 1: For AP SRS triggering, support Opt.2 (i.e. Reference slot is the slot indicated by the legacy triggering offset).

Proposal 2: For partial frequency sounding with reduced number of RBs, it should be applied to both frequency hopping and non-frequency hopping. And sequence structure in case of non-frequency hopping can be studied considering PAPR issue.

Proposal 3: Regarding the value of 1PFmSRS, BSRS, support Alt 4 (Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2).

Proposal 4: The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs should be aligned with a multiple of 4.

Proposal 5: The maximum number of cyclic shifts should be based on the SRS sequence length, for example, a function of KTC and PF. And for Comb-8, the maximum number of CSs is 6.

Proposal 6: For RPFS SRS in Rel-17, support Alt 1: Generate length -12PFmSRS, BSRSComb ZC sequence.

- References

- Chairman’s notes, RAN1#104b-e, April 12th – April 20th, 2021.