3GPP TSG RAN WG1 #106-e		              R1-2107163

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.11.1.1

Source:	Lenovo, Motorola Mobility

Title:	Sidelink resource allocation for power saving

Document for:	Discussion

# Introduction

During RAN1#105-e, following agreements and conclusions were archived:

Agreement: - For the set of Preserve values in periodic-based partial sensing, - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList. - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList - This is per mode 2 Tx resource pool (pre-)configuration - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx - FFS whether the monitoring can be mandatory Agreement: · In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources. o The processing time restriction includes Tproc,0SL and Tproc,1SL. o Aspects relating to sensing during SL DRX are to be discussed separately - Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking Agreement: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE Agreement: - For random resource selection, - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation. - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any) - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected. - FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions). - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.). Agreement: In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative - TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc) - FFS details - FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

In this contribution, we provide our views on the sidelink resource allocation procedure for power saving for vulnerable road user and commercial D2D.

# Discussion

## Contiguous-based partial sensing

In NR V2X, the traffic type includes both periodic traffic and aperiodic traffic. Since the transmission pattern of the aperiodic traffic is not predictable, it may not be sensed by UE. Therefore, it is very likely for the aperiodic transmission to collide with resource selected based on sensing for V2X sidelink communication, particularly for partial sensing based P2X related sidelink communication.

The period of NR periodic traffic can one of {1:99 ms, 100 ms, 200 ms, 300 ms, 400 ms, 500 ms, 600 ms, 700ms, 800ms, 900 ms, 1000 ms}. The LTE partial sensing is designed based on smallest period of 100ms (for 20ms/50ms period the resource reservation is scaled). Due to much more short resource reservation period and aperiodic traffic in NR, the probability of resource collision between  aperiodic and periodic traffic will be increased if LTE scheme is adopted. To reduce the probability of resource collision one additional sensing occasion needs to be introduced beside the periodic partial sensing window. The additional sensing range is based on the selection of Y slots as defined in LTE partial sensing.

n

T0

sensing window

Tproc,0

T2, T2min≤T2≤remaining PDB

Y slots

selection window

Proposal 1: The legacy partial sensing mechanism needs to be enhanced for both periodic traffic and aperiodic traffic.

Besides sensing mechanism, resource re-evaluation was introduced in NR V2X. For the power sensitive UE which performs partial sensing, whether re-evaluates the selected resource needs be further discussed.

Option 1: the selected resource is based on 	Contiguous-based partial sensing/short-term sensing and it can avoid the collision with aperiodic traffic to some extent but may be difficult to avoid the long-term periodic resource reservation.

Option 2: the selected resource is based on long-term partial sensing and it can avoid the collision with periodic traffic, but it may be further re-evaluated to avoid the collision with aperiodic traffic same as the resource re-evaluation motivation in NR V2X.

Proposal 2: Define a new short-term sensing window and/or reuse/enhance resource re-evaluation mechanism for power sensitive UE.

When re-evaluating the selected resource for partial sensing UE, the resource re-evaluation window should consider the following cases:

Case 1: The resource selection is triggered (at time slot n) after the partial sensing window boundary as shown in figure 2-1. The resource re-evaluation mechanism can be same as existing resource re-evaluation mechanism. And the resource re-evaluation sensing window is started after the resource selection timeslot n, until resource re-evaluation occurs at time slot m-T3.

Figure 2-1, resource selection is triggered after the partial sensing window boundary

Case 2: The resource selection is triggered (at time slot n) before the partial sensing window boundary as shown in figure 2-2. In this case, Rx UE does not perform reception between the resource selection timeslot n and partial sensing window boundary. That is, the resource re-evaluation window may be started from partial sensing window boundary, until resource re-evaluation occurs at time slot m-T3. The resource re-evaluation performance is decreased due to less re-evaluation duration. Hence, whether the performance degradation is acceptable or advances the starting point of resource re-evaluation window to time slot n, can be considered.

Figure 2-2, resource selection is triggered before the partial sensing window boundary

Proposal 3: Consider resource re-evaluation performance for partial sensing UE.

## Impact of sidelink DRX on resource allocation

Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - This work should consider the impact of sidelink DRX, if any. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

As mentioned in WID scope, DRX mechanism will be introduced for NR sidelink, and it can be designed not only for communication between vehicle UE and pedestrian UE but also for the communication among multiple commercial UEs. DRX configuration and alignment mechanism will be studied by RAN2, but from RAN1 perspective, there may have some potential impacts on RAN1.

For example, in mode 1, sidelink Tx UE follows the scheduling from network. RAN1 needs to evaluate whether existing time gap indication in DCI can ensure that the scheduled sidelink is transmitted in DRX on-duration of Rx UE and the time gap can be set according to the DRX on-duration. If the Mode 1 grant from gNB is not inside the active duration of Rx UE then Tx UE may skip the grant otherwise use the grant for transmission to any Rx UEs whose active period is within the grant, other improvements such as gNB to be aware of the sidelink DRX on-duration to prepare the mode 1 grant may upto consideration in RAN2. Also, in mode 2, the transmission resource is based on Tx UE autonomous resource selection. RAN1 needs to consider whether the resource selection/re-evaluation window should further determine based on the DRX on-duration of Rx UE, i.e., the sidelink transmission on selected resources should be transmitted within the DRX on-duration of Rx UE. For example, input to the sensing and resource selection procedure may require DRX on-duration period of the Rx UE. Resource (re)selection is triggered when UE is not able to find any candidate resource for sidelink transmission within the DRX on-duration. One issue might arise when a transmitter UE configured to perform contiguous partial sensing with TA negative value when its relationship with that of DRX on-duration timer is not clear. Hence, RAN1 needs to discuss this issue of contiguous partial sensing with TA negative value when a DRX on-duration of the transmitter UE is not perfectly aligned with that of n+ TA, when TA  is configured with a negative value.

Proposal 4: Consider the potential impacts on RAN1 when DRX is introduced. For example, scheduling time gap and resource selection/re-evaluation.

Proposal 5: RAN1 further discuss the issue of DRX on-duration alignment problem arising due to contiguous partial sensing with negative TA negative value.

For the sake of power saving, Rel-14 LTE sidelink partial sensing is introduced as baseline to Rel-17 NR sidelink resource allocation in mode 2. Since SL DRX is also designed for UE aiming at saving power, it is possible for both partial sensing and SL DRX to be configured for one UE simultaneously. Generally, DRX is a mechanism in which UE gets into sleep mode for a certain period of time (i.e. DRX OFF duration) and wakes up for another period of time (i.e. DRX ON duration). In case of partial sensing, UE performs sensing within time duration defined by partial sensing window. On one hand, it leads to ambiguity for UE behavior when DRX OFF duration and partial sensing window overlap, or partially overlap, with each other. On the other hand, misaligned DRX ON duration and partial sensing window will extend sensing duration to UE and thus increase power consumption. To this end, a coordination is needed in order to provide UE procedure without ambiguity as well as to guarantee power saving for UE.

Observation: For mode 2, partial sensing configuration and DRX on-duration for a Tx UE must be aligned as much as possible to capture all reservations for each given resource reservation.

In LTE-V Rel-15 partial sensing configuration contain bitmap of sensing slots and with the introduction of SL DRX in NR SL Rel-17, the relation between partial sensing configuration defined in LTE-V Rel 15 and SL-DRX needs to be clarified. There could be two possible ways to handle relation between partial sensing configuration and SL-DRX configuration: In the first option, partial sensing configuration contains multiple DRX cycle configuration, where each DRX configuration corresponds to each resource reservation periodicity configured in the resource pool so that UE performs sensing by monitoring and decoding SCI according to the multiple DRX configuration. While in the second option, UE can perform sensing only during their configured DRX active period and the partial sensing configuration is not provided for this case. If a UE requires more sensing duration, then a UE can also be configured with a separate DRX cycle configuration especially for the sensing purpose according to one or more reservation period values.

Proposal 6: Additional DRX configuration can be configured for Tx UE performing periodic partial sensing considering multiple resource reservation periods

Power saving benefit of entering early DRX sleep within the DRX on-duration/active period when there is not enough data for transmission/reception should be investigated through any signaling enhancement in SCI. As an example, Tx UE may transmit no more data to be transmitted to a destination using go-to-sleep indication to Rx UEs as an assistance message to make the Rx UEs enter the DRX Sleep state earlier.

Proposal 7: RAN1 study on the transmission of assistance indication like go-to-sleep to aid Rx UE(s) enter early DRX sleep state.

Cross slot scheduling enhancement in the sidelink can be introduced for power saving purpose by indicating the time slot offset between 2nd SCI and data from that of 1st SCI. Sidelink UE does not need to decode as well as buffer the 2nd SCI and data if it is not the intended recipient of the data. As an example,

an information present in the 1st SCI provide information on the intended recipient of the message like P2P , P2V, V2P, P2V and accordingly Rx UE decide to buffer and decode the 2nd SCI and data.

Proposal 8: RAN1 study the cross-slot scheduling enhancement with a time gap specified between data(+2nd SCI) and 1st SCI, 1st SCI contains information whether the intended recipient is a pedestrian or Vehicular UEs for power saving purposes

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

RAN2 is currently discussing on how DRX cycle can be (pre)configured to UEs which in one option shall be based on the PQI value. When many UEs tries to transmit in an active period of a DRX cycle results in congestion mechanism for that active period. Hence the congestion mechanism introduced in NR Rel-16 should be modified to detect the congestion mechanism in each DRX cycle active period. The relation between CBR measurement window and DRX can be defined in two ways: in the first option the CBR measurement window can be aligned with each of the DRX cycle and in the second option, CBR measurement window can include active periods from more than one DRX cycle. The first option can detect the congestion in each active period of a DRX cycle accurately. Similar mechanism should be adopted for reporting CR.  Congestion control mechanism of restricting PSSCH/PSCCH TX parameters is performed per active period of a DRX cycle. Finally, reconfiguration of sidelink DRX offset, on-duration for UEs in an active period of a DRX cycle are also possible based on the CBR/CR measurement.

Proposal 9: Relation between CBR measurement window and active period of a DRX cycle should be discussed and detection of congestion for each active period shall be defined.

Proposal 10: Restriction of transmission parameter based on the CBR measurement is performed per active period of a DRX cycle.

## Other improvements on resource allocation

For short term sensing and partial sensing-based resource re-evaluation, the time length of sensing window is limited. So the received resource reservation indication information in limited reception duration may be not long enough to detect resource reservation with various period. So to design additional resource reservation indication/signaling, which is transmitted in the limited reception duration, can be considered including its time/frequency resource location or physical channel.

Proposal 11: Design additional resource reservation indication/signaling for collision avoidance.

Resource selection based on partial sensing facilitates power saving for vulnerable road users (VRU) by allowing for monitoring only in a subset of resources. Since Rel-17 SL communication is additionally supposed to support reception function for VRU, it is desired for time duration of transmission/reception of the VRU to be known by other UE. From signaling overhead perspective, configuring an identical subset of resources, referred to as communication window, for both transmission and reception is needed for information sharing purpose.

Furthermore, if communication window is left for UE implementation, it would be difficult for communication windows selected by different pairs of UEs in communication to be aligned. Consequently, any two pairs of UEs would probably suffer collision of resource reserved in overlapping window from each other due to misalignment of resources for sending resource reservation. That is to say, an alignment among communication windows would achieve a good compromise between energy consumption and resource collision avoidance. To this end, a mechanism on how to define communication window for alignment purpose is provided as follows.

Define a resource pool partition, which provides information on how to partition a resource pool. From signaling overhead perspective, an efficient way is to divide a resource pool into a disjoint set of resource patterns, as illustrated by Figure 2-3. Each resource pattern, denoted by PATTi, contains components periodically allocated within the resource pool period. Each component contains a set of contiguous resources in the time domain, measured by Lengthi sidelink slots. Each resource pattern can be further configured with features about controlling selection opportunities for different type of services and thus facilitate resource collision avoidance. A communication window can be configured to comprise one or more resource patterns.

Figure 2-3 An example of resource pool partition

A resource pool partition can be configured per resource pool and contained in the resource pool configuration. Wherein a parameter of identity can be defined to indicate each resource pattern among multiple resource patterns within the resource pool partition.

For a resource pool selected for use by a UE, the UE can further determine one or more resource patterns as communication window for power saving purpose. The (re-)selection of a resource pattern can be triggered by following conditions: (1) A VUR is configured to transmit and/or receive P2X related SL communication. (2) A non-power sensitive VUE is configured to transmit P2X related SL communication (3) A UE re-selects a resource pattern when the resource pattern in use cannot satisfy transmission requirements. Resource pattern (re-)selection can be based on sensing results. Detailed method on how to (re-)select resource pattern(s) is left for UE implementation.

Proposal 12: Support SL Tx/Rx performed in a power saving manner by configuring a resource pool partition for resource alignment among multiple UEs.

- A resource pool partition is configured by a set of disjoint resource patterns.

- Each resource pattern can be configured with features about controlling selection opportunities for different type of services and thus facilitating resource avoidance.

- For a resource pool selected for use, a UE can further (re-)select resource pattern(s) based on sensing results.

- Resource alignment can be performed by indicating identity of resource pattern among UEs.

Further, cooperative feature among UEs can be exploited to facilitate resource allocation based on partial sensing while guaranteeing power saving for UE. The cooperation among UEs can be done by sharing of sensing results from other UE (such as V-UE) to VRU.  Thus, a set of rules and configurations are needed in order to enable sharing of sensing results among UEs.

RSU can assist VRU with the resource selection mechanism by providing feedback on the resource occupancy status in a resource pool or subset of candidate resource in a resource pool for transmission thereby minimizing the effort of VRU on sensing procedure.

Proposal 13: Mechanism of sensing result sharing by RSU or other UE can be considered for VRU to achieve power saving.

# Conclusion

In this contribution, we discuss the resource allocation mechanisms for power saving. We have the following proposals:

Proposal 1: The legacy partial sensing mechanism needs to be enhanced for both periodic traffic and aperiodic traffic.

Proposal 2: Define a new short-term sensing window and/or reuse/enhance resource re-evaluation mechanism for power sensitive UE.

Proposal 3: Consider resource re-evaluation performance for partial sensing UE.

Proposal 4: Consider the potential impacts on RAN1 when DRX is introduced. For example, scheduling time gap and resource selection/re-evaluation.

Proposal 5: RAN1 further discuss the issue of DRX on-duration alignment problem arising due to contiguous partial sensing with negative TA negative value.

Proposal 6:  Additional DRX configuration can be configured for Tx UE performing periodic partial sensing considering multiple resource reservation periods.

Proposal 7: RAN1 study on the transmission of assistance indication like go-to-sleep to aid Rx UE(s) enter early DRX sleep state.

Proposal 8: RAN1 study the cross-slot scheduling enhancement with a time gap specified between data(+2nd SCI) and 1st SCI, 1st SCI contains information whether the intended recipient is a pedestrian or Vehicular UEs for power saving purposes.

Proposal 9: Relation between CBR measurement window and active period of a DRX cycle should be discussed and detection of congestion for each active period shall be defined.

Proposal 10: Restriction of transmission parameter based on the CBR measurement is performed per active period of a DRX cycle.

Proposal 11: Design additional resource reservation indication/signaling for collision avoidance.

Proposal 12: Support SL Tx/Rx performed in a power saving manner by configuring a resource pool partition for resource alignment among multiple UEs.

- A resource pool partition is configured by a set of disjoint resource patterns.

- Each resource pattern can be configured with features about controlling selection opportunities for different type of services and thus facilitating resource avoidance.

- For a resource pool selected for use, a UE can further (re-)select resource pattern(s) based on sensing results.

- Resource alignment can be performed by indicating identity of resource pattern among UEs.

Proposal 13: Mechanism of sensing result sharing by RSU or other UE can be considered for VRU to achieve power saving.

# References

- RP-201385, WID revision: NR sidelink enhancement