3GPP TSG RAN WG1 #106-e			                        R1-2107170

e-Meeting, August 16th – 27th, 2021

Source:	CAICT

Title:	Discussion on other enhancements for on-demand PRS and INACTIVE mode positioning

Agenda Item:	8.5.6

Document for:	Discussion and Decision

- Introduction

In RAN1#105 meeting, NR Positioning Enhancements for DL-AoD are discussed, and the following agreements are made:

- For potential signaling of one or more parameters (such signaling is not yet agreed) for both UE- and LMF- initiated on-demand DL PRS request, consider at least the following (all parameters will not necessarily be supported and all parameters may not be applicable to both UE-initiated and LMF-initiated on-demand DL PRS request)

- Start/end time of DL PRS transmission

- DL PRS resource bandwidth

- DL-PRS resource set IDs

- DL PRS resource IDs

- DL PRS transmission periodicity and offset

- DL PRS resource repetition factor

- Number of DL PRS symbols per DL PRS resource

- DL PRS muting patterns

- DL PRS QCL information

- Number of TRPs

- Number of PRS resources per PRS resource set

- Number frequency layers or frequency layer indicator

- Beam directions

- Combsize, start PRB, Point A of DL PRS

- ON/OFF indicator of the DL PRS

- FFS additional parameters indicated for UE and/or LMF initiated on-demand DL PRS request

In this contribution, we share our views on other enhancements for on-demand PRS and INACTIVE mode positioning.

- Discussions

  - INACTIVE mode positioning

Without supporting RRC_IDLE/ RRC_INACTIVE mode positioning in Rel-16, UEs in RRC_IDLE/ RRC_INACTIVE mode have to first enter into RRC connected mode, which leads to increase lots of signaling overhead, additional latency and high power consumption.

For DL positioning, the DL PRS configuration for measurement can be obtained from the broadcast system information in Rel-16 specification, the UE based DL positioning can be supported. While to support the UE assisted DL positioning, the DL positioning measurement has to be reported to the LMF, to support UEs in RRC_INACTIVE state to transport the DL positioning measurement, the measurement report transmission can be specified in RAN2 UL small data transmission.

For UL positioning, the UL positioning reference signal configuration need to be informed to the UEs. For UEs in RRC_CONNECTED state, network need to provide the configuration information. While for UEs in RRC_INACTIVE state, the configuration information can be informed to the UEs by a new paging message which transmitting the UL positioning reference signal configuration, or a new random access process to help UEs to obtain the configuration information of UL positioning reference signal configuration.

Proposal 1: For UL positioning in RRC_IDLE state, a new paging message or a new random access process need to be specified.

  - On-demand PRS enhancement

For on demand PRS enhancement, the multiple sets of configuration of DL PRS can be supported considering the different positioning QoS requirements. The multiple sets of configuration of DL PRS include multiple sets of PRS frequency bands, PRS time-frequency resources, participating positioning base station information and beam direction information configurations.

2.2.1. System information broadcast

The multiple sets of DL PRS configuration can be broadcasted in the system information, and for the UE initiated on demand DL PRS, the UEs could request the required DL PRS configuration considering on the positioning QoS requirements. the UEs could request the concrete set of DL PRS configuration through message 1 (MSG1) or message 3 (MSG3) in the random access process, the UE sends MSG1 according to the SIB1 system configuration to request DL PRS configuration or sends MSG3 based on a special preamble sequence to request the required DL PRS configuration. When the request is based on MSG1, the gNB receives MSG1, then sends MSG2 transmitting the random access response confirmation information to UEs, and then broadcast the UE requested DL PRS configuration. When the request is based on MSG3, the gNB sends MSG4 transmitting confirmation to the UEs, and then broadcast the UE requested DL PRS configuration. On the other hand, When no request of one or more sets of DL PRS configuration are transmitted by any UEs, the one or more sets of DL PRS configuration can be configured as OFF state, and are not transmitted to UEs by gNBs.

2.2.2. RRC signaling request

The multiple sets of DL PRS configuration can also be requested by RRC configuration, the procedure is shown in  Figure 1.

Step 1, the UE sends the DL PRS request to the LMF.

Step 2, the LMF sends the request of UE to the serving gNB and neighboring gNB.

Step 3, the serving gNB and the neighboring gNB send response to the LMF.

Step 4: the location server sends a response to the UE.

Step 5: the serving gNB and neighboring gNB send the on-demand DL PRS to the UE.

Figure 1. the procedure of RRC signalling to request DL PRS

Proposal 2: For on demand PRS, the multiple sets of configuration of DL PRS can be supported, and the potential signaling of parameters can include system information broadcasting or RRC signalling.

- Conclusion

In this contribution, we give following proposals of enhancements for on-demand PRS and INACTIVE mode positioning:

Proposal 1: For UL positioning in RRC_IDLE state, a new paging message or a new random access process need to be specified.

Proposal 2: For on demand PRS, the multiple sets of configuration of DL PRS can be supported, and the potential signaling of parameters can include system information broadcasting or RRC signalling.

References

- 3GPP RAN1 #105-e meeting chairman’s notes