3GPP TSG RAN WG1 Meeting #106-e	R1-2107182

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.7.1.1

Source: 	Lenovo, Motorola Mobility

Title: 	Paging enhancement for UE power saving

Document for:	Discussion and Decision

# Introduction

During RAN1 #105-e meeting, RAN1 made the following observations regarding Paging Early Indication (PEI) candidate designs [1]:

Agreement:

For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

Conclusion:

To down-select one solution for PEI physical-layer channel/signal in RAN1 #106-e, using below as a starting point:

- PDCCH-based PEI

- SSS-based PEI

- TRS/CSI-RS-based PEI

Note: Additional details for each of the above 3 solutions are encouraged for more informed down-selection

Note: further refinement of the above list is possible, e.g., by merging/further splitting, depending on significance of the commonality and/or differences

Agreement:

Observation:

Dynamically sharing PDCCH resources of Rel-15 UEs (whether or not this is an important aspect to consider for PEI is FFS)

  - For PDCCH-based PEI,

    - PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates

      - Exact number of multiplexed/impacted Rel-15 PDCCH candidates depends on AL used for PDCCH-based PEI and relative size of PDCCH CORESET, etc.

  - For SSS-based PEI and for the case of partial overlap of CORESET and PEI

    - For interleaved CORESET (such as CORESET#0), SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity

    - For non-interleaved CORESET, SSS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at granularity of one or more candidates

      - Exact number of impacted Rel-15 PDCCH candidates depends on relative size and location of PDCCH CORESET, etc.

  - For TRS/CSI-RS-based PEI and for the case of partial overlap of CORESET and PEI

    - For interleaved CORESET (such as CORESET#0), TRS/CSI-RS-based PEI can dynamically share resources with PDCCH for Rel-15 UEs only at CORESET-level granularity

    - For non-interleaved CORESET, TRS/CSI-RS-based can dynamically share resources with PDCCH for Rel-15 UEs within a PDCCH CORESET at candidate level granularity

      - Exact number of impacted Rel-15 PDCCH candidates depends on CSI-RS mapping pattern, relative size and location of PDCCH CORESET, etc.)

Agreement:

For paging indication to the subgroups in a PO,

- For PDCCH-based PEI, subgroups in a PO are indicated by one PEI

  - One bit in the DCI payload indicating one UE subgroup is supported

    - FFS: Whether code-point based mapping is utilized, and, if so, how to map to the subgroups in a PO

- For SSS-based PEI, subgroups in a PO are indicated by a set of sequence realizations

  - FFS: Sequence mapping design for supporting up to 8 subgroups per PO

  - Physical-layer configuration(s) and sequence generation design are subject to no impact to initial access and RRM measurements of legacy UEs

- For TRS/CSI-RS-based PEI, subgroups in a PO can be indicated by the following alternatives

  - Alt 1: One TRS sequence with orthogonal cover as PEI transmitted in the PEI monitoring occasion where one orthogonal cover of the PEI indicates one subgroup or combination of subgroups

    - FFS: Design details for the orthogonal cover

  - Alt 2: A set of TRS sequences indicating the subgroups with one selected sequence transmitting in one TRS resource

    - FFS: Sequence mapping design for supporting up to 8 subgroups per PO and combination of subgroups

  - Alt 3: Multiple TRS/CSI-RS resources FDMed/TDMed /CDMed in the same monitoring occasion where one TRS/CSI-RS resource indicates one subgroup

    - Reuse Rel-15/16 CSI-RS FDM/TDM/CDM patterns for supporting up to 8 subgroups per PO

- Note : It is RAN1 understanding that Physical-layer configuration(s) for paging early indication to the subgroups is subject to the same idle-mode reception bandwidth as CORESET-0 frequency span

In this document, we discuss detailed designs for Paging Early Indication.

# Discussion on Paging Early Indication (PEI)

In Rel-15/16 NR, a UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption and monitors one paging occasion (PO) per DRX cycle [2]. For SI acquisition [3], UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with a common search space on the active BWP to monitor paging.

A paging early indication (PEI) can implicitly or explicitly indicate a UE to skip monitoring paging DCI on a paging occasion(s), in order to reduce UE power consumption from unnecessary SS/PBCH block (SSB) measurements and PDCCH decoding efforts.

In NR, multi-beam operation is supported. A SS burst set is a set of SSBs being transmitted within 5ms window of SSB transmission. The number of actual transmitted SSBs is determined according to ssb-PositionsInBurst. Each of these SSBs may correspond to a particular beam direction. In multi-beam operations, an NR PO consists of one or multiple sets of ‘S’ consecutive PDCCH monitoring occasions, where 'S' is the number of actual transmitted SSBs. The k-th PDCCH monitoring occasion (MO) for paging DCI in the PO corresponds to the k-th transmitted SSB and the k-th beam direction. Considering the beam sweeping for a SSB and a corresponding PDCCH MO for paging DCI in NR, the PEI should be designed with beam sweeping with quasi-co-location (QCL) association among PEI, MO, and SSB, as illustrated in Figure 1. Similar to PO in multi-beam operations, UE may assume that the same message of PEI is repeated with all beams of the sweeping pattern, and a PEI occasion can be QCLed with an SSB of actual transmitted SSBs.

Figure 1. QCL association among an SSB, a paging DCI MO, and a PEI occasion

Proposal 1: Support repetition of PEI with multiple beams, where each PEI occasion is QCLed with one SSB of transmitted SSBs.

## Sequence based PEI

The benefits of sequence-based paging early indication include short detection delay and small power consumption of detection. The complexity of decoding sequence-based indication process is low because of the non-coherent detection. Besides, it can be designed not relying on DL synchronization, since the receiving has much lower synchronization requirement. In addition, the sequence used for indication can be designed for time/frequency synchronization to help UEs get synchronization quickly.

Similar to sequence-based paging early indication, a sequence-based Wake Up Signal (WUS) has been introduced in Rel-15 MTC/NB-IoT and has been enhanced for UE-grouping in Rel-16 MTC/NB-IoT. For the UE that supports WUS and WUS configuration is provided, the UE shall monitor WUS to decide whether to monitor the next PO(s) or not. If the UE does not detect WUS the UE is not required to monitor the following PO(s). The function of sequence-based WUS introduced in MTC/NB-IoT is consistent with the current objective. Sequence-based indication in NR can save more power because of less detection of SSB. WUS has been discussed in depth in MTC/NB-IoT, and the design is mature. So, WUS in NB-IoT/MTC can be considered as a reference for the design of sequence-based indication.

For the configuration of MTC/NB-IoT WUS, a non-zero gap between the end of the WUS duration and the start of accordingly PO is defined for WUS detection and the gap is used for PDCCH detection warming up time. Similar to the non-zero gap of WUS, a non-zero gap at least for PEI detection/processing is necessary for NR PEI. As stated above, multi-beam operation is supported in NR. The configuration of gap for NR PEI can take the multi-beam operation as a consideration. The gap can be defined as from the end of PEI to the corresponding PO even to the corresponding MO as shown in Figure 2. The gap illustrated in Figure 2 is from the end of PEI to the corresponding PO. The gap1 and gap2 are the gaps from the end of PEI1 to the MO1 and the end of PEI2 to the MO2 separately.

If the gap is defined as from the last PEI to the corresponding PO, it is straightforward and simple. PEI for different beams can be transmitted next to each other. UE can determine the position of a PEI by the start position of the corresponding PO, the gap and the order of the PEI in the sequence. If the gap is defined as from each PEI to the corresponding MO, it can give network more flexibility to transmit each PEI corresponding to a specific beam. UE can determine the position of a PEI by the start position of the corresponding MO and the gap between the PEI and the corresponding MO.

Figure 2. Illustration of different gap for PEI

Proposal 2: A non-zero gap between the PEI and the corresponding PO or MO is configured for sequence based NR PEI.

## DCI based PEI

Another design approach for PEI is DCI based indication. Similar to Rel-16 power saving PDCCH (i.e. DCI format 2_6), an idle or inactive UE may receive a PDCCH search space configuration for a DCI format of Paging Power Saving (PPS)-PDCCH.

### Signaling of PPS-PDCCH search space configuration

A PPS-PDCCH search space configuration may be included in a system information message (e.g. SIB1). An idle and/or inactive mode UE may monitor PPS-PDCCH in order to determine whether to monitor paging DCI on a corresponding PO of a given DRX cycle. A connected mode UE may monitor PPS-PDCCH in order to determine whether to monitor paging DCI on any monitoring occasion within a system information modification period.

A PPS-PDCCH search space configuration may be included in an RRC release message including the parameter ‘suspendConfig’ (i.e. information required for an RRC inactive mode). If included, an inactive mode UE may monitor PPS-PDCCH according to the PPS-PDCCH search space configuration signaled in the RRC release message, in order to determine whether to monitor paging DCI on a corresponding PO of a given DRX cycle.

Proposal 3: For PDCCH based PEI, Paging Power Saving (PPS)-PDCCH search space configuration can be signaled in SIB1 or in an RRC release message.

### Payload size and monitoring occasions of PPS-PDCCH

Depending on intended UEs for a PPS-PDCCH, a payload size and a monitoring occasion(s) of a PPS-PDCCH may need to be configured differently. It is expected that there is a trade-off between a payload size of a PPS-PDCCH and the number of PPS-PDCCH transmissions per DRX/paging cycle. The payload size of the PPS-PDCCH is dependent on whether the PPS-PDCCH is intended to all UEs in a paging area, a group of UEs associated with a particular set of PFs, or a group of UEs associated with a particular set of POs.

If a PPS-PDCCH is intended to all UEs in a paging area, a PPS-PDCCH monitoring occasion(s) for a given PO may be determined based on a paging cycle, e.g. before a start of a corresponding paging cycle. A time offset value with respect to the start of the corresponding paging cycle in terms of the number of subframes and/or the number of slots can be configured, where the time offset value indicates a time, where the UE starts monitoring a PPS-PDCCH prior to the start of the paging cycle.

If a PPS-PDCCH is intended to a group of UEs associated with a particular set of Paging Frames (PFs), a PPS-PDCCH monitoring occasion(s) for a given PO may be configured based on a reference PF (e.g. the earliest PF of a particular set of consecutive PFs), e.g. before a start of a reference PF in a given paging cycle. A time offset value with respect to the start of the reference PF in terms of the number of subframes and/or the number of slots can be configured. A UE can identify a PPS-PDCCH monitoring occasion(s) intended for the UE based on a reference PF of the UE and a configured timing offset value.

If a PPS-PDCCH is intended to a group of UEs associated with a particular set of POs, a PPS-PDCCH monitoring occasion(s) for a given PO may be configured based on a reference PO (e.g. the earliest PO of a particular set of consecutive POs), e.g. before a start of a reference PO in a given paging cycle. A time offset value with respect to the start of the reference PO in terms of the number of subframes and/or the number of slots can be configured. A UE can identify a PPS-PDCCH monitoring occasion(s) intended for the UE based on a reference PO of the UE and a configured timing offset value.

In Rel-15/16 NR, the maximum configurable number of PFs per DRX/paging cycle is equal to the number of radio frames in a DRX/paging cycle, and the maximum configurable number of radio frames for a DRX/paging cycle is 256. Further, the maximum configurable number of POs per PF is 4, and the maximum number of page records in a paging message is 32. Thus, the maximum number of UEs that can be paged in a DRX cycle is 32768 UEs (= 256 PFs per DRX cycle × 4 POs per PF × 32 page records per PO). With 8 UE sub-groups for a given PO, the maximum 8192 bits (e.g. 8 bits per PO) are required for paging early indication intended to all UEs in a paging area. Considering that the maximum DCI size for DCI format 2_6 (i.e. maxDCI-2-6-Size-r16) is set to 140 bits, a DCI size of a PPS-PDCCH may need to be limited up to 120-140 bits, in order to guarantee similar coverage and detection performance as DCI format 2_6 (i.e. DCI based power saving indication for connected mode UEs). Thus, a PPS-PDCCH intended to a group of UEs associated with a particular set of consecutive PFs may be appropriate to balance a payload size of the PPS-PDCCH and the number of PPS-PDCCH transmissions per DRX cycle. A size of a PF set (i.e. the number of PFs per set) for a PPS-PDCCH may be configurable dependent on selected paging configuration parameter (e.g. the DRX cycle, the number of PFs per DRX cycle, and the number of POs per PF) values. For example, if the PF set size is 1, the expected payload size of the PPS-PDCCH is from 1 bit (i.e. 1 PO per PF, 1 bit per PO) to 32 bits (i.e. 4 POs per PF, 8 bits per PO).

Proposal 4: A PDCCH carrying PEI is intended to a group of UEs associated with a set of paging frames. A size of the set of paging frames may be dependent on selected paging configuration parameter values.

Proposal 5: A PPS-PDCCH monitoring occasion(s) for a given PO may be configured based on a reference PF of the given PO (e.g. the earliest PF of a particular set of consecutive PFs associated with the given PO), e.g., before a start of a reference PF in a given paging cycle.

### RNTI and bit field configuration of PPS-PDCCH

A UE may receive information of DCI configuration such as the total number of bits in DCI, the number of bits per Paging Frame, and/or the number of bits per Paging Occasion for PPS-PDCCH. Further, the UE may determine locations of a bit field and a bit(s) within the bit field intended to the UE based on the DCI configuration information and/or a UE identity (e.g. 5G-S-TMSI mod 1024 and/or a UE group identity). For example, each bit field in DCI may comprises N bits, where each of N bits correspond to a sub-group of UEs from the group of UEs associated with a particular PO of a particular PF. The bit field for the UE starts at the k-th bit, where k=NPO∙F∙Ns+is, where F is a PF index (or a PF index within a set of PFs associated with a given PPS-PDCCH) for UE’s PF, is is a PO index for UE’s PO, Ns is the number of POs within a PF, NPO is the number of bits in a bit field corresponding to a PO, and the indices, k, F, and is start from the value zero.

A RNTI value used for scrambling a CRC in a PPS-PDCCH that indicates whether to monitor paging DCI in the next paging cycle may be predefined and fixed. Alternatively, an RNTI value used for scrambling a CRC in a PPS-PDCCH is determined based on a PF index and/or a PO index. For example, the RNTI (e.g. referred to as PPS-RNTI) is determined as follows:

PPS-RNTI = C + F∙Ns,

where C is a predefined, configured, or a hash value dependent on SFN (and/or Hyper-SFN) of a particular PF (e.g. a reference PF), F is a PF index for the particular PF, and Ns is the number of POs within a PF.

Proposal 6: RAN1 further discusses DCI configuration information and RNTI for PPS-PDCCH.

# Paging Early Indication for RedCap UE

Configuring paging related parameters of a reduced capability (RedCap) UE(s) separately from paging related parameters of a normal UE(s) may be beneficial for RedCap UE power saving and efficient resource utilization. For example, a DRX cycle (i.e. paging cycle) of the RedCap UE(s) may be configured to be longer than a DRX cycle of the normal UE(s). The number of paging frames per DRX cycle and the number of Paging Occasions per Paging Frame may be separately configured based on the number of RedCap UEs in a cell.

Further, a DCI format of PPS-PDCCH may consist of a bit field(s) intended to a normal UE(s) and a bit field(s) intended to a RedCap UE(s). A paging power saving configuration  may include an indication of a starting bit position in the DCI format for the RedCap UE(s). Alternatively, PPS-PDCCH can be transmitted separately for a RedCap UE(s) and a normal UE(s) based on separate monitoring occasions and/or separate RNTI values.

Proposal 7: Study how to support Paging Early Indication for reduced capability UEs.

# Conclusion

In summary, we observe and propose the followings for Rel-17 NR paging enhancement for UE power saving:

Proposal 1: Support repetition of PEI with multiple beams, where each PEI occasion is QCLed with one SSB of transmitted SSBs.

Proposal 2: A non-zero gap between the PEI and the corresponding PO or MO is configured for sequence based NR PEI.

Proposal 3: For PDCCH based PEI, Paging Power Saving (PPS)-PDCCH search space configuration can be signaled in SIB1 or in an RRC release message.

Proposal 4: A PDCCH carrying PEI is intended to a group of UEs associated with a set of paging frames. A size of the set of paging frames may be dependent on selected paging configuration parameter values.

Proposal 5: A PPS-PDCCH monitoring occasion(s) for a given PO may be configured based on a reference PF of the given PO (e.g. the earliest PF of a particular set of consecutive PFs associated with the given PO), e.g., before a start of a reference PF in a given paging cycle.

Proposal 6: RAN1 further discusses DCI configuration information and RNTI for PPS-PDCCH.

Proposal 7: Study how to support Paging Early Indication for reduced capability UEs.

# References

- “RAN1 Chairman’s Notes”, RAN1 105-e meeting, May 10th – 27th, 2021.

- 3GPP TS 38.304, “User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 16)”.

- 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 16)”.