3GPP TSG RAN WG1 #106-e      	                                                                          R1-2107213

e-Meeting, August 16th – 27th, 2021

Source:	OPPO

Title:	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Agenda Item:	8.5.1

Document for:	Discussion and Decision

- Introduction

The Rel-17 WID of NR positioning enhancement was approved in RAN#90e [1], focusing on RAN1-centric objectives. Then, it was updated to include more objectives in RAN1#91e [2].

Mitigating of Rx/Tx timing delays is one of the key objectives. RAN1#105e had an extensive discussion on it and got some agreements as below [3]

Agreement: - For mitigating UE Tx timing errors for UL TDOA, support one of the following options: - Option 1: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs. - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs - Option 2: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs. - Support the serving gNB to forward the association information provided by the UE to the LMF - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs - FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF Agreement: Send an LS to RAN2/RAN3 (cc SA2), including the following content: - RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning. - Notes: - The term “positioning reference unit (PRU)” is only used as a terminology in this discussion. PRU does not necessarily mean an introduction of a new network node. - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2. The positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF. Agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

In this contribution, we will continue to discuss the enhancement for positioning to resolve the issues of UE Rx/Tx and gNB Rx/Tx timing delays from the perspective of positioning methods, measurement, signaling and procedures.

- Discussion on timing error group (TEG)

In RAN1#104bis-e and RAN1#105e meeting, there were some discussions on the association of TEG and MIMO SRS port, but no consensus was made.  From the perspective of specification, MIMO SRS can be used for some positioning, e.g., UL-RTOA. However, from UE side, there are only four usages for MIMO SRS resource(s) and positioning is not one of these four usages according to the NR specifications. In this sense, whether a MIMO SRS is used for positioning or not can be only known by network and is totally transparent to UE.

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]]

}

Thus, we have the following observation:

Observation 1: From the UE perspective, whether a MIMO is used for positioning or not is totally transparent

From the perspective of implementation, UE may transmit MIMO SRS by using different antenna virtualization or precoding according to the usage configured by RRC signaling. As MIMO SRS is widely used to support various NR features, we need to carefully check the potential impact on UE implementation of MIMO SRS for various use cases and scenarios.

Usage: non-codebook based UL MIMO. Let’s take a 2-Tx UE as an example.

- If each port is associated with one Tx, the timing difference of two SRS ports is related to the timing difference between different RF chains/antennas.

- If the two ports are implemented by vector [1,  1] and [1, -1] respectively, the timing difference of two ports is likely to be the same.  (e.g., used for non-codebook based UL MIMO)

Obviously, the two different mapping of SRS will have different assumption of timing difference and UE doesn’t know whether it will be used for positioning or not.  Moreover, the mapping of SRS port and physical antenna/PA may be different in different transmission occasions as UE will adjust the precoder according to the current DL channel status.

Usage: codebook based UL MIMO. Let consider a 4-Tx UE that supports UL Full power transmission. If a gNB configures 2-port SRS for UL full power transmission (Codebook based UL MIMO), then UE may have different mapping of MIMO SRS

- Alt.1: each port is mapped to one Tx

- Alt.2: each port is mapped to two Tx via CDD or precoding.

TEG information is likely to be different for Alt.1 and Alt.2. Moreover, UE implementation of Alt.1 or Alt.2 is transparent to gNB/LMF.

Usage: antenna switching. Let’s assume antenna switching of 2T4R.We cannot assume the timing difference between the two 2 Tx chains/antennas for the two transmissions (each corresponding to one 2-port SRS resource) are the same since antennas are in different positions and different switchers are used. That is to say, there may exist such case:  timing difference between the 2 ports of one SRS resource is within a margin, whereas timing difference between the 2 ports of another SRS resource is larger than this margin.

Based on the discussion, we can have the following observation:

Observation 2: At least for some usage(s) of MIMO SRS resource, the TEG information may be varying for different transmission occasions.

On the other hand, for a MIMO SRS resource configured with one usage in Rel-15/Rel-16, UE knows the intention of this SRS resource and may do some dedicated mapping for this SRS. If additional information on the MIMO SRS used for positioning is received, UE has to make some decision, for example

- Option1: Prioritize the configured usage

- Option2: Prioritize positioning

- Option3: Try to make some optimization to achieve a good trade-off between the configured usage and positioning

Different options will have different impact on the UE implementation and the performance of MIMO and positioning, which is not friendly from UE perspective. Thus, the benefits to asscociate TEG with MIMO SRS port are not well justified.

Proposal 1: Rel-17 doesn’t support the association of TEG with MIMO SRS port(s).

- TEG based method for DL Positioning

In RAN1#104bis-e, an agreement for the enhancement of DL-TDOA positioning by down-selecting some basic schemes out of 17 alternatives:

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

According to the agreement, the basic enhancement scheme for DL positioning is determined to mitigate the UE Rx timing errors and/or TRP Tx timing errors.

One of the remaining issues is that how to signal the information of TEG.  Each DL RSTD measurement is related to a pair of TRPs, where one is the reference TRP and the other is the target TRP. Thus, the UE Rx TEG should be indicted for these two TRPs for each DL RSTD measurement. According to the Rel-16 design, the reference TRP is common for the DL RSTD measurements within a report. As a result, one Rx TEG for the reference TRP in a report is sufficient.

In Rel-16 positioning, UE also supports to report DL RSTD measurement for additional path(s) between the same pair of TRPs. From the perspective of reception, UE can use different panels to receive the signals from the same TRP and each panel can correspond a different path. It may be also beneficial to indicate the Rx TEG information for the measurement associated with additional path(s).

Based on the discussion, we have the following proposal:

Proposal 2: Subject to UE capability, UE can report the following information in a DL TDOA measurement report (NR-DL-TDOA-SignalMeasurementInformation)

  - One Rx TEG ID for the RSTD reference TRP

    - It can be achieved by setting nr-RSTD and nr-RSTD-ResultDiff to be zero for the corresponding DL RSTD measurement

  - One Rx TEG ID for each DL RSTD measurement (NR-DL-TDOA-MeasElement)

  - One Rx TEG ID for each DL RSTD measurement for an additional path (NR-DL-TDOA-AdditionalMeasurementElement)

There was also a discussion on whether UE can measure the same DL PRS resource from a TRP with different UE Rx TEGs. From our perspective, this mechanism is not useful in practical deployment due to the following reasons:

- Whether a UE can or not measure the same signal with different panels for different directions depends on the wireless environment. It is not very likely for a UE to use different panels to measure the same signal with good quality

- If the main intention is to facilitate LMF to estimate the Rx timing error of UE, then

  - We don’t get any information whether the Rx timing error is constant or varying. We need the related information from RAN4.

    - If Tx timing error is varying at each reception, or within a short time, LMF cannot estimate a relatively accurate timer error.

    - If the Tx timing error is relatively varying, UE can compensate it by implementation and doesn’t need to report multiple measurement to LMF

  - The measurement result consists of both Rx timing error and the difference of the different paths. For a UE with multiple panels, different panels are usually for different directions. Thus, the timing difference of the different paths associated with different panels will be large. In this case, LMF/UE cannot estimate the Rx timing error with a satisfying accuracy.

Proposal 3: Rel-17 doesn’t support UE to report multiple DL RSTD measurement which are measured on the same DL PRS resource from a TRP with different UE Rx TEGs.

- TEG based method UL Positioning method

In RAN1#104bis-e, an agreement for the enhancement of UL-TDOA positioning by down-selecting some schemes out of 10 alternatives:

Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the Signaling, procedures, and UE capability

One of the remaining issues is about the MIMO SRS. As we discussed in Section 2, whether a MIMO resource is used for positioning or not is transparent to UE. The analysis of potential impacts on positioning and MIMO are also applicable for this issue. Thus, we propose not to support the association of MIMO SRS resources and TEG information.

Proposal 4: Rel-17 doesn’t support the association of TEGs with MIMO SRS resources.

Another open issue is how to report the association of TEG and Positioning SRS resources. The following two alternatives were discussed in RAN1#105e and RAN1 agreed to down select one of them:

- Alt.1: The association information is sent directly from UE to LMF

- Alt.2: The association information is first provided to gNB and then forwarded to LMF

The SRS resource for positioning is configured by gNB and signaled to UE via RRC. That means the SRS configuration for each are “locally” configured with in the serving cell of the gNB. As a result, there may be some ambiguity from LMF side on the associated TRPs of the SRS for Alt.1. For example, two gNBs (gNB A, gNB B) with long distance has UE A and UE B, respectively. If each gNB configures two SRS resources for its own UEs for positioning and these SRS configuration for UE A and UE B are the same, then same LMF will cannot differentiate these SRS resource for positioning, e.g., whether the reported association information for UE A is applicable for gNB A or gNB B.  In order to address this issue, some new design will be needed and the design principle of Rel-16 signaling mechanism will be broken. Thus, Alt.1 is not a good approach for the signaling of association information.

Observation 3: Alt.1 that the association information is sent directly from UE to LMF may lead to ambiguity issue at LMF.

According to the above discussion, we suggest to select Alt.2 to move forward.

Proposal 5: For the association information of TEGs and SRS resources for positioning, Rel-17 supports UE to report it to gNB and gNB to forward it to LMF via NRPPa, i.e.g,

  - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

  - Support the serving gNB to forward the association information provided by the UE to the LMF

There is an FFS part : Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs. For UL TDOA, the calculation of location information is done by LMF. Thus, it is sufficient for gNB to know the related TEG information.

Proposal 6: R17 doesn’t support LMF to forward the association Tx TEG information of a UE from the serving gNB to the neighboring gNBs

Regarding the Rx TEG information of TRP, we should have a counterpart one of Proposal 2:

Proposal 7: Rel-17 supports TRP/gNB to report a Rx TEG ID for each UL TROA measurement result.

- TEG based method Multi-RTT Positioning Method

In RAN1#104bis-e, an agreement for mitigating UE Rx/Tx timing errors in DL+UL positioning by down-selecting some basic schemes out of 9 alternatives. In RAN1#105e meeting, these schemes were refined as two options as below:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE

  - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

For DL-TDOA and UL-TDOA positioning method, the Rx TEG and Tx TEG are used. As a general framework for all positioning method is preferred. Moreover, the signaling of the association information can reuse that of DL-TDOA and UL-TDOA, rather than some new design based on RxTx TEG. Thus, we suggest to support Option 2 for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning.

Proposal 8: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Option 2, i.e.,

- UE provides the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG ID, Tx TEG ID} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS

For the association of Tx TEG ID, there were three alternatives identified in RAN1#105e meeting:

  - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

  - Alt. 2: the Tx timing of the Rx-Tx measurement

  - Alt. 3: one or more UL SRS resources for positioning

From our understanding, the down-selection depends on the definition of Rx-Tx time difference measurements. Thus, we have the following proposal:

Proposal 9: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, a Tx TEG ID is associated with the Tx timing of the Rx-Tx measurement if there is no modification on the definition of Rx-Tx time difference measurements (Alt.2).

In RAN1#104bis-e, an agreement for mitigating UE Rx/Tx timing errors in DL+UL positioning by down-selecting some basic schemes out of 8 alternatives. In RAN1#105e meeting, these schemes were refined as two options as below:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures

Based on the same reason for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, we suggest to support Alt.1 for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning as well. Thus, we have the following proposal:

Proposal 10: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG ID, Tx TEG ID } to LMF

- Multiple results for a report

In RAN1#104e meeting, it was also agreed to report one or more measurement instances in a single measurement report to LMF.

From UE perspective, the current LPP signaling has supported multiple measurement results in a report. For example, a UE can report up to 4 results for NR Multi-RTT measurements and NR DL-TDOA measurements. The corresponding signaling is copied as below

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-UE-RxTxTimeDiff-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-Multi-RTT-AdditionalMeasurements-r16 NR-Multi-RTT-AdditionalMeasurements-r16 OPTIONAL, ... } NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ... } NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

Therefore, the current signaling can support this feature with an exception that more than 4 results may be needed.

Proposal 11: The current LPP signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF, with potential extension to support a larger number than 4.

From network perspective, the current NRPPa signaling has supported multiple measurement results in a report. For example, a UE can report up to 16384 results for NR Multi-RTT measurements and NR UL RTOA measurements. The corresponding signaling is copied as below

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Measured Result Item; 1 .. <maxnoPosMeas>. >CHOICE Measured Results Value; M. >>UL Angle of Arrival; M; 9.2.38. >>UL SRS-RSRP; M; INTEGER (0..126). >>UL RTOA; M; 9.2.39. >>gNB Rx-Tx Time Difference; M; 9.2.40. >Time Stamp; M; 9.2.42. >Measurement Quality; O; 9.2.43. >Measurement Beam Information; O; 9.2.57. Range bound; Explanation. maxnoPosMeas; Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384..

Therefore, the current NRPPa signaling can support  reporting of multiple measurement instances. For the same type of measurement instances, the current mechanism seems sufficient. However, for the different types of measurement instances, there should be some association between them. For example, the TROA and AoA results are measured on the same SRS resource(s).

Observation 4 The current NRPPa signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF

Proposal 12: No enhancement is needed for the current NRPPa signaling to support the feature that TRP reports one or more measurement instances with the same quantity in a single measurement report to LMF.

Proposal 13:  Enhancement on the association of measurement instances should be introduced to support the feature that TRP reports one or more measurement instances with different quantities in a single measurement report to LMF.

One remaining issue is how to determine the timestamp of each measurement instance. There were two options discussed in the last meeting:

- Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource set/PRS resource (or the last SRS resource set/SRS resource for the positioning purpose) that are used to determining the measurement instance.

- Option 2: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the first and last DL-PRS resource set/PRS resource (or the first and last SRS resource set/SRS resource for the positioning purpose) that are used to determining the measurement instance.

In general, Option 1 will give a timestamp with less uncertainty. Thus, LMF and UE can have a better alignment on the timestamp. For a Dl-PRS resource set, there may be multiple PRS resources transmitted different slots. How to determine the reception time of the last DL-PRS resource set. In order to make it clear, we can set the reception time of the last PRS resource within the last DL-PRS resource set, where the last PRS resource is used for the measurement. Thus, we have the following proposal:

Proposal 14:  For the timestamps for the measurement instances in a measurement report, support Option 1 with the following modification:

  - Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource set/PRS resource (or the last SRS resource set/SRS resource for the positioning purpose) that are used to determining the measurement instance.

- Conclusion

In this contribution, we presented our views on the impact of Tx/Rx timing delays on the DL, UL and DL/UL positioning and we also discussed the potential enhancements to address the issues. Based on the discussions, the following observations and proposal were made:

Observation 1: From the UE perspective, whether a MIMO is used for positioning or not is totally transparent

Observation 2: At least for some usage(s) of MIMO SRS resource, the TEG information may be varying for different transmission occasions.

Observation 3: Alt.1 that the association information is sent directly from UE to LMF may lead to ambiguity issue at LMF.

Observation 4 The current NRPPa signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF

Proposal 1: Rel-17 doesn’t support the association of TEG with MIMO SRS port(s).

Proposal 2: Subject to UE capability, UE can report the following information in a DL TDOA measurement report (NR-DL-TDOA-SignalMeasurementInformation)

  - One Rx TEG ID for the RSTD reference TRP

    - It can be achieved by setting nr-RSTD and nr-RSTD-ResultDiff to be zero for the corresponding DL RSTD measurement

  - One Rx TEG ID for each DL RSTD measurement (NR-DL-TDOA-MeasElement)

  - One Rx TEG ID for each DL RSTD measurement for an additional path (NR-DL-TDOA-AdditionalMeasurementElement)

Proposal 3: Rel-17 doesn’t support UE to report multiple DL RSTD measurement which are measured on the same DL PRS resource from a TRP with different UE Rx TEGs.

Proposal 4: Rel-17 doesn’t support the association of TEGs with MIMO SRS resources.

Proposal 5: For the association information of TEGs and SRS resources for positioning, Rel-17 supports UE to report it to gNB and gNB to forward it to LMF via NRPPa, i.e.g,

  - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

  - Support the serving gNB to forward the association information provided by the UE to the LMF

Proposal 6: R17 doesn’t support LMF to forward the association Tx TEG information of a UE from the serving gNB to the neighboring gNBs

Proposal 7: Rel-17 supports TRP/gNB to report a Rx TEG ID for each UL TROA measurement result.

Proposal 8: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Option 2, i.e.,

- UE provides the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG ID, Tx TEG ID} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS

Proposal 9: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, a Tx TEG ID is associated with the Tx timing of the Rx-Tx measurement if there is no modification on the definition of Rx-Tx time difference measurements (Alt.2).

Proposal 10: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG ID, Tx TEG ID } to LMF

- Proposal 11: The current LPP signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF, with potential extension to support a larger number than 4.

Proposal 12: No enhancement is needed for the current NRPPa signaling to support the feature that TRP reports one or more measurement instances with the same quantity in a single measurement report to LMF.

Proposal 13:  Enhancement on the association of measurement instances should be introduced to support the feature that TRP reports one or more measurement instances with different quantities in a single measurement report to LMF.

Proposal 14:  For the timestamps for the measurement instances in a measurement report, support Option 1 with the following modification:

  - Option 1: The timestamp of the UE (or TRP) measurement instance corresponds to the reception time of the last DL-PRS resource set/PRS resource (or the last SRS resource set/SRS resource for the positioning purpose) that are used to determining the measurement instance.

- Reference

RP-202900 New WID on NR Positioning Enhancements

RP-210903, Revised WID on NR Positioning Enhancements

Chairman’s note of RAN1#105e