3GPP TSG RAN WG1 #106-e R1-2107235 e-Meeting, August 16th – 27th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 16.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PDSCH-to-HARQ feedback timing indicator field values. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_unlic-Core; Date:; 2021-8-16. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In current spec, for a DCI format, other than DCI format 1_0, scheduling a PDSCH reception or a SPS PDSCH release, the PDSCH-to-HARQ_feedback timing indicator field values are configured by high layer signaling To support Type 3 HARQ-ACK codebook, DCI requesting Type3 HARQ-ACK codebook report but not scheduling a PDSCH is introduced. However, it is not clear how to determine PDSCH-to-HARQ_feedback timing indicator field values for DCI requesting Type3 HARQ-ACK codebook report but not scheduling a PDSCH. It is straightforward to reuse the same solution as DCI scheduling a PDSCH reception or a SPS PDSCH release, i.e. configured by high layer signaling.. -. Summary of change:; PDSCH-to-HARQ_feedback timing indicator field values for DCI requesting Type3 HARQ-ACK codebook report but not scheduling a PDSCH is configured by high layer signaling.. Consequences if not approved:; PDSCH-to-HARQ_feedback timing indicator field values for DCI requesting Type3 HARQ-ACK codebook report but not scheduling a PDSCH cannot be determined and UE may determine incorrect slot for HARQ-ACK feedback.. Clauses affected:; 9.2.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR has isolated impact on PDSCH-to-HARQ_feedback timing indicator field values for DCI requesting Type3 HARQ-ACK codebook report but not scheduling a PDSCH. If gNB is implemented according to this CR and UE is not, or, if UE is implemented in accordance to this CR while gNB is not, UE may determine incorrect slot for HARQ-ACK feedback.. This CR's revision history:; This is the first version for this CR..

- 9.2.3	UE procedure for reporting HARQ-ACK

<Unchanged part omitted>

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, scheduling a PDSCH reception or a SPS PDSCH release, or requesting Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception as described in Clause 9.1.4, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1.

<Unchanged part omitted>