3GPP TSG RAN WG1 #106-e R1-2107261 e-Meeting, August 16th – 27th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 16.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on spatial domain transmission filter for PUCCH. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_2step_RACH-Core; Date:; 2021-8-6. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In current spec, regardless of random access scheme, spatial domain transmission filter for PUCCH is always the same as PUSCH transmission scheduled by a RAR UL grant. However, when 2-step RACH is applied, there is no PUSCH transmission scheduled by a RAR UL grant. The spatial domain transmission filter for PUCCH can be not be determined. In 2-step RACH procedure, PUSCH for Type-2 random access procedure has similar function and transmission parameter as PUSCH scheduled by a RAR UL grant in legacy random access procedure. So, it is straightforward to apply the same spatial domain transmission filter for PUCCH as PUSCH for Type-2 random access procedure when 2-step RACH is applied.. -. Summary of change:; PUCCH uses the same spatial domain transmission filter as for PUSCH for Type-2 random access procedure.. Consequences if not approved:; Spatial domain transmission filter determination for PUCCH is missed when 2-step RACH is applied.. Clauses affected:; 9.2.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR has isolated impact on spatial domain transmission filter determination for PUCCH when 2-step RACH is applied. If gNB is implemented according to this CR and UE is not, or, if UE is implemented in accordance to this CR while gNB is not, UE may use incorrect spatial domain transmission filter for PUCCH leading to PUCCH transmission failure.. This CR's revision history:; This is the first version for this CR..

- 9.2.1	PUCCH Resource Sets

<Unchanged part omitted>

The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3 or PUSCH for Type-2 random access procedure as described in Clause 8.1A

<Unchanged part omitted>