3GPP TSG RAN WG1 #106-e R1-2107270 e-Meeting, August 16th – 27th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 16.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on scheduling and HARQ. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_URLLC_enh; Date:; 2021-8-6. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In RAN1 105e meeting, companies show different views on the current spec behavior regarding intermediate multiplexing during intra-UE prioritization procedure. Moreover, most companies think that the intermediate multiplexing checking complicates the UE implementation. So, some simple solutions were discussed. To avoid intermediate multiplexing checking and make sure enough processing time for cancellation. In R1-2107271, Intra-UE prioritization procedure is suggested: - The UE uses the final HP PUCCH/PUSCH channel(s) to cancel LP channel(s) - Extended multiplexing timeline is Tproc,2+d1 for any PUCCH or PUSCH of larger priority index overlapping with a PUCCH or PUSCH of low priority index.. -. Summary of change:; Intra-UE prioritization procedure is modified.. Consequences if not approved:; Intra UE prioritization procedure is unnecessarily mis-interpreted and increases UE implementation complexity.. Clauses affected:; 9. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR has isolated impact on PUCCH or PUSCH transmission: If gNB is implemented according to this CR and UE is not, or, if UE is implemented in accordance to this CR while gNB is not, UE may transmit PUCCH or PUSCH, which is not expected by gNB.. This CR's revision history:; This is the first version for this CR..

- 9    UE procedure for reporting control information

<unchanged text omitted>

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller the same priority index as described in clauses 9.2.5 and 9.2.6. Then,

-	if a transmission of a first PUCCH of larger priority index scheduled by corresponding to a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission

-	if a transmission of a first PUSCH of larger priority index scheduled by corresponding to a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission

where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6

- the overlapping is applicable as described in clauses 9.2.5, and any PUCCH or PUSCH of larger priority index in the overlapping group overlaps with a PUCCH or PUSCH of smaller priority index would not start before Tproc,2 +d1 after a last symbol of the corresponding PDCCH reception for the PUCCH or PUSCH of larger priority index.

-	any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception.

-	Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability

<unchanged text omitted>