3GPP TSG RAN WG1 #106-e,		 R1-2107277

e-Meeting, August 16th -27th, 2021

Source:	OPPO

Title:	Discussion on cross-carrier scheduling from SCell to PCell

Agenda Item:	8.13.1

Document for:	Discussion and Decision

Introduction

The following was captured in RAN1 Chairman notes in RAN1 #105-e meeting for cross-carrier scheduling from SCell to PCell/PSCell:

For at least Type B UE, downselect from one of the BD/CCE limit handling options below

- [based on Option A/C] When UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1)

  - On P(S)Cell (for self-scheduling)

    - UE is not required to monitor more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

    - UE is not required to monitor more than

      - Alt1

        - α*MPDCCHmax,slot,μ PDCCH BD candidates per P(S)Cell slot

      - Alt2

        - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

  - On sSCell (for cross-carrier scheduling to P(S)Cell)

    - UE is not required to monitor more than minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per slot of sSCell

    - UE is not required to monitor more than

      - Alt1

        - β*MPDCCHmax,slot,μ PDCCH BD candidates per P(S)Cell slot

      - Alt2:

        - β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

  - At least case of α+β≤1 is supported.

    - FFS case of  α, β,α+β>1

    - FFS multi-TRP case

  - FFS following

    - Selection between Alt1 vs. Alt2 above

    - Whether separate α and  β  are configured by RRC or if β=1-α and only  α is configured

    - How the PDCCH BD candidates are distributed between multiple sSCell slots overlapping a P(S)Cell slot when μ<μ1 and whether the BD limits for sSCell are specified per sSCell slot or per P(S)Cell slot

- [based on Option B] When UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1)

  - On P(S)Cell (for self-scheduling)

    - UE is not required to monitor more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per slot of P(S)Cell

  - On sSCell (for cross-carrier scheduling to P(S)Cell)

    - UE is not required to monitor more than minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per slot of sSCell

  - Considering both PDCCH BD candidates for P(S)Cell self-scheduling on P(S)Cell and PDCCH BD candidates for sSCell to P(S)SCell cross-carrier scheduling on sSCell

    - UE is not required to monitor more than

      - Alt 1

        - MPDCCHmax,slot,μ  PDCCH BD candidates per P(S)Cell slot

      - Alt 2

        - minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot

  - FFS: selection between Alt-1 and Alt-2

- FFS: whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell

In this contribution, we provide our further views on cross-carrier scheduling scheme.

Discussion

For Rel-15/16 cross-carrier scheduling, one SCell is scheduled by PCell or another scheduling SCell, and the PCell or the scheduling SCell has to be scheduled by itself. In Rel-17, to support cross-carrier scheduling from a SCell to PCell/PSCell, RAN1 agrees that PCell/PSCell can be scheduled by both itself and the sSCell. The PDCCH monitoring capability for PCell and the search spaces configuration /linkage need further discussion.

MPDCCHtotal,slot,μ determination

According to the spec 38.213, the MPDCCHtotal,slot,μ is MPDCCHmax,slot,μ or γ∙MPDCCHmax,slot,μ when μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, or is Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  when μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap. In order to simplify the discussion, NcellsDL,μ is used to instead Ncells,0DL,μ+γ∙Ncells,1DL,μ, assuming there is no M-TR. The definition of NcellsDL,μ is ‘the number of configured DL-CCs whose scheduling cell is with active DL BWP having SCS configuration ’, and one cell can be scheduled by only one cell. If CCS is configured, PCell may have two scheduling cells, so the main point is how to count PCell, as one scheduled cell of PCell and SCell(Alt. 1), or as one scheduled cell of PCell and one scheduled cell of sSCell(Alt. 2), or as γμ1 of one scheduled cell of PCell and γμ2 of one scheduling cell of sSCell. And in RAN1 #105e meeting, three candidates calculation scheme is mentioned for ‘CA scaling calculation’ by FL.

- Alt1 - P(S)Cell is counted once - sSCell is counted once - Alt 2 - P(S)Cell is counted once - sSCell is counted twice (with same SCS as sSCell) - Alt 3 - Use scaling factors γμ1 and γμ2 with SCS configuration μ1 and μ2 respectively for P(S)Cell and sScell (the scaling factors may be less than 1)

When PCell and sSCell are configured with same SCS, Alt.1 can be used. The monitoring capability for PCell on PCell and sSCell are limited by the total capability with the configured SCS. And Alt.1 will not impact the capability for other scheduling cells configured for the UE. When PCell and sSCell are configured with different SCS, Alt.1, Alt.2 and Alt.3 shows different limitation for monitoring capability for PCell and Alt.2 may increase the total number of scheduling cells when calculate MPDCCHtotal,slot,μ.

Figure 1 An example for different SCS case

The details for different SCS case are showed in an example as Figure 1. There are 5 cells configured for a UE. The SCS configuration is μ1=15kHz and μ2=30kHz. In baseline case, PCell is only scheduled by itself. In case 1, PCell can be scheduled by PCell and sSCell simultaneous. The NcellsDL,μ and MPDCCHtotal,slot,μ are showed in Table 1 for each case. NcellsDL,μ1 is the number of scheduled cells with scheduling cell configured with SCS μ1. NcellsDL,μ2 is the number of scheduled cells with scheduling cell configured with SCS μ2. ‘a1’ and ‘a2’ are PDCCH monitoring times for PCell, ‘a1’ is monitored on PCell and ‘a2’ is monitored on sSCell. ‘a3’ is PDCCH monitoring times on sSCell for self-scheduling. ‘a4’, ‘a5’ and ‘a6’ are PDCCH monitoring times for SCell 1, SCell 2 and SCell 3.

Table 1 NcellsDL,μ and monitoring capability limitation

|  | Baseline | Alt. 1 | Alt. 2 | Alt. 3 |
| --- | --- | --- | --- | --- |
| NcellsDL,μ1 | 1 | 1 | 1 | γμ1 |
| NcellsDL,μ2 | 3 | 3 | 4 | γμ2+3 |
| NcellsDL,μ3 | 1 | 1 | 1 | 1 |
| MPDCCHtotal,slot,μ1 | 4*44*1/(1+3+1)=35 | 4*44*1/(1+3+1)=35 | 4*44*1/(1+4+1)=29 | 4*44*γμ1/(γμ1+γμ2+3+1) |
|  | Upper limit for a1 | Upper limit for a1+a2 | Upper limit for a1 | Upper limit for a1 |
| MPDCCHtotal,slot,μ2 | 4*36*3/(1+3+1)=86 | 4*36*3/(1+3+1)=86 | 4*36*4/(1+4+1)=96 | 4*44*(γμ2+3)/(γμ1+γμ2+3+1) |
|  | Upper limit for a3+a4+a5 | Upper limit for a3+a4+a5 | Upper limit for a2+a3+a4+a5 | Upper limit for a2+a3+a4+a5 |
| MPDCCHtotal,slot,μ3 | 4*22*1/(1+3+1)=17 | 4*22*1/(1+3+1)=17 | 4*22*1/(1+4+1)=14 | 4*22*1/(γμ1+γμ2+3+1)=17 |
|  | Upper limit for a6 | Upper limit for a6 | Upper limit for a6 | Upper limit for a6 |

For Alt.1

The PCell is calculated as one scheduled cell for PCell and sSCell. And the reference SCS is the SCS configuration of PCell. As PCell is counted based on reference SCS, monitoring capability of PDCCH on PCell and sSCell are also counted in the total capability with reference SCS. The total capability determination procedure is similar with Rel-15/16, but there is additional capability ‘a2’ is considered because of sSCell scheduling PCell. In case 1, ‘a1’ and ‘a2’ are limited by the capability calculated with SCS μ1, where ‘a1’ is PDCCH monitoring times on PCell and ‘a2’ is PDCCH monitoring times on sSCell.

For Alt.2

The PCell is counted as one scheduled for PCell and one scheduled cell for sSCell. In the examples, the total number of scheduled cells used for capability calculation is 6, larger than the real number of configured cells 5. ‘a1’ is limited by the total capability with SCS μ1, and ‘a2’ is limited by the total capability with SCS μ2. And the total capacity for scheduling cells with SCS μ3 is decreased because of the total number of cells is increased.

For Alt.3

The PCell is counted as a scaling value γμ1 for PCell and γμ2 for sSCell. In the examples, the total number of scheduled cells used for capability calculation is 5, same with the number of configured cells. Similar with Alt.2, the monitoring capability for PCell on PCell and sSCell are respectively counted in the total capability for SCS μ1 and μ2.

Table 1 shows monitoring capability limitation for difference cases. In Alt.1, the capability for PCell is limited by only one MPDCCHtotal,slot,μ , and the results are same with baseline case. In Alt.2 and Alt.3, the capabilities for PCell are limited respectively with different SCS. Alt.2 increases the total number of scheduling cells, it may impact the capability for other scheduling cells, e.g., the total capability for cells with SCS μ3 is decreased. So Alt.2 is not preferred. As the capability for PCell is defined with a total capability on PCell and sSCell, Alt.1 is preferred because the limitation for PCell and sSCell comes from only one MPDCCHtotal,slot,μ.

Proposal 1: When PCell and sSCell are configured with same SCS, Alt. 1 can be used. When PCell and sSCell are configured with different SCS, Alt.1 is preferred.

- Alt.1: The PCell is calculated as one scheduled cell for both PCell and sSCell.

BD/CCE limit handling

In RAN1 #105e meeting, there were 2 options of BD/CCE limit handling mentioned, option A/C and option B. In option A/C, two explicit capabilities are defined respectively monitoring on PCell and monitoring on sSCell used for scheduling PCell, and they are fixed for each slot(s). In option B, one total capability is defined for monitoring on PCell and monitoring on sSCell used for scheduling PCell, and the capability on PCell and sSCell can be changed in different slot(s). In this way, the monitoring for PCell can be independently on PCell and sSCell for option A/C. For option B, it need considered together for the PDCCH candidates on PCell and sSCell. Similar restrictions exist for the PDCCH dropping/overbooking procedure.

Selection between Alt.1 and Alt.2

There are two candidate formulas mentioned for the capability on PCell, α*MPDCCHmax,slot,μ and α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ. For the capability on sSCell, the candidate formulas are β*MPDCCHmax,slot,μ and β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ. The total capability on PCell and sSCell is limited, and it is also limited by MPDCCHtotal,slot,μ when Alt.1 is used for MPDCCHtotal,slot,μ. In this way, minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ should be used.

PDCCH capability distribution of multiple sSCell slots(μPCell≤μsSCell)

In option A/C, the capabilities for PCell and sSCell are defined based on the slot of PCell. When the SCS configurations for PCell and sSCell are different and the SCS of PCell is the larger one, there are several slots on sSCell overlapped with one slot on PCell. In this case, the capability for sSCell is defined based on multi-slots, which is different from Rel-15/16. It is better to define the capability on PCell based on a slot of PCell, and to define the capability on sSCell based on a slot of sSCell. The capability per sSCell slot can be equally distributed based on the overlapped PCell slot. It can be reflected by the restriction of α and β, e.g. α+2μ2-μ1*β≤1.

Proposal 2: Option A/C is preferred with the clarification and additional restrictions.

- The capability on PCell is α*minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 per PCell slot

- The capability on sSCell is β*minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 per sSCell slot

- α+2μ2-μ1*β≤1

- μ1 is the SCS configuration for PCell , μ2 is the SCS configuration for sSCell and μ1≤μ2

Numerology for PCell and sSCell

In DSS scenario, PCell shares spectrum with LTE cell. And the SCS configuration should be same for PCell and LTE cell (15kHz). The case SCS of PCell is higher than sSCell could not be considered.

Proposal 3: The DSS WI only considers the case where PCell has no larger SCS than sSCell when SCell-to-PCell CCS is configured.

Search space configuration

In Rel-15/16, when cross-carrier scheduling is configured, search space ID is linked between scheduling cell and scheduled cell. The detail configuration of search space is defined in scheduling cell expect for nrofCandidates. For the case of CCS in Rel-17, the similar mechanism can be used. Search spaces on both PCell and sSCell can be configured for PCell. For the search spaces on PCell used for self-scheduling, the configurations are same with Rel-16. For the search space used for PCell on sSCell, the monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot and duration are the same as the configuration for self-scheduling on sSCell. nrofCandidates is configured independently for PCell and sSCell with the linked search space ID.

Proposal 4: The same Rel-16 search space configuration mechanism is reused for SCell-to-PCell CCS in Rel-17.

Conclusion

In this contribution, we show our views on cross-carrier scheduling with following proposals:

Proposal 1: When PCell and sSCell are configured with same SCS, Alt. 1 can be used. When PCell and sSCell are configured with different SCS, Alt.1 is preferred.

- Alt.1: The PCell is calculated as one scheduled cell for both PCell and sSCell.

Proposal 2: Option A/C is preferred with the clarification and additional restrictions.

- The capability on PCell is α*minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 per PCell slot

- The capability on sSCell is β*minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 per sSCell slot

- α+2μ2-μ1*β≤1

- μ1 is the SCS configuration for PCell , μ2 is the SCS configuration for sSCell and μ1≤μ2

Proposal 3: The DSS WI only considers the case where PCell has no larger SCS than sSCell when SCell-to-PCell CCS is configured.

Proposal 4: The same Rel-16 search space configuration mechanism is reused for SCell-to-PCell CCS in Rel-17.

Reference

- R1-2106356 Summary#5 of Email discussion [105-e-NR-DSS-01]

- R1-2102416 Discussion on cross-carrier scheduling for Scell to Pcell

- R1-2104806 Discussion on cross-carrier scheduling