3GPP TSG-RAN WG1 Meeting #106-e	R1-2107295

e-Meeting, August 16th – 27th, 2021

Source:	FGI, Asia Pacific Telecom

Title:	Discussion on Intra-UE multiplexing/prioritization

Agenda item:	8.3.3

Document for: 	Discussion and Decision

# Introduction

In RAN1 #105-e, intra-UE multiplexing/prioritization was discussed and the following agreements were made [1].

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options:

  - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

  - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

- For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1.

- FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority.

- Rel-15 design (for PF0 and PF1) is baseline.

- Note: Qualcomm has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. Qualcomm accepts the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

In this contribution, we provide our views on intra-UE multiplexing/prioritization.

# Discussion

## Reliability of high priority UCI when multiplexing high priority PUCCH and low priority PUCCH

PUCCH resource set determination

Considering different code rates may be used for encoding high priority UCI and low priority UCI when multiplexed in a PUCCH, the following was proposed to determine UCI payload size for PUCCH resource determination:

•	Option 1: UCI payload size = the number of HP UCI bits + the number of LP UCI bits

•	Option 2: UCI payload size = the number of HP UCI bits + the number of LP UCI bits * scaling factor.

o	FFS: The scaling factor can be a function of code rate for HP UCI and LP UCI.

•	FFS: the number of LP UCI bits should be the actual number of LP UCI bits that UE generates based on DCI or RRC configuration or a reference number of LP UCI bits

In our view, option 2 should be supported as the encoded UCI bits can better fit the PUCCH resources in the PUCCH resource set selected based on the option, which can make configuratioin of PUCCH resource sets simpler.

Proposal 1: The following option is adopted for determination of PUCCH resource set.

- Option 2: UCI payload size = the number of HP UCI bits + the number of LP UCI bits * scaling factor.

Since the DCI scheduling low priority PUCCH may not be transmitted with the high priority that is used for transmitting the DCI scheduling high priority PUCCH, there may be some miss detection of DCI scheduling low priority PUCCH. In some cases, miss detection of DCI may result in UE selecting a different PUCCH resource set from the PUCCH resource set selected by gNB, which in turn results in UE selecting a different PUCCH resource from the PUCCH resource selected by gNB. This discrepancy between UE and gNB should be avoided. To address this issue, following options has been proposed.

- Option 1: Configure a dedicated PUCCH resource for HP+LP in the second PUCCH-Config

- Option 2: PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource

- Option 3a: The LP type 2 codebook size is quantized/rounded up to a nearest reference size. FFS reference size granularity.

- Option 3b: Configuration of semi-static size reservation for LP HARQ-ACK payload is provided by RRC. LP HARQ-ACK semi-static size reservation is used instead of determined LP HARQ-ACK codebook size when selecting the PUCCH resource set.

- Option 4: Additional DCI field in DCI corresponding HP HARQ-ACK or HP PUSCH for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH.

- Option 5: Provide indication on at least the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

- Other solutions are not precluded.

- FFS ambiguity cases.

- FFS whether the issue needs to be considered for Type-1 HARQ-ACK codebook.

In our view, option 1 may make scheduling of PUCCH resources more complex and a rule for determination of the PUCCH resources may be needed. For option 2, it is not clear how it resolves the issue. For option 3a, whether the issue can be resolved depends on the granularity. Option 3b, Option 4, and Option 5 should be further considered as they may better resolve the issue.

Proposal 2: Further consider the following options to resolve the issue of DCI miss detection.

- Option 3b: Configuration of semi-static size reservation for LP HARQ-ACK payload is provided by RRC. LP HARQ-ACK semi-static size reservation is used instead of determined LP HARQ-ACK codebook size when selecting the PUCCH resource set.

- Option 4: Additional DCI field in DCI corresponding HP HARQ-ACK or HP PUSCH for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH.

- Option 5: Provide indication on at least the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

Explicit indication for multiplexing

Using an explicit indication to indicate UE whether to multiplex overlapping high priority PUCCH and low priority PUCCH should be supported since it gives gNB full control over the reliability and latency that can be achieced by the multiplexed PUCCH or high priority PUCCH that is not multiplexed with low priority UCI. It can also resolve the ambiguity that may result from determination of whether multiplexing is performed based on coding rate, timeline, etc., since the precision of coding rate calculation and TA may be different between gNB and UE. Dynamic indication should at least be supported for the above reasons.

Proposal 3: Dynamic indication is supported for indicating whether to multiplex overlapping high priority PUCCH and low priority PUCCH.

Multiplexing HP PUCCH and LP PUCCH with total bits up to 2 bits

To provide high priority UCI with better reliability, for multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUCCH, when the total number of LP and HP HARQ-ACK bits is 2 bits, for multiplexing on a PUCCH format 0, HARQ-ACK values to CS indices mapping with unequal distance between mapped CS indices should be supported. For example, for a fixed LP HARQ-ACK bit value, the distance between mapped CS indices for different HP HARQ-ACK bit value can be 6.

Proposal 4: For multiplexing LP HARQ-ACK and HP HARQ-ACK on a PUCCH format 0, HARQ-ACK values to CS indices mapping with unequal distance between mapped CS indices should be supported.

## Reliability of high priority data/UCI when multiplexing high priority PUCCH/PUSCH and low priority PUCCH/PUSCH

Beta offsets and scaling

To ensure reliability of high priority PUCCH can be achieved when multiplexed in a PUSCH, and to ensure reliability of high priority PUSCH can be achieved when a low priority PUCCH is multiplexed in the PUSCH, separate beta offsets and scalings should be configured, which can be used to ensure the allocated number of resource elements for high priority UCI is enough for acheiving the target reliability, and can be used to ensure the allocated number of resource elements for low priority UCI is not too much to degrade the reliability of high priority data and UCI.

Proposal 5: Separate beta offsets and scalings can be configured for low priority UCI multiplexed in low priority PUSCH, for low priority UCI multiplexed in high priority PUSCH, for high priority UCI multiplexed in low priority PUSCH, and for high priority UCI multiplexed in high priority PUSCH.

Explicit indication for multiplexing

Explicit indication can givee gNB full control over the reliability and latency that can be achieced by the multiplexed PUSCH and PUCCH or high priority PUCCH/PUSCH that is not multiplexed with low priority data/UCI. It is proposed that beta offset value may be used to implicity indicate whether to multiplex low priority UCI in a high priority PUSCH. However, if semi-static beta offsets are configured, the method cannot work. Therefore, another indication should be defined for the purpose.

Proposal 6: Dynamic indication is supported for indicating whether to multiplex overlapping high priority PUSCH and low priority PUCCH. FFS the indication method when semi-static beta offsets are configured.

## CG-UCI

If a CG-UCI is multiplexed a high priority CG PUSCH, it should be jointly encoded with high priority HARQ-ACK if there is high priority HARQ-ACK to be multiplexed in the CG PUSCH. CG-UCI multiplexed in a high priority CG PUSCH is treated as high priority HARQ-ACK, i.e. mapped to resource elements that may be used for mapping high priority HARQ-ACK, if there is no high priority HARQ-ACK to be multiplexed in the CG PUSCH.

Proposal 7: CG-UCI multiplexed in a high priority CG PUSCH is treated as high priority HARQ-ACK and jointly encoded with high priority HARQ-ACK, if there is high priority HARQ-ACK to be multiplexed in the CG PUSCH.

# Conclusions

In this contribution, we discussed the issues regarding intra-UE multiplexing/prioritization. Based on the discussion in section 2, we have proposal as follows.

Proposal 1	The following option is adopted for determination of PUCCH resource set.

-  Option 2: UCI payload size = the number of HP UCI bits + the number of LP UCI bits * scaling factor.

Proposal 2	Further consider the following options to resolve the issue of DCI miss detection.

-  Option 3b: Configuration of semi-static size reservation for LP HARQ-ACK payload is provided by RRC. LP HARQ-ACK semi-static size reservation is used instead of determined LP HARQ-ACK codebook size when selecting the PUCCH resource set.

-  Option 4: Additional DCI field in DCI corresponding HP HARQ-ACK or HP PUSCH for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH.

-  Option 5: Provide indication on at least the number of RBs and/or PUCCH resource set index to be used in the PUCCH transmission, where the indication is included in the high-priority DL assignment.

- Proposal 3	Dynamic indication is supported for indicating whether to multiplex overlapping high priority PUCCH and low priority PUCCH.

- Proposal 4	For multiplexing LP HARQ-ACK and HP HARQ-ACK on a PUCCH format 0, HARQ-ACK values to CS indices mapping with unequal distance between mapped CS indices should be supported.

- Proposal 5	Separate beta offsets and scalings can be configured for low priority UCI multiplexed in low priority PUSCH, for low priority UCI multiplexed in high priority PUSCH, for high priority UCI multiplexed in low priority PUSCH, and for high priority UCI multiplexed in high priority PUSCH.

- Proposal 6	Dynamic indication is supported for indicating whether to multiplex overlapping high priority PUSCH and low priority PUCCH. FFS the indication method when semi-static beta offsets are configured.

- Proposal 7	CG-UCI multiplexed in a high priority CG PUSCH is treated as high priority HARQ-ACK and jointly encoded with high priority HARQ-ACK, if there is high priority HARQ-ACK to be multiplexed in the CG PUSCH.

# References

RAN1 Chairman’s Notes, RAN1 #105-e