3GPP TSG RAN WG1 #106-e		     R1-2107322

e-Meeting, Aug. 16th – Aug. 27th, 2021

Agenda item:	7.2.12

Source: 	Qualcomm Incorporated

Title: 	Remaining issues for 1Tx-2Tx switching

Document for:	Discussion/Decision

# Introduction

In the past couple RAN1 meetings, RAN1 discussed the technical solution to enable UL Tx switching together with CA-based SRS carrier switching. However, due to missing the pre-requisite agreements on SRS CR discussion, no progress was achieved. In this paper, we provide our views on this topic.

# UL Tx switching together with CA-based SRS switching

In R1-2103149, we clarified why the current specifications are with ambiguity and submitted draft text proposals to TS 38.214 to fully solve the issue when SRS carrier switching is configured together with UL Tx switching.

The blow text is the ambiguity analysis submitted in RAN1 #104b-e, and we copy it below for information.

We note that in the existing specification for SRS carrier switching, the following text is repeated multiple times: “… that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]”. In general, this text is ambiguous, and we propose to clarify it at least for UL Tx switching together with SRS carrier switching. In the following, we give an example where the ambiguity arises. The original intent of the text is to cover cases as the following example: The UE indicates the following CA band combination capabilities: - Band Combination 1: - Band X + Band Y DL CA together with Band X + Band Y UL CA, with 1 UL port in Band X and 2 UL ports in Band Y - Band Combination 2: - Band X + Band Y DL CA with no UL CA, together with SRS carrier switching with Band X being source and Band Y being target for the switching. Assume that this UE is being configured with CA and SRS carrier switching according to Band Combination 2. This UE will not require a switching gap for SRS carrier switching, since the simultaneous transmission doesn’t exceed the UEs UL CA capability as indicated in Band Combination 1. On the other hand, another UE that indicates Band Combination 2 capability but not Band Combination 1 capability would require a switching gap. Now consider the following case. The UE indicates the following CA band combination capabilities: - Band Combination 1: - Band X + Band Y DL CA together with Band X + Band Y UL CA, with 1 UL port in Band X and 2 UL ports in Band Y - Band Combination 3: - Band X + Band Y + Band Z DL CA together with Band X + Band Z UL CA with 1 UL port in Band X and 2 UL ports in Band Z, together with SRS carrier switching with Band X being source and Band Y being target for the switching. Now assume that this UE is being configured with DL and UL CA according to Band Combination 3. Suppose at a particular time instance, the UE is not configured to transmit in Band Z but is required to transmit PUSCH in Band X and SRS in Band Y at the same time. Strictly speaking, this instance would not exceed the UEs indicated UL CA capability as indicated in Band Combination 1. However, obviously this UE is not capable of simultaneous transmission in Band X and Band Y, since transmit chain(s) are committed to Band Z, even if there is no Band Z transmission is configured at the given instance. Therefore, there is an ambiguity in how to interpret what is exceeding a UEs UL CA capability. We propose to clarify this by adding an explanation that for the purposes of evaluating what exceeds the UEs UL CA capability, transmission on all configured UL CCs need to be assumed, irrespective of whether actual transmission is taking place at a given instance or not. We note that this clarification would be useful for the general case of CA with more than two CCs, but at least it should be clarified for the case of UL Tx switching together with SRS carrier switching involving three carriers.

During the email discussion in RAN1 #104b-emeeting, companies agreed that we could wait for the conclusion of email thread [104b-e-NR-7.1CRs -02] which is trying to solve similar ambiguity issue. However, the email thread [104b-e-NR-7.1CRs -02] didn’t conclude in RAN1 #104b-e.

In RAN1 #105e-meeting the dependent CR [105-e-NR-7.1CRs-12] failed to get final conclusion again. We objected to an incomplete proposal as some of the key issues are not  solved, for example if C3 is configured with UCI, it should be with higher priority than SRS of C1. We can’t find this statement in the current specification.

As Rel-16 UL Tx switching has been delayed for several meetings already and the ambiguity of SRS carrier switching might not be able to be solved in a short time, we propose to conclude that the combination of SRS carrier switching and UL Tx switching is not supported in R16. Furthermore, we would suggest solving the issues in Rel-17 for this combined feature.

Proposal 1: We propose to conclude that the combination of SRS carrier switching and UL Tx switching is not supported in Rel-16.

Proposal 2: We suggest solving the issues in Rel-17 for the combined feature of SRS carrier switching and UL Tx switching.

# Conclusions

As Rel-16 UL Tx switching has been delayed for several meetings already and the ambiguity of SRS carrier switching might not be able to be solved in a short time, we propose to conclude that the combination of SRS carrier switching and UL Tx switching is not supported in R16. Furthermore, we would suggest solving the issues in Rel-17 for this combined feature.

Proposal 1: We propose to conclude that the combination of SRS carrier switching and UL Tx switching is not supported in Rel-16.

Proposal 2: We suggest solving the issues in Rel-17 for the combined feature of SRS carrier switching and UL Tx switching.