3GPP TSG RAN WG1 #106-e	R1-2107330

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.2.1

Source: 	Qualcomm Incorporated

Title: 	Initial access aspects for NR to support operation between 52.6 GHz and 71 GHz

Document for:	Discussion/Decision

# Introduction

In RAN #90e meeting, a Rel-17 work item for NR operation in a frequency regime between 52.6 GHz and 71 GHz was approved, and the WID was further revised in RAN #92-e [1]. As part of the work item, the study on the physical layer aspects of initial access includes the following:

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion. - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum Note 5: FR2 is extended to cover 24.25GHz to 71GHz with FR2-1 for 24.25-52.6GHz and FR2-2 for 52.6-71GHz.

The Appendix lists the agreements made in RAN1#104e, RAN1#104b-e, and RAN1#105-e related to initial access.

In this contribution, based on the WID and the additional RAN1 agreements, we present our views on the initial access aspects for NR operation in the frequency regime between 52.6GHz and 71GHz, including design aspects for:

- SSB

- SSB and CORESET0 multiplexing patterns

- RACH

# Discussion

## SSB Design Aspects

### SSB Numerology

In RAN1#104b-e, the following agreement was made:

- For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

Note: Strive to minimize specification impact due to the new SCS for SSB

In RAN #92-e, the following agreements were made:

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

  - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access)

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

  - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

To summarize, the following observation can be made:

Observation 1: for FR2-2, the following numerologies are supported for the SSB (in addition to SCS 120 kHz):

- SSB SCS of 480 kHz for initial access case

- SSB SCS of 480 kHz and 960 kHz for non-initial access case

  - SSB location and SCS are explicitly provided to the UE

  - SSB does not configure Type-0 PDCCH

- SSB SCS of 480 kHz and 960 kHz for ANR case

- UE support of SSB SCS of 480 kHz and 960 kHz is optional

### SSB Pattern

According to the WID, up to 64 SSB beams is to be supported in this frequency range.

In RAN1#105-e, the following agreement was made:

For 480kHz/960kHz SSB, select one of the following alternatives:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

    - FFS: exact value of X and Y

- ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- Values of n for 480kHz and 960kHz for ALT 1 and 2

  - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT)

  - FFS: exact values of ‘n’ for each SCS

  - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs

  - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

For SSBs using SCS = 480 kHz or 960 kHz, a shorter symbol length means a shorter CP length. A gNB may need to have some time to switch from one beam to another which can be in the order of [100-200 ns] (this number is to be determined based on RAN4 response to LS [2]). In NR Rel 16, back-to-back SSBs from different beams was possible because the beam switching time can be absorbed in the CP and hence no explicit switching gap is needed (CP for SCS 120 and 240 kHz = 596 and 293 ns, respectively which is > [100 ns] needed). However, for higher SCS SSBs, the CP is smaller (CP for SCS 480 kHz and 960 kHz are 146.5 ns and 73.2 ns, respectively) and the beam switching time may be larger than the CP or takes a considerable portion of the CP. Hence, an explicit switching gap may be needed. Therefore, a simple extension of the existing patterns to the new numerologies may not work and a new pattern may be needed.

The gNB beam switching delay may be in the order of [100-200 ns]. The CP length for SCS 480 kHz and 960 kHz are 146.5 ns and 73.2 ns, respectively, while the usable symbols lengths are 2083.3 ns and 1041.7 ns, respectively. Therefore, a beam switching gap may be in the order of 1 symbol.

Hence, we prefer adopting ALT 1 in the above agreement.

Observation 2: explicit beam switching gap between SSBs with larger SCS (SCS = 480 kHz and 960 kHz) may be required. This beam switching gap may be of symbol-level (e.g., 1 symbol)

In addition, due to the larger SCS (hence shorter symbols), the DL/UL switching delay is not trivial and may take more than few symbols (e.g., [7.015 µsec] depending on the RAN4 response to LS [2]). Therefore, to account for UL segments within an SSB burst, a considerable number of symbols need to be reserved for DL/UL switching delays.

Observation 3: for larger SSB SCS (480 kHz and 960 kHz), accommodating UL segments within the SSB burst may require accounting for DL/UL switching delays taking considerable number of symbols

To accommodate DL/UL switching, it is recommended that the values of ‘n’ in the agreement above, shall not be all consecutive integer values.

In addition, control symbols (e.g., PDCCH) may be also included in the SSB slots. These control symbols may be generic and may not need to be associated with the corresponding SSB beam if the design allows for a gap between the control symbols and an SSB of different beams.

Another option is to have a self-contained structure where the SSB and the corresponding CORESET0 may be confined in the same SSB slot.

Some examples for an SSB slot are shown in Figure 1.

Figure 1: “SSB Slot” Examples

If 2 SSBs are allocated in an SSB slot, 32 SSB slots are needed for all 64 beams to be swept. In case all SSB slots are back-to-back with no gap slots in between, the 32 slots span 0.5 ms for 960 kHz SCS. While it is much faster than the 5 ms for SCS 120 kHz, the 0.5 ms may be too long and may block URLLC traffic. Hence, the design may consider having gap slots within the SSB slots of the burst. An example is shown in Figure 2.

Figure 2: SSB Pattern with “Gap Slots” Example

The number of SSB slots and the gap slots in a burst/span may be such that:

- The burst/span is small enough so the DMTC window is short (not longer than Rel-16)

- The number of gap slots is reasonable for DL/UL transmission TDM multiplexing

- The pattern of SCS 480 and 960 kHz matches in time domain (i.e., number of SSB/gap slots for SCS 960 = 2 x number of SSB/gap slots for SCS 480)

Based on the above discussions and observations, the following can be proposed:

Proposal 1: for 480kHz/960kHz SSB, select the following alternative:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

  - X = 2 and Y = 9

- Values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

### DBTW Related Aspects

In RAN1#105-e, the following was agreed:

FFS: Support DBTW at least for 120kHz

- FFS whether DBTW will be applicable for 480/960 kHz SSB SCS

  - If DBTW is supported for 480/960kHz SSB:

    - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW,  NSSBQCL, and DBTW length) are supported by dedicated signaling.

- For 120kHz SSB, support mechanism to distinguish at least the following scenarios:

  - Case 1) (Unlicensed with LBT off) + DBTW disabled

  - Case 2) (Unlicensed with LBT on) + DBTW enabled

  - Case 3) (Unlicensed with LBT on) + DBTW disabled

  - Case 4) (Licensed) + DBTW disabled

  - FFS: Whether/how LBT on/off is indicated in MIB

    - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI

  - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling

  - FFS: whether all above cases need an explicit indication

  - FFS: Whether a single indication can be used for combination of more than one cases

- For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods:

  - Option 1) signaling in MIB

    - Option 1-1) disabling DBTW is jointly coded with NSSBQCL

    - Option 1-2) indicated by other bit fields in MIB

    - FFS: among options 1-1 and 1-2

  - Option 2) distinct GSCN used by the SSB

  - Option 3) By comparing the value of  NSSBQCL in MIB and DBTW length after UE reads SIB1 or by comparing the value of  NSSBQCL in MIB and default DBTW length of 5 ms before UE reads SIB1.

  - FFS: whether to support option 1, 2, 3, or any combination of the options.

  - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods

If DBTW is supported,

- Working assumption: MIB signaling to support

  - Alt A) indication of NSSBQCL at least for 120kHz SSB

    - In this case, the total number of values of NSSBQCL to not exceed 4

  - Alt B) Explicit indication of SSB index and/or SSB candidate location

    - FFS on the details of signaling

  - FFS between Alt A, or B, or supporting both

- Supported DBTW lengths

  - Alt 1) 0.5, 1, 2, 3, 4, 5 msec

    - Note: same as Rel-16 FR1 NR-U

  - Alt 2) maximum 5 msec

    - FFS other values

  - FFS between Alt 1 and 2

- Number of candidate positions when DBTW is enabled

  - For 120kHz SSB

    - FFS between 64 or 80

  - If DBTW is additionally supported for 480/960kHz SSB

    - FFS between 64 or 128

A transmission may be classified as a short control signal transmission if its duration does not exceed 10 ms within an observation period of 100 ms. For SCS = 480 kHz and 960 kHz, assuming 64 beams and SSB periodicity of 20 ms, the SSB transmission duration satisfies the allowed limit to classify as short control signal transmission.

Proposal 2: do not support discovery burst transmission window (DBTW) for SSB for SCS 480 and 960 kHz

For SCS 120 kHz, although there may not be a strong motivation to support SSB DBTW, in the following we present our views in the case it is concluded that it will be supported.

Based on the agreement from RAN1#104b-e, the changes needed to support the DBTW should not include any additional PBCH payload or any additional DMRS sequences. This means that the bits needed to signal the Q factor and to enable/disable DBTW should come from either the existing fields or by reinterpreting some values in the existing fields. In both cases, some restrictions may be needed to the existing fields. Thus, to reduce this restriction impact, it may be desirable to reduce as much as possible the number of bits needed (e.g., to 1 or 2 bits) for Q and thus the allowed values (e.g., to 2 or 4 values).

In addition, there may not be a need to explicitly reserve bits for DBTW enabling/disabling as it can be implicitly assumed from the Q factor value.

Consider the 4-bits used to signal controlResourceSetZero. We can restrict it to be only 3-bits (8 possible values) to spare one bit for Q. In the current specs, only 3 bits are used for 120 kHz SCS SSB for the valid combinations for this WI. However, this may not become available if more combinations are supported (e.g., 96 RBs).

The second bit (if needed) may come from the 4-bits for searchSpaceZero field. For multiplexing patterns 2 and 3, only one value is used, hence, 1 bit is easily available. For multiplexing pattern 1, restrictions to the available configurations may be needed to get the 1 extra bit.

Another option to get the second bit (if needed) is from ssb-SubcarrierOffset by restricting the available values.

One more option to consider is the subCarrierSpacingCommon bit, since now SCS of the SSB = SCS of the CORESET0.

As for the number of candidate SSB positions per DBTW, there may not be a strong motivation to increase it from the available 64 for FR2, i.e., there may not be a need to define new candidate SSB positions outside the FR2 Case D pattern. This is mainly to maintain the gaps needed for UL and/or URLCC traffic.

The following however may be considered: the gNB may do an LBT before an SSB burst:

- If the LBT passes, then the gNB sends all SSBs in the burst​

- If the LBT fails, the gNB may send the SSBs by dividing the SSB positions in an SSB burst into the following subsets:​

  - Subset 1: includes SSB beams that are transmitted under the short control signal assumption​

  - Subset 2: includes SSB beams that may be transmitted​

    - Option 1: one SSB candidate position can be assigned per SSB ​

      - If LBT fails on 1 opportunity, no transmission occurs for that beam in this burst​ (best effort)

    - Option 2: more than 1 SSB candidate positions can be assigned per SSB ​

      - If LBT fails on 1 opportunity, other opportunities are available in the burst​ (requires Q)

Proposal 3: for an unlicensed band that requires LBT, if DBTW for SSB is adopted for 120 kHz SSB:

- MIB signaling to support indication of NSSBQCL for 120 kHz SSB

- Minimize the number of bits needed to signal NSSBQCL (1 or 2 bits) and thus the values (2 or 4 values)

- Enabling/disabling DBTW can be implicit in the NSSBQCL value

- Consider getting the bits needed from one or more of the following: controlResourceSetZero, searchSpaceZero, ssb-SubcarrierOffset, subCarrierSpacingCommon

- Number of candidate positions when DBTW is enabled = 64 for 120 kHz SSB

- Consider having a subset of the SSBs (< 64) transmitted under the short control signal assumption while another subset can be best effort or have multiple positions per beam (have a NSSBQCL within the subset)

In NR Rel-15/16, fallback DCI 0_0 and 1_0 contain a field that is only specific for NR-U.

From 38.212: “ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise”

It means that the DCI size is different between NR-U and NR. In the current NR, however, the UE would know that the system is NR vs NR-U from the frequency band. This may not be possible in FR2-2 since the band is common. If nothing is done in FR2-2, the UE would need to decode DCI using 2 possible sizes which adds complexity to the UE.

To mitigate this, we can consider increasing the size of the DCI 0_0 and 1_0 for NR licensed to align with the size of the corresponding DCIs for the NR-U. In this case the UE would only do a single size hypothesis.

Proposal 4: consider increasing the size of the DCI 0_0 and 1_0 for NR licensed, by adding a field, to align with the size of the corresponding DCIs for the NR-U

### ANR NCGI Reporting

The following agreement was made in RAN #92-e:

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

  - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting

The NCGI is used a global identifier to resolve any possible confusion between cells having the same PCI. The NCGI is sent on SIB1 of the neighbor cell, i.e., the UE (if instructed to report the NCGI) it has to read the neighbor's SIB1 (which is scheduled by the SSB/CORESET0). Current NR support is enough and there may not be a need to have any additional methods to support signaling the NCGI.

Proposal 5: for ANR, do not consider additional methods (compared to current NR) to signal the NCGI

## SSB/CORESET0 Multiplexing Aspects

### Allowed SCS combinations

The following was agreed in RAN #92-e:

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

  - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960).

Based on the agreements, the following observation can be made:

Observation 4: for FR2-2, CORESET0 SCS = SSB SCS for all SCSs

### SSB/CORESET0 Multiplexing Patterns

The following was agreed in RAN #92-e:

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

For higher SCS SSB, as discussed earlier, beam switching gaps may be needed. This “explicit” beam switching gap may become an overhead and ways to reduce this effect may be desirable.

Observation 5: for higher SCS SSBs, explicit beam switching gaps may be needed adding additional overhead, hence ways to reduce this overhead may be needed

One of the ways to reduce this overhead is to use multiplexing pattern 3 where the SSB and CORESET0/RMSI are FDMed and beam switching gaps are not duplicated (e.g., between different beam SSBs and between different beam CORESET0’s).

Another way to consider is grouping (TDMing) the SSB and CORESET0 symbols where the grouping may help reduce the gap overhead/duplication. In section 2.1.2, we proposed the possibility that control symbols may be multiplexed with SSB in the same SSB slot (example 2 in Figure 1). This design may have several advantages: e.g., beam switching gaps may be minimized, self-contained structure, and generic control symbols may be multiplexed with the SSB.

Proposal 6: consider minimizing the overhead of beam switching gaps by:

- Supporting multiplexing pattern 3

- Introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where TDM grouping of the SSB and the corresponding CORESET0 is considered

## RACH Design Aspects

### PRACH Sequence Length

Similar to unlicensed band in FR1, there will be a PSD limit and an EIRP constraint in FR2-2. For example, in use cases other than outdoor, there is PSD limit of 23 dBm/MHz with up to 40 dBm EIRP. While some mobile terminals may have TRP of 23 dBm, some other user terminals (e.g., CPE) may be able to operate at higher EIRP up to 40 dBm. For these high capability UEs, to fully take advantage of the 40 dBm EIRP limit, ~50 MHz transmission bandwidth is needed under the 23 dBm/MHz PSD limit.

To accommodate this, an increase of the BW of the PRACH preamble may be considered, and a longer sequence length (compared to FR2) was considered. In RAN1#104-e, it was agreed that “for initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.”

However, for higher SCS PRACH, the agreement was: “For non-initial access use cases, if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively. FFS: support of sequence length L = 571, 1151.”

Table 1 shows the required PRACH preamble BW for multiple SCS and sequence lengths (LRA) options.

Table 1: PRACH Preamble BW

| SCS (kHz) | LRA | BW (MHz) |
| --- | --- | --- |
| 120 | 139 | 16.7 |
|  | 571 | 68.5 |
|  | 1151 | 138.1 |
| 480 | 139 | 66.7 |
|  | 571 | 274.1 |
|  | 1151 | 552.5 |
| 960 | 139 | 133.4 |
|  | 571 | 548.2 |
|  | 1151 | 1105.0 |

Observation 6: SCS = 480/960 kHz with sequence length = 139 is enough to achieve the desired BW requirement for the maximum EIRP allowed

Proposal 7: consider only using PRACH sequence length = 139 for SCS = 480 kHz and 960 kHz

### RO Considerations

In RAN1#104b-e, the following agreement was made:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

The following agreement was further done in RAN1#105-e:

For 480kHz and 960kHz PRACH,

- Down-select among option 1 and 2

  - Option 1) The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

    - FFS: supported values of the starting PRACH slot index nslotRA  within reference slot and whether or not the ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

  - Option 2) Each 120kHz RO corresponds to 4 and 8 candidate RO positions for 480kHz and 960kHz PRACH, respectively. Information about the number and locations of 480/960kHz candidate RO(s) are configured or pre-selected within each 120kHz RO. The reference 120kHz RO is determined by the current PRACH configuration method in Rel-15/16 specification.

- Following alternatives are considered on PRACH density

  - ALT 1) At least the same density (i.e. number of PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher PRACH slot density (number of PRACH slots per reference slot)

  - ALT 2) at least the same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher RO density

  - An “example” illustration of PRACH slots for 480/960kHz is shown below:

- FFS: whether and how to account for LBT in RO configuration (if needed)

- FFS: whether and how to account for beam switching gap in RO configuration (if needed)

In NR Rel-16, up to 8 RACH occasions (RO) can be FDM’ed. For higher bands, as agreed in RAN1#104-e, a longer preamble sequence length of 571 or 1151 may be used for SCS = 120 kHz and if all 8 FDM’ed ROs are configured, the overall BW would be 548 MHz and 1.105 GHz, respectively, which are higher than the 400 MHz maximum BW for SCS 120 kHz. Hence:

Proposal 8: a maximum of 4 and 2 FD multiplexed ROs for SCS = 120 kHz and sequence length = 571 and 1151, respectively

Due to the possible reduction in the maximum number of FDM’ed ROs, more time domain ROs may be needed to maintain the same RACH capacity​. We can have several options to do this:

- Option A: Re-use the existing design but use larger association period​

  - This may slow down initial access​ and increase UE power consumption

- Option B: Explicitly add more reference slots in a configuration period in Table 6.3.3.2-4 in TS 38.211​

  - Non-trivial spec work/time​

- Option C: Add more reference slots in a configuration period by:

  - Alt 1: adding N additional slots every M reference slot​

    - Reuse existing Table 6.3.3.2-4 in TS 38.211​ (minimal spec impact)

    - N and M can be specified or indicated​

    - Example: PRACH Config. Index 0:​

      - Current table: Slot number = 4,9,14,19,24,29,34,39​

      - New values (N = 1, M = 2): Slot number = 4,5, 9,14,15,19,24,25, 29,34,35,39​

  - Alt 2: adding one or more offseted version(s) (offset = L) of the slot number pattern to the existing one​

    - Reuse existing Table 6.3.3.2-4 in TS 38.211​ (minimal spec impact)

    - L can be specified or indicated and can be either added or subtracted to the existing slot number​

    - Example: PRACH Config. Index 0:​

      - Current table: Slot number = 4,9,14,19,24,29,34,39​

      - New values (L = 2): Slot number = 2,4,7,9,12,14,17,19,22,24,27,29,32,34,37,39​

Proposal 9: for SCS = 120 kHz, if the maximum number of FD ROs are reduced, consider ways to increase the TD ROs (to maintain the same capacity) with minimal specification impact

In the time domain, the same argument presented for the SSB beam switching delays may apply for PRACH. NR Rel-16 can multiplex RO’s back-to-back because the gNB beam switching delay can be absorbed by the CP (i.e., the CP is long enough to absorb the beam switching delays). However, in higher bands and SCS (e.g., SCS = 480 and 960 kHz), the CP length may not be long enough to absorb this beam switching delay requirement.

Observation 7: for higher RACH SCS (480 and 960 kHz), the gap and CP length may not be long enough to absorb the gNB beam switching delay requirement

To mitigate this issue, the following options may be considered:

- Option A: Do not include an explicit gap (reuse NR Rel16 design)

  - Some PRACH symbol repetitions may be lost

  - Leave it to gNB implementation on how to handle any “possible” repetition preamble losses

- Option B: Include a PRACH symbol level gap between ROs

  - This makes sure all repetitions are usable by the gNB and the coverage can be predicted

- Option C: Include a RO-level gap between ROs

  - This has larger gaps between ROs

  - May have an advantage of scheduling other UL traffic (e.g., for other users) in the gap

  - gNB may also chose to interlace the gaps and ROs in adjacent cells to reduce interference

To reduce the initial access delay and possible losses, and to reduce any possible gap overhead, option B is preferred.

Proposal 10: for higher RACH SCS (480 and 960 kHz), consider including a symbol-level gap between ROs to allow for gNB beam switching delay

Figure 3 shows examples for introducing symbol level gaps between ROs of different beams. This can be done by introducing N ≥ 1 symbol-level gaps between ROs:

- Option A: TDM "RO + gap" until all required number of ROs are satisfied (even if they extend to next slot)

- Option B: split the number of ROs evenly among multiple slots such that the pattern is the same for all slots (distribute the "RO + gap" among slots)

- Option C: use the current starting position of first RO, keep adding "RO + gap" until the last symbol of the original last RO pattern without gap

  - The span of ROs is within the original design (i.e., will not extend to the next slot)

  - However, may have less ROs

Figure 3: Symbol-level Gaps Between ROs Examples

Regarding the number of 480/960 kHz RACH slots within a RACH reference slot, consider the following situations:

- A longer PRACH format may be needed (e.g., for coverage). In this case, the same number of ROs defined for the 120 kHz SCS cannot be contained in the reference slot. Therefore, having the flexibility of going beyond 2 RACH slots per reference slot can be useful

- If beam switching and/or LBT gaps are introduced, the number of ROs (per RACH slot) that was defined for 120 kHz may not fit within the RACH slot, and the ROs may need to extend to a following slot​. Hence, the currently defined 1 or 2 RACH slots in a RACH reference slot may not be enough for the higher SCSs

The 2 situations above may not necessarily lead to an increase of RACH processing load. Also, these may lead to RO extension into more RACH slots. Hence, to maintain the RACH density, the same ROs in a reference slot need to be maintained (not the RACH slots which may increase).

Observation 8: for higher RACH SCS (480 and 960 kHz), due to gaps and/or coverage enhancement needs, more than 2 RACH slots per RACH reference slots may be needed. This may not necessarily lead to an increase of RACH processing load if the number of ROs in a reference slot is maintained

For the reference slot duration, to have a unified approach for other FRs and to minimize the specification changes, using 60 kHz reference slot SCS may be desirable.

From the argument above, the following proposals can be made:

Proposal 11: for 480kHz and 960kHz PRACH:

- The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA, corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot

- ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

- The same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

# Conclusions

In this contribution, based on the WID, we presented our views on the initial access aspects for NR operation in the frequency regime between 52.6GHz and 71GHz, including design aspects for:

- SSB

- SSB and CORESET0 multiplexing patterns

- RACH

The following observations were made:

Observation 1: for FR2-2, the following numerologies are supported for the SSB (in addition to SCS 120 kHz):

- SSB SCS of 480 kHz for initial access case

- SSB SCS of 480 kHz and 960 kHz for non-initial access case

  - SSB location and SCS are explicitly provided to the UE

  - SSB does not configure Type-0 PDCCH

- SSB SCS of 480 kHz and 960 kHz for ANR case

- UE support of SSB SCS of 480 kHz and 960 kHz is optional

Observation 2: explicit beam switching gap between SSBs with larger SCS (SCS = 480 kHz and 960 kHz) may be required. This beam switching gap may be of symbol-level (e.g., 1 symbol)

Observation 3: for larger SSB SCS (480 kHz and 960 kHz), accommodating UL segments within the SSB burst may require accounting for DL/UL switching delays taking considerable number of symbols

Observation 4: for FR2-2, CORESET0 SCS = SSB SCS for all SCSs

Observation 5: for higher SCS SSBs, explicit beam switching gaps may be needed adding additional overhead, hence ways to reduce this overhead may be needed

Observation 6: SCS = 480/960 kHz with sequence length = 139 is enough to achieve the desired BW requirement for the maximum EIRP allowed

Observation 7: for higher RACH SCS (480 and 960 kHz), the gap and CP length may not be long enough to absorb the gNB beam switching delay requirement

Observation 8: for higher RACH SCS (480 and 960 kHz), due to gaps and/or coverage enhancement needs, more than 2 RACH slots per RACH reference slots may be needed. This may not necessarily lead to an increase of RACH processing load if the number of ROs in a reference slot is maintained

The following proposals can be made:

Proposal 1: for 480kHz/960kHz SSB, select the following alternative:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

  - X = 2 and Y = 9

- Values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

Proposal 2: do not support discovery burst transmission window (DBTW) for SSB for SCS 480 and 960 kHz

Proposal 3: for an unlicensed band that requires LBT, if DBTW for SSB is adopted for 120 kHz SSB:

- MIB signaling to support indication of NSSBQCL for 120 kHz SSB

- Minimize the number of bits needed to signal NSSBQCL (1 or 2 bits) and thus the values (2 or 4 values)

- Enabling/disabling DBTW can be implicit in the NSSBQCL value

- Consider getting the bits needed from one or more of the following: controlResourceSetZero, searchSpaceZero, ssb-SubcarrierOffset, subCarrierSpacingCommon

- Number of candidate positions when DBTW is enabled = 64 for 120 kHz SSB

- Consider having a subset of the SSBs (< 64) transmitted under the short control signal assumption while another subset can be best effort or have multiple positions per beam (have a NSSBQCL within the subset)

Proposal 4: consider increasing the size of the DCI 0_0 and 1_0 for NR licensed, by adding a field, to align with the size of the corresponding DCIs for the NR-U

Proposal 5: for ANR, do not consider additional methods (compared to current NR) to signal the NCGI

Proposal 6: consider minimizing the overhead of beam switching gaps by:

- Supporting multiplexing pattern 3

- Introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where TDM grouping of the SSB and the corresponding CORESET0 is considered

Proposal 7: consider only using PRACH sequence length = 139 for SCS = 480 kHz and 960 kHz

Proposal 8: a maximum of 4 and 2 FD multiplexed ROs for SCS = 120 kHz and sequence length = 571 and 1151, respectively

Proposal 9: for SCS = 120 kHz, if the maximum number of FD ROs are reduced, consider ways to increase the TD ROs (to maintain the same capacity) with minimal specification impact

Proposal 10: for higher RACH SCS (480 and 960 kHz), consider including a symbol-level gap between ROs to allow for gNB beam switching delay

Proposal 11: for 480kHz and 960kHz PRACH:

- The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA, corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot

- ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

- The same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

# References

[1] RP-211584, “Revised WID: Extending current NR operation to 71GHz”, RAN #92e, Qualcomm, Intel

[2] R1-2102202, “LS on beam switching gap for 60 GHz band”, RAN1 #104-e

# Appendix

## RAN1#104-e Agreements

Agreement:

Send an LS to RAN4 to get input on gap required for gNBs and UEs for beam switching and for UL/DL and DL/UL switching.

Agreement:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Agreement:

For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS

- If DB supported

  - FFS: What signals/channels are included in DB other than SS/PBCH block

- If DBTW is supported

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

- The following points are additionally FFS:

  - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size

  - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

  - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

Agreement:

For CORESET#0 and Type0-PDCCH search space configured in MIB:

- Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz

  - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz.

    - FFS: Supporting additional values

  - FFS: Supported values for SSB to CORESET#0 offset RBs

- If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz

- If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz

- If 240 kHz SSB SCS is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz

- FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960)

  - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz)

Agreement:

For 480 kHz and 960 kHz SSB SCS (if agreed)

- Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels)

  - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz.

- Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern

- Study should account for inputs from RAN4

Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

- For non-initial access use cases,

  - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively.

    - FFS: support of sequence length L = 571, 1151

- FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access

Agreement:

If 480 and/or 960 kHz PRACH SCS is supported, RAN1 should study whether or not the current RA-RNTI calculation and PRACH identification in RAR correctly provides unique identification of PRACH.

## RAN1#104b-e Agreements

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

## RAN1#105-e Agreements

Agreement:

For 480kHz/960kHz SSB, select one of the following alternatives:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

    - FFS: exact value of X and Y

- ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- Values of n for 480kHz and 960kHz for ALT 1 and 2

  - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT)

  - FFS: exact values of ‘n’ for each SCS

  - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs

  - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

Agreement:

For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access)

- Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives

  - Alt 1) Using dedicated signaling

  - Alt 2) Using configuration in MIB

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Agreement:

For 480kHz and 960kHz PRACH,

- Down-select among option 1 and 2

  - Option 1) The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index,  , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

    - FFS: supported values of the starting PRACH slot index  within reference slot and whether or not the ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

  - Option 2) Each 120kHz RO corresponds to 4 and 8 candidate RO positions for 480kHz and 960kHz PRACH, respectively. Information about the number and locations of 480/960kHz candidate RO(s) are configured or pre-selected within each 120kHz RO. The reference 120kHz RO is determined by the current PRACH configuration method in Rel-15/16 specification.

- Following alternatives are considered on PRACH density

  - ALT 1) At least the same density (i.e. number of PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher PRACH slot density (number of PRACH slots per reference slot)

  - ALT 2) at least the same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher RO density

  - An “example” illustration of PRACH slots for 480/960kHz is shown below:

- FFS: whether and how to account for LBT in RO configuration (if needed)

- FFS: whether and how to account for beam switching gap in RO configuration (if needed)

Agreement:

FFS: Support DBTW at least for 120kHz

- FFS whether DBTW will be applicable for 480/960 kHz SSB SCS

  - If DBTW is supported for 480/960kHz SSB:

    - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW,  , and DBTW length) are supported by dedicated signaling.

- For 120kHz SSB, support mechanism to distinguish at least the following scenarios:

  - Case 1) (Unlicensed with LBT off) + DBTW disabled

  - Case 2) (Unlicensed with LBT on) + DBTW enabled

  - Case 3) (Unlicensed with LBT on) + DBTW disabled

  - Case 4) (Licensed) + DBTW disabled

  - FFS: Whether/how LBT on/off is indicated in MIB

    - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI

  - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling

  - FFS: whether all above cases need an explicit indication

  - FFS: Whether a single indication can be used for combination of more than one cases

- For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods:

  - Option 1) signaling in MIB

    - Option 1-1) disabling DBTW is jointly coded with

    - Option 1-2) indicated by other bit fields in MIB

    - FFS: among options 1-1 and 1-2

  - Option 2) distinct GSCN used by the SSB

  - Option 3) By comparing the value of   in MIB and DBTW length after UE reads SIB1 or by comparing the value of   in MIB and default DBTW length of 5 ms before UE reads SIB1.

  - FFS: whether to support option 1, 2, 3, or any combination of the options.

  - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods

Agreement:

If DBTW is supported,

- Working assumption: MIB signaling to support

  - Alt A) indication of  at least for 120kHz SSB

    - In this case, the total number of values of  to not exceed 4

  - Alt B) Explicit indication of SSB index and/or SSB candidate location

    - FFS on the details of signaling

  - FFS between Alt A, or B, or supporting both

- Supported DBTW lengths

  - Alt 1) 0.5, 1, 2, 3, 4, 5 msec

    - Note: same as Rel-16 FR1 NR-U

  - Alt 2) maximum 5 msec

    - FFS other values

  - FFS between Alt 1 and 2

- Number of candidate positions when DBTW is enabled

  - For 120kHz SSB

    - FFS between 64 or 80

  - If DBTW is additionally supported for 480/960kHz SSB

    - FFS between 64 or 128