| 3GPP TSG-RAN WG1 Meeting #106 e-Meeting, August 16th – 27th, 2021 | R1-2107351 |
| --- | --- |

Agenda item:	8.6.1.1

Source: 	Qualcomm Incorporated

Title: 	BW Reduction for RedCap UE

Document for:	Discussion/Decision

# Introduction

To meet the generic and use case specific requirements of mid-range use cases, potential UE complexity reduction techniques were identified and analyzed for wearables, industry wireless sensors and surveillance cameras [1]. A revised WID [2] was approved in RAN-92e meeting, and a main objective of the WI is to specify the support for the following UE complexity reduction features in RAN1, RAN2 and RAN4:

- Reduced maximum UE bandwidth:

  - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz

In RAN1#105, the following agreements were made for the non-initial BWP of RedCap UE in FR1:

Agreements:

- A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE.

  - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability.

    - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs.

Besides, a clarification was made for the initial UL BWP of RedCap UE and non-RedCap UE when the carrier BW is wider than the max BW of RedCap UE:

Agreements:

- Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed.

In this contribution, we continue to discuss the BWP configuration/operation rules for R17 RedCap UE during and after initial access. UE’s complexity issues related to FG 6-1a and resource fragmentation mitigation of non-RedCap UE are also studied.

# Supporting BW Reduction of RedCap UE in FR1

## Initial BWP Configuration

### CORESET0 and Initial DL BWP

In FR1, 20 MHz is the maximum BW supported by a RedCap UE during and after initial access [2].  Since the max BW of SSB, CORESET0 and SIB1 of non-RedCap UE are contained within 20 MHz during initial access,  RedCap UE and non-RedCap UE can share the same SSB, CORESET0 and SIB1. Since the initial DL BWP of non-RedCap UE is defined by CORESET0, RedCap UE and non-RedCap UE can also share the initial DL BWP during initial access. The other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

Proposal 1: During initial access, RedCap and non-RedCap UEs share the SSB, CORESET#0, SIB1 and initial DL BWP.

Proposal 2: Other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

### PRACH and Initial UL BWP

In NR R15/16, a non-RedCap UE is provided an initial UL BWP configuration by UplinkConfigCommonSIB IE in SIB1 and the BW of initial UL  BWP can be wider than 20 MHz. When msg3 or PUCCH (HARQ feedback for msg4) is scheduled during initial access, a RedCap UE cannot interpret the UL grant appropriately, if the FDRA and frequency hopping  are determined with respect to an initial UL BWP wider than 20 MHz. Therefore, when a RedCap UE is allowed to access a cell whose initial UL BWP for non-RedCap UE is wider than 20 MHz, it is necessary for the network to identify the RedCap UE before sending the RAR grant, so that the RedCap UE can be scheduled appropriately with a valid UL grant. To facilitate early identification of RedCap UE before msg2/msg3, dedicated PRACH resources should be allocated for RedCap UE within its initial UL BWP. Furthermore, with early indication in dedicated PRACH resources, BS could estimate the amount of coverage recovery needed on UL and DL,  since the pathloss between BS and RedCap UE is a function of RTT/TA.

Observation 1: By configuring dedicated PRACH resources for RedCap UE, NW is able to identify the RedCap UE during initial access. Based on the PRACH preamble transmitted by RedCap UE, NW is able to:

- detect the presence of RedCap UE

- schedule msg3/PUCCH (HARQ feedback for msg4/msgB) properly within the initial UL BWP of RedCap UE

- enable coverage recovery necessary for msg2/msg3/msgB/msg4/PUCCH by re-using the solutions available in NR R15/16 and R17 coverage enhancement WI

In NR R15/16, the initial UL and DL BWPs are required to be aligned at center frequency in TDD bands. In NR R17, RedCap UE should follow the same rule for BWP configuration. In addition, the RACH occasions (ROs) configured for RedCap UE should be contained within its initial UL BWP, as shown in Figure 1.

Figure 1: Initial UL BWP and RO Configuration for RedCap UE

For the example shown in Figure 2, we assume the initial UL BWP of non-RedCap UE is 40 MHz, and there are 8 FDMed ROs shared by all UEs (non-RedCap and RedCap), which spans 17.28 MHz for SCS 15 kHz and 34.56 MHz for SCS 30 kHz. If the starting PRB of RedCap UE’s initial UL BWP is aligned with that of non-RedCap UE, and RedCap UE needs to select a RO/PRACH preamble outside its UL BWP (e.g. M>3), it is infeasible for RedCap UE to reach the desired RO through center frequency change/retuning. To illustrate, Figure 2 shows three BWPs (A, B and C) with 20 MHz BW. In particular, BWP B and C are obtained by retuning/switching the center frequency of initial BWP. However, the two BWPs partially overlap with the ROs configured by NW. Therefore, there is a conflict in the RO configurations for BWP B, C and the initial UL BWP of non-RedCap UE. To transmit on the desired RO, RedCap UE can switch to BWP A, which requires spec change in RAN1/RAN2/RAN4 to support BWP switching during initial access.

Figure 2: RO Configuration/Selection Outside Initial UL BWP of RedCap UE

Observation 2: Fast RF switching (changing center frequency of initial UL BWP only) cannot solve the RO selection issue of RedCap UE, if the ROs configured for RedCap UE are outside its initial UL BWP.

To illustrate the increase of UE complexity and power, Figure 3 shows the UL BWP switching during 4-step RACH. If RedCap UE is configured with a “virtual” BWP wider than its max BW (e.g. virtual UL BWP of RedCap UE = initial UL BWP of non-RedCap UE shown in Figure 3) and BWP switching is allowed during initial access, RedCap UE needs to perform multiple retuning/switching in UL and DL before being able to establish a RRC connection with NW. Due to the spec impacts and complexity increase for RedCap UE, BWP switching/retuning should NOT be allowed during initial access.

Figure 3: BWP Switching/Retuning Increases RedCap UE’s Complexity and Power Consumption

Observation 3: Specifying multiple initial UL BWPs for RedCap UE and allowing BWP switching during initial access will increase the complexity and power consumption of UE.

Observation 4: When the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE, NW should configure a separate initial UL BWP for RedCap UE and ensure the ROs configured for RedCap UE are within the initial UL BWP of RedCap UE.

Observation 5: PRACH resources valid for RedCap UE should be configured within its initial UL BWP. By configuration, NW should avoid/reduce the overlapping between SSB and RO configured for RedCap UE.

Proposal 3: Confirm the working assumption for initial UL BWP configuration of RedCap UE during initial access. A RedCap UE is NOT configured with an initial UL BWP wider than its max BW.

Proposal 4: Confirm the working assumptions for initial UL BWP configuration of RedCap UE when the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE is separately configured

- PRACH resources dedicated to RedCap UE is configured within its initial UL BWP to enable early indication in msg1 (or msgA preamble, if 2-step RACH is supported)

- the initial UL BWP of RedCap UE can be nested within the initial UL BWP of non-RedCap UE

- the initial UL BWP of RedCap UE can be configured by SIB1, or by rules defined in standards

Proposal 5: When RedCap UE and non-RedCap UE co-exist in the same cell, and the initial UL BWP of non-RedCap UE is no wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE and non-RedCap UE can be shared, or separately configured

- dedicated PRACH resources for RedCap UE can be configured within its initial UL BWP,  to enable early indication in msg1 (or msgA preamble, if 2-step RACH is supported)

Figure 4: BWP Switching/Retuning Impacts RedCap UE’s Timeline and Co-existence of Different UE Types

In studying the RF impacts of BWP adaptation [3], RAN4 concluded that for intra-band operation in FR1, transition time of 50~ 200 usec is needed for the center frequency change of BWP/RF retuning. The retuning time impacts the timeline of UE during and after initial access. To illustrate, Figure 4 shows UE’s timeline for msg1/msg3/PUCCH transmissions in 4-step RACH, wherein the minimum values for Tgap, Toffset, k2 and k1 have been specified in NR R15/16 for non-RedCap UEs without accounting for the extra time required for BWP/RF retuning. If BWP/RF retuning is allowed for RedCap UE, the timeline for UL transmission and DL reception need to be re-visited in NR R17. As a result, multiplexing of different UE types on DL and UL become problematic, although there is no relaxation for processing capabilities (N1 and N2) of R17 RedCap UE. It is worth noting the RF/BWP retuning incurs switching gaps on both directions (DL to UL switching, UL to DL switching), which is similar to Type-B HD-FDD of LTE and has been excluded from the WI objectives. Such standardization efforts are not justified for RedCap UE, especially when it leads to increase of UE’s complexity and power consumption.

Furthermore, channel reciprocity is a feature inherent to TDD system. By keeping the DL BWP and its pairing UL BWP aligned at the center frequency, channel reciprocity can be exploited by both UE and NW in TDD. Therefore, the UE can estimate UL channel based on DL measurements, and NW can estimate DL channel by measuring UL without requesting additional feedback. However, channel reciprocity is lost in TDD operation of RedCap UE, if the center frequencies of its DL/UL BWP pair are not aligned as shown by Figure 5. As a consequence of the loss in channel reciprocity, latency and signaling overhead will increase for TDD operation of RedCap UE.

Figure 5: Channel Reciprocity in TDD vs Center Frequency of DL/UL BWP

Observation 6: Changing center frequency of  DL/UL BWP in TDD leads to:

- loss of channel reciprocity

- increased latency similar to Type-B HD-FDD of LTE

- increased UE complexity

- non-trivial spec impacts and increased workload for RAN1/2/4

Proposal 6: Fast RF/BWP retuning is not supported for RedCap UE during initial access.

Proposal 7: During initial access, a RedCap UE operating on unpaired spectrum does not expect to receive a configuration where the center frequency for an initial DL BWP is different than the center frequency for an initial UL BWP.

Similar to NR R15/16 non-RedCap UEs, RedCap UE can re-use the long (LRA=839) and short (LRA=139) PRACH preamble sequences. To ensure the ROs of RedCap UE can be associated with the best/appropriate SSB beams, gNB can configure dedicated PRACH resources for RedCap UE within its initial UL BWP. When the initial UL BWP of non-RedCap UE is wider than 20 MHz, the RRC parameters for RACH procedure of RedCap UE, such as msg1-FDM, msg1-FrequencyStart, ra-ResponseWindow, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and SSB-to-RO association pattern period can be separately configured when necessary. Therefore, we have the following proposals:

Proposal 8: RedCap UE re-uses the long and short PRACH preamble sequences introduced in NR R15.

- In FD-FDD or Type-A HD-FDD operation, RedCap UE re-use the PRACH configurations for FR1 on paired spectrum (Table 6.3.3.2-2, TS 38.211).

- In TDD operation, RedCap UE re-use the PRACH configurations for FR1 on unpaired spectrum (Table 6.3.3.2-3, TS 38.211).

Proposal 9: To facilitate early indication of RedCap UE during initial access, PRACH resource dedicated to RedCap UE is configured within its  initial UL BWP. The following options can be considered:

- Option 1: ROs for RedCap UE and non-RedCap UE are separately configured, without overlapping in time or frequency

- Option 2: Preamble groups for RedCap UE and non-RedCap UE are separately configured on shared RO

- Option 3: PRACH preamble formats for RedCap UE and non-RedCap UE are separately configured

## BWP Operation after Initial Access

- For non-RedCap UE, the BW of initial UL BWP can be wider than 20 MHz, which is beyond the BW capability of RedCap UE during initial access

- When intra-slot frequency hopping is enabled for msg3, and the frequency span of two hops is wider than 20 MHz:

  - RedCap UE requires Tgap for retuning

  - Non-RedCap UE does not require a retuning gap

- Without early RedCap indication by PRACH, gNB does not know the presence of RedCap UE and cannot identify the origin of msg3

- When gNB attempts to decode msg3 from a RedCap UE and interprets Tgap as part of the 2nd hop of a non-RedCap UE, channel estimation is messed up and msg3 decoding fails

- gNB cannot decode msg3 of RedCap UE

- RedCap UE cannot access the network

After RRC connection is established, RedCap UE can switch to an active DL/UL BWP by re-using the BWP switching mechanism in NR R15/16. In the DL BWP (initial, active, default) configured for RedCap UE, it is desirable for NW to schedule a SSB (on or off the NR sync raster) to reduce UE’s complexity of RRM measurements for the serving cell.

In TDD bands, a DL BWP of RedCap UE with index provided by BWP-Id is linked with a UL BWP of the RedCap UE with the same BWP-Id. Besides, a RedCap UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP with the same BWP-Id.

Similar to a non-RedCap UE, a RedCap UE can be configured by higher layers a set of at most four BWPs for receptions by the UE (DL BWP set) in a DL bandwidth by RRC parameters BWP-Downlink-RedCap or initialDownlinkBWP-RedCap by BWP-DownlinkCommon-RedCap and BWP-DownlinkDedicated-RedCap, and a set of at most four BWPs for transmissions by the UE (UL BWP set) in an UL bandwidth by RRC parameters BWP-Uplink-RedCap or initialUplinkBWP-RedCap configured by BWP-UplinkCommon-RedCap and BWP-UplinkDedicated-RedCap.

Out of the considerations for power saving, interference management and traffic offloading, the BWP switching procedures specified for non-RedCap UE should be supported by RedCap UE. Since the use cases of RedCap UE are not latency-sensitive, the Type-2 timeline of DCI based BWP switching (shown in Table 1) should be adopted as a baseline. DL (or UL) BWP with different starting PRB positions should be configured with different BWP-Id. By default, a RedCap UE shall support the intra-slot frequency hopping and inter-slot frequency hopping within its initial or active UL BWP.

Table 1: Type 1 and Type 2 BWP Switching Timeline Based on DCI

BWP switching can occur within or beyond the max BW of RedCap UE. Switching between BWPs with different starting PRB and same BW resembles sub-band based frequency hopping, but the guard time required for BWP switching is much longer than that required for intra-BWP frequency hopping.  It is not necessary for RedCap UE to support a BWP switching timeline faster than Type-1, since it is beyond the capabilities of non-RedCap UE and the latency of RRC re-configuration is more than the guard period required by RF re-tuning. Furthermore, frequent RF/BWP switching can compromise the performance of both NW and UE. On UE side, it increases power consumption, complexity of link maintenance, signaling overhead for measurements/reporting. On NW side, it worsens resource fragmentation and imposes more challenges for interference management.

To illustrate the diversity gain achievable by frequency hopping within 100 MHz channel BW, Figure 6 shows two scenarios with different BW configurations. To decouple the coding gain from HARQ/IR combining, RV0 is applied to the two repetitions with and without frequency hopping. The example on the left is a RedCap UE with ~1 MHz BW (6 RBs, 15 kHz SCS)  hopping on 20 MHz channel BW, and the right one is a RedCap UE with ~20 MHz BW (48 RBs, 30 kHz SCS) hopping on 100 MHz channel BW. Three different MCS are evaluated. For two adjacent hops, the frequency offset between the starting PRBs is twice the UE BW.

Compared with the baseline of 2 repetitions without frequency hopping, frequency hopping provides ~2 dB gain for the 1 MHz  BW at 10% BLER, whereas the gain for 20 MHz BW is marginal. This is because in a TDL-C channel with 300ns delay spread, 20 MHz BW is sufficient to glean the gain in frequency diversity.

A similar observation holds for frequency hopping of PUSCH on a wider channel BW.

Figure 6: Gain of Frequency Hopping Under Two Scenarios. Left: 1 MHz/hop over 20 MHz Channel BW. Right: 20 MHz/hop over 100 MHz Channel BW.

Observation 7: For DL or UL coverage recovery of RedCap UE, the gain of frequency hopping diminishes as the BW per hop increases. To improve the SINR of RedCap UE, HARQ combining (based on IR or chase combining) and repetitions with RV cycling can provide more significant gains and less overhead (RS for demodulation and CSI measurements, retuning gap, etc.) than frequency hopping.

Observation 8: DL (or UL) BWPs with different starting PRB should be configured with different BWP-Id by higher layer. A RedCap UE is not expected to support more than 4 RRC configured DL/UL BWPs on the serving cell.

Observation 9:  Fast and frequent BWP/RF switching degrades the energy/spectral efficiency of UE and NW, due to the  increased overhead of switching gap, increased UE power consumption, and increased complexity in channel estimation, measurements, rate matching, and collision/priority handling.

Proposal 10: RedCap UE should support inter-slot or intra-slot frequency hopping within its initial or active UL BWP. Cross-carrier BWP switching is not supported.

Proposal 11: RedCap UE should re-use the BWP switching mechanism of non-RedCap UE. For DCI based switching, RedCap UE should support Type-2 switching delay capability as a baseline.

Proposal 12: For RedCap UE, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. A DL BWP is not linked with more than one UL BWPs with the same BWP-id. A UL BWP is not linked with more than one DL BWPs with the same BWP-id.

Proposal 13: RedCap UE should not support BWP switching/hopping/retuning faster than Type-1 switching delay for DCI based switching.

Proposal 14: RedCap UE is not expected to support BWP configuration procedure and switching mechanisms different from non-RedCap UE of NR R15/16.

Proposal 15: Confirm the working assumption for DL BWP#0 configuration for RedCap UE after initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment). A RedCap UE is not expected to operate in a  DL BWP#0 wider than its maximum UE bandwidth for either option 1 or option 2.

Proposal 16: Confirm the working assumption for UL BWP#0 configuration of RedCap UE after initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment). For the scenario where the UL BWP#0  for non-RedCap UE is configured to be wider than the max BW of RedCap UE, the RedCap UE is configured with a separate UL BWP#0  no wider than its max UE BW.

Proposal 17: After initial access, a RedCap UE operating on unpaired spectrum does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.

Proposal 18: Fast RF/BWP retuning is not supported for RedCap UE after initial access.

## DL RS Configuration for Measurements

Similar to non-RedCap UE of NR R15/16, RedCap UE needs to measure SSS or TRS of the serving cell after initial access for synchronization. To support cell level mobility, RedCap UE needs to measure SSB of the serving cell and neighbour cells as well. In serving cell of RedCap UE, if the DL RS for link maintenance or measurements are not configured in the active DL BWP, NW needs to configure periodic measurement gaps for RedCap UE via RRC signalling. When the measurement gap starts, RedCap UE suspends the reception of DL control/data in the active BWP, and switches to another BWP configured with DL RS, as shown by the example in Figure 7.

Figure 7: Measurements of RedCap/Non-RedCap UE in Active DL BWP without SSB (FG 6-1a)

According to TS 38.214, a non-RedCap UE in RRC connected state is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. For RedCap UE operating in an active DL BWP without SSB, periodic TRS should be configured. To mitigate load imbalance, throughput degradation and UE complexity/power increase resulted from measurement gaps, other DL RS for link maintenance and measurements can also be configured in the active DL BWP without SSB, if RedCap UE supports FG 6-1a as an optional feature.

Observation 10:  If neither SSB nor TRS/CSI-RS is configured in the active DL BWP (for RedCap or non-RedCap UE), UE has to switch to the initial DL BWP of the serving cell for link maintenance and measurements. When the number of UEs simultaneously switching to initial DL BWP is large (e.g. minimum periodicity of measurements = periodicity of SSB), load imbalance occurs in the serving cell. To mitigate load imbalance issue and save UE’s power, at least periodic TRS/CSI-RS needs to be configured in the active DL BWP without SSB.

Proposal 19: For link maintenance in active BWP, RedCap UE is expected to be configured with periodic DL RS resource(s) for RLM, L1-RSRP, BFD, BFR and  time/frequency tracking, wherein the DL RS configurations are explicitly signalled to RedCap UE.

Proposal 20: If a RedCap UE supports FG 6-1a and operates in an active DL BWP without SSB, it can request L3 gap via intra-FreqNeedForGap.

Proposal 21: If RedCap UE supports FG 6-1a  and operates in an active DL BWP without CORESET0 or SSB, it expects to receive:

- periodic TRS for time/frequency tracking

- dedicated RRC signalling for SI update

- dedicated BFR-CSIRS-RACH resource,  if BFR-CSI-RS is configured in the active BWP

## Mitigation of Resource Fragmentation

When the initial UL BWP of non-RedCap UE is wider than the max UE BW of RedCap UE, the initial UL BWP of RedCap UE is separately configured, as shown by Figure 8. For initial access of non-RedCap UE, default PUCCH resource sets are used and frequency hopping for PUCCH is enabled (if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon). If RedCap UE follows the same rule for PUCCH transmission as R15/16 non-RedCap UE, a few companies expressed concerns for potential resource fragmentation of non-RedCap UE. To mitigate this issue, they proposed to disable PUCCH frequency hopping for RedCap UE during and after initial access, as shown by Figure 9.

Figure 8: PUCCH Hopping Enabled in Initial/Active UL BWP of RedCap UE

Figure 9: PUCCH Hopping Disabled in Initial/Active UL BWP of RedCap UE

Nevertheless, UL resource fragmentation is a pre-existing issue for R15/16 non-RedCap UEs, if NW supports BWPs with different BW. To support features and use cases introduced in NR R16/17 (e.g. 2-step RACH, power saving, RedCap UE, coverage enhancement and SDT), it is desirable for NW to adopt a scalable and forward-compatible configuration for BWP and frequency hopping. Besides, configuring additional PRACH resources to facilitate early differentiation of UE types/capabilities are beneficial to optimize NW’s resource utilization efficiency, energy efficiency and interference management, without compromising UE’s link budget, power and complexity.

Observation 11: UL resource fragmentation is a pre-existing issue for R15/16 non-RedCap UEs. To support features and use cases introduced in NR R16/17 (e.g. 2-step RACH, power saving, RedCap UE, coverage enhancement and SDT), it is desirable for NW to adopt a scalable and forward-compatible solution based on early indication of UE types/capabilities and adaptive resource configuration for PUCCH/PUSCH.

Proposal 22: During initial access, frequency hopping and UL coverage enhancement for RedCap UE can be indicated by:

- Option 1: dynamic signalling in DCI or PDSCH

- Option 2: SI dedicated to RedCap UE

# Supporting BW Reduction of RedCap UE in FR2

In RAN#90e meeting, a Rel-17 work item for support of reduced capability NR devices was approved and the WID was updated in RAN#92e [2]. As part of the work item, it is agreed to specify support for the following for FR2 BW reduction:

- Reduced maximum UE bandwidth:

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz

In this section, based on the WID, we present our views on the FR2 reduced maximum UE bandwidth

## Partial CORESET0 Handling

In RAN1#105 meeting, the following working assumption was agreed:

Working assumption: At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access

- …

- FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring

- …

Figure 10 shows the Rel 15/16 FR2 SSB/CORESET0 multiplexing patterns.

As can be seen, some configurations are either larger than 66 RBs (the maximum # RBs for 100 MHz BW UE), or larger than 100 MHz. In both cases, the RedCap UE cannot support these as they are, and some solutions need to be defined for these cases.

To illustrate this, the left figure of Figure 11, shows an example for SSB+CORESET0 SCS = 240+120 kHz, config index 7 (48 RBs/1 symbol), where the PDCCH candidate mapping is shown for AL = 4, num of PDCCH candidates = 2, interleaved CCE-to-REG mapping. For this example, the UE will not receive half of the CCEs (CCEs 1,3,5,7 in the example) since they lie outside the UE BW, which reduces the performance by 3 dB (effectively reducing the AL by half, from 4 to 2 in the example).

Figure 10: Rel15/16 FR2 SSB/CORESET0 Multiplexing Patterns

Figure 11: Example of Configuration Exceeding RedCap UE max BW

For CORESET0, the following configurations are allowed:

  - REG bundle = 6​

  - AL = 4, 8, 16​

  - CCE-to-REG mapping is interleaved​

  - Max number of PDCCH candidates = 4, 2, 1 for AL = 4, 8, 16

Multiple solutions can be considered to resolve this issue.

In case part of the configured CORESET0 falls outside the UE DL BWP or outside the UE maximum BW, the UE may apply one or more of the following options to handle the CCEs that lie outside the UE’s DL BWP or UE’s maximum BW:​

- Option A: the NW does not configure such case

- Option B: the UE would not receive those CCEs​ (left figure in Figure 11)

  - Some performance reduction is expected since the effective AL is reduced​

- Option C: the UE would recalculate the CCE-to-REG mapping based on the number of RBs/CCEs available in the UE BWP or BW​ (right figure in Figure 11)

- Option D: the UE would assume different CCE-to-REG mapping type​ (right figure in Figure 11)

  - Interleaving vs non-interleaving​

- Option E: after reading the SSB, the UE hops in frequency to cover the complete CORESET0​

Some trade-off between PDCCH capacity and performance to consider:

- For the non-interleaving case and/or the recalculation based on reduced Num RB​

  - Less number of PDCCH candidates (1 in the example), hence less PDCCH capacity​

  - AL will not be affected (AL = 4 in the example), hence performance will be preserved​

- For the interleaving case:

  - Same number of PDCCH candidates (2 in the example), hence the same PDCCH capacity​

  - AL will be affected (AL = 2 in the example), hence performance will be reduced​

Proposal 23: For FR2, consider solutions to handle the case where CORESET0 is partially included in the UE BW

## Partial RO Handling

Rel-15/16 allows for up to 8 ROs to be FDMed. For SCS = 120 kHz, the BW for an RO = 17.28 MHz. For 8 FDMed ROs, the BW would be 8 x 17.28 = 138.24 MHz which is larger than the max UE BW (=100 MHz). If the gNB configures all 8, some ROs will be outside the maximum BW for a RedCap UE.

In RAN1#105 meeting, the following working assumption was agreed:

For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs.

- Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

However, what if, due to beam SSB-to-RO association, only a subset of the ROs that fall in the RedCap BW can be used by the RedCap UE. E.g., say ROs 0-7 are configured for the non-RedCap UE in FD. The RedCap UE BWP covers ROs 2-5, but only ROs 3 and 4 are useable for the RedCap UE, in this case, there needs to be some additional RedCap specific signaling to indicate that.

Hence, we propose that gNB can optionally configure a RedCap UE in RMSI (e.g., SIB1) or in dedicated RRC message with:

- An offset for the start of the RACH Occasions (ROs)

- The number of consecutive ROs or RBs it can use for the ROs

Proposal 24: For FR2, consider signaling an offset for the start and number of consecutive ROs that the RedCap UE can use in its separate initial UL BWP

## Transitioning to a Narrow Active BWP (NBWP)

The WID defines the following data rates:

- Industrial Wireless Sensors: < 2 Mbps (UL heavy)

- Video Surveillance: UL dominated (Economic: 2 – 4 Mbps, High End: 7.5 - 25 Mbps)

- Wearables: Reference DL/UL = 5-50/2-5 Mbps, peak DL/UL = 150/50 Mbps

Table 2 shows the maximum data rates that can be achieved for different BWs for 1 MIMO layer.

Table 2: Peak Data Rates (Mbps) for SCS 120 kHz (1 Layer) Based on TDD DL:UL = 3:1

| BW (MHz) | DL 64QAM | UL 64QAM |
| --- | --- | --- |
| 25 | 78 | 29 |
| 50 | 156 | 57 |
| 100 | 317 | 116 |

From the table, it can be noted that the data rates required for RedCap use cases may be achieved with smaller BW than the maximum UE BW of 100 MHz. For some use cases, it can be achieved with a BW much less than 100 MHz (e.g., 25 RB).

Observation 12: For FR2, the required data rates for RedCap use cases can be achieved with BW less than 100 MHz

Hence, a UE BW of 100 MHz may not be needed after initial cell search. To reduce the UE BW and thus save power, the UE may switch into a narrower BW active BWP (NBWP).

Due to the possibly large number of RedCap UEs, they may need to be distributed among several NBWPs (i.e., a carrier BW includes multiple NBWPs). To reduce signaling, the network may choose to allow the UEs to implicitly transition into a NBWP after initial access. This can be done by having the UE select a NBWP from a pool of NBWPs (e.g., with equal probability). Another approach is to have UEs select the NBWP based on some hashing function based on a UE ID. This may result in some uniform distribution of the UEs among the NBWPs and hence reduce the overloading and be more resource efficient.

Another aspect to consider is that a UE, based on its measurements (e.g., for interference) or capability may prefer a certain NBWP where the interference is low. Hence it may be desirable to have the option for the UE to initiate/request a preferred NBWP and/or BW.

The initial transition of a UE to one of the NBWPs can be:

- Network initiated/controlled

  - This is already existing in NR R15/16

- Implicit

  - Based on a random selection or some UE ID hashing function

- UE initialed/requested

  - UE may send a preferred max UE BW (≤ 100 MHz) to be used after initial access

  - UE may send a preferred BWP to be used after initial access

Proposal 25: For FR2, to save UE power and complexity, consider switching the UE to a narrow active BWP (NBWP) after initial access is complete. The switching may be:

- Network initiated/controlled (similar to existing NR)

- Implicit (e.g., based on a random selection or some UE ID hashing function)

- UE initiated/requested

## Virtual NBWP Hopping

There are however certain aspects that need to be considered due to the BW reduction of the active BWP.

- Reduce narrowband interference effects

- Get frequency diversity gains

- Optimize operation due to the reduced BW

For a narrow BW UE, to achieve frequency diversity gains, frequency hopping is one of the methods that can be used. However, in FR2, due to beamforming at both gNB and UE, in addition to smaller cells, the delay spread is smaller compared to FR1. This leads to a larger coherence BW and hence less gain using frequency hopping (if the hopping was within a limited frequency range). For FR2, to get the frequency diversity hopping gains, the UE may need to hop across a larger system frequency range (across larger system BW). For example, in case the network supports larger operation bandwidth, e.g., using CA, the UE which only supports up to 100 MHz and single CC may hop in frequency over larger frequency span which may include multiple CCs from gNB point of view.

In addition, for positioning, a design may be considered where the UE uses multiple frequency resources in TDM fashion, to achieve a higher BW capability (e.g., stitch 8x100 MHz to get 800 MHz) thus obtaining a higher positioning accuracy.

Hopping within a limited system BW, however, may still be beneficial to mitigate persistent interference because narrow BW operation may be more prone to such interference (affecting a large portion of the active BWP). This is even more exemplified for stationary devices where the interference is not randomized by the UE movements and may be persisting. It may be beneficial to have some sort of NBWP hopping mechanism, where we consider a “virtual” NBWP that is hopping in the frequency domain (Figure 12), where:

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

This simplifies the scheduler and the specification development by having a transparent hopping mechanism.

To illustrate the gains of the NBWP hopping, link and system level evaluations were considered.

Figure 12: “Virtual” BWP Hopping for RedCap UEs

### UL Link-level Simulations

The following assumptions were used for the UL LLS:

| Parameter | Value |
| --- | --- |
| Physical Channel | PUSCH |
| SCS | 120 kHz |
| Channel (Delay Spread) | TDL-A (30 ns) / CDL-A (30 ns) / CDL-B (50 ns) |
| Carrier frequency | 28 GHz |
| Number of RBs (BW) | 14 (20.16 MHz) and 34 (48.96 MHz) |
| Hopping | Inter-slot |
| HARQ RV | 0213 (re-tx on different hops) |
| Num Antenna | 1 Tx /2 Rx |
| Num Layers | 1 |
| 100 MHz Hop RB Start | 20 MHz Allocation: 1, 33, 17, 49 (1st hop distance = 47.52 MHz) 50 MHz Allocation: 1, 17, 8, 32 (1st hop distance = 24.48 MHz) |
| 200 MHz Hop RB Start | 1, 65, 33, 97 (1st hop distance = 93.6 MHz) |
| 400 MHz Hop RB Start | 1, 129, 65, 193 (1st hop distance = 185.76 MHz) |

We have considered 2 allocation BWs (20 MHz and 50 MHz). We have considered the 50 MHz allocation to align with the TS 38.101 requirement of the minimum UE BW support for FR2. However, a UE may be allocated smaller BWs and can choose to reduce its BW to further reduce power. For example, a CORESET0 can be as small as 24 RBs (34.6 MHz) and a UE may choose to use that BW for its BB/RF if allocation does not require any more BW.

Table 3 shows the UL SNR gains that can be achieved from hopping over different BWs (100, 200, 400 MHz) using 1 Tx and 2 Rx. Figure 13 to Figure 18 show the corresponding BLER curves.

The following observations can be made from the results:

Observation 13: For FR2:

- UL hopping across 100 MHz can yield SNR link level PUSCH gains of few dBs (up to 3.5 dB)

  - Hopping for 50 MHz allocation also yields gains (up to 3 dB)

- Some channels (e.g., CDL-A) has larger coherence BW and needs larger BW hop (e.g., 200 MHz) to achieve more gains

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Table 3: PUSCH SNR Gains for Hopping over Different BWs

|  |  |  | SNR Gain (dB) @ 1% BLER |  |  |  |  | SNR Gain (dB) @ 10% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocation | Channel | MCS | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) |
| 20 MHz | CDL-A | 0 | 2.0 | 3.2 | 3.4 | 1.4 | 0.1 | 0.4 | 1.3 | 1.4 | 1.0 | 0.1 |
|  |  | 9 | 2.1 | 3.4 | 3.6 | 1.5 | 0.2 | 0.1 | 0.9 | 1.1 | 1.0 | 0.2 |
|  | CDL-B | 0 | 3.5 | 4.1 | 3.5 | 0.0 | -0.6 | 1.6 | 2.0 | 1.7 | 0.1 | -0.3 |
|  |  | 9 | 3.4 | 3.6 | 3.1 | -0.2 | -0.5 | 1.4 | 1.7 | 1.4 | 0.0 | -0.2 |
|  | TDL-A | 0 | 1.8 | 2.4 | 2.8 | 1.0 | 0.4 | 0.9 | 1.5 | 1.7 | 0.7 | 0.2 |
|  |  | 9 | 1.6 | 2.3 | 2.3 | 0.7 | 0.0 | 0.8 | 1.1 | 1.4 | 0.6 | 0.3 |
| 50 MHz | CDL-A | 0 | 1.0 | 2.8 | 3.0 | 2.0 | 0.2 | 0.2 | 1.3 | 1.4 | 1.3 | 0.1 |
|  |  | 9 | 0.8 | 2.7 | 2.9 | 2.1 | 0.2 | 0.2 | 1.2 | 1.4 | 1.2 | 0.2 |
|  | CDL-B | 0 | 0.8 | 2.1 | 1.8 | 1.1 | -0.3 | 0.4 | 1.2 | 1.2 | 0.7 | -0.1 |
|  |  | 9 | 0.5 | 1.6 | 1.4 | 0.9 | -0.2 | 0.3 | 1.0 | 0.9 | 0.6 | -0.1 |
|  | TDL-A | 0 | 0.5 | 1.4 | 1.9 | 1.5 | 0.5 | 0.5 | 1.1 | 1.3 | 0.9 | 0.2 |
|  |  | 9 | NA | 1.3 | 1.7 | NA | 0.3 | 0.2 | 0.5 | 0.9 | 0.8 | 0.4 |

| Figure 13: Hopping for CDL-A, 20 MHz Allocation | Figure 14: Hopping for CDL-A, 50 MHz Allocation |
| --- | --- |
| Figure 15: Hopping for CDL-B, 20 MHz Allocation | Figure 16: Hopping for CDL-B, 50 MHz Allocation |
| Figure 17: Hopping for TDL-A, 20 MHz Allocation | Figure 18: Hopping for TDL-A, 50 MHz Allocation |

### DL Link-level Simulations

The following assumptions were used for the DL LLS:

| Parameter | Value |
| --- | --- |
| Physical Channel | PDSCH |
| SCS | 120 kHz |
| Channel | TDL-A / CDL-A |
| Carrier frequency | 28 GHz |
| Number of RBs (BW) | 14 (20.16 MHz) |
| Desired delay spread | 30 ns |
| Hopping | Inter-slot |
| HARQ RV | 0213 (re-tx on different hops) |
| Num Layers/Rx Antenna | 1 |
| 100 MHz Hop RB Start | 1, 33, 17, 49 (1st hop distance = 47.52 MHz) |
| 200 MHz Hop RB Start | 1, 65, 33, 97 (1st hop distance = 93.6 MHz) |
| 400 MHz Hop RB Start | 1, 129, 65, 193 (1st hop distance = 185.76 MHz) |
| Antenna configuration for CDL channel model (indoor scenario of Rel-17 CE WI) |  |
| BS antenna | (Mg,Ng,M,N,P) = (1,1,8,8,2) with (0.5 dv, 0.5 dH) |
| UE antenna | (Mg,Ng,M,N,P) = (1,1,2,2,2) with (0.5 dv, 0.5 dH) * Only a single polarization is used for 1 Rx |

Table 4 shows the DL SNR gains that can be achieved from hopping of a 20 MHz allocation over different BWs (100, 200, 400 MHz) using 2 Tx and 1 or 2 Rx. Figure 19 to Figure 26 show the corresponding BLER curves.

The following observations can be made from the results:

Observation 14: For FR2:

- Hopping across over 100 MHz can yield SNR link level PDSCH performance gains of few dBs (1-5 dB)

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Table 4: PDSCH SNR Gains for Hopping over Different BWs

|  |  |  | SNR Gain (dB) @ 1% BLER |  |  |  |  | SNR Gain (dB) @ 10% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Num Rx | MCS | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) |
| TDL-A | 1 | 0 | 3.1 | 3.5 | 4.1 | 1.1 | 0.6 | 1.6 | 2.1 | 2.4 | 0.8 | 0.3 |
|  |  | 9 | 2.7 | 3.7 | 3.9 | 1.2 | 0.3 | 1.4 | 1.8 | 2.4 | 1.0 | 0.6 |
|  | 2 | 0 | 1.6 | 2.4 | 2.6 | 0.9 | 0.2 | 0.9 | 1.4 | NA | NA | NA |
|  |  | 9 | 1.5 | 2.1 | 2.3 | 0.8 | 0.2 | 0.7 | 1.2 | 1.3 | 0.6 | 0.1 |
| CDL-A | 1 | 0 | 5.0 | 6.8 | 6.9 | 1.9 | 0.1 | 1.6 | 2.8 | 2.9 | 1.3 | 0.1 |
|  |  | 9 | NA | NA | NA | 2.0 | -0.1 | 1.5 | 2.8 | 2.6 | 1.2 | -0.2 |
|  | 2 | 0 | 2.3 | 3.8 | 3.6 | 1.3 | -0.2 | 1.1 | 2.0 | 2.0 | 0.9 | 0.0 |
|  |  | 9 | 2.1 | 3.4 | 3.4 | 1.4 | 0.0 | 0.9 | 1.6 | 1.6 | 0.8 | 0.0 |

| Figure 19: Hopping for TDL-A, 1Rx, MCS0 | Figure 20: Hopping for CDL-A, 1Rx, MCS0 |
| --- | --- |
| Figure 21: Hopping for TDL-A, 1Rx, MCS9 | Figure 22: Hopping for CDL-A, 1Rx, MCS9 |
| Figure 23: Hopping for TDL-A, 2Rx, MCS0 | Figure 24: Hopping for CDL-A, 2Rx, MCS0 |
| Figure 25: Hopping for TDL-A, 2Rx, MCS9 | Figure 26: Hopping for CDL-A, 2Rx, MCS9 |

### UL System-level Simulations

The following assumptions were used for the UL SLS:

| Parameters | Value |
| --- | --- |
| Deployment | Indoor hotspot (8, 24 UEs/cell), UMi (10, 20 UEs/cell) |
| Beam Management | Disabled |
| Antenna array | Indoor hotspot (128), UMi (256) |
| Allocation BW | 20 MHz |
| Hopping | Hop over 4 sub-bands (25/100 MHz apart for 100/400 MHz hopping) |
| Antenna array | Indoor hotspot (32/128), UMi (64/256) |
| EIRP = Tx Power + Antenna gain + Array gain | 60 dBm = 28 dBm (TxP) + 8 dBi (ant gain) + 24 dB (array gain = 10*log10(256)) |
| Carrier frequency | 28 GHz |
| UE Tx Power | 14 dBm |
| Polarization | 2 |
| MIMO | SU-MIMO only (max Rank = 2) |
| Traffic | Video surveillance (480p@30fps = 2.5Mbps) |
| Frame structure | DDDSU |
| Direction | UL |
| Max Modulation | 64QAM |

Figure 27 and Figure 28 shown the SINR and mean UPT distributions for different deployments and number of UEs.

| Figure 27: SLS SINR Distributions | Figure 28: SLS UPT Mean Distributions |
| --- | --- |

The following observations can be made from the results:

Observation 15: For FR2:

- BWP hopping can yield overall SINR and mean UPT gains

- Hopping across larger BW (e.g., 400 MHz) may be needed to get SINR gains for some small delay spread cases (e.g., UMi)

Proposal 26: For FR2, consider introducing “virtual” BWP hopping to achieve frequency diversity gains and reduce the NB interference effects

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

- Consider supporting hopping across larger BW (e.g., > 100 MHz) to achieve frequency diversity gains for low delay spread deployments

## Reducing the Effect of BWP Switching

UE hopping across frequency (e.g., using NBWP hopping) may lead to utilization issues in time due to the switching gaps defined in TS 38.133 (based on UE capability). RedCap is a delay tolerant system and some delays may be acceptable. However, it may be desirable to consider techniques to reduce the effect of the hopping switching gaps on messages within the gap. Some example methods to consider:

- Network implementation:

  - Other UEs may be scheduled during the gap, thus not affecting the overall system capacity

- Simplified BWP switching: Define simplified BWP switching by preconfiguring the switching/hopping and by using similar “virtual” BWP hop parameters

  - For DCI-based BWP switching, the switching time/gap is mainly due to UE DCI processing time, modem L1 processing time (loops re-initializations and settling, etc...), and RF retuning time.

  - Reduce the UE complexity by reducing the need for UE DCI and modem L1 processing. This can be achieved by pre-configuring the switches and using the same parameters

- BWP grouping:

  - A BWP group may contain some specific BWPs that when switching among them, the UE may be able to do faster RF retuning as compared to others​ (this can be based on UE capability)

  - For example, typically, a UE may be able to switch faster if the source and target BWP frequencies are close

- Variable BWP hop time (extension):

  - In case a transmission falls into a BWP switching gap, the previous BWP (before the switch) is extended to cover that transmission

- BWP hop skipping/modification:

  - UE may skip or modify BWP hops based on certain conditions that are either signaled to the UE (using RRC/MAC-CE/DCI) or specified. E.g.: no periodic/dynamic signals/messages scheduled in these hops

Proposal 27: For FR2, consider ways to reduce the impact of the BWP switching delays, examples include:

- Simplified BWP switching

  - By preconfiguring the switching/hops and by using similar BWP hop parameters

  - Consider RAN4’s input on this

- BWP grouping

  - Intra-BWP group switching times is smaller than inter-BWP group switching times

  - Consider RAN4’s input on this

- Variable BWP hop time (extension)

- BWP hop skipping/modification

# Conclusions

## BW Reduction in FR1

In this contribution, we have shared our views on the BW reduction for R17 RedCap devices. To summarize, we have the following observations and proposals for FR1:

Observation 1: By configuring dedicated PRACH resources for RedCap UE, NW is able to identify the RedCap UE during initial access. Based on the PRACH preamble transmitted by RedCap UE, NW is able to:

- detect the presence of RedCap UE

- schedule msg3/PUCCH (HARQ feedback for msg4/msgB) properly within the initial UL BWP of RedCap UE

- enable coverage recovery necessary for msg2/msg3/msgB/msg4/PUCCH by re-using the solutions available in NR R15/16 and R17 coverage enhancement WI

Observation 2: Fast RF switching (changing center frequency of initial UL BWP only) cannot solve the RO selection issue of RedCap UE, if the ROs configured for RedCap UE are outside its initial UL BWP.

Observation 3: Specifying multiple initial UL BWPs for RedCap UE and allowing BWP switching during initial access will increase the complexity and power consumption of UE.

Observation 4: When the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE, NW should configure a separate initial UL BWP for RedCap UE and ensure the ROs configured for RedCap UE are within the initial UL BWP of RedCap UE.

Observation 5: PRACH resources valid for RedCap UE should be configured within its initial UL BWP. By configuration, NW should avoid/reduce the overlapping between SSB and RO configured for RedCap UE.

Observation 6: Changing center frequency of  DL/UL BWP in TDD leads to:

- loss of channel reciprocity

- increased latency similar to Type-B HD-FDD of LTE

- increased UE complexity

- non-trivial spec impacts and increased workload for RAN1/2/4

Observation 7: For DL or UL coverage recovery of RedCap UE, the gain of frequency hopping diminishes as the BW per hop increases. To improve the SINR of RedCap UE, HARQ combining (based on IR or chase combining) and repetitions with RV cycling can provide more significant gains and less overhead (RS for demodulation and CSI measurements, retuning gap, etc.) than frequency hopping.

Observation 8: DL (or UL) BWPs with different starting PRB should be configured with different BWP-Id by higher layer. A RedCap UE is not expected to support more than 4 RRC configured DL/UL BWPs on the serving cell.

Observation 9:  Fast and frequent BWP/RF switching degrades the energy/spectral efficiency of UE and NW, due to the  increased overhead of switching gap, increased UE power consumption, and increased complexity in channel estimation, measurements, rate matching, and collision/priority handling.

Observation 10:  If neither SSB nor TRS/CSI-RS is configured in the active DL BWP (for RedCap or non-RedCap UE), UE has to switch to the initial DL BWP of the serving cell for link maintenance and measurements. When the number of UEs simultaneously switching to initial DL BWP is large (e.g. minimum periodicity of measurements = periodicity of SSB), load imbalance occurs in the serving cell. To mitigate load imbalance issue and save UE’s power, at least periodic TRS/CSI-RS needs to be configured in the active DL BWP without SSB.

Observation 11: UL resource fragmentation is a pre-existing issue for R15/16 non-RedCap UEs. To support features and use cases introduced in NR R16/17 (e.g. 2-step RACH, power saving, RedCap UE, coverage enhancement and SDT), it is desirable for NW to adopt a scalable and forward-compatible solution based on early indication of UE types/capabilities and adaptive resource configuration for PUCCH/PUSCH.

Proposal 1: During initial access, RedCap and non-RedCap UEs share the SSB, CORESET#0, SIB1 and initial DL BWP.

Proposal 2: Other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

Proposal 3: Confirm the working assumption for initial UL BWP configuration of RedCap UE during initial access. A RedCap UE is NOT configured with an initial UL BWP wider than its max BW.

Proposal 4: Confirm the working assumptions for initial UL BWP configuration of RedCap UE when the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE is separately configured

- PRACH resources dedicated to RedCap UE is configured within its initial UL BWP to enable early indication in msg1 (or msgA preamble, if 2-step RACH is supported)

- the initial UL BWP of RedCap UE can be nested within the initial UL BWP of non-RedCap UE

- the initial UL BWP of RedCap UE can be configured by SIB1, or by rules defined in standards

Proposal 5: When RedCap UE and non-RedCap UE co-exist in the same cell, and the initial UL BWP of non-RedCap UE is no wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE and non-RedCap UE can be shared, or separately configured

- dedicated PRACH resources for RedCap UE can be configured within its initial UL BWP,  to enable early indication in msg1 (or msgA preamble, if 2-step RACH is supported)

Proposal 6: Fast RF/BWP retuning is not supported for RedCap UE during initial access.

Proposal 7: During initial access, a RedCap UE operating on unpaired spectrum does not expect to receive a configuration where the center frequency for an initial DL BWP is different than the center frequency for an initial UL BWP.

Proposal 8: RedCap UE re-uses the long and short PRACH preamble sequences introduced in NR R15.

- In FD-FDD or Type-A HD-FDD operation, RedCap UE re-use the PRACH configurations for FR1 on paired spectrum (Table 6.3.3.2-2, TS 38.211).

- In TDD operation, RedCap UE re-use the PRACH configurations for FR1 on unpaired spectrum (Table 6.3.3.2-3, TS 38.211).

Proposal 9: To facilitate early indication of RedCap UE during initial access, PRACH resource dedicated to RedCap UE is configured within its  initial UL BWP. The following options can be considered:

- Option 1: ROs for RedCap UE and non-RedCap UE are separately configured, without overlapping in time or frequency

- Option 2: Preamble groups for RedCap UE and non-RedCap UE are separately configured on shared RO

- Option 3: PRACH preamble formats for RedCap UE and non-RedCap UE are separately configured

Proposal 10: RedCap UE should support inter-slot or intra-slot frequency hopping within its initial or active UL BWP. Cross-carrier BWP switching is not supported.

Proposal 11: RedCap UE should re-use the BWP switching mechanism of non-RedCap UE. For DCI based switching, RedCap UE should support Type-2 switching delay capability as a baseline.

Proposal 12: For RedCap UE, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. A DL BWP is not linked with more than one UL BWPs with the same BWP-id. A UL BWP is not linked with more than one DL BWPs with the same BWP-id.

Proposal 13: RedCap UE should not support BWP switching/hopping/retuning faster than Type-1 switching delay for DCI based switching.

Proposal 14: RedCap UE is not expected to support BWP configuration procedure and switching mechanisms different from non-RedCap UE of NR R15/16.

Proposal 15: Confirm the working assumption for DL BWP#0 configuration for RedCap UE after initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment). A RedCap UE is not expected to operate in a  DL BWP#0 wider than its maximum UE bandwidth for either option 1 or option 2.

Proposal 16: Confirm the working assumption for UL BWP#0 configuration of RedCap UE after initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment). For the scenario where the UL BWP#0  for non-RedCap UE is configured to be wider than the max BW of RedCap UE, the RedCap UE is configured with a separate UL BWP#0  no wider than its max UE BW.

Proposal 17: After initial access, a RedCap UE operating on unpaired spectrum does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.

Proposal 18: Fast RF/BWP retuning is not supported for RedCap UE after initial access.

Proposal 19: For link maintenance in active BWP, RedCap UE is expected to be configured with periodic DL RS resource(s) for RLM, L1-RSRP, BFD, BFR and  time/frequency tracking, wherein the DL RS configurations are explicitly signalled to RedCap UE.

Proposal 20: If a RedCap UE supports FG 6-1a and operates in an active DL BWP without SSB, it can request L3 gap via intra-FreqNeedForGap.

Proposal 21: If RedCap UE supports FG 6-1a  and operates in an active DL BWP without CORESET0 or SSB, it expects to receive:

- periodic TRS for time/frequency tracking

- dedicated RRC signalling for SI update

- dedicated BFR-CSIRS-RACH resource,  if BFR-CSI-RS is configured in the active BWP

Proposal 22: During initial access, frequency hopping and UL coverage enhancement for RedCap UE can be indicated by:

- Option 1: dynamic signalling in DCI or PDSCH

- Option 2: SI dedicated to RedCap UE

## BW Reduction in FR2

We have the following observations and proposals for FR2:

Observation 12: For FR2, the required data rates for RedCap use cases can be achieved with BW less than 100 MHz

Observation 13: For FR2:

- UL hopping across 100 MHz can yield SNR link level PUSCH gains of few dBs (up to 3.5 dB)

  - Hopping for 50 MHz allocation also yields gains (up to 3 dB)

- Some channels (e.g., CDL-A) has larger coherence BW and needs larger BW hop (e.g., 200 MHz) to achieve more gains

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Observation 14: For FR2:

- Hopping across over 100 MHz can yield SNR link level PDSCH performance gains of few dBs (1-5 dB)

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Observation 15: For FR2:

- BWP hopping can yield overall SINR and mean UPT gains

- Hopping across larger BW (e.g., 400 MHz) may be needed to get SINR gains for some small delay spread cases (e.g., UMi)

Proposal 23: For FR2, consider solutions to handle the case where CORESET0 is partially included in the UE BW

Proposal 24: For FR2, consider signaling an offset for the start and number of consecutive ROs that the RedCap UE can use in its separate initial UL BWP

Proposal 25: For FR2, to save UE power and complexity, consider switching the UE to a narrow active BWP (NBWP) after initial access is complete. The switching may be:

- Network initiated/controlled (similar to existing NR)

- Implicit (e.g., based on a random selection or some UE ID hashing function)

- UE initiated/requested

Proposal 26: For FR2, consider introducing “virtual” BWP hopping to achieve frequency diversity gains and reduce the NB interference effects

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

- Consider supporting hopping across larger BW (e.g., > 100 MHz) to achieve frequency diversity gains for low delay spread deployments

Proposal 27: For FR2, consider ways to reduce the impact of the BWP switching delays, examples include:

- Simplified BWP switching

  - By preconfiguring the switching/hops and by using similar BWP hop parameters

  - Consider RAN4’s input on this

- BWP grouping

  - Intra-BWP group switching times is smaller than inter-BWP group switching times

  - Consider RAN4’s input on this

- Variable BWP hop time (extension)

- BWP hop skipping/modification

# References

- 3GPP TR 38.875, “Study on Support of Reduced Capability NR Devices,” December 2020.

- RP-211574, “Revised WID on Support of Reduced Capability NR Devices,” 3GPP TSG RAN#92e, June,  2021.

- R4-1702029, “Reply LS on UE RF Bandwidth Adaptation in NR,”3GPP TSG RAN4#82, February, 2017.