| 3GPP TSG-RAN WG1 Meeting #106 e-Meeting, August 16th – 27th, 2021 | R1-2107353 |
| --- | --- |

Agenda item:	8.6.1.3

Source: 	Qualcomm Incorporated

Title: 	Type-A HD-FDD for RedCap UE

Document for:	Discussion/Decision

# Introduction

To meet the generic and use case specific requirements of mid-range use cases, potential UE complexity reduction techniques were identified and analyzed for wearables, industry wireless sensors and surveillance cameras [1]. The RAN1 study for R17 RedCap devices was completed in Q4 2020, and a revised WID [2] was approved in RAN-92e meeting. A main objective of the WI is to specify support for the following UE complexity reduction features in RAN1 and RAN4:

- Duplex operation:

  - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

Based on the discussion in R17 RedCap SI [1], the average cost reduction achievable by Type-A HD-FDD is ~7%, and the gain of cost saving scales up with multi-band support.  In addition, the noise figure and insertion loss of the switch are less than that of duplexer, and the transceiver chain can be put in a low-power state when the communication is going on in the opposite direction.

Different from HD-FDD operation of  LTE, Type-A HD-FDD of NR aims for more flexible switching positions and shorter switching gap. An LS has been sent to RAN4 to confirm the RX-to-TX and TX-to-RX switching gaps in FR1. Besides, RAN1 has discussed the collision handling rules when UE’s DL reception overlaps with UL transmission. The following agreements and conclusions have been made for R17 RedCap UE:

Agreements:

- For HD-FDD, for cases (if any) where collision handling needs to be specified, then the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum are used as a starting point if deemed applicable.

Agreements:

- For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI

  - No special handling on the priority rule for PDCCH carrying ULCI

Conclusion:

- No consensus of specification support of semi-static UL/DL pattern to HD-FDD RedCap UEs in Rel-17.

Conclusion:

- Enhancement for potential UL and DL collision handling due to TA misalignment is not considered for Type-A HD-FDD operation of RedCap UEs

In this contribution, we shared our views on the guard symbol configuration for RX-to-TX switching,  the validation rule for RACH occasion (RO), and the collision handling procedures for Type-A HD-FDD operation of R17 RedCap UE.

# RX/TX Switching Time

According to Table 4.3.2-3 in TS 38.211 specification, the transition time between UE TX and UE RX amounts to 13 usec in FR1. This is based on the minimum ON-OFF transition time of 10 usec in sub-clause 6.3.3.2 of  TS 38.101-1 (shown in Figure 1), with underlying assumptions that various RF blocks in the transceiver can remain ON [3].

Figure 1: ON/OFF transition time mask for NR R15/16 UE assuming other RF blocks remain ON

Observation 1:  For a NR non-RedCap UE not capable of full-duplex operation, the minimum TX/RX switching time in Table 4.3.2-3 of TS 38.211 specification is based on the assumption  that various RF blocks in the transceiver can remain ON.

Observation 2:  To save RedCap UE’s power in Type-A HD-FDD operation of NR, the TX PLL can be shut down when UE is not transmitting.

Figure 2: Impacts of TX/RX PLL Locking Time on the Last Symbol of Cross Link

# Guard Symbol Configuration

In NR R15/16 TDD operation, the effects of TA and RX-to-TX switching can be accounted for by the flexible symbol(s) inserted between DL and UL. As shown in Table 1, at least one flexible symbol (nf≥1) is inserted before the UL symbols when UE needs to switch from DL to UL. For TX-to-RX switching, guard symbol is not needed because the fixed TA offset of 13 usec is always maintained between UL and DL in NR FR1.

Despite the differences in slot format determination in TDD and Type-A HD-FDD, a RedCap UE is subject to the same scheduling constraints in switching between RX and TX directions. Similar to NR TDD, at least one guard symbol (ng ≥1) is needed in Type-A HD-FDD to accommodate RF retuning from DL to UL and timing advance on UL. Due to the TA offset between UL and DL carriers of NR FDD bands, it is not necessary to configure additional guard symbols when UE switches from UL to DL. The guard symbol configuration for Type-A HD-FDD is shown in Figure 3.

Observation 3:  Similar to NR R15/16 TDD operation, at least one guard symbol needs to be configured for RX-to-TX switching in Type-A HD-FDD operation of NR, to account for the effects of:

- RF retuning from DL to UL

- timing advance (TA) on UL

Observation 4:  Similar to NR R15/16 TDD operation,  the 13 usec TA offset between UL carrier and DL carrier is sufficient for TX-to-RX switching of UE in Type-A HD-FDD operation of NR.

Table 1: NR TDD Slot Format Specified in TS 38.213

Figure 3: Guard Symbols for RX-to-TX Switching Gap of Type-A HD-FDD

Since UE is not expected to transmit and receive simultaneously on DL and UL in HD-FDD operation, a loss in DL and UL peak data rate w.r.t. FD-FDD is expected for HD-FDD as shown by Table 2 (a), assuming 1T1R antenna configuration and 20 MHz BW for RedCap UE. The impacts of ng on  peak data rates are shown in Table 2-(c). Compared with TDD slot format DDSU with one flexible symbol (nf=1), there is no loss in DL and UL peak data rates for Type-A HD-FDD when ng=1 and SCS is 15 kHz. When ng=2, the loss in DL peak data rates is around 3% for 30 kHz SCS. Since no guard symbol is configured for TX-to-RX switching, there is no loss in UL peak data rates for Type-A HD-FDD when compared with its TDD counterpart shown in Table 2-(b).

Table 2: Peak Data Rates in Different Duplex Modes

Proposal 1:  Support the following rules for guard symbol configuration of RedCap UE in Type-A HD-FDD operation of NR:

- No guard symbol is configured for TX-to-RX switching of UE.

  - The switching time specified in Table 4.3.2-3 of TS 38.211 is accounted for by the timing advance of UE.

- At least one guard symbol is configured for RX-to-TX switching of UE.

  - The time duration of the guard symbol corresponds to the minimum SCS configuration (μref) of the DL BWP and the UL BWP.

  - Down-select from the following options to determine the minimum number of guard symbols (ng, min) for RX-to-TX switching:

    - Option 1:  ng, min=1  for μref=0 or 1

    - Option 2:  ng,min=1 ,    μref=0 2 ,    μref=1

Proposal 2:  In Type-A HD-FDD operation of NR, the switching position determination is not expected to override:

- the minimum number of guard symbol(s) between DL and UL

- the collision handling rules for DL, UL the guard symbol between DL and UL

# Collision Handling Rules

When RedCap and non-RedCap UEs co-exist on NR FDD carriers, new collision handling rules need to be specified for UEs incapable of full duplex operation (FD-incapable). To handle collisions between DL, UL and guard symbol, companies have agreed to use the NR R15/16 TDD rules as a starting point.

However, DL/UL collision handling in TDD is much simpler than Type-A HD-FDD. To name a few,

- UE can be configured with semi-static slot formats by SI and RRC

  - UE always follows semi-static D/U configurations of the slot format, and ignores the potential collisions in the opposite direction, regardless they are grant-based (dynamically scheduled) or grant-free (SPS, CG)

- UE does not expect to deal with collisions between SSB and valid RO

- UE can fulfil RRM measurement requirements by prioritizing SSB over UL transmissions

- RO validation rules specified for non-RedCap UE can be re-used by RedCap UE

Since there was no consensus to support semi-static D/U patterns for Type-A HD-FDD in NR R17, RedCap UE cannot exploit the RRC configured slot formats to simplify the collisions handling procedures for HD-FDD.

In previous RAN1 meetings, RO validation rule was discussed for Type-A HD-FDD RedCap UE. Some companies proposed to reuse the RO validation rule of FD-capable UE for FD-incapable UE, so that NW does not need to configure a separate RO or a different SSB-to-RO mapping pattern for different UE types. Nevertheless, NW should be able to support separate ROs and/or separate SSB-to-RO mapping patterns for RedCap UE. In NR 16, NW is required to support separate RO configurations for Type-2 RACH procedure, as specified in Clause 8.1 of  TS 38.213. Furthermore, TS 38.213 and TS 38.331 have introduced a new IE “msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB”  to facilitate co-existence of multiple configurations for ROs and SSB-to-RO mapping patterns in the same cell, wherein the ROs dedicated to newly introduced Type-2 RACH can be configured in time/frequency by {msgA-PRACH-ConfigurationIndex, msgA-RO-FDM, msgA-RO-FrequencyStart}, which are different from {msg1-PRACH-ConfigurationIndex, msg1-RO-FDM, msg1-RO-FrequencyStart } of Type-1 RACH procedure. Therefore, NW should be able to support a separate RO and SSB-to-RO mapping pattern for RedCap UE with minor spec impacts.

Observation 5:  NW should be able to support a separate RO configuration and SSB-to-RO mapping pattern for RedCap UE with minor spec impacts.

Example of PRACH Resource Configuration for FR1 and Paired Spectrum

Figure 4:  SSB and RO Collision for Type-A HD-FDD

(PRACH configuration index =185, Table 6.3.3.2-2, TS 38.211)

Since a FD-incapable UE cannot transmit PRACH and receive SSB simultaneously, additional collision handling rules have to be specified for HD-FDD operation.  Some companies proposed to include the guard symbol between DL and UL in specifying the collision handling rules between “valid RO” and DL receptions.  However,  this does not solve the problem inherent to HD-FDD, as shown by the example in Figure 4.  Assume PRACH configuration index 185 is selected by the NW,  and PRACH format B1 is used by RedCap and non-RedCap UEs for msg1 transmission of 4-step RACH.  For FD-capable UEs, the valid ROs in a PRACH slot can be mapped to all the SSBs transmitted within a 20ms association period.  If the ROs valid for FD-capable UE are considered as valid for FD-incapable UE as well, there will be persistent collision between SSB beams and ROs due to the overlapping illustrated by Figure 4. Since RRM measurement relaxations are not specified for the serving cell of RedCap UE, and current RAN4 RRM measurement requirements are derived assuming 2 RX branches, if the HD UE prioritizes PRACH transmission over SSB reception, it may not be able to meet the performance requirements for RRM measurements. On the other hand, if the HD UE prioritizes SSB reception over PRACH transmission (similar to NR TDD UE), the UE may not be able to transmit on the ROs associated with the best SSB beams.

Observation 6:  In NR TDD bands, UE does not expect SSB to collide with a valid RO, wherein the valid RO does not precede a SSB  in the PRACH slot and starts at least Ngap symbols after a last SSB symbol.

Observation 7:  In NR FDD bands, all the configured ROs are valid for non-RedCap UE (FD-capable). Overlapping between SSB and RO is not excluded in NW configuration.

Observation 8:  For RedCap UE in Type-A HD-FDD operation, overlapping between SSB and RO (with or without accounting for the guard symbol between DL and UL) should be avoided by NW configuration, instead of resorting to a collision handling rule or leaving to UE implementation.

- If the collision handling rule specified for RedCap UE prioritizes SSB reception over PRACH transmission (similar to NR TDD UE), the UE may not be able to transmit on the ROs associated with the best SSB beams within a SSB to RO association period.

- If the collision handling rule specified for RedCap UE prioritizes PRACH transmission over SSB, UE may not be able to meet the RRM measurement requirements in the serving cell.

- If the collision handling is up to UE implementation, there is an ambiguity for UE implementation so that UE cannot transmit on the ROs associated with the best beams and/or cannot meet the RRM measurement requirements in the serving cell.

Observation 9:  In NR FDD bands, the RO validation rule for non-RedCap (FD-capable) UE is NOT suitable for RedCap (FD-incapable) UE.

Observation 10:  In Type-A HD-FDD operation, the RO validation rule for RedCap (FD-incapable) UE can be defined as a trade-off between NR TDD and FD-FDD:

- guard symbol between SSB and PRACH in accounted for in RO validation

- RO overlapping with SSB is not valid

- RO overlapping other DL symbols (SSB excluded) is not excluded in RO validation

Proposal 3:  For a PRACH occasion configured for HD-FDD RedCap UE, the RO is valid if:

- it is within the initial UL BWP of UE

- it does not precede a SS/PBCH block in the PRACH slot and starts at least ng,min  symbols after a last SS/PBCH block symbol, where  ng,min≥1

  - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1 of TS 38.213

Proposal 4:  Based on the RO validation rule specified in Proposal 3:

- Collision between SSB and valid RO is not expected by a HD-FDD RedCap UE

- No need to introduce extra rules to handle overlapping between SSB and valid RO

- To ensure RedCap UE can transmit PRACH on ROs associated with the best SSB beam within a SSB-to-RO association period, NW can select an appropriate PRACH configuration applicable to both RedCap and non-RedCap UE, or configure separate PRACH resources for RedCap UE within its initial UL BWP

Proposal 5:  In Type-A HD-FDD operation of UE, SSB reception is prioritized over dynamically scheduled UL transmissions, if a dynamically scheduled  UL transmission overlaps with an SSB.

- PDCCH ordered PRACH is excluded from the dynamically scheduled UL transmissions discussed above

Proposal 6: In Type-A HD-FDD operation of UE,  SSB reception is prioritized over UL transmissions configured by higher layers, if an UL transmission configured by higher layers overlaps with an SSB.

- PRACH initiated by higher layer is excluded from the UL transmissions discussed above

Proposal 7:  In Type-A HD-FDD operation of UE, leaving to UE implementation to determine whether to receive the configured PDCCH or transmit PRACH on the valid RO, if a valid RO overlaps with configured PDCCH in Type 0/0A/1/2 CSS set.

Proposal 8:  In Type-A HD-FDD operation of UE,  overlapping between valid RO and UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS) is not expected by UE and will be treated as a configuration error of NW.

Proposal 9:  In Type-A HD-FDD operation of UE,  overlapping between valid RO and dynamically scheduled DL reception is not expected by UE and will be treated as a configuration error of NW.

# Conclusions

In this contribution, we have shared our views on the support of Type-A HD-FDD operation for R17 RedCap devices. To summarize, we have the following observations and proposals:

Observation 1:  For a NR non-RedCap UE not capable of full-duplex operation, the minimum TX/RX switching time in Table 4.3.2-3 of TS 38.211 specification is based on the assumption  that various RF blocks in the transceiver can remain ON.

Observation 2:  To save RedCap UE’s power in Type-A HD-FDD operation of NR, the TX PLL can be shut down when UE is not transmitting.

Observation 3:  Similar to NR R15/16 TDD operation, at least one guard symbol needs to be configured for RX-to-TX switching in Type-A HD-FDD operation of NR, to account for the effects of:

- RF retuning from DL to UL

- timing advance (TA) on UL

Observation 4:  Similar to NR R15/16 TDD operation,  the 13 usec TA offset between UL carrier and DL carrier is sufficient for TX-to-RX switching of UE in Type-A HD-FDD operation of NR.

Observation 5:  NW should be able to support a separate RO configuration and SSB-to-RO mapping pattern for RedCap UE with minor spec impacts.

Observation 6:  In NR TDD bands, UE does not expect SSB to collide with a valid RO, wherein the valid RO does not precede a SSB  in the PRACH slot and starts at least Ngap symbols after a last SSB symbol.

Observation 7:  In NR FDD bands, all the configured ROs are valid for non-RedCap UE (FD-capable). Overlapping between SSB and RO is not excluded in NW configuration.

Observation 8:  For RedCap UE in Type-A HD-FDD operation, overlapping between SSB and RO (with or without accounting for the guard symbol between DL and UL) should be avoided by NW configuration, instead of resorting to a collision handling rule or leaving to UE implementation.

- If the collision handling rule specified for RedCap UE prioritizes SSB reception over PRACH transmission (similar to NR TDD UE), the UE may not be able to transmit on the ROs associated with the best SSB beams within a SSB to RO association period.

- If the collision handling rule specified for RedCap UE prioritizes PRACH transmission over SSB, UE may not be able to meet the RRM measurement requirements in the serving cell.

- If the collision handling is up to UE implementation, there is an ambiguity for UE implementation so that UE cannot transmit on the ROs associated with the best beams and/or cannot meet the RRM measurement requirements in the serving cell.

Observation 9:  In NR FDD bands, the RO validation rule for non-RedCap (FD-capable) UE is NOT suitable for RedCap (FD-incapable) UE.

Observation 10:  In Type-A HD-FDD operation, the RO validation rule for RedCap (FD-incapable) UE can be defined as a trade-off between NR TDD and FD-FDD:

- guard symbol between SSB and PRACH in accounted for in RO validation

- RO overlapping with SSB is not valid

- RO overlapping other DL symbols (SSB excluded) is not excluded in RO validation

Proposal 1:  Support the following rules for guard symbol configuration of RedCap UE in Type-A HD-FDD operation of NR:

- No guard symbol is configured for TX-to-RX switching of UE.

  - The switching time specified in Table 4.3.2-3 of TS 38.211 is accounted for by the timing advance of UE.

- At least one guard symbol is configured for RX-to-TX switching of UE.

  - The time duration of the guard symbol corresponds to the minimum SCS configuration (μref) of the DL BWP and the UL BWP.

  - Down-select from the following options to determine the minimum number of guard symbols (ng, min) for RX-to-TX switching:

    - Option 1:  ng, min=1  for μref=0 or 1

    - Option 2:  ng,min=1 ,    μref=0 2 ,    μref=1

Proposal 2:  In Type-A HD-FDD operation of NR, the switching position determination is not expected to override:

- the minimum number of guard symbol(s) between DL and UL

- the collision handling rules for DL, UL the guard symbol between DL and UL

Proposal 3:  For a PRACH occasion configured for HD-FDD RedCap UE, the RO is valid if:

- it is within the initial UL BWP of UE

- it does not precede a SS/PBCH block in the PRACH slot and starts at least ng,min  symbols after a last SS/PBCH block symbol, where  ng,min≥1

  - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1 of TS 38.213

Proposal 4:  Based on the RO validation rule specified in Proposal 3:

- Collision between SSB and valid RO is not expected by a HD-FDD RedCap UE

- No need to introduce extra rules to handle overlapping between SSB and valid RO

- To ensure RedCap UE can transmit PRACH on ROs associated with the best SSB beam within a SSB-to-RO association period, NW can select an appropriate PRACH configuration applicable to both RedCap and non-RedCap UE, or configure separate PRACH resources for RedCap UE within its initial UL BWP

Proposal 5:  In Type-A HD-FDD operation of UE, SSB reception is prioritized over dynamically scheduled UL transmissions, if a dynamically scheduled  UL transmission overlaps with an SSB.

- PDCCH ordered PRACH is excluded from the dynamically scheduled UL transmissions discussed above

Proposal 6: In Type-A HD-FDD operation of UE,  SSB reception is prioritized over UL transmissions configured by higher layers, if an UL transmission configured by higher layers overlaps with an SSB.

- PRACH initiated by higher layer is excluded from the UL transmissions discussed above

Proposal 7:  In Type-A HD-FDD operation of UE, leaving to UE implementation to determine whether to receive the configured PDCCH or transmit PRACH on the valid RO, if a valid RO overlaps with configured PDCCH in Type 0/0A/1/2 CSS set.

Proposal 8:  In Type-A HD-FDD operation of UE,  overlapping between valid RO and UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS) is not expected by UE and will be treated as a configuration error of NW.

Proposal 9:  In Type-A HD-FDD operation of UE,  overlapping between valid RO and dynamically scheduled DL reception is not expected by UE and will be treated as a configuration error of NW.

# References

- 3GPP TR 38.875, “Study on Support of Reduced Capability NR Devices,” December 2020.

- RP-211574, “Revised WID on Support of Reduced Capability NR Devices,” 3GPP TSG RAN#92e, June,  2021.

- R4-2111424, “Type-A HD-FDD RedCap UE RF Issues,” Qualcomm, 3GPP TSG-WG4 Meeting RAN4#99-e, May 2021.