3GPP TSG RAN WG1 #106e							                R1-2107363

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.3

Source: 	Qualcomm Incorporated

Title: 	Type-A PUSCH repetition for Msg3

Document for:	Discussion/Decision

# Introduction

In this contribution, we shall discuss our views on Type-A PUSCH repetition for Msg3.

# Msg3 PUSCH Repetition Request by UE

RAN1 105e made the following agreements on UE request for Msg3 PUSCH repetition:

Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

In particular, it was agreed to use a shared RO configured by the same PRACH configuration index with legacy UEs for the request. Therefore, such resources for the request can also be used for existing procedures (e.g., RO for 4-step RACH only or RO for 2-step RACH only or both). Hence, it is critical to define a shared RO for the UE to request Msg3 repetition:

Proposal 1: The shared RO for requesting Msg3 PUSCH repetition can be an RO with preamble(s) for 4-step RACH only or an RO with preambles for both 4-step RACH and 2-step RACH.

- Whether an RO with preamble(s) for 4-step RACH only or an RO with preambles for both 4-step RACH and 2-step RACH is used can be indicated to the UE.

The use of a separate RO for repetition request was also discussed in the past RAN1 meetings. If a separate RO for repetition request is introduced, RAN1 should discuss whether this separate RO can also be used for new procedures in R17 (e.g., RedCap identification in Msg1).

Proposal 2: If the Msg3 PUSCH repetition request is indicated by a separate RO, RAN1 should discuss whether this RO can be shared for Redcap UE to send identification via PRACH or not.

For supporting UE request for Msg3 repetition, some triggering condition should be defined. RAN1#105e made the following agreements:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold.

It is important to enable gNB to have some control over the average number of repetition requests from the UEs (by configuring the triggering condition). In particular, the conditions can be based on SS-RSRP and/or UE power class since there can be various types of Tx power at the device. For example, in NTN, CPE/VSAT devices will have Tx power of 31dBm while handheld devices have Tx power of 23dBm.

Proposal 3: The RSRP threshold depends on SS-RSRP and/or UE power class.

- FFS: whether RSRP thresholds are separate for NUL and SUL.

For FFS on indicating UE capability of supporting Msg3 PUSCH repetition after initial access procedure, the usage of this indication is unclear. If it is for RACH resource optimization, it may be better to use history of actual RACH uses for such optimization.

Observation 1: Benefit of indicating UE capability of supporting Msg3 PUSCH repetition after initial access procedure is unclear.

In addition, this FFS is not critical for enabling Msg3 repetition while there are fundamental aspects to be discussed, we prefer to discuss it later under UE capability discussion if necessary.

Proposal 4: Whether the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure can be discussed later under UE capability discussion if necessary.

# Msg3 PUSCH Repetition Indication

The number of Msg3 repetitions is indicated in UL grant scheduling Msg3 for initial transmission and in DCI format 0_0 with CRC scrambled by TC-RNTI for retransmission. The indication is via reinterpreting the existing fields in the corresponding grant. There was some discussion in the past RAN1 meetings on whether there is signalling from the NW to signal UE whether the fields should be reinterpreted or be in the legacy interpretation. From our viewpoint, as long as the UE requests the Msg3 repetition, it will reinterpret the respective fields to obtain the number of Msg3 repetition.

Proposal 5: If the UE requests Msg3 repetition, it shall interpret the respective field(s) to obtain the number of Msg3 repetition.

For initial transmission, the number of repetitions may be indicated in the UL grant scheduling initial Msg3 transmission as agreed in RAN1 104b:

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted. - FFS additionally using MAC RAR for indication.

Furthermore, RAN1#105e made the following working assumption:

Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

We propose to re-interpret TPC command field which currently has value from -6dB to 8dB with a step size of 2dB to indicate the number of Msg3 repetition for initial Msg3 transmission. For the UEs requesting Msg3 PUSCH repetition, it would not make sense to have negative values for TPC command. An example of such joint indication is provided in Table 1. To enhance the scheduling flexibility, we can consider jointly reinterpreting TPC command and MCS in the UL grant scheduling initial Msg3 transmission for the repetition indication.

Table 1: An example of TPC command table for UEs requesting Msg3 PUSCH repetition

Proposal 6: For initial Msg3 transmission, the number of repetitions is indicated by reinterpreting TPC command in the UL grant scheduling initial Msg3 transmission.

- Alternatively, the number of repetitions is indicated by jointly reinterpreting TPC command and MCS in the UL grant scheduling initial Msg3 transmission.

If the initial transmission fails, the number of repetitions for the Msg3 retransmission can be indicated in DCI format 0_0 with CRC scrambled by TC-RNTI as agreed in RAN1 104b:

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

RAN1#105e

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

One approach to convey such indication is by repurposing the reserved field e.g., HARQ process number (4 bits) in the DCI.

Proposal 7: For Msg3 retransmission, repurpose reserved field “HARQ process number” in DCI 0_0 with CRC scrambled by TC-RNTI to indicate the number of repetitions.

# Spatial Domain Transmission Relation

In NR, the spatial domain transmission filter for Msg3 PUSCH transmission may be the same or different from the filter for PRACH transmission in the initial transmission. Furthermore, for retransmission, the UE assumes to use the same filter for PUSCH retransmission as the one used to receive PDCCH carrying DCI format 0_0 with CRC scrambled by TC-RNTI. Now the question is whether all repetition occasions within a particular transmission (initial or retransmission) should use the same spatial domain transmission filter or not. Having the same spatial filters across repetitions within a transmission may simplify the gNB implementation. On the other hand, having different spatial filters across repetitions within a transmission may provide the gNB opportunities to perform beam refinement which may improve Msg4 performance.

Proposal 8: Consider one of the following options on spatial domain transmission relation for Msg3 PUSCH transmission:

- Option 1: The UE transmits the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the same spatial domain transmission relation.

- Option 2: The UE may transmit the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the different spatial domain transmission relations.

# Frequency Hopping

In Rel-15/16, the frequency hopping is enabled for Msg3 transmission via a frequency hopping flag in RAR as specified in TS 38.213 and copied below. Such frequency hopping is essentially the intra-slot frequency hopping.

Now the question is that if intra-slot frequency hopping is enabled, how it should be applied to the repetitions within a transmission. For simplicity, the UE may assume the same starting RB and the same frequency offset for Msg3 PUSCH repetitions within a transmission.

Proposal 9: Support intra-slot frequency hopping for Msg3 PUSCH repetitions in initial and re-transmission.

- When intra-slot frequency hopping is configured, the UE assumes the same starting RB and the same frequency offset across Msg3 PUSCH repetitions.

- Intra-slot frequency hopping and inter-slot frequency hopping are not enabled at the same time.

- Use the existing field “Frequency hopping flag” in RAR for Msg3 initial transmission and DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 retransmission for indicating intra-slot frequency hopping.

In addition to the intra-slot frequency hopping, it was agreed in RAN1#104e that inter-slot frequency hopping between repetitions is also supported.

Agreements: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. FFS details, e.g., signaling etc.

Regarding signalling to configure the UE with inter-slot frequency hopping, we can indicate such configuration by reinterpreting a field in UL grant scheduling initial Msg3 transmission for Msg3 initial transmission and repurposing the reserved field “New data indicator” in DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 retransmission.

Proposal 10: Indicate inter-slot frequency hopping flag by

- Reinterpreting a field in UL grant scheduling initial Msg3 transmission for Msg3 initial transmission,

- Repurposing the reserved field “New data indicator” in DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 retransmission.

# Available Slot for Msg3 PUSCH Repetitions

The following working assumption was agreed for counting the number of repetitions on the basis of available slots for Type A PUSCH repetitions for Msg3.

Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3. - FFS: the determination of available slots.

Proposal 11: Confirm working assumption with clarification that counting the number of repetitions on basis of available slots is the only counting method for Type A PUSCH repetitions for Msg3.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

The use of flexible symbols for possible Msg3 repetition transmission may help reducing the Msg3 transmission latency. The gNB should have a good view of the flexible symbols or special slots that may be available for the UE to transmit the Msg3. Hence, we make the following proposal:

Proposal 12: For determination of available slots for Msg3 PUSCH repetitions in unpaired spectrum, if flexible symbols can be used for determining available slot, a bit map is signaled to the UE for indicating whether flexible symbols in a slot is considered for PUSCH repetition transmission or not.

- FFS: How to define the bit map.

# Joint Chanel Estimation for Msg3 PUSCH Repetitions

Joint channel estimation for Msg3 PUSCH repetitions requires the UE to maintain phase continuity across PUSCH repetitions. Such phase continuity maintenance depends heavily on UE capability.

Proposal 13: If JCE is supported for Msg3 PUSCH repetitions with subject to power consistency and phase continuity requirements, only back-to-back PUSCH transmission is supported and the UE indicates its capability of supporting JCE during RACH procedure.

For joint channel estimation, it is important that UE maintains phase continuity. However, maintaining phase continuity may become impossible for the UE if UE needs to perform DL reception between two Msg3 PUSCH repetitions. Therefore, it is useful to avoid unnecessary DL reception in the middle of Msg3 repetitions. Monitoring for Msg2 PDCCH, when UE has received RAR (including UL grant) and has not finished transmission of Msg3 yet, is not necessary.

Proposal 14: UE is not expected to monitor Msg2 PDCCH between the repetitions of Msg3.

Whether UE maintains phase continuity or gNB estimates phase jumps across different Msg3 repetitions, it is important for joint channel estimation that UE does not switch UL transmit beam in the case that gNB wants to perform joint channel estimation.

Proposal 15: gNB should be able to expect no UL beam switching among repetitions of Msg3.

- FFS whether it is configured via SIB1

- FFS whether no UL beam switching should be expected for all Msg3 repetitions or only for the case of no frequency hopping or among the repetitions of the same bundle.

For joint channel estimation, in many cases it is important for gNB to be able to estimate phase jumps across repetitions and compensate them (when phase continuity is not perfect). Inclusion of PTRS can help gNB to achieve this. Also, in case of no capability signaling in PRACH for UE capability of phase continuity, PTRS presence is useful for the gNB to detect even the presence of phase continuity or estimate its quality (to decide whether it can apply DMRS bundling or not).

Proposal 16: Support transmission of PTRS inside Msg3 repetitions

- FFS whether the presence of PTRS for Msg3 repetitions and its parameters is configured by SIB1

- FFS whether it is applicable only for FR2 or also for FR1.

# TB Transmission over Multi-slot PUSCH

There was some discussion in RAN1#104b on whether TB transmission over multi-slot PUSCH (TBoMS) can also be applicable to Msg3 repetition. From our perspectives, having TBoMS feature for Msg3 is out of scope of Msg3 PUSCH repetition.

Observation 2: TBoMS is out of scope for Msg3 PUSCH repetition.

# Conclusion

This contribution discusses our views on PUSCH repetitions for Msg3. In particular, we have made the following observations and proposals:

Observation 1: Benefit of indicating UE capability of supporting Msg3 PUSCH repetition after initial access procedure is unclear.

Observation 2: TBoMS is out of scope for Msg3 PUSCH repetition.

Proposal 1: The shared RO for requesting Msg3 PUSCH repetition can be an RO with preamble(s) for 4-step RACH only or an RO with preambles for both 4-step RACH and 2-step RACH.

- Whether an RO with preamble(s) for 4-step RACH only or an RO with preambles for both 4-step RACH and 2-step RACH is used can be indicated to the UE.

Proposal 2: If the Msg3 PUSCH repetition request is indicated by a separate RO, RAN1 should discuss whether this RO can be shared for Redcap UE to send identification via PRACH or not.

Proposal 3: The RSRP threshold depends on SS-RSRP and/or UE power class.

- FFS: whether RSRP thresholds are separate for NUL and SUL.

Proposal 4: Whether the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure can be discussed later under UE capability discussion if necessary.

Proposal 5: If the UE requests Msg3 repetition, it shall interpret the respective field(s) to obtain the number of Msg3 repetition.

Proposal 6: For initial Msg3 transmission, the number of repetitions is indicated by reinterpreting TPC command in the UL grant scheduling initial Msg3 transmission.

- Alternatively, the number of repetitions is indicated by jointly reinterpreting TPC command and MCS in the UL grant scheduling initial Msg3 transmission.

Proposal 7: For Msg3 retransmission, repurpose reserved field “HARQ process number” in DCI 0_0 with CRC scrambled by TC-RNTI to indicate the number of repetitions.

Proposal 8: Consider one of the following options on spatial domain transmission relation for Msg3 PUSCH transmission:

- Option 1: The UE transmits the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the same spatial domain transmission relation.

- Option 2: The UE may transmit the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the different spatial domain transmission relations.

Proposal 9: Support intra-slot frequency hopping for Msg3 PUSCH repetitions in initial and re-transmission.

- When intra-slot frequency hopping is configured, the UE assumes the same starting RB and the same frequency offset across Msg3 PUSCH repetitions.

- Intra-slot frequency hopping and inter-slot frequency hopping are not enabled at the same time.

- Use the existing field “Frequency hopping flag” in RAR for Msg3 initial transmission and DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 retransmission for indicating intra-slot frequency hopping.

Proposal 10: Indicate inter-slot frequency hopping flag by

- Reinterpreting a field in UL grant scheduling initial Msg3 transmission for Msg3 initial transmission,

- Repurposing the reserved field “New data indicator” in DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 retransmission.

Proposal 11: Confirm working assumption with clarification that counting the number of repetitions on basis of available slots is the only counting method for Type A PUSCH repetitions for Msg3.

Proposal 12: For determination of available slots for Msg3 PUSCH repetitions in unpaired spectrum, if flexible symbols can be used for determining available slot, a bit map is signaled to the UE for indicating whether flexible symbols in a slot is considered for PUSCH repetition transmission or not.

- FFS: How to define the bit map.

Proposal 13: If JCE is supported for Msg3 PUSCH repetitions with subject to power consistency and phase continuity requirements, only back-to-back PUSCH transmission is supported and the UE indicates its capability of supporting JCE during RACH procedure.

Proposal 14: UE is not expected to monitor Msg2 PDCCH between the repetitions of Msg3.

Proposal 15: gNB should be able to expect no UL beam switching among repetitions of Msg3.

- FFS whether it is configured via SIB1

- FFS whether no UL beam switching should be expected for all Msg3 repetitions or only for the case of no frequency hopping or among the repetitions of the same bundle.

Proposal 16: Support transmission of PTRS inside Msg3 repetitions

- FFS whether the presence of PTRS for Msg3 repetitions and its parameters is configured by SIB1

- FFS whether it is applicable only for FR2 or also for FR1.

# Reference

- RP-202928, New WID on coverage enhancements, RAN 90e, December 2020.