3GPP TSG RAN WG1 #106-e                                                      R1-2107385

e-Meeting,  August 16th – 27th, 2021

Agenda Item:	8.6.3

Source:	Spreadtrum Communications, Apple, CEPRI

Title:	Discussion on scaling factor for RedCap

Document for:	Discussion and decision

Introduction

The higher layer capability enhancement for RedCap UEs is under discussion in RAN2 [1]. Scaling factor discussion was ongoing in RAN2 email thread to reduce the L2 soft buffer size.

In this contribution, we discuss the scaling factor for RedCap from RAN1 perspective.

Discussion

Background for R15/16 NR scaling factor

The scaling factor was firstly introduced in RAN1#91, and according to the previous T-doc [2][3][4], the use cases of the scaling factor are as follows:

- Scaling factor is used to reflect the association of capability mismatch between the baseband capability and RF capability for both SA UE and NSA UE.

- Scaling factor is used to scale down maximum throughput of NSA UEs operating in EN-DC scenario that share hardware resources between LTE and NR.

For the first use case, it assumes that the UE may have good RF capability, for example, support of 256 QAM, however, the UE may not be able to process the peak data rates associated with 256 QAM in baseband when the assigned resource is very large.

For the second use case, considering the UE share hardware resources between LTE and NR, the aggregate maximum data rate from both sides should be limited within the hardware capability, therefore, the scaling factor is applied on NR side.

Observation 1: In Rel15/16, the scaling factor in spec is used to reflect the association of capability mismatch between the baseband capability and RF capability or scale down maximum throughput of NSA UEs operating in EN-DC scenario.

The scaling factor is given by higher layer parameter scalingFactor (in UE capability IEs) and can take the values 1, 0.8, 0.75, and 0.4.  Information on how to use the scaling factor can be found in the following spec [5][6]:

TS 38.306 clause 4.1.2 Supported max data rate For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL NOTE: Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL. The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations. For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅f(j) is no smaller than 4. NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and f(j)=1.

TS 38.331 clause 6.3.3 UE capability information elements FeatureSetDownlink information element -- ASN1START -- TAG-FEATURESETDOWNLINK-START FeatureSetDownlink ::= SEQUENCE { featureSetListPerDownlinkCC SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id, intraBandFreqSeparationDL FreqSeparationClass OPTIONAL, scalingFactor ENUMERATED {f0p4, f0p75, f0p8} OPTIONAL, … FeatureSetUplink information element -- ASN1START -- TAG-FEATURESETUPLINK-START FeatureSetUplink ::= SEQUENCE { featureSetListPerUplinkCC SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id, scalingFactor ENUMERATED {f0p4, f0p75, f0p8} OPTIONAL,

Note1: There is a restriction highlight in yellow. Note2: For the highlight in blue part, it can be implicitly deduced that if the scalingfactor is not reported, the scalingfactor is 1.

Observation 2: For single carrier NR SA operation, there is a restriction when use the scaling factor, i.e. the multiplication of MIMO layer, modulation order and scaling factor should no smaller than 4.

Motivations on scaling factor for Rel-17 RedCap

- From RAN 1 perspective

In R17, three use cases were considered for RedCap [7], and the requirements are summarized below.

Table 1: Use cases and requirements of R17 RedCap

| Use cases | Data rate | Latency | Reliability | Battery life |
| --- | --- | --- | --- | --- |
| Industrial wireless Sensors | Less than 2 Mbps | Communications: <100 ms Safety: 5-10 ms | 99.99% | At least few years. |
| Video surveillance | Economic video: 2-4 Mbps, High-end video: 7.5-25 Mbps | < 500 ms | 99%-99.9% | N/A |
| Wearables | Reference bitrate: 5-50 Mbps in DL and 2-5 Mbps in UL. Peak bitrate: up to 150 Mbps for DL and up to 50 Mbps for UL. | N/A | N/A | Up to 1-2 weeks |

Based on the table above, it can be observed that the peak data rate requirements for these three use cases are quite different (i.e. from 2 Mbps to 150 Mbps). It is worth noticing that different data rate requirements will translate into different hardware (e.g. memory) requirements, and hence result in different device cost.

However, there is only one UE type was defined for these three use cases in Rel.17, and for this UE type, the minimum capabilities of BW, antenna number, MIMO layer and modulation order are 20MHz, 1 Rx, 1 MIMO layer and 64QAM, respectively. With these minimum capabilities, the supported max data rate of the current RedCap can still up to 80Mbps, which is overdesigned for industrial wireless sensors and video surveillance.

Observation 3: Even with reduced BW, MIMO layer, antenna number and modulation order, the current RedCap is still overdesigned for industrial wireless sensors and video surveillance.

When a scaling factor (e.g. the current scaling factor, 0.4, 0.75, 0.8 and 1) is used, the peak data rate can be limited to different levels to match different RedCap use cases.

Table 2: Scaling factors and Approx. Data Rate

| SCS | BW | NRB | #Layer | Mod | Ts | OH(DL/UL) | SF | Approx. Data Rate(DL/UL) | Supported Use cases |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30KHz | 20M | 51 | 1 | 6 | 3.57e-5 | 0.14/0.08 | 1 | 81.9 Mbps/87.6 Mbps | Sensors Video surveillance Wearables |
| 30KHz | 20M | 51 | 1 | 6 | 3.57e-5 | 0.14/0.08 | 0.8 | 65.6 Mbps/70.1 Mbps | Sensors Video surveillance Wearables |
| 30KHz | 20M | 51 | 1 | 6 | 3.57e-5 | 0.14/0.08 | 0.75 | 61.5 Mbps/65.7 Mbps | Sensors Video surveillance Wearables |
| 30KHz | 20M | 51 | 1 | 6 | 3.57e-5 | 0.14/0.08 | 0.4* | 32.8 Mbps/35.04 Mbps | Sensors Video surveillance |
| Note: Under the current restriction” vLayers(j)⋅Qmj⋅f(j) is no smaller than 4”, 0.4 cannot be used for RedCap, since the vLayers(j)=1, Qmj=6, vLayers(j)⋅Qmj⋅f(j)=1*6*0.4=2.4 < 4. |  |  |  |  |  |  |  |  |  |

Due to the restriction, 0.4 cannot be used for RedCap, then the benefits of reusing the current scaling factor for matching the required data rate of different redcap use cases are limited.

In addition, it can be seen from above, even with the smallest one (i.e. 0.4), the data rate in DL and UL are still very large for sensors and economic video scenarios (data rate requirement is less than 4 Mbps). So the current values (even 0.4 can be used) may not be enough to cover all Rel.17 RedCap use cases, smaller values, e.g. 0.1, may need to be introduced. With scaling factor value 0.1, the approximated data rate for DL and UL are around 8.2 Mbps and 8.76 Mbps, respectively, which is more suitable for sensors and economic video surveillance.

Observation 4: From RAN1 perspective, introducing scaling factor for Rel-17 RedCap will be beneficial to matching the required data rate of different redcap use cases.

Observation 5: Due to the restriction, the benefits of reusing the current scaling factor for matching the required data rate of different redcap use cases are limited.

Observation 6: Even the restriction can be removed, the current scaling factor values may not be enough to cover all Rel.17 RedCap use cases.

- From RAN 2 perspective

From RAN 2 perspective [1], introducing scaling factor for Rel-17 RedCap is mainly focus on L2 buffer size reduction, and the discussion is ongoing in RAN2 email thread. Scaling factor can be used to scale down the supported peak data rate (MaxDLDataRate/MaxULDataRate), and according to the calculation in TS 38.306 [5], with peak data rate reductions, L2 buffer requirements for RedCap UEs are implicitly reduced accordingly. Further tightly coordination between two WGs is needed.

Observation 7: From RAN2 perspective, introducing scaling factor for Rel-17 RedCap will be potentially beneficial to L2 buffer size reduction, which is being discussed in RAN2.

Solutions on scaling factor for Rel-17 RedCap

Based on the discussion in chapter 2.1 and 2.2, three candidate schemes can be considered for Rel.17 RedCap and listed below, and the possible specification modifications can be found in appendix.

- Scheme 1: Reuse the current scaling factor, and remove the restriction for RedCap.

- Scheme 2: Reuse the current scaling factor, remove the restriction for RedCap, and introduce new value 0.1 for RedCap, e.g. the scaling factor values of RedCap are {0.1, 0.4, 0.8}.

- Scheme 3: Define a new scaling factor (New IE) to scale down maximum throughput of RedCap UEs. e.g. Scalingfactor-RedCap, and the scaling factor values are {0.1, 0.4}.

The comparison of these schemes is summarized below:

Table 3: Schemes comparison

| Schemes | Cost/ Complexity reduction | Spec impacts | Compatibility | NW impacts |
| --- | --- | --- | --- | --- |
| Scheme 1 | Medium | Low | Yes | Low |
| Scheme 2 | High | Medium | Yes | Medium |
| Scheme 3 | High | High | Yes | Medium |

Take into consideration various factors (cost/complexity reduction, Spec and NW impacts, compatibility), we recommend scheme 2 to be adopted for Rel.17 RedCap.

Proposal 1: Reuse the current scaling factor, remove the restriction for RedCap, and introduce new value 0.1 for RedCap, e.g. the scaling factor values of RedCap are {0.1, 0.4, 0.8}.

Conclusion

In this contribution, we discuss the background of R15/16 NR-UE Scaling Factor, the motivation on introducing scaling factor for R17 RedCap, and the possible solutions from RAN1 perspective. Based on the analyses and discussions, we have the following observations and proposals:

Observation 1: In Rel15/16, the scaling factor in spec is used to reflect the association of capability mismatch between the baseband capability and RF capability or scale down maximum throughput of NSA UEs operating in EN-DC scenario.

Observation 2: For single carrier NR SA operation, there is a restriction when use the scaling factor, i.e. the multiplication of MIMO layer, modulation order and scaling factor should no smaller than 4.

Observation 3: Even with reduced BW, MIMO layer, antenna number and modulation order, the current RedCap is still overdesigned for industrial wireless sensors and video surveillance.

Observation 4: From RAN1 perspective, introducing scaling factor for Rel-17 RedCap will be beneficial to matching the required data rate of different redcap use cases.

Observation 5: Due to the restriction, the benefits of reusing the current scaling factor for matching the required data rate of different redcap use cases are limited.

Observation 6: Even the restriction can be removed, the current scaling factor values may not be enough to cover all Rel.17 RedCap use cases.

Observation 7: From RAN2 perspective, introducing scaling factor for Rel-17 RedCap will be potentially beneficial to L2 buffer size reduction, which is being discussed in RAN2.

Proposal 1: Reuse the current scaling factor, remove the restriction for RedCap, and introduce new value 0.1 for RedCap, e.g. the scaling factor values of RedCap are {0.1, 0.4, 0.8}.

Reference

- R2-2107676, Email discussion report on [105][RedCap] Capabilities (Intel).

- R1-1804682, Discussion on L1 data rate, RAN1#92b, Intel.

- R1-1803648, Discussion on UE peak data rate, RAN1#92b, Huawei, HiSilicon.

- R1-1807651, Reply LS on formula or table for L1 data rate, RAN1#93.

- TS 38.306, User Equipment (UE) radio access capabilities.

- TS 38.331, Radio Resource Control (RRC) protocol specification.

- RP-211574, Revised WID on support of reduced capability NR devices.

Appendix

The possible specification modifications of different schemes are listed in A.1-A.3.

- A.1 Scheme 1

Scheme 1: Reuse the current scaling factor, and remove the restriction for RedCap.

- Modifications in 38.306

--------------------------------------------------- Start of the modifications --------------------------------------------------

4.1.2	Supported max data rate for DL/UL

…

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅f(j) is no smaller than 4.

- NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and f(j)=1.

- NOTE: This restriction is not applied when the UE is indicated as a RedCap UE.

…

--------------------------------------------------- Unchanged parts omitted --------------------------------------------------

--------------------------------------------------- End of the modifications --------------------------------------------------

- A.2 Scheme 2

Scheme 2: Reuse the current scaling factor, remove the restriction for RedCap, and introduce new value 0.1 for RedCap, e.g. the candidate values of RedCap are {0.1, 0.4, 0.8}.

- Modifications in 38.306

--------------------------------------------------- Start of the modifications --------------------------------------------------

4.1.2	Supported max data rate for DL/UL

…

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅f(j) is no smaller than 4.

- NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and f(j)=1.

- NOTE: This restriction is not applied when the UE is indicated as a RedCap UE.

…

--------------------------------------------------- Unchanged parts omitted --------------------------------------------------

--------------------------------------------------- End of the modifications --------------------------------------------------

- Modifications in 38.331

--------------------------------------------------- Start of the modifications --------------------------------------------------

6.3.3	UE capability information elements

…

–	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                 SEQUENCE {

featureSetListPerDownlinkCC        SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL          FreqSeparationClass                        OPTIONAL,

scalingFactor                      ENUMERATED {f0p4, f0p75, f0p8}             OPTIONAL,

…

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

supportedSRS-Resources Indicates supported SRS resources for SRS carrier switching to the band associated with this FeatureSetDownlink. The UE is only allowed to set this field for a band with associated FeatureSetUplinkId set to 0.

scalingFactor For Non-RedCap UEs, f0p4, f0p75, f0p8 corresponding to 0.4, 0.75, and 0.8 respectively. For RedCap UEs, f0p4, f0p75, f0p8 corresponding to 0.1, 0.4, and 0.8 respectively.

…

–	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=               SEQUENCE {

featureSetListPerUplinkCC      SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                  ENUMERATED {f0p4, f0p75, f0p8}                 OPTIONAL,

…

FeatureSetUplink field descriptions

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

scalingFactor For Non-RedCap UEs, f0p4, f0p75, f0p8 corresponding to 0.4, 0.75, and 0.8 respectively. For RedCap UEs, f0p4, f0p75, f0p8 corresponding to 0.1, 0.4, and 0.8 respectively.

--------------------------------------------------- Unchanged parts omitted --------------------------------------------------

--------------------------------------------------- End of the modifications ---------------------------------------------------

- A.3 Scheme 3

Scheme 3: Define a new scaling factor (New IE) to scale down maximum throughput of RedCap UEs. e.g. Scalingfactor-RedCap, and the candidate values are {0.1, 0.4}.

- Modifications in 38.306

--------------------------------------------------- Start of the modifications --------------------------------------------------

4.1.2	Supported max data rate for DL/UL

For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

data ratein Mbps=10-6∙j=1J(vLayersj∙Qmj∙fR(j)∙fj∙Rmax∙NPRBBWj,μ∙12Tsμ∙(1-OH(j)))

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink.

is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4.

fR(j)is the scaling factor given by higher layer parameter scalingFactor-RedCap and can take the values 1, 0.4 and 0.1.

…

--------------------------------------------------- Unchanged parts omitted --------------------------------------------------

--------------------------------------------------- End of the modifications --------------------------------------------------

- Modifications in 38.331

--------------------------------------------------- Start of the modifications --------------------------------------------------

6.3.3	UE capability information elements

…

–	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                ENUMERATED {f0p4, f0p75, f0p8}              OPTIONAL,

scalingFactor-RedCap         ENUMERATED {f0p1, f0p4,}                    OPTIONAL,

…

–	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor              ENUMERATED {f0p4, f0p75, f0p8}                     OPTIONAL,

scalingFactor-RedCap       ENUMERATED {f0p1, f0p4,}                           OPTIONAL,

…

--------------------------------------------------- Unchanged parts omitted --------------------------------------------------

--------------------------------------------------- End of the modifications --------------------------------------------------