3GPP TSG RAN WG1 #106-e			R1-2107388

e-Meeting, August 16th – 27th, 2021

Agenda item:	5.1

Title:	Discussion on Rel-17 UL Tx switching

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN1#105-e meeting, the switching mechanism between two carriers were agreed, leaving the mechanism between band A (1 carrier) and band B (2 carriers) as one of the remaining issues. We would like to discuss it in this contribution.

- Uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

It was getting consensus in the last meeting that, the 2 continuous carriers in band B “should be considered as a single uplink carrier for the purpose of UL Tx switching”, with the following details:

  - FFS: with respect to the determination of uplink switching triggering, the presence of transmission occasion on any one uplink carrier on Band B is equivalent to the presence of transmission occasion on any other uplink carrier(s) on Band B.

  - FFS: no uplink switching is triggered if the presence of transmission occasion is on one uplink carrier on Band B and the preceding uplink transmission occasion is on other uplink carrier(s) on Band B.

Like the figure below.

This is a simple situation and very much like the “2 carriers” case, thus we suggest reusing the agreed 2 carriers’ principle, with appropriate update reflecting “2 bands” reality.

2.1 SUL

For SUL, the agreement for “2 carriers” case in the last meeting was:

Agreement:

- For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused.

Now update to “2 bands”:

- For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink bands, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused.

Proposal 1: For 2 bands case on SUL,  update the agreed principle for 2 carriers with “bands” instead of “carriers”

The specification in S6.1.6.3 of TS 38.214 can be updated like:

6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers belonging to two bands respectively with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink which belongs to a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks.

Proposal 2: Introduce the draft TP for Rel-17 2Tx-2Tx switching between two uplink bands for SUL in S6.1.6.3 of TS38.214.

2.2 CA Option 1 & 2

Applying the same idea to CA option 1 and 2.

Agreement: - For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink bands, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on. - When the UE is to transmit a 2-port transmission on one uplink band and if the preceding uplink transmission is a 2-port transmission on another uplink band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two bands. Agreement: - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink bandsis configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 within band A and 1Tx on carrier 2 within band B, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 within band A and 2Tx on carrier 2 within band B, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 within band A and 0Tx on carrier 2 within band B, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control.

Proposal 3: For 2 bands case on CA option 1 & 2,  update the agreed principle for 2 carriers with “bands” instead of “carriers”

- Conclusions

Proposal 1: For 2 bands case on SUL,  update the agreed principle for 2 carriers with “bands” instead of “carriers”

Proposal 2: Introduce the draft TP for Rel-17 2Tx-2Tx switching between two uplink bands for SUL in S6.1.6.3 of TS38.214.

Proposal 3: For 2 bands case on CA option 1 & 2,  update the agreed principle for 2 carriers with “bands” instead of “carriers”

- References

- RP‑202799, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1), RAN#90e.

- Section 5.1, RAN1 Aspects for RF requirements for NR frequency range 1, Chair's Notes RAN1#105-e