3GPP TSG RAN WG1 #106-e                                                                            R1-2107389

e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | xxxx | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDCCH DMRS scrambling sequence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-8-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current 38.211, the scrambling sequence of PDCCH channel is inititalized based on nID which equals the higher layer parameter pdcch-DMRS-ScramblingID for UE-specific search space. 38.211 section 7.3.2.3 - for a UE-specific search space as defined in clause 10 of [5, TS 38.213], equals the higher-layer parameter pdcch-DMRS-ScramblingID if configured, - nID=NIDcell otherwise. However, in section 7.4.1.3.1 for the scrambling sequence of PDCCH DMRS, the current 38.211 is unclear whether higher layer parameter pdcch-DMRS-ScramblingID is only used for UE-specific search space or not. |
| Summary of change: |  | Clarify that pdcch-DMRS-ScramblingID for PDCCH DMRS sequence generation is only used for UE specific search space. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For PDCCH DMRS scrmabiling sequence generation, it is unclear whether higher layer parameter pdcch-DMRS-ScramblingID is only used for UE-specific search space or not |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The same higher layer parameter pdcch-DMRS-ScramblingID should be naturally used for UE specific search space for both PDCCH channel and PDCCH DMRS, so it is expected no isolated impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

#### 7.4.1.3	Demodulation reference signals for PDCCH

##### 7.4.1.3.1	Sequence generation

The UE shall assume the reference-signal sequence  for OFDM symbol  is defined by

.

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot,  is the slot number within a frame, and

-	for a UE-specific search space as defined in clause 10 of [5, TS 38.213],  is given by the higher-layer parameter pdcch-DMRS-ScramblingID if provided

-	 otherwise.

<Unchanged parts are omitted>