3GPP TSG RAN WG1 #106-e		R1-2107411

e-Meeting, August 16th – 27th, 2021

Source: 	CMCC

Title:	Discussion on potential modification of existing DCI formats

Agenda item:	8.6.1.4

Document for:	Discussion & Decision

- Introduction

In RAN1#105-e meeting, the following agreements were made regarding the DCI formats of RedCap UEs [1].

Agreement:

- Redcap UE is mandated to support at least DCI format 0_0/1_0.

Agreement:

Regarding DCI format 0_1/1_1 and DCI format 0_2 and 1_2,

- DCI format 0_1/1_1 are mandatory as in legacy. DCI 0_2/1_2 are optionally supported.

In this contribution, considerations on potential modification of existing DCI formats and related specs influences are discussed and proposals are given.

- Discussion on potential modification of existing DCI formats

Last RAN1 meeting has agreed to mandatory support the existing DCI formats x_0/x_1, DCI format x_2 are optionally supported, and potential modification on fields of existing DCI formats needs to be further discussed.

During initial access, RedCap UEs can reuse existing DCI formats 0_0/1_0. After initial access, RedCap UEs can use DCI formats 0_1/1_1 or compact DCI formats 0_2/1_2. There are some fields with fixed bitwidth in DCI 1_1 and 1_2, as shown by the non-bold part in Table 1. Specificially, for DCI 1_1 and 1_2, when the bandwidth of RedCap UEs is 20MHz and SCS is 30kHz, the bitwidth of frequency domain resource assignment field is 11 bits.

Besides, there are some fields with flexible bitwidth that can be configured by higher layer parameters, including carrier indicator, bandwidth part indicator, time domain resource assignment etc, which correspond to the bold part in Table 1. Compared with existing DCI 1_1 and 1_2, the potential modification of bitwidth of fields for RedCap UEs is listed as follow:

- For DCI 1_1 and 1_2, the bit field of carrier indicator can be ignored since CA is not allowed for RedCap UEs.

- For DCI 1_1 and 1_2, when separate initial BWP of RedCap UEs has different center frequency from legacy UEs, RedCap UEs may switch to initial BWP of legacy UEs to receive SSB and SI. If the number of RedCap UEs is small and traffic offloading is not necessary, RedCap UEs do not need to stay on separate initial BWP and may switch to initial BWP of legacy UEs. In this case, the bitwidth of bandwidth part indicator is 1, 2.

- For DCI 1_1, the bitwidth of downlink assignment index is 0, 2, 4 since CA is not allowed. For DCI 1_2, the bitwidth of downlink assignment index is 0, 1, 2 since CA is not allowed and total DAI is not necessary.

- For DCI 1_1, the bit fields of one-shot HARQ-ACK request, PDSCH group index, new feedback indicator, number of requested PDSCH group(s) are used for enhanced Type-2 HARQ-ACK codebook and Type-3 HARQ-ACK codebook. These two codebooks are designed for NR-U to improve HARQ-ACK feedback reliability. The motivation of introducing enhanced Type-2 and Type-3 HARQ-ACK codebook for RedCap UEs is not clear, thus these bit fields can be ignored.

- For DCI 1_1 and 1_2, the bit fields of transport block 2, CBGTI and CBGFI can be ignored since the maximum RedCap UE bandwidth is comparatively small and the packet size is similar to that in LTE network.

- For DCI 1_1, the bitwidth of ChannelAccess-CPext depends on whether RedCap UEs support shared spectrum channel access. If RedCap UEs do not support shared spectrum channel access, the bit field of ChannelAccess-CPext can be ignored.

- For DCI 1_1, the bit field of SCell dormancy indication can be ignored since CA is not allowed.

Proposal 1: In DCI 1_1 and 1_2 for RedCap UEs, the bitwidth of frequency domain resource assignment field can be reduced due to reduced maximum RedCap UE bandwidth.

Proposal 2: In DCI 1_1 and 1_2 for RedCap UEs, the bit field of carrier indicator can be ignored.

Proposal 3: In DCI 1_1 and 1_2 for RedCap UEs, the bitwidth of downlink assignment index can be reduced since CA is not allowed.

Proposal 4: In DCI 1_1 and 1_2 for RedCap UEs, the bit fields of transport block 2, CBGTI and CBGFI can be ignored.

Proposal 5: In DCI 1_1 for RedCap UEs, the bit fields of one-shot HARQ-ACK request, PDSCH group index, new feedback indicator, number of requested PDSCH group(s), SCell dormancy indication can be ignored.

In summary, the detail of DCI 1_1 and 1_2 for RedCap UEs is given in Table 1.

Table 1. Potential modification of DCI 1_1 and 1_2 for RedCap UEs

| DCI bit field | Bitwidth of field in existing DCI 1_1 | Bitwidth of field in DCI 1_1 for RedCap UEs | Bitwidth of field in existing DCI 1_2 | Bitwidth of field in DCI 1_2 for RedCap UEs |
| --- | --- | --- | --- | --- |
| Identifier for DCI formats | 1 | 1 | 1 | 1 |
| Frequency domain resource assignment | 16bit (273RB) | 11bit (50RB) | 16bit (273RB,K2=1) | 11bit (50RB,K2=1) |
| VRB-to-PRB mapping | 0,1 | 0,1 | 0,1 | 0,1 |
| transport block 1 | 8 | 8 | 6,7,8 | 6,7,8 |
| HARQ process number | 4 | 4 | 0,1,2,3,4 | 0,1,2,3,4 |
| TPC command for scheduled PUCCH | 2 | 2 | 2 | 2 |
| PUCCH resource indicator | 3 | 3 | 0,1,2,3 | 0,1,2,3 |
| DMRS sequence initialization | 1 | 1 | 0,1 | 0,1 |
| Carrier indicator | 0,3 | 0 | 0,1,2,3 | 0 |
| Bandwidth part indicator | 0,1,2 | 0,1,2 | 0,1,2 | 0,1,2 |
| Time domain resource assignment | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 |
| PRB bundling size indicator | 0,1 | 0,1 | 0,1 | 0,1 |
| Rate matching indicator | 0,1,2 | 0,1,2 | 0,1,2 | 0,1,2 |
| ZP CSI-RS trigger | 0,1,2 | 0,1,2 | 0,1,2 | 0,1,2 |
| transport block 2 | 8 | 0 | 0 | 0 |
| Downlink assignment index | 0,2,4,6 | 0,2,4 | 0,1,2,4 | 0,1,2 |
| PDSCH-to-HARQ_feedback timing indicator | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 |
| One-shot HARQ-ACK request | 0,1 | 0 | 0 | 0 |
| PDSCH group index | 0,1 | 0 | 0 | 0 |
| New feedback indicator | 0,1,2 | 0 | 0 | 0 |
| Number of requested PDSCH group(s) | 0,1 | 0 | 0 | 0 |
| Antenna port(s) | 4,5,6 | 3,4 (1Rx) 4,5 (2Rx) | 0,4,5,6 | 0,3,4 (1Rx) 0,4,5 (2Rx) |
| Transmission configuration indication | 0,3 | 0,3 | 0,1,2,3 | 0,1,2,3 |
| SRS request | 2,3 | 2,3 | 0,1,2,3 | 0,1,2,3 |
| CBGTI | 0,2,4,6,8 | 0 | 0 | 0 |
| CBGFI | 0,1 | 0 | 0 | 0 |
| Priority indicator | 0,1 | 0,1 | 0,1 | 0,1 |
| ChannelAccess-CPext | 0,1,2,3,4 | 0 | 0 | 0 |
| Minimum applicable scheduling offset indicator | 0,1 | 0,1 | 0 | 0 |
| SCell dormancy indication | 0,1,2,3,4,5 | 0 | 0 | 0 |
| Minimum total bitwidth | 49 | 35 (50RB,1Rx) | 25 | 20 (50RB,1Rx) |
| Maximum total bitwidth | 104 | 61 (50RB,1Rx) | 70 | 58 (50RB,1Rx) |

According to Table 1, through higher layer configuration, the total bitwidth of modified DCI 1_1 for RedCap UEs is much smaller than the total bitwidth of DCI 1_1 for legacy UEs. The total bitwidth of modified DCI 1_2 for RedCap UEs is smaller than the total bitwidth of modified DCI 1_1 for RedCap UEs. Thus, The bitwidth of DCI 1_1/1_2 for RedCap UEs can be reduced by higher layer configuration.

Proposal 6: The bitwidth of DCI 1_1/1_2 for RedCap UEs can be reduced by higher layer configuration.

Preparing UL received DCI and parallel

Preparing UL received DCI and parallel

无法加载全部结果

重试

正在重试…

正在重试…

- Conclusions

In this contribution, considerations on potential modification of existing DCI formats are discussed, and the following proposals are made.

Proposal 1: In DCI 1_1 and 1_2 for RedCap UEs, the bitwidth of frequency domain resource assignment field can be reduced due to reduced maximum RedCap UE bandwidth.

Proposal 2: In DCI 1_1 and 1_2 for RedCap UEs, the bit field of carrier indicator can be ignored.

Proposal 3: In DCI 1_1 and 1_2 for RedCap UEs, the bitwidth of downlink assignment index can be reduced since CA is not allowed.

Proposal 4: In DCI 1_1 and 1_2 for RedCap UEs, the bit fields of transport block 2, CBGTI and CBGFI can be ignored.

Proposal 5: In DCI 1_1 for RedCap UEs, the bit fields of one-shot HARQ-ACK request, PDSCH group index, new feedback indicator, number of requested PDSCH group(s), SCell dormancy indication can be ignored.

Proposal 6: The bitwidth of DCI 1_1/1_2 for RedCap UEs can be reduced by higher layer configuration.

- References

- [1]	RAN1 Chairman’s Notes, RAN1#105-e, e-Meeting, 10th May–27th May, 2021.