3GPP TSG RAN WG1 #106-e		R1-2107413

e-Meeting, August 16th – 27th, 2021

Source: 	CMCC

Title:	Discussion on other aspects of RedCap UE

Agenda item:	8.6.3

Document for:	Discussion & Decision

- Introduction

In RAN1#105e, the following working assumptions related to separate initial UL/DL BWP are made,

Working assumption: Both during and after initial access, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs.

- RO sharing between RedCap and non-RedCap is not precluded.

Working assumption: At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access

- FFS the details of the configuration/definition

  - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB.

  - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s)

  - whether part of the configuration can be defined instead of signaled

- If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment).

  - FFS during the initial access

- FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring

- FFS: supported bandwidths in the separate initial DL BWP

- FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs

- FFS: FDD case

When separate initial UL/DL BWP is configured, some BWP related behaviour needs to be discussed, including default BWP for BWP switching, and where to monitor paging. In this contribution, such issues are discussed and proposals are made.

- Discussion on initial BWP related behaviour

In this section, we first discuss the definition of BWP with id=0 when separate initial DL/UL BWP is configured for RedCap UE, and then discuss whether UE can monitor paging on separate initial DL BWP.

Discussion on BWP Id=0 for RedCap UEs

According to working assumption, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs. For this scenario, the motivation of configuring a separate initial UL BWP may be for traffic offloading. When the number of UEs is reduced, offloading will become not necessary. So BWP switching can be used to switch RedCap UEs from separate initial BWP to the same initial BWP as non-RedCap devices. The number of initial BWPs maintained by the base station can be reduced when the traffic load is light.

On the other hand, for connected UEs, if the data rate demand decreases, it should be able to work on narrow bandwith BWP to achieve energy saving. When the data rate requirement is high, it can switch to a larger BWP. Therefore, it is proposed that the RedCap UE can support BWP switching. And all the three BWP switching mechanisms such RRC configuration, timer-based and DCI-based can be supported.

Proposal 1: RRC configuration based, timer-based and DCI-based BWP switching are supported by RedCap UE.

To support BWP switching, the definition of BWP with Id = 0 needs to be discussed. According to current specification, the initial BWP is referred to by BWP-Id = 0. When the number of BWPs configured by RRC is less than or equal to 3, DCI can trigger a BWP switch to BWP 0. BWP switching can also be triggered by BWP inactivity timer,

UE switches its active BWP to default BWP upon expiry of the BWP inactivity timer. If the default BWP is not configured, the UE uses the initial BWP as default BWP. Therefore, it is necessary to clarify which initial BWP corresponds to BWP Id=0 for RedCap UEs when separate initial BWP is configured.

When the initial UL/DL BWP of the non-RedCap UE is wider than 20MHz, the redcap UE cannot work on the non-redcap initial UL/DL BWP, so for this case, its BWP 0 should be the separate initial UL/DL BWP.

When the initial UL/DL BWP of the non-RedCap terminal is no wider than 20MHz, the redcap terminal can work on the non-RedCap initial UL/DL BWP, which means the non-RedCap initial UL/DL BWP can be one BWP of RedCap UE. If the redcap UE accesses the network through the separate initial BWP, the BWP 0 can be the separate initial BWP. In this case, if the network wants the flexibility of switching UE to initial DL/UL BWP of non-RedCap UE, it can reconfigure the non-RedCap initial UL/DL BWP to be a RRC configured BWP. There is another possibility that, RedCap UE may access the network through the non-RedCap initial UL/DL BWP since its bandwidth is no wider than 20MHz, in this case, it’s better to set the non-RedCap initial UL/DL BWP to be BWP 0.

Therefore, for a RedCap UE, its BWP 0 is proposed to be the initial BWP which it uses for initial access, and the BWP 0 can be the separate initial DL/UL BWP or the initial DL/UL BWP for non-RedCap UE if they are no wider than 20MHz.

Proposal 2: For a RedCap UE, its BWP 0 is the initial BWP that UE used for initial access.

Discussion on initial DL BWP for paging monitoring

When separate initial UL BWP is configured, random access can be initiated on the separate initial UL BWP, and the UE can receive the transmission of Msg2 and Msg4 during initial access on the separate initial DL BWP accordingly. But whether paging can be detected on the separate initial DL BWP needs to be discussed.

For UE before RRC connection, the initial DL BWP is equal to CORESET#0, so the bandwidth of initial DL BWP for idle UE is always smaller than 20MHz. When separate initial DL BWP is configured for RedCap UEs, there will be two initial DL BWPs for RedCap UEs. Considering that the RedCap service characteristics may be different from the eMBB/URLLC service, if different DRX parameters can be configured on the separate initial DL BWP to better match the redcap service characteristics, unnecessary RedCap wakeup can be avoided and energy saving can be achieved. So paging monitoring on separate initial DL BWP can be supported for RedCap idle/inactive UEs.

Proposal 3: Paging monitoring on separate initial DL BWP can be supported for idle/inactive RedCap UEs.

- Conclusions

In this contribution, considering that separate initial DL/UL BWP may be configured for RedCap UEs, some BWP related behaviours such as default BWP or where to monitor paging for idle RedCap UE are discussed. The following proposals are made,

Proposal 1: RRC configuration based, timer-based and DCI-based BWP switching are supported by RedCap UE.

Proposal 2: For a RedCap UE, its BWP 0 is the initial BWP that UE used for initial access.

Proposal 3: Paging monitoring on separate initial DL BWP can be supported for idle/inactive RedCap UEs.

- References

[1]. Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0