3GPP TSG RAN WG1 #106-e																																																				R1-2107430

e-Meeting, August 16th – 27th, 2021

Source:	CMCC

Title:	Enhancements on time and frequency synchronization for IoT NTN

Agenda Item:	8.15.1

Document for:	Discussion and Decision

Introduction

In RAN#92-e meeting, the Rel-17 work item of NB-IoT/eMTC support for Non-Terrestrial Networks was approved with the following RAN1 objectives [1].

RAN1 Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763: - UE pre-compensation including ephemeris format (orbital / Position -Velocity) - UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris - Timing advance formula (granularity of the timing advance may be different) - Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any. Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763: - Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments. - Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects - DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB. - GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763: - Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763 - UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD - Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

In this contribution, we will discuss some time and frequency synchronization related aspects for IoT over NTN.

Discussion

UE pre-compensation including ephemeris format (orbital / Position -Velocity)

In RAN1#105-e meeting, it was agreed to support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements [2].

Agreement: - Specifications should support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

Figure 1: Short, sporadic transmissions for IoT over NTN [3].

As shown in Figure 1, for a typical short, sporadic transmissions for IoT over NTN, an IoT UE may transmit its payload once every 8 hrs, once every 24 hrs, etc; after transmitting the payload, the UE's connection is released, and it goes back into deep sleep mode, until before the next transmission occasion.

For the above short, sporadic connection paradigm, short-term but accurate ephemeris information for the serving-satellite and long-term but cursory ephemeris information for the whole satellite constellation are both useful, where, the former one can be used to assist its current transmission, while the latter one is helpful to tell UE when to awake after long-term deep sleep.

For the former functionality, i.e., providing short-term but accurate ephemeris information of the serving-satellite, satellite position and velocity state vectors based ephemeris format seems more fit for a low cost UE to do short-term orbital propagation.

While for the latter functionality, i.e., providing long-term but cursory ephemeris information for the whole satellite constellation, Kepler orbital parameter based ephemeris format seems more fit to reduce signaling overhead.

Proposal 1: Support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

Proposal 2: Support serving-satellite ephemeris broadcast based on satellite position and velocity state vectors with high accuracy for a short-term.

Proposal 3: Support whole satellite constellation ephemeris broadcast based on orbital parameter ephemeris format with low accuracy for a long-term.

Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments

In RAN1#104b-e meeting, it was agreed to support segmented UE TA pre-compensation for long PUSCH and PRACH transmission [4].

Agreement: UE pre-compensation done per N time units for long PUSCH is the baseline solution. - The pre-compensation does not vary within a block of N time units - FFS: the definition and value of N Agreement: UE pre-compensation done per N time units for long PRACH is the baseline solution. - The pre-compensation does not vary within a block of N time units - FFS: the definition and value of N

In RAN1#105-e meeting, there was an extensive discussion on how to support segmented UE TA pre-compensation mechanism, with the following issues identified [2].

Agreement: - A specification change is needed for UL transmission with repetitions R>1. - For segmented UE pre-compensation how the following is handled can be further discussed - Phase discontinuity at subframe boundary when applying new pre-compensation - Coherence time limitation due to delay/frequency drift rate during segment - Signal overlapping between different TA segments - FFS: Need for more frequent new UL gaps during long transmission - FFS: Whether sampling frequency adjustment to avoid new UL gaps can be achieved by implementation - FFS: Value of N for the number of time units and what is the time unit for the segmented UE pre-compensation

Phase discontinuity at subframe boundary when applying new pre-compensation

For single subcarrier NB-IoT, TS36.211 section 10.1.5 defines the waveform in a way that ensures phase continuity between transmitted symbols and slots. The phase continuity between symbols reduces the PAPR of the waveform. The relevant part of the specification is copied below:

TS 36.211, section 10.1.5 For , the time-continuous signal for sub-carrier index in SC-FDMA symbol in an uplink slot is defined by for where parameters for and are given in Table 10.1.5-1, is the modulation value of symbol , and the phase rotation is defined by ϕk,j=ρl mod 2+φkl ρ=π2 for BPSKπ4 for QPSK φkl=0 l=0φkl-1+2πΔfk+1/2N+NCP,lTs l>0 l=0,1,…,NTBMrepNPUSCHNRUNslotsULNsymbUL-1 l= l mod NsymbUL where NTB is the number of transport blocks defined in 16.5.1 of TS 36.213 [4].

Moderator had a calculation on the phase discontinuity effect over 1 RU as following [5].

Phase discontinuity [degree] = delay drift per subframe * sampling frequency * 360 degree Assuming total delay drift is 100 us/s , then in 1 ms subframe the delay drift is 0.1 us/ms. Then, for PUSCH we get - NB-IoT SCS=15 kHz (2*0.5 ms slot): phase discontinuity= 0.1 us/s * 180 kHz/2 * 360 degrees = 3.36 degree - NB-IoT SCS=3.75 kHz (16*2 ms slot): phase discontinuity = 16*2 * 0.1 us/s * 180 kHz/2 * 360 degrees = 103.68 degree - eMTC: phase discontinuity = 0.1 us * 1.4 MHz/2 * 360 degrees = 25.2 degrees

It is noted that if segmented UE TA correction is applied every N time units, the phase continuity will be kept within a block of N time units, while phase discontinuity may occur at the boundary of each block.

The phase discontinuity at the block boundary may cause PA efficiency loss due to increased PAPR. In order to reduce the PA efficiency loss due to phase discontinuity, the following three alternativity can be considered:

- Alt 1: adopt small block duration (i.e., 1 ms) to reduce the phase discontinuity difference at each block boundary;

- Alt 2: adopt large block duration (i.e., 27.9 ms for NB-IoT and 7.5 ms for eMTC) to reduce the frequency when phase discontinuity effect occurs within a given time duration (e.g., 256ms);

- Alt 3: spec enhancement to keep phase continuity from TA correction action. E.g., modify φkl updating equation to take TA correction action into account.

Note that Alt 1 and Alt 2 may have no or less impact to spec with the cost of PA efficiency loss.

The relationship between the average PA efficiency loss within a time long duration (e.g., 256ms) and the block duration needs further study, before way forward to spec enhancement.

Proposal 4: Regarding phase discontinuity issue when applying segmented UE TA correction, there following can be further studied.

- Alt 1: adopt small block duration to reduce the phase discontinuity difference at each block boundary

- Alt 2: adopt large block duration to reduce the frequency when phase discontinuity effect occurs

- Alt 3: spec enhancement to keep phase continuity from TA correction action

Conclusions

In this contribution, we share our views on related issues on UL time and frequency synchronization enhancements for IoT over NTN. The observations and proposals are summarised as follows:

Proposal 1: Support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

Proposal 2: Support serving-satellite ephemeris broadcast based on satellite position and velocity state vectors with high accuracy for a short-term.

Proposal 3: Support whole satellite constellation ephemeris broadcast based on orbital parameter ephemeris format with low accuracy for a long-term.

Proposal 4: Regarding phase discontinuity issue when applying segmented UE TA correction, there following can be further studied.

- Alt 1: adopt small block duration to reduce the phase discontinuity difference at each block boundary

- Alt 2: adopt large block duration to reduce the frequency when phase discontinuity effect occurs

- Alt 3: spec enhancement to keep phase continuity from TA correction action

References

- RP-211601, “NB-IoT/eMTC support for Non-Terrestrial Networks”, e-Meeting, June 14 - 18, 2021.

- Chairman's Notes RAN1#105-e, May 10th – 27th, 2021.

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)”, V17.0.0 (2021-06).

- Chairman's Notes RAN1#104b-e, April 12th – 20th, 2021.

- R1-2106289, “Summary #4 of AI 8.15.2 Enhancements to time and frequency synchronization”, e-Meeting, May 10th – 27th, 2021.