3GPP TSG RAN WG1 #106-e	                         	R1-2107439

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.2.5

Source: 	LG Electronics

Title: 	PDSCH/PUSCH enhancements to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss and provide views on multi-PDSCH/PUSCH scheduling and corresponding HARQ operation, reference signals including PT-RS, and time line related aspects adapted to each of 480 kHz and 960 kHz SCSs, to support NR in high frequency range from 52.6 GHz to 71 GHz.

- Multi-PDSCH/PUSCH scheduling

Agreement: (RAN1#104bis-e) - The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz. - FFS: Further restrictions for 480 kHz to 4 - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. - The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8. - FFS: Further restrictions for 120 kHz and 480 kHz SCS - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

For the FFS point regarding SCS-dependent maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI, we suggest not to restrict the maximum number to 4 (or less) for 120 and/or 480 kHz SCS, since up to 8 PDSCHs or PUSCHs that are scheduled by a single DCI can be beneficial irrespective of SCS and gNB is allowed to configure the maximum number as 4 whenever scheduling up to 4 PDSCHs or PUSCHs can be suitable in some cases.

For the FFS point regarding the applicability of multi-PDSCH DCI to 120 kHz, it would be preferable to apply multi-PDSCH scheduling DCI to all SCSs including 480 and 960 kHz, considering the usefulness of multi-PDSCH scheduling DCI in terms of DCI overhead reduction.

Proposal #1: Do not restrict the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI to less than 8 for 120 and/or 480 kHz SCS.

Proposal #2: Apply scheduling multiple PDSCHs by single DL DCI to all SCSs including 480 and 960 kHz.

Agreement: (RAN1#105-e) - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

It was agreed that PDSCH (or PUSCH) which is overlapped with semi-static UL (or DL) symbols is considered as “invalid”. Thus, we need to discuss how to handle HARQ process number, RV, and NDI scheduled by multi-PDSCH/PUSCH scheduling DCI, for invalid PDSCH or PUSCH.

Firstly, for HARQ process number, it was agreed that indicated HARQ process number in multi-PDSCH/PUSCH scheduling DCI applies to the first scheduled PDSCH/PUSCH and the number is incremented by one for subsequent PDSCHs/PUSCHs. If some PDSCH(s)/PUSCH(s) can be skipped due to the collision with semi-static UL/DL symbol(s), it would be reasonable to not increase HARQ process number for skipped PDSCH(s)/PUSCH(s).

Secondly, for NDI and RV values, it was agree to indicate NDI/RV per PDSCH/PUSCH in multi-PDSCH/PUSCH scheduling DCI. Similar to the case of HARQ process number signalling, it seems straight-forward not to indicate NDI and RV values for skipped PDSCH(s)/PUSCH(s).

Proposal #3: If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the HARQ process number indicated by the DCI is incremented by one by excluding the PDSCH and NDI/RV fields corresponding to the PDSCH are absent in the DCI.

Proposal #4: If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the HARQ process number indicated by the DCI is incremented by one by excluding the PUSCH and NDI/RV fields corresponding to the PUSCH are absent in the DCI.

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

As to whether or not to support CBG-based transmission for 480/960 kHz SCS, we prefer the unified behaviour for multi-PDSCH/PUSCH scheduling irrespective of SCS. In other words, CBG-based (re)transmission can be configured subject to UE capability and gNB configuration not only for 120 kHz but also for 480/960 kHz SCSs. Additionally, if CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs (or PDSCHs) are scheduled, but is present when a single PUSCH (or PDSCH) is scheduled, for multi-PUSCH (or multi-PDSCH) scheduling DCI.

Proposal #5: Support CBG-based (re)transmission for 480/960 kHz SCS, subject to optional UE capability.

Proposal #6: For multi-PDSCH (or multi-PUSCH) scheduling DCI, if CBG-based (re)transmission is configured, CBGTI field is not present when more than one PDSCHs (or PUSCHs) are scheduled, but is present when a single PDSCH (or PUSCH) is scheduled, for all SCSs.

Agreement: (RAN1#105-e) For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Regarding the maximum gap between scheduled PDSCHs or scheduled PUSCHs, several companies proposed to restrict the maximum gap to a specific value considering channel conditions such as coherence time. However, defining the proper value for the maximum gap might be controversial since channel conditions can vary depending on deployment scenario, carrier frequency, and so on. Instead, network may be able to control the gap between PDSCHs or PUSCHs based on channel conditions, traffic load, etc. Therefore, it would be more desirable for the maximum gap between PDSCHs or PUSCHs to be left up to network implementation, without specifying the corresponding gap values.

Observation #1: Adjustment of the gap between PDSCHs (or PUSCHs) for multi-PDSCH (or multi-PUSCH) scheduling DCI can be left up to network implementation.

Regarding the method to indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, several options were discussed in RAN1#105-e meeting [1]. Considering the scheduling flexibility, we prefer Option 1 or 1a, i.e., indicating scheduling offset for each individual PDSCH/PUSCH in a row of TDRA table. In detail of Option 1a, a row index of TDRA table is signalled with {K0, SLIV, mapping type} for the first PDSCH and {D, SLIV, mapping type} for each of next PDSCH(s) where D corresponds to slot level gap between adjacent PDSCHs. For example, row index 0 can be signalled with {K0=0, SLIV#1, mapping type A}, {D=0, SLIV#2, mapping type B}, {D=2, SLIV#3, mapping type A}. In this case, if multi-PDSCH scheduling DL DCI indicates row index 0 in slot k, the 1st and 2nd PDSCHs will be transmitted in slot k and the 3rd PDSCH will be transmitted in slot k+2.

Between Option 1 ({SLIV, mapping type, scheduling offset K0/K2} for each PDSCH/PUSCH in a row of TDRA table) and Option 1a ({SLIV, mapping type, distance between PXSCHs} for each PDSCH/PUSCH in a row of TDRA table), we slightly prefer Option 1a considering lower RRC signalling overhead.

Proposal #7: In order to support non-continuous resource allocation in time-domain, the following options can be considered for TDRA enhancements and Option 1a is preferred.

  - Option 1: {SLIV, mapping type, scheduling offset K0/K2} for each PDSCH/PUSCH in a row of TDRA table

  - Option 1a: {SLIV, mapping type, distance between PXSCHs} for each PDSCH/PUSCH in a row of TDRA table

In RAN1#105-e meeting [1], whether or not to allow more than one TDMed PDSCHs/PUSCHs in a slot was discussed. In our view, this feature should be supported at least for 120 kHz SCS, as in Rel-15/16. Furthermore, it would be beneficial to allow more than one TDMed PDSCHs/PUSCHs also for 480/960 kHz SCS, e.g., considering more stringent requirement on low latency.

Proposal #8: For NR FR2-2, support TDMed PDSCHs (or PUSCHs) in a slot, subject to UE capability.

  - Multi-PUSCH scheduling with a single UL DCI

Agreement: (RAN1#104-e) - For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling. - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled). - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline). - TDRA: Down-select among - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI. - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs - Applicability to multi-PDSCH scheduling in Rel-17. - Note: Other enhancements are not precluded.

Based on the above agreements made in RAN1#104-e, the following can be considered for the design of multi-PUSCH scheduling DCI.

  - URLLC related fields such as priority indicator and/or open loop power control parameter set indication: A simple way would be to apply them to all of scheduled PUSCHs. Alternatively, those fields can be present if one PUSCH is scheduled, but can be absent if more than one PUSCHs are scheduled.

For FDRA field (if enhancement is supported), DCI overhead reduction can be considered by increasing RBG size for RA type 0 or changing allocation granularity for RIV based RA type 1, only if more than one PUSCHs are scheduled.

Proposal #9: For the multi-PUSCH scheduling in Rel-17,

  - URLLC related fields such as priority indicator and/or open loop power control parameter set indication

    - Alt 1: Apply to all of scheduled PUSCHs.

    - Alt 2: Present if only a single PUSCH is scheduled, but absent otherwise.

  - Multi-PDSCH scheduling with a single DL DCI

Agreement: (RAN1#104bis-e) For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

Based on the above agreements made in RAN1#104bis-e, the followings can be considered for the design of multi-PDSCH scheduling DCI.

  - Rate matching indicator and ZP-CSI-RS trigger: This can be applied to all or part of scheduled PDSCHs (e.g., the first PDSCH)

  - Priority indicator: A simple way would be to apply this to all of scheduled PDSCHs. Alternatively, this field can be present if one PDSCH is scheduled, but can be absent if more than one PDSCHs are scheduled.

Proposal #10: For multi-PDSCH scheduling with a single DCI,

  - Rate matching indicator and ZP-CSI-RS trigger: This can be applied to all or part of scheduled PDSCHs (e.g., the first PDSCH).

  - Priority indicator:

    - Alt 1: Apply to all of scheduled PDSCHs.

    - Alt 2: Present if only a single PDSCH is scheduled, but absent otherwise.

In RAN1#105-e meeting [1], whether or not to allow 2-TB transmission for NR FR2-2 was discussed. In Rel-15 NR, 2-TB transmission is applied only when more than 4 layers are transmitted. So, the issue was whether more than 4-layer transmission could be a typical scenario in FR2-2 or not. However, depending on the form factor size of device, more than 4-layer transmission can be utilized and it should be noted that based on RAN4 discussion [2], FWA and vehicular device other than handheld type of UE can be considered for NR FR2-2. Therefore, 2-TB transmission for NR FR2-2 should not be prohibited. Nevertheless, if support of 2-TB transmission can significantly increase DCI overhead for multi-PDSCH scheduling case, we can consider to allow 2-TB transmission only if single PDSCH is scheduled but disallow 2-TB transmission otherwise.

Proposal #11: For NR FR2-2, support 2-TB transmission, subject to UE capability. Taking DCI overhead into account, 2-TB transmission can be allowed only when a DCI schedules a single PDSCH.

- HARQ operation

  - Type-1 HARQ-ACK codebook

Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - FFS: details of further pruning of the set of SLIVs - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot - FFS impact of time domain bundling, if supported

One discussion point for the details of further pruning of the set of SLIVs is how to reflect skipped PDSCH(s) due to the collision with semi-static UL symbol(s). A simple way is to exclude the SLIV corresponding to a skipped PDSCH for SLIV pruning procedure. This way would not cause any ambiguity issue for UE to construct type-1 HARQ-ACK codebook since the collision due to UL symbol(s) can be determined based on semi-static configuration.

Proposal #12: Do not consider the SLIV corresponding to a PDSCH skipped due to the collision with semi-static UL symbols, for pruning procedure of type-1 HARQ-ACK codebook generation.

  - Type-2 HARQ-ACK codebook

Agreement: (RAN1#104-e) For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback Conclusion: (RAN1#104bis-e) The following is observed for alternative 1 from prior agreement. - For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI - T-DAI in UL DCI: - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured - HARQ-ACK codebook generation: - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s) - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group. - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs. - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: (RAN1#104bis-e) The following is observed for alternative 2 from prior agreement. - For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR. - FFS: details on increment of DAI field size - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI - HARQ-ACK codebook generation: - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs. - FFS: ordering of the PDSCHs for DAI counting - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Conclusion: (RAN1#104bis-e) The following is observed for alternative 3 from prior agreement. - For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed. - Else if M equals to 1, Alt 3 is the same with Alt 2. - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that - The number of HARQ-ACK bits corresponding to each DAI increases by M times. - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M. - FFS: details on DAI field size - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell - In addition, new RRC parameter to configure M needs to be introduced. - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH Agreement: (RAN1#105-e) If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: (RAN1#105-e) If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

Based on the above agreement/conclusions for type-2 HARQ-ACK codebook, we suggest not to further consider Alt 3 considering the large amount of specification impact is anticipated for Alt 3, compared to Alts 1 and 2.

Furthermore, for Alt 2, to prevent from increasing C-DAI size in DCI format 1_0 (as per the agreement), introducing two sub-codebooks where first sub-codebook is for single PDSCH scheduling case and second sub-codebook is for multi-PDSCH scheduling case can be considered. In this case, bit-width of C/T-DAI in DL DCI is increased for multi-PDSCH DCI but not increased for single-PDSCH DCI. In UL DCI, additional UL DAI field (with increased bit-width compared to legacy UL DAI) is required for multi-PDSCH DCI, for all serving cells including a serving cell not configured with multi-PDSCH DCI. In detail, increased bit-width of each DAI field can be equal to 2 + ceiling of log2{the configured maximum number of PDSCHs across carriers in a PUCCH cell group}. If CBG is additionally configured, three individual sub-codebooks can be considered, e.g., where first sub-codebook is for TB-based single-PDSCH scheduling case, second sub-codebook is for CBG-based PDSCH scheduling case, and third sub-codebook is for multi-PDSCH scheduling case.

Alternatively, single codebook can be considered by quantizing the DAI value in fallback DCI. For example, if C-DAI size in DCI format 1_1 equals to 4 bits (assuming up to 4 PDSCHs can be scheduled by a single DCI), DAI indication {00, 01, 10, 11} in DCI format 1_0 can correspond to DAI values {4, 8, 12, 16}, respectively, which can avoid codebook size mismatch caused by DCI missing but lead to HARQ-ACK feedback overhead increase.

For Alt 1, two sub-codebooks should be supported to consider that HARQ-ACK payload size with two sub-codebooks is less than that with single codebook and two sub-codebook concatenation was already introduced from Rel-15 NR. If CBG is additionally configured, the number of sub-codebooks can be kept as two, e.g., where first sub-codebook is for TB-based single-PDSCH scheduling case and second sub-codebook is for CBG-based PDSCH scheduling and multi-PDSCH scheduling cases.

Proposal #13: For (enhanced) type-2 HARQ-ACK codebook,

  - If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted, two sub-codebooks where one is for single PDSCH scheduling case and the other is for multi-PDSCH scheduling case are introduced. If CBG is additionally configured, the number of sub-codebooks is kept as two and HARQ-ACK corresponding to CBG-based PDSCH scheduling and multi-PDSCH scheduling cases is merged into the same sub-codebook.

  - If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted, two sub-codebooks where one is for single PDSCH scheduling case and the other is for multi-PDSCH scheduling case are introduced to prevent from increasing C-DAI size in DCI format 1_0. If CBG is additionally configured, the number of sub-codebooks is increased to three where first sub-codebook is for TB-based single-PDSCH scheduling case, second sub-codebook is for CBG-based PDSCH scheduling case, and third sub-codebook is for multi-PDSCH scheduling case.

As to time bundling operation, the following methods can be considered:

  - Method 1: Time domain HARQ-ACK bundling operation per M PDSCHs

  - Method 2: Time domain HARQ-ACK bundling operation per N slots

  - Method 3: Time domain HARQ-ACK bundling operation per PDSCH group where the number of groups is configured to L and scheduled PDSCHs are evenly divided into L groups.

Since the motivation of introducing time domain bundling is to consider channel correlation in time domain, time bundling window (in which HARQ-ACK bits can be bundled) should not be dependent of how many PDSCHs are scheduled, but dependent of the absolute time of scheduled PDSCHs. In this sense, Method 1 or 2 is preferred. For instance of Method 2, if time duration spanned by scheduled PDSCHs is shorter than N slots, 1 bit is generated, otherwise, 2 bits are generated. In addition, if time bundling operation is introduced for type-2 HARQ-ACK codebook, it should also be considered for type-1 HARQ-ACK codebook.

Proposal #14: Consider the following methods if time bundling operation is introduced:

  - Method 1: Time domain HARQ-ACK bundling operation per M PDSCHs

  - Method 2: Time domain HARQ-ACK bundling operation per N slots

  - Two PUCCHs corresponding to a single multi-PDSCH scheduling DCI

Agreement: (RAN1#104-e) - For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1, - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. - It is noted that granularity of K1 can be separately discussed. - FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

In RAN1#104bis-e meeting, it was discussed that two separate PUCCHs can be triggered by a multi-PDSCH scheduling DCI, which may lead to enhancing HARQ feedback latency of preceding PDSCHs among PDSCHs scheduled by the DCI. However, in order to reduce several slots of HARQ-ACK latency, at least the followings should be additionally considered:

- How to separately allocate resource for two PUCCHs (e.g., K1, PRI, etc)

- How to signal individual DAI values corresponding to two PUCCHs

- Under which condition(s) two PUCCHs are indicated by the DCI (e.g., in case more than N PDSCHs are scheduled)

Proposal #15: Further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI can be carried by two different PUCCHs, at least considering the follows:

  - How to separately allocate resource for two PUCCHs (e.g., K1, PRI, etc)

  - How to signal individual DAI values corresponding to two PUCCHs

  - Under which condition(s) two PUCCHs are indicated by the DCI (e.g., in case more than N PDSCHs are scheduled)

- PT-RS enhancements

Regarding the new PTRS pattern support, we attempted a simple evaluation that reconfirms the limitations of the current PTRS structure by comparing the CPE-only compensation and the de-ICI filtered compensation using the existing distributed PTRS pattern and de-ICI filtering. The results are shown in Figure 1, Figure 2 and Figure 3. For all considered SCS (i.e., 120/480/960 kHz), the difference between ideal cases (no PN or ideal filter estimation) and simple LS ICI compensation based on Rel-15 PT-RS with appropriately selected PT-RS density and filter size is well within 1 dB. This means that any further improvement, either by advanced estimation algorithms (e.g., MMSE) or by enhanced PT-RS pattern will be within this 1 dB, and most likely, much less. As an example, PT-RS pattern with nulling in Figure 2 and Figure 3 provides stable, but not very large improvement in comparison with Rel-15 PT-RS pattern, but in case of higher MCS(e.g., MCS 28) and may provide improvement up to 0.2 - 0.3 dB. These results pose a very strong limitation for further improvement of the phase noise compensation, either by advanced estimation algorithms or by corresponding PT-RS enhancements.

Figure 1. BLER performance with different PN compensation (120 kHz SCS)

Figure 2. BLER performance with different PN compensation (480 kHz SCS)

Figure 3. BLER performance with different PN compensation (960 kHz SCS)

Proposal #16: Retain the existing distributed PT-RS structure for all SCS in FR2-2. It seems that further improvement either by advanced estimation algorithms or by new PT-RS pattern cannot achieve a noticeable improvements.

The following agreement was captured in 104bis-e meeting.

Agreement: - In Rel-17, for NR operation in 52.6 – 71 GHz, conclude that increased PTRS frequency density is not supported for CP-OFDM at least for Rel-15 PTRS pattern when the allocated number of RB > 32 - Companies are encouraged to study whether to increase PTRS frequency density for small RB allocations for CP-OFDM for NR operation in 52.6 to 71 GHz with respect to phase noise compensation performance - CPE and ICI PN compensation - Note: Results for CPE compensation-only are to be reported for reference - (K = 0.5, L = 1), (K = 1, L = 1), (K = 2, L = 1), - Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols - Number of RBs: 8, 16, 32 - Other values of K and number of RBs are not precluded - Study on other aspects of potential PTRS enhancement (e.g., decreased PTRS frequency density) is not precluded

With the legacy PTRS density values (K=2 and K=4), for a small number of RB allocated, the number of available PTRS will be low to the point that is impossible to estimate filter taps due to insufficient number of data. This is especially true in the case of the advanced PTRS schemes that require RE grouping in chunks. Thus, increasing the number of PTRS frequency density may improve the filter estimate and potentially lead to performance increase over CPE-only compensation. To investigate the potential gain from the increased PTRS density, we compared two additional PTRS density values K = 1 and K = 0.5 with the legacy density K = 2 in our companion paper [3], where the evaluation assumptions and more evaluation results can be found.

Figure 4, Figure 5 and Figure 6 show the BLER performance for the several combinations of the PTRS density and receiving algorithm. By comparing the curves for different combinations of PTRS parameters and SCS, we can observe that the increase of PTRS density for the case of small RB allocations, e.g., 8 or 16 RBs, still does not provide enough performance gain for 3 tap de-ICI filtering. In addition, for CPE-only compensation, increasing the PTRS density does not provide any significant improvement. Moreover, K=0.5 do not provide any BLER improvement in comparison with K=2 since the increased overhead is compensated by the code rate. So, in our view, additional support for PTRS density K values such as K=0.5 and K=1 is not required.

Figure 4. BLER performance (120 kHz, MCS 16 and 22, 8 RB allocated)

Figure 5. BLER performance (120 kHz, MCS 16 and 22, 16 RB allocated)

Figure 6. BLER performance (120 kHz, MCS 16 and 22, 32 RB allocated)

Observation #2: Based on the performed evaluation research, PTRS frequency density K=0.5 or 1 does not provide enough performance gain, or provide marginal gain by 0.1 dB in very specific situations.

Proposal #17: Do not support K=0.5 and K=1

One more thing to be considered is how much the nulling PTRS allocation could reduce the computational complexity compared to legacy PTRS allocation. In our companion paper [x], we presented this in detail and draw the following observation.

Observation #3: Complexity analysis of the de-ICI filtering approaches has shown that the main contribution to the overall number of operations comes from the application of the filter and not from the taps estimation procedure. Thus, for the advanced Nulling (or zero-padded) PTRSs scheme, the computation gain with respect to the baseline Rel-15 is typically less than 3%.

- DM-RS enhancements

Agreement: - Existing DMRS patterns are supported for NR operation in 52.6 to 71 GHz with 120 kHz SCS. - At least existing DMRS patterns are supported for NR operation in 52.6 to 71 GHz with 480 kHz and/or 960 kHz SCS - Further study on whether to introduce different DMRS pattern with increased frequency domain density (in number of subcarriers) than the existing DMRS patterns for NR operation in 52.6 to 71 GHz with 480 kHz and/or 960 kHz SCS - Further study on whether and how to restrict DMRS port configuration (e.g., the number of DMRS ports) as in FR2 for NR operation in 52.6 to 71 GHz with 480 kHz and/or 960 kHz SCS Agreement: Further study on at least the following aspects of potential DMRS enhancement with respect to FD-OCC: - whether to support a configuration of DMRS in which FD-OCC is not applied for 480 kHz and 960 kHz SCS - Applicability to Type-1 and/or Type-2 DMRS - Details on whether and how to indicate that FD-OCC is not applied to DMRS port - Impact to UE multiplexing capacity and inter-UE interference in MU-MIMO

In FR2-2, the effect of phase noise on PDSCH/PUSCH reception is seriously negative, and for this reason, it may be necessary to use PT-RS for all SCSs. Not to mention the need for PT-RS for 120 kHz SCS, it seems necessary for high SCS such as 960 kHz, at least to compensate CPE. On the other hand, the current specifications do not allow some DM-RS ports to be configured at the same time as the PT-RS (e.g., for PDSCH reception, any DM-RS ports among 1004-1007 or 1006-1011 for DMRS type-1 or type-2). Therefore, when using PT-RS, the UE does not expect an index that contains these ports from the Table which is associated to the antenna port field in DCI (e.g., values 16-19, 22, 23 in Figure 7). It may be beneficial to study how these unused indexes can be reused for high SCS in FR2-2 when PT-RS is transmitted to UE.

Figure 7. Values (in yellow) that are not used simultaneously with PT-RS among values for antenna port field in DCI (partially excerpted from Table 7.3.1.2.2-2 in 38.212)

If the coherence bandwidth of the channel is less than DM-RS spacing, the orthogonality with DM-RS port to which FD-OCC is applied would be broken and the channel estimation accuracy would be degraded. In this reason, FD-OCC should be disabled and the details on how to indicate that FD-OCC is not applied to DM-RS ports should be specified at least for high SCS such as 960 kHz. Currently, TD-OCC is not applied to DM-RS ports when PT-RS is transmitted to UE. With the similar way, FD-OCC will be not applied to DM-RS ports without a separate DCI indication. For example, when PDSCH is transmitted with high MCS in FR2-2 channel with large delay spread, PT-RS should be transmitted together. If high SCS is also configured to this PDSCH, the reception performance may be degraded due to FD-OCC and turning FD-OCC off may be required. In this case, with combinations of configured SCS, MCS and PT-RS, whether FD-OCC is on or off can be indicated to UE. As an additional method indicating FD-OCC turned-off implicitly, the unused value of antenna port field explained above can be reused for this purpose.

Proposal #18: DM-RS configuration without FD-OCC should be supported for high SCS.

Proposal #19: Further study on how to indicate implicitly that FD-OCC is not applied to DM-RS port is required.

- Time-line related aspects for 480 kHz and 960 kHz SCSs

Agreement: - RAN1 use the absolute time duration for 120 kHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 kHz and 960 kHz SCS for NR operation in 52.6 to 71 GHz - RAN1 strives to reduce the absolute time durations from the upper bound if feasible - FFS: How to derive timeline values - Case by case study - FFS: model-based approach for selected timelines, e.g. exponential models, projection based on log-linear regression, etc. Agreement: Further study at least the following aspects of timelines to support both single PDSCH/PUSCH and multi-PDSCH/PUSCH scheduling for NR operation in 52.6 GHz to 71 GHz. - Time unit and applicability to selected timelines - Value and/or range of value - Potential impact on UE capability Agreement: - The following UE processing timelines are prioritized for discussion - PDSCH processing time (N1), PUSCH preparation time (N2), HARQ-ACK multiplexing timeline (N3) - configuration(s)/default values of k0 (PDSCH), k1 (HARQ), k2 (PUSCH) - CSI processing time, Z1, Z2, and Z3, and CSI processing units - Note: the order of the above sub-bullets represents the priority for discussion in descending order - Companies are encouraged to provide preferred values/ranges of timelines for discussion Agreement: FFS: The need for enhancements and standardization, of the following additional processing timelines: - UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH - SRS, PUCCH, PUSCH, PRACH cancellation with dynamic SFI - ZP CSI Resource set activation/deactivation - Application delay of the minimum scheduling offset restriction - timing aspects related to cross carrier operation

Some timeline aspects, e.g., k0/k1/k2, were prioritized for discussion as agreement above. We discuss and provide our views on how to determine the value ranges for such timeline aspects to flexibly support both single- and multi-PDSCH/PUSCH scheduling with high SCS such as 480 kHz or 960 kHz. In addition, the potential enhancement to CPU availability check are discussed.

- Additional time margin for PDSCH processing capability (i.e., N1 symbols)

The effect of phase noise to PDSCH reception could be seriously negative and ICI compensation might be necessary for performance enhancement in FR2-2. Especially, PDSCH reception performance using high MCS for 120 kHz SCS may be seriously deteriorated, and thus the UE would have to compensate for ICI with the help of, for example, de-ICI filtering. Due to the use of additional signal processing in UE for ICI mitigation, additional time margin would be required to timeline requirement for PDSCH processing and HARQ-ACK preparation. In this regard, whether ICI mitigation is required or not may be determined according to the SCS configured for UE. For instance, in the case of a 120 kHz SCS, ICI compensation is required, and in the case of a 960 kHz SCS, it may not be necessary. Similarly, for the MCS, the amount of signal processing required by the receiver may be different depending on the case of QPSK/16QAM and the case of using an MCS of 64QAM or higher. For this reason, it should be investigated that the time required for PDSCH processing for ICI compensation necessitates an additional time margin.

Proposal #20: Consider additional UE PDSCH processing procedure time (i.e., N1 symbols) when UE is required to perform both of CPE and ICI compensation, e.g., for 120 kHz SCS and 64 QAM.

- Time unit and value ranges for k0/k1/k2

For high SCS newly introduced in FR2-2, PDCCH/PDSCH processing or PUSCH preparation in a shortened slot duration may become a burden to UE. The candidate values supported in current specification for k0, k1 and k2, may not be sufficient to meet UE processing capability with high SCS. Moreover, it may be needed to increase the range of k0/k1/k2 values by RRC/DCI in order to support both single-PDSCH/PUSCH scheduling and multi-PDSCH/PUSCH scheduling flexibly. Therefore, value range of k0/k1/k2 should be revisited for high SCS. One direction would be defining new time unit for them with multi-slot granularity. It may be considered to define a slot-group consisting of M slots and interpret k0/k1/k2 as multiplied by M from the value configured/indicated by RRC or DCI. M can be determined separately according to each SCS. For example, for 480 kHz or 960 kHz, M may be determined to be 4 or 8 to align with the slot duration for 120 kHz.

Proposal #21: Indicated (or configured) value(s) for k0/k1/k2 can be interpreted as multiplied by M where M denotes the number of slots in a slot-group (if configured).

Another direction can be adopting the offset to the currently supported values. With the same time unit for k0/k1/k2, the increased range of values can be achieved by adding the offset to the indicated k0/k1/k2 values. For example, N1-dependent value may be added to the indicated k1 as an offset.	For large SCS (e.g., 960 kHz), PDSCH processing in a few slots could be burden to UE. Currently, N1 for 120 kHz can be 20 or 24 symbols and moreover for 480 kHz or 960 kHz SCS, if defined, this could be much larger than that for 120 kHz. In this case, small k1 values would rarely be configured when UE has a large N1. It is obviously an inefficient way to use limited number of k1 values. Simple solution to this is adding N1-dependant offset such as N1/14 or N1/14 to the indicated k1. With such offset, while maintaining the overhead to indicate k1, it is possible to substitute a value with a low probability of use with a realistic value considering N1. In addition, same offset can be adopted to adjust the value range of PDSCH-to-HARQ_feedback timing indicator field in DCI. Without any changes in DCI field, UE may be indicated a more practical PDSCH processing time with respect to large N1. The same approach can be also applicable to k2 which can be adjusted based on increased N2 value for 480 kHz or 960 kHz SCS. For k0, slot offset can be considered especially when cross-carrier scheduling between different numerologies for PDCCH and PDSCH is configured.

Proposal #22: The configured and default value of k1 (or PDSCH-to-HARQ_feedback), should be adjusted to practical value considering the increased N1, e.g., ceil(N1/14) or floor(N1/14).

Proposal #23: The configured and default value of k2 should be adjusted to practical value considering the increased N2, e.g., ceil(N2/14) or floor(N2/14).

Proposal #24: The configured value of k0 should be adjusted to practical value considering the UE PDSCH reception preparation time with cross carrier scheduling with different numerologies for PDCCH and PDSCH.

In addition, it is also worth considering how to determine k0 and k1 in conjunction with each other. To ensure scheduling flexibility for multi-PDSCH scheduling by single DCI with high SCS, value ranges of k0 and k1 needs to be increased. However, in the case where the UL slot is semi-statically determined such as UL/DL TDD configuration, k0 and k1 may not need to be configured with large value at the same time. In this case, the range of the required k0 value can be determined based on the indicated k1 value. In other words, it can be beneficial to determine the offsets to be applied to k0 and k1 taking into account the dependencies between each other. For instance, smaller offset to k1 can be applied if larger values of k0 are indicated, vice versa.

Proposal #25: Consider the dependence of each other when determining the value range of k0 and k1.

- CSI processing unit (CPU) availability check for multiple numerologies across active BWPs in different component carriers

High SCSs 480 kHz and 960 kHz may also cause problems to check the CPU occupancy in unit of symbol. Given these shortened symbol lengths, symbol-group level processing instead of symbol-by-symbol processing could be beneficial with respect to simplified UE implementation and potential power consumption reduction.

When a UE is configured to process CSI reports, each report is allocated to one or multiple available CPU(s). If there are not enough CPUs available due to that the UE is already processing other CSI reports, the CSI reporting for which required CPUs are not allocated does not have to be calculated/updated by the UE. In addition, if a UE is configured to process multiple CSI reports with different numerologies, the processing time would be different depending up on the corresponding numerologies for each CSI report. In other words, CPU occupancy time for high SCS such as 960 kHz is relatively shorter than that of low SCS like 120 kHz. If multiple CSI reports are configured with multiple SCSs, the chance for high SCS to get available CPU at the time of checking unoccupied CPU may be less than that for low SCS. For instance, when all of CPUs are occupied for several symbol duration (in unit of symbol duration for 120 kHz) by CSI report processing for 120 kHz SCS, CSI report for 960 kHz SCS may not occupy CPUs for a long time (in unit of symbol duration for 960 kHz) although the required CPU processing time for 960 kHz SCS is relatively very short. Conversely, CPU available check for 120 kHz SCS may be dropped if all of CPUs are occupied by CSI processing for 960 kHz. In this case, the next CPU availability check for 120 kHz SCS should be postponed until its own symbol boundary even if CPUs are no longer occupied by 960 kHz SCS. This is an inefficient management for limited CPU resources and seems to need improvement.

To address these inefficient scenarios for checking CPU availability, potential enhancement for CPU management may be needed for the case of multiple numerologies across multiple BWPs. One simple solution for that is use of reference SCS for all CPU availability check regardless of actual SCS. Reference SCS can be the lower SCS than the configured SCS, especially when high SCS is configured for UE. Similarly, it can be considered that symbol-group can be used as a time unit for CPU availability check. For example, by using 8 symbol duration for 960 kHz as a time unit for CPU check, the use of the common time unit for CPU check is possible for both 960 kHz and 120 kHz SCSs. In addition, giving priority for CPU occupancy according to the CSI report type and associated SCS can be considered to address these situations.

Proposal #26: Consider CSI processing timeline enhancements for better availability for CPUs for multiple CSI reports associated with different numerologies.

- Conclusions

In this contribution, multi-PDSCH/PUSCH scheduling and corresponding HARQ operation, reference signals including PT-RS, and time line related aspects adapted to each of 480 kHz and 960 kHz SCSs, to support NR in the frequency range from 52.6 GHz to 71 GHz were discussed. The derived proposals and observations are as follows,

Proposal #1: Do not restrict the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI to less than 8 for 120 and/or 480 kHz SCS.

Proposal #2: Apply scheduling multiple PDSCHs by single DL DCI to all SCSs including 480 and 960 kHz.

Proposal #3: If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the HARQ process number indicated by the DCI is incremented by one by excluding the PDSCH and NDI/RV fields corresponding to the PDSCH are absent in the DCI.

Proposal #4: If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the HARQ process number indicated by the DCI is incremented by one by excluding the PUSCH and NDI/RV fields corresponding to the PUSCH are absent in the DCI.

Proposal #5: Support CBG-based (re)transmission for 480/960 kHz SCS, subject to optional UE capability.

Proposal #6: For multi-PDSCH (or multi-PUSCH) scheduling DCI, if CBG-based (re)transmission is configured, CBGTI field is not present when more than one PDSCHs (or PUSCHs) are scheduled, but is present when a single PDSCH (or PUSCH) is scheduled, for all SCSs.

Observation #1: Adjustment of the gap between PDSCHs (or PUSCHs) for multi-PDSCH (or multi-PUSCH) scheduling DCI can be left up to network implementation.

Proposal #7: In order to support non-continuous resource allocation in time-domain, the following options can be considered for TDRA enhancements and Option 1a is preferred.

  - Option 1: {SLIV, mapping type, scheduling offset K0/K2} for each PDSCH/PUSCH in a row of TDRA table

  - Option 1a: {SLIV, mapping type, distance between PXSCHs} for each PDSCH/PUSCH in a row of TDRA table

Proposal #8: For NR FR2-2, support TDMed PDSCHs (or PUSCHs) in a slot, subject to UE capability.

Proposal #9: For the multi-PUSCH scheduling in Rel-17,

  - URLLC related fields such as priority indicator and/or open loop power control parameter set indication

    - Alt 1: Apply to all of scheduled PUSCHs.

    - Alt 2: Present if only a single PUSCH is scheduled, but absent otherwise.

Proposal #10: For multi-PDSCH scheduling with a single DCI,

  - Rate matching indicator and ZP-CSI-RS trigger: This can be applied to all or part of scheduled PDSCHs (e.g., the first PDSCH).

  - Priority indicator:

    - Alt 1: Apply to all of scheduled PDSCHs.

    - Alt 2: Present if only a single PDSCH is scheduled, but absent otherwise.

Proposal #11: For NR FR2-2, support 2-TB transmission, subject to UE capability. Taking DCI overhead into account, 2-TB transmission can be allowed only when a DCI schedules a single PDSCH.

Proposal #12: Do not consider the SLIV corresponding to a PDSCH skipped due to the collision with semi-static UL symbols, for pruning procedure of type-1 HARQ-ACK codebook generation.

Proposal #13: For (enhanced) type-2 HARQ-ACK codebook,

  - If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted, two sub-codebooks where one is for single PDSCH scheduling case and the other is for multi-PDSCH scheduling case are introduced. If CBG is additionally configured, the number of sub-codebooks is kept as two and HARQ-ACK corresponding to CBG-based PDSCH scheduling and multi-PDSCH scheduling cases is merged into the same sub-codebook.

  - If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted, two sub-codebooks where one is for single PDSCH scheduling case and the other is for multi-PDSCH scheduling case are introduced to prevent from increasing C-DAI size in DCI format 1_0. If CBG is additionally configured, the number of sub-codebooks is increased to three where first sub-codebook is for TB-based single-PDSCH scheduling case, second sub-codebook is for CBG-based PDSCH scheduling case, and third sub-codebook is for multi-PDSCH scheduling case.

Proposal #14: Consider the following methods if time bundling operation is introduced:

  - Method 1: Time domain HARQ-ACK bundling operation per M PDSCHs

  - Method 2: Time domain HARQ-ACK bundling operation per N slots

Proposal #15: Further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI can be carried by two different PUCCHs, at least considering the follows:

  - How to separately allocate resource for two PUCCHs (e.g., K1, PRI, etc)

  - How to signal individual DAI values corresponding to two PUCCHs

  - Under which condition(s) two PUCCHs are indicated by the DCI (e.g., in case more than N PDSCHs are scheduled)

Proposal #16: Retain the existing distributed PT-RS structure for all SCS in FR-X. It seems that further improvement either by advanced estimation algorithms or by new PT-RS pattern cannot achieve a noticeable improvements.

Observation #2: Based on the performed evaluation research, PTRS frequency density K=0.5 or 1 does not provide enough performance gain, or provide marginal gain by 0.1 dB in very specific situations.

Proposal #17: Do not support K=0.5 and K=1.

Observation #3: Complexity analysis of the de-ICI filtering approaches has shown that the main contribution to the overall number of operations comes from the application of the filter and not from the taps estimation procedure. Thus, for the advanced Nulling (or zero-padded) PTRSs scheme, the computation gain with respect to the baseline Rel-15 is typically less than 3%.

Proposal #18: DM-RS configuration without FD-OCC should be supported for high SCS.

Proposal #19: Further study on how to indicate implicitly that FD-OCC is not applied to DM-RS port is required.

Proposal #20: Consider additional UE PDSCH processing procedure time (i.e., N1 symbols) when UE is required to perform both of CPE and ICI compensation, e.g., for 120 kHz SCS and 64 QAM.

Proposal #21: Indicated (or configured) value(s) for k0/k1/k2 can be interpreted as multiplied by M where M denotes the number of slots in a slot-group (if configured).

Proposal #22: The configured and default value of k1 (or PDSCH-to-HARQ_feedback), should be adjusted to practical value considering the increased N1, e.g., ceil(N1/14) or floor(N1/14).

Proposal #23: The configured and default value of k2 should be adjusted to practical value considering the increased N2, e.g., ceil(N2/14) or floor(N2/14).

Proposal #24: The configured value of k0 should be adjusted to practical value considering the UE PDSCH reception preparation time with cross carrier scheduling with different numerologies for PDCCH and PDSCH.

Proposal #25: Consider the dependence of each other when determining the value range of k0 and k1.

Proposal #26: Consider CSI processing timeline enhancements for better availability for CPUs for multiple CSI reports associated with different numerologies.

- References

R1-2106105, “Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ),” Moderator (LG Electronics), RAN1#105-e

R4-2107973, “WF on 60 GHz UE Tx requirements,” Intel Corporation, RAN4#99-e

R1-2107442, “Link level evaluation results and complexity analysis for NR FR2-2 PTRS design”, RAN1#106-e, LG Electronics