3GPP TSG RAN WG1 #106-e                               R1-2107446

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.3.3

Source: 	LG Electronics

Title: 	Discussion on Intra-UE multiplexing/prioritization

Document for:	Discussion and decision

- Introduction

In RAN1#105-e, following agreements were made to support intra-UE multiplexing in Rel-17 [1].

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. - Rel-15 design (for PF0 and PF1) is baseline.

In this contribution, we discuss and provide our views on the intra-UE multiplexing and prioritization, in terms of processing, multiplexing, and mapping UCIs with different priority on PUCCH and PUSCH.

- Multiplexing of UCI/PUSCH with different priority

Regarding the multiplexing of UCIs with different priority on a same PUCCH/PUSCH, several aspects need to be taken into account as the followings, in terms of UCI processing/multiplexing/mapping and overall UCI multiplexing procedure.

  - General aspects

- Encoding of different priority UCIs

It was agreed to support separate encoding for low priority (LP) HARQ-ACK and high priority (HP) HARQ-ACK multiplexed on a same PUCCH, in case when the total number of LP and HP HARQ-ACK bits is more than 2. Under the agreement, there are several FFS points as below.

- FFS point 1: How to perform encoding for the case with HP HARQ-ACK of up to 2 bits and/or LP HARQ-ACK of up to 2 bits (e.g., PUSCH UCI encoding scheme or RM coding with bit-padding)

- FFS point 2: How to handle CSI (with part 1 and/or part 2) if the CSI would multiplex with HP HARQ-ACK on a same PUCCH

- FFS point 3: Whether to keep the (maximum) number of encoding chains for UCI multiplexing on PUCCH same as that in Rel-15/16

- FFS point 4: How to multiplex (or perform encoding for) HP HARQ-ACK and LP HARQ-ACK on PUCCH format 2

Firstly, regarding FFS point 1, currently separate encoding for the UCI of up to 2 bits on PUCCH is not supported and not defined in the specification. In this case, if the encoding scheme (e.g., repetition coding) used for UCI on PUSCH is applied, the maximum UCI coding rate currently configured considering the coding gain of RM code and polar code, needs to additionally cover that of the repetition code. For this reason, to minimize impacts to the specification as well as UE implementation, it can be considered to apply RM coding with simple bit-padding (to make 3-bit payload) so that current UCI coding chains/procedures are reused as it is without any spec change.

Proposal #1: Consider to apply RM coding with bit-padding for HP/LP HARQ-ACK of up to 2 bits (in case when the total number of LP and HP HARQ-ACK bits is more than 2), in order to minimize impacts to the specification as well as UE implementation.

Secondly, regarding FFS point 2, just dropping both CSI part 1 and 2 might be able to make UE behaviour a bit simpler, but by doing so, the gNB would not be able to acquire sufficient CSI feedback used for scheduling of PDSCH toward the same UE. For this reason, it is preferred to support multiplexing of HP HARQ-ACK and at least CSI part 1 (and/or LP HARQ-ACK) on a PUCCH (at least for PUCCH format 3/4). For an example, in case with HP HARQ-ACK and CSI (without LP HARQ-ACK), the HP HARQ-ACK and CSI part 1 can be separately encoded where CSI part 2 is dropped. For another example, in case with HP HARQ-ACK and LP HARQ-ACK and CSI, the LP HARQ-ACK and CSI part 1 can be jointly encoded and the HP HARQ-ACK can be solely encoded where CSI part 2 is dropped.

Proposal #2: Consider the following UE behaviour for the multiplexing of CSI at least on PUCCH format 3/4.

- In case with HP HARQ-ACK and CSI (without LP HARQ-ACK), the HP HARQ-ACK and CSI part 1 can be separately encoded where CSI part 2 is dropped.

- In case with HP HARQ-ACK, LP HARQ-ACK and CSI, the LP HARQ-ACK and CSI part 1 can be jointly encoded and the HP HARQ-ACK can be solely encoded where CSI part 2 is dropped.

Thirdly, regarding FFS point 3, currently the number of encoding chains used for UCI multiplexing on PUCCH is up to 2 (except for PUCCH format 2). Thus, to minimize impacts to the specification as well as UE implementation, it is preferred to keep the (maximum) number of encoding chains for UCI on PUCCH as 2 (at least for PUCCH format 3/4).

Lastly, regarding FFS point 4, since currently at most 1 encoding chain is used for UCI multiplexing on PUCCH format 2, there can be several alternatives to multiplex (or perform encoding for) HP HARQ-ACK and LP HARQ-ACK on the PUCCH format 2 as below.

- Alt 1: apply separate encoding for HP HARQ-ACK and LP HARQ-ACK

- Alt 2: encode/transmit HP HARQ-ACK only by dropping LP HARQ-ACK

- Alt 3: apply joint encoding for HP HARQ-ACK and LP HARQ-ACK

With Alt 1, LP HARQ-ACK can be multiplexed/transmitted with HP HARQ-ACK, but the (maximum) number of encoding chains on PUCCH format 2 would need to be increased to 2. With Alt 2, current number of encoding chains on PUCCH format 2 can be kept, but LP HARQ-ACK would be lost in gNB side even though short PUCCH format 2 is likely to be used for URLLC. With Alt 3, LP HARQ-ACK can be multiplexed/transmitted with HP HARQ-ACK similarly with Alt 1, but it would cause making an exceptional case reverting the above agreement on separate encoding.

Among three alternatives, Alt 1 is preferred in terms of supporting to multiplex/transmit HP and LP HARQ-ACK on a PUCCH as well as respecting the agreement to support separate coding for the two HARQ-ACKs. Since up to 2 encoding chains for UCI multiplexing has already been adopted/specified for other PUCCH format 3/4 (and limiting the number of encoding chains on PUCCH format 2 to 1 was originally from the discussion on CSI reporting on short PUCCH in Rel-15), increasing the number of encoding chains on PUCCH format 2 into 2 might not incur UE complexity.

Regarding the UCI RE mapping on PUCCH with separate coding for HP UCI and LP UCI, firstly the HP UCI is required to occupy the REs on the PUCCH resource first (based on the maximum UCI coding rate configured for HP), then the LP UCI could be mapped to the remaining REs not occupied by the HP UCI (based on the maximum UCI coding rate configured for LP). Similarly, for the encoding of UCIs with different priority on PUSCH, it is also needed to consider how to map the separately-encoded UCI REs on the PUSCH resource, based on the beta offset configured per each of LP and HP.

For the UCI RE mapping on PUCCH format 2, in order to guarantee the reliable UCI performance by achieving frequency diversity, HP (coded) UCI needs to be mapped over distributed REs on the PUCCH resource. To be specific, following two options can be considered for the mapping of HP (coded) UCI REs on PUCCH format 2.

- Option 1

  - Splitting HP UCI bits (as well as LP UCI bits) into two parts

  - Mapping each HP UCI part over distributed REs on each of two PUCCH symbols

- Option 2

  - Concatenating the REs on two PUCCH symbols (virtually) into one (concatenated) symbol

  - Mapping all HP UCI bits over distributed REs on the concatenated symbol

Proposal #3: Consider to support separate encoding for HP HARQ-ACK and LP HARQ-ACK on PUCCH format 2.

- HP (coded) UCI is firstly mapped over distributed REs on the PUCCH resource (to guarantee the reliable HP UCI performance by achieving frequency diversity), then LP UCI is mapped to the remaining REs not occupied by the HP UCI.

As another aspect, for multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH format 0 or 1 when the total HARQ-ACK payload size is 2 bits, the related details are to be provided. Regarding this aspect, one remaining detail would be how to map cyclic shift value on PUCCH format 0 and QPSK constellation point on PUCCH format 1 for 1-bit HP HARQ-ACK and 1-bit LP HARQ-ACK. For this, simply, HP UCI bit and LP UCI bit can be mapped to MSB and LSB, respectively, on 2-bit UCI payload. Another remaining issue is how to determine PUCCH resource (between HP PUCCH and LP PUCCH) for the 2-bit UCI payload. For this, consistently (as for the case of more than 2 bits with HP HARQ-ACK and LP HARQ-ACK), HP PUCCH format 0/1 is used to transmit the 2-bit UCI.

Proposal #4: Consider the following aspects for the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH format 0/1 with the total UCI payload size of 2 bits.

- HP UCI bit and LP UCI bit are mapped to MSB and LSB, respectively.

- HP PUCCH format 0/1 resource is selected for the UCI multiplexing.

- Inter-priority multiplexing condition

Considering inter-priority multiplexing of UCIs and different latency requirement for the UCIs, it may need to consider additional condition to check the processing timeline for the inter-priority multiplexing and the latency requirement especially for the HP UCI.

For example, a processing time (symbol) margin may require to be added to the current timeline (for intra-priority multiplexing in Rel-16) considering additional inter-priority multiplexing. For another example, the timing of the last symbol in the PUCCH resource selected to multiplex the LP UCI and HP UCI, may need to be checked whether it is allowable in terms of latency from the perspective of the HP UCI.

Proposal #5: Consider additional condition for the processing of inter-priority multiplexing and the latency requirement for HP UCI.

- Overall multiplexing procedure

Regarding the inter-priority multiplexing of UCIs on PUCCH/PUSCH, the overall multiplexing procedures or steps (including the intra-priority multiplexing in Rel-16) is to be taken into account.

One way could be that, some important LP UCI type such as LP HARQ-ACK is treated as HP UCI, and the intra-priority multiplexing is applied per each of LP UCI and HP UCI as in Rel-16. Then, the multiplexed LP UCI would be dropped if it collided with the multiplexed HP UCI (as in Rel-16), but the LP HARQ-ACK could be multiplexed/transmitted (with HP UCI) without dropping.

Another approach (i.e., Approach 1) could be that, the intra-priority multiplexing defined in Rel-16 is applied per each of LP UCI and HP UCI as the first step, then after that, as the second step, the inter-priority multiplexing is performed for the outcomes of the first step, specifically between the (intra-priority) multiplexed LP UCI and the (intra-priority) multiplexed HP UCI.

Another approach (i.e., Approach 2) could be that, the (intra-priority and) inter-priority UCI multiplexing is performed only for the PUCCHs (except for the PUSCHs) as the first step, then after that, the piggybacking of the (inter-priority) multiplexed UCI on PUSCH is done as the second step.

Another approach (i.e., Approach 3) could be that, both intra-priority and inter-priority UCI multiplexing is performed for all the PUCCHs/PUSCHs with same or different priority by single step.

Related to this multiplexing procedure aspect, a working assumption was made in RAN1#104-e to reuse Rel-15 PUCCH/PUSCH multiplexing timeline requirements for Rel-17 PUCCH/PUSCH multiplexing with different priorities, and whether to specify different behaviour from Rel-15 for the case when the timeline requirements are not met is FFS.

On the FFS point, if it is concluded not to specify different behaviour from Rel-15, it means that the case when the timeline requirements are not met would be treated as an error case and then the UE behaviour would be remained as undefined in this case. However, it seems to be undesirable since even in case where the UL multiplexing between different priorities is enabled by gNB, the gNB should be able to schedule urgent HP DL/UL traffic without considering the timeline for multiplexing with LP PUCCH/PUSCH, for example, by overriding the LP resources already. In other words, it would not be reasonable that HP PUCCH/PUSCH is treated as an error just due to not satisfying the timeline requirements with LP PUCCH/PUSCH.

With the above observation, it would be desirable to proceed the multiplexing and transmission at least for HP PUCCH/PUSCH (if the timeline requirements among the HP PUCCH/PUSCH are met) even in case when the timeline requirements with LP are not met, for example, as the followings.

- In case with the above Approach 1,

  - If the intra-priority multiplexing timeline requirements are met per each of LP PUCCH/PUSCH and HP PUCCH/PUSCH but the inter-priority multiplexing timeline requirements are not met, then the UE would proceed the multiplexing/transmission only for the HP by dropping the LP.

- In case with the above Approach 2/3,

  - If the inter-priority multiplexing timeline requirements are not met, then the UE would proceed the multiplexing/transmission only for the HP by dropping the LP if the intra-priority multiplexing timeline requirements are met among HP PUCCH/PUSCH.

Proposal #6: Consider the overall procedures for the inter-priority multiplexing of UCIs on PUCCH/PUSCH.

- It is desirable to proceed the multiplexing and transmission at least for HP PUCCH/PUSCH (if the timeline requirements among the HP PUCCH/PUSCH are met) even in case when the timeline requirements with LP are not met.

Among the Approach 1/2/3, Approach 1 is preferred in terms of having the benefits to reuse Rel-16 timeline checking procedure (in the first step), and by doing so, the UE can early judge whether the inter-priority multiplexing is possible for the PUCCHs/PUSCHs with different priority. Furthermore, in case when simultaneous PUCCH+PUSCH transmission is enabled, by performing timeline check and multiplexing procedure only per priority (by omitting those for different priority), dropping of UCI (and loss of PUSCH code rate) and timeline checking complexity could be avoided/reduced with Approach 1.

Proposal #7: Consider the following approach as the procedure for the inter-priority multiplexing of UCIs on PUCCH/PUSCH, in order to avoid/reduce dropping of UCI (and loss of PUSCH code rate) and timeline checking complexity.

- The intra-priority multiplexing defined in Rel-16 is applied per each of LP PUCCH/PUSCH and HP PUCCH/PUSCH as the first step.

- As the second step, the inter-priority multiplexing is performed for the outcomes of the first step, specifically between the (intra-priority) multiplexed LP PUCCH/PUSCH and the (intra-priority) multiplexed HP PUCCH/PUSCH.

As other aspect, for multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH or for multiplexing of HARQ-ACK on PUSCH with different priority, a mechanism supporting to enable/disable the multiplexing need to be decided. Regarding this aspect, considering potential UE complexity due to dynamic change of inter-priority processing behaviour (e.g. multiplexing LP and HP or dropping LP as in Rel-16) and unreliable UCI/PUSCH performance caused by misalignment between UE and gNB in terms of whether the multiplexing is enabled or disabled, RRC configuration is preferred for supporting the reliable mechanism to enable/disable the multiplexing.

In case of dynamic DCI indication mechanism, there seems to be some additional consideration points, for example, how to handle the semi-static/semi-persistent UCI/PUSCH (e.g. P/SP-CSI, SR, SPS HARQ-ACK, CG PUSCH) and the UCI/PUSCH scheduled by fallback DCI format not having the indication to enable/disable the multiplexing (e.g. follow other non-fallback DCI’s indication or apply pre-defined/configured default behaviour), and how to handle the case where UE receives multiple DCIs indicating different behaviour (e.g. some DCIs indicate to enable the multiplexing while other DCIs indicate to disable the multiplexing).

Proposal #8: Consider RRC configuration for the mechanism to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH or the multiplexing of HARQ-ACK on PUSCH with different priority, with consideration of potential UE complexity and UCI/PUSCH reliability.

  - PUCCH-specific aspects

- Handling on UCI coding rate

Considering to multiplex LP UCI and HP UCI on a same PUCCH, it is required to handle the case where actual UCI coding rate based on total payload size of LP UCI and HP UCI (and PUCCH resource size) exceed the maximum UCI coding date configured for the PUCCH. A reasonable way would be to drop LP UCI (entirely or partially) and/or apply bundling for LP HARQ-ACK.

In case of bundling for LP HARQ-ACK, considering potential NACK(DTX)-to-ACK error in case when a DL DCI is missed by UE, it would be desirable to apply bundling in spatial domain and/or CBG domain. In case of partial dropping for LP HARQ-ACK in terms of dropping order, it may have dependency with the HARQ-ACK codebook type. For example, HARQ-ACK corresponding to the last SLIV could be dropped first in case of Type-1 codebook while HARQ-ACK corresponding to the last DAI could be dropped first in case of Type-2 codebook.

Moreover, the bundling and the dropping for LP HARQ-ACK could be sequentially applied. For example, the bundling is applied first in case of exceeding the maximum UCI coding date, after that, the dropping is additionally applied in case of exceeding the maximum UCI coding date even based on the bundled LP HARQ-ACK payload size.

Proposal #9: Consider the following LP HARQ-ACK handling for the case of exceeding the maximum UCI coding rate on PUCCH.

- HARQ-ACK bundling for LP HARQ-ACK in spatial domain and/or CBG domain.

- Partial dropping for LP HARQ-ACK according to HARQ-ACK codebook type.

Regarding the maximum UCI coding rate used for LP UCI multiplexed on HP PUCCH, it was discussed whether an additional maximum coding rate dedicated for this case needs to be configured or the existing maximum coding rate configured for LP UCI on LP PUCCH is reused for this case. On this issue, there seems no essential reason to use different maximum coding rate for same LP UCI according to whether it is transmitted on LP PUCCH or HP PUCCH since HP UCI would occupy the REs on HP PUCCH earlier than LP UCI and the LP UCI would be mapped on the remaining REs without any impact to HP UCI mapping.

Proposal #10: Consider to reuse the existing maximum UCI coding rate (configured for LP UCI on LP PUCCH) for the LP UCI multiplexed on HP PUCCH.

- PUCCH resource determination

On this aspect, it was agreed in RAN1#104-e to use a PUCCH resource in the HP PUCCH configuration for the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH, at least in case when total payload size of the LP/HP HARQ-ACKs is more than 2 bits. And, there are three FFS points: 1) to configure/use additional PUCCH resource set dedicated for LP/HP HARQ-ACK multiplexing, 2) in case when total payload size of the LP/HP HARQ-ACKs is 2 bits, and 3) on details for determination of a PUCCH resource in the HP PUCCH configuration.

Regarding the first FFS point, configuring/using the dedicated PUCCH resource set would be reasonable for both UE and gNB since the difference between total payload size of LP + HP (and required REs to convey it) and the payload size of LP/HP only (and required REs to convey it) could be too large to cover them with a same HP PUCCH resource set. In addition, on the second FFS point, (as already mentioned earlier) it seems there is no critical reason or motivation to differentiate between the case of more than 2 bits and the case of 2 bits in term of PUCCH resource determination. Moreover, regarding the third FFS point, at least following two issues need to be discussed and decided.

- Issue 1: How to select one of multiple HP PUCCH resource sets

  - E.g. based on total size of HP UCI payload and LP UCI payload or based on total size of HP UCI payload and scaled LP UCI payload (e.g., scaling factor x LP UCI payload)

- Issue 2: How to determine a PUCCH resource in the selected HP PUCCH resource set

  - E.g. based on the PRI indicated in the last DCI corresponding to HP HARQ-ACK or based on the PRI indicated in the last DCI

For Issue 1, it is straightforward to select PUCCH resource set based on the total LP+HP UCI payload size as done so far, rather than based on scaled total LP+HP UCI payload size by scaling LP UCI payload since the UE would anyhow use the minimum number of PRBs required for UCI transmission in the selected PUCCH resource even if a PUCCH resource larger than UCI payload size is selected. Moreover, if the scaling factor is used, additional handling would be required in some cases/aspects, for example, how to determine PUCCH resource (set) in case where HP UCI is 1 bit and LP UCI is N bit (N > 1) but the scaled LP UCI payload size would be less than or equal to 1 bit.

Proposal #11: Consider to determine HP PUCCH resource set based on the total UCI payload size of HP UCI and LP UCI as done so far.

For Issue 2, it may seem to be straightforward to determine a HP PUCCH resource based on the PRI indicated in the last DCI corresponding to HP HARQ-ACK as in Rel-16. On the other hand, it may be necessary to address the case when UE didn’t receive any DCI indicating HP (but received DCI indicating LP), for example, by using a PUCCH resource in HP PUCCH resource set based on the PRI indicated in the last DCI only for this case. Note that, dropping the LP HARQ-ACK just due to the reason that the HP HARQ-ACK corresponds to SPS PDSCH, is not desirable from technical perspective considering potential DL resource overhead required for PDSCH retransmission and/or potential gNB scheduling restriction to avoid the collision with HP SPS HARQ-ACK corresponding to short SPS PDSCH periodicity.

Proposal #12: Consider the following to determine a PUCCH resource in the HP PUCCH resource set selected based on total UCI payload size.

- In case when at least one HP DL DCI is received by the UE, the PUCCH resource corresponding to the PRI indicated in the last HP DCI is selected.

- In case when LP DL DCI is only received by the UE, the PUCCH resource corresponding to the PRI indicated in the last LP DCI is selected.

- Multiplexing of HARQ-ACK and/or SR

Regarding the multiplexing between LP HARQ-ACK and HP HARQ-ACK on PUCCH (or PUSCH), it is necessary to consider the HARQ-ACK codebook type (e.g. Type-1/2/3 codebook) configured/indicated for the LP/HP HARQ-ACKs. Considering CA situation, there would be two types of serving cell as below, according to the configuration of priority indicator in DL DCI formats for each cell.

- Cell type 1

  - Priority indicator is not configured in DL DCI formats for the cell.

  - PDSCH in the cell is scheduled only with LP, and thus HARQ-ACK for the PDSCH reception corresponds to LP only.

  - K1 and TDRA are configured only for LP, and all the HARQ process IDs are scheduled only with LP.

- Cell type 2

  - Priority indicator is configured in DL DCI formats for the cell.

  - PDSCH in the cell can be scheduled with either LP or HP, and thus HARQ-ACK for the PDSCH reception can correspond to either LP or HP.

  - K1 and TDRA are configured per each of LP-dedicated DCI format and HP-schedulable DCI format, and all or part of HARQ process IDs can be scheduled with HP.

In case of Type-1 or Type-3 codebook, for the multiplexing of LP/HP HARQ-ACKs, it may need to consider/decide how to generate HARQ-ACK payload per each of LP and HP, especially for the HARQ-ACKs corresponding to the above cell type 2 configured with priority indicator in DL DCI (e.g. mapping the HARQ-ACK for cell type 2 into both LP payload and HP payload, or mapping it only into HP payload by omitting it from LP payload).

Especially for Type-1 codebook, it may also need to consider for entire HARQ-ACK codebook construction on whether/which DCI is scheduled/received for a given priority. For example, in case when no DCI or only single fallback DCI is scheduled/received for a given priority, it is to be decided whether/how to generate HARQ-ACK payload for the priority.

In case of Type-2 codebook, since DAI is signalled/counted per each of LP/HP HARQ-ACKs, consequently HARQ-ACK payload for LP and HP can be separately generated as a sub-codebook based on the received DAIs per priority, and thus there seems to be no issue.

Proposal #13: Consider how to generate the HARQ-ACK payload per each of LP and HP for the multiplexing of LP/HP HARQ-ACK on PUCCH (or PUSCH), according to HARQ-ACK codebook type (e.g. Type-1/2/3 codebook).

Moreover, it was discussed how to handle potential ambiguity on the presence of LP HARQ-ACK feedback or the size of LP HARQ-ACK codebook due to DCI missing in case of multiplexing HP HARQ-ACK and the LP HARQ-ACK on a same PUCCH/PUSCH. On this issue, it is reasonable to consider an additional field in the DL/UL DCIs corresponding to HP HARQ-ACK/PUSCH for determining the number of LP HARQ-ACK bits multiplexed (with HP HARQ-ACK bits) on PUCCH/PUSCH.

To be specific, in case of Type-2 codebook based LP HARQ-ACK, multiple candidate payload sizes such as {X-bit, Y-bit, Z-bit, W-bit} can preconfigured by RRC (where X < Y < Z < W), and one of the candidate sizes can be indicated by DCI. Then, the UE generates LP HARQ-ACK payload based on the indicated size potentially with padding of NACK bits or dropping of some HARQ-ACK according to the difference between the indicated size and actual payload from UE perspective.

In addition, in case of Type-1 codebook based LP HARQ-ACK, three candidate payload sizes such as {full codebook (corresponding to all the TDRA entries and all the K1 values), fallback HARQ-ACK (corresponding to the DCI scheduling Pcell with DAI = 1), no HARQ-ACK} can be considered (without RRC configuration), and one of the candidate sizes can be indicated by DCI. Then, the UE generates LP HARQ-ACK payload just based on the indicated size.

Proposal #14: Consider to introduce an additional field in the DL/UL HP DCIs for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH for both Type-1 and Type-2 codebooks, in order to handle potential ambiguity on the presence of LP HARQ-ACK feedback or the size of LP HARQ-ACK codebook.

Regarding the multiplexing between HP SR and LP HARQ-ACK of up to 2 bits on PUCCH (format 0 or format 1, e.g., PF0 or PF1), following three cases may need to be enhanced to guarantee HP SR reliability related to PUSCH (scheduling) latency.

- Case 1: SR PF0 + HARQ-ACK PF0

  - Currently, an offset of 3 or 1 is added to the sequence CS values on HARQ-ACK PF0 for HARQ-ACK with 1-bit or 2-bit, respectively, in case when SR is positive.

- Case 2: SR PF0 + HARQ-ACK PF1

  - Currently, HARQ-ACK is only transmitted on PF1 by dropping SR.

- Case 3: SR PF1 + HARQ-ACK PF0

  - Currently, the same behavior is applied as in the above Case 1.

For Case 1 and Case 3, in case of 2-bit HARQ-ACK, the CS gap between negative SR and positive SR becomes too small (since the CS gap is 1), and thus it would cause degradation of HP SR reliability under certain channel condition. For Case 2, obviously due to the dropping of SR, it would be undesirable in terms of ensuring HP SR reliability.

One possible way to address the above issues is that, firstly LP HARQ-ACK of up to 2 bits is reduced or compressed to 1-bit, and then the (reduced/compressed) 1-bit LP HARQ-ACK and 1-bit HP SR are multiplexed on the HARQ-ACK PUCCH.

On this HP SR+LP HARQ-ACK multiplexing on PF0/1, it was agreed in RAN1#104-e to study several options for the above Case 1/2/3. Among the options, firstly, considering to ensure HP SR reliability as well as to keep PUCCH resource overhead, the following Opt 2b (as proposed in the above) is preferred. And secondly, if LP HARQ-ACK performance also needs to be guaranteed on top of HP SR reliability, the following Opt 2a is also preferable.

- Opt 2b

  - Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit (for Case 1/3).

  - Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit (for Case 2).

- Opt 2a

  - If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource (for Case 1/2/3).

Moreover, specifically to Case 3, the following Opt 3 is also preferable since all of HP SR reliability, PUCCH resource overhead, and LP HARQ-ACK performance could be addressed where the behaviour for positive SR is the same as for the case of SR PF1+HARQ-ACK PF1. Furthermore, specifically to Case 2, the following Opt 4 can also be considered just for the similar behaviour with Rel-15/16 (but less preferable in the context of supporting LP+HP multiplexing).

- Opt 3

  - For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource (for Case 3).

- Opt 4

  - For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource (for Case 2).

Proposal #15: Consider to support Opt 2b for the combinations of {SR PF0 + HARQ-ACK PF0} and {SR PF0 + HARQ-ACK PF1} and {SR PF1 + HARQ-ACK PF0}, to ensure HP SR reliability as well as to keep PUCCH resource overhead.

Proposal #16: Consider to support Opt 2a for the combinations of {SR PF0 + HARQ-ACK PF0} and {SR PF0 + HARQ-ACK PF1} and {SR PF1 + HARQ-ACK PF0}, to guarantee LP HARQ-ACK performance on top of HP SR reliability.

  - PUSCH-specific aspects

- UCI RE mapping (order) on PUSCH

In previous meeting, it was discussed how to multiplex HP UCI and LP UCI on LP PUSCH or HP PUSCH in terms of mapping UCI REs or dropping some UCI, and it needs to consider whether the CSI to be multiplexed on PUSCH consists of two parts or single part, and whether the PUSCH for multiplexing of the UCIs is conveying UL-SCH or not. For example, the above discussion/consideration at least addressed the case when HP HARQ-ACK and LP HARQ-ACK and LP CSI are to be multiplexed on LP PUSCH, and the case when HP HARQ-ACK and LP HARQ-ACK and HP CSI are to be multiplexed on HP PUSCH. Thus, including the above cases, overall UCI RE mapping rule (order) on PUSCH with different priority can be summarized as below, according to various combinations of UCI and PUSCH (here, {A, B, C} implies the mapping order that, B is mapped after mapping of A and C is mapped after mapping of B).

- Case H1: {HP HARQ-ACK, LP HARQ-ACK, HP CSI} on HP PUSCH

  - Case H1-1: HP CSI consists of two parts

    - {HP HARQ-ACK, LP HARQ-ACK, HP CSI part 1} are multiplexed on HP PUSCH, by dropping HP CSI part 2

      - If HP PUSCH is not conveying UL-SCH where encoding of the UL-SCH is skipped, then {HP HARQ-ACK, LP HARQ-ACK, HP CSI part 1, HP CSI part 2} can be all multiplexed on the HP PUSCH without UCI dropping

  - Case H1-2: HP CSI consists of single part

    - {HP HARQ-ACK, LP HARQ-ACK, HP single-part CSI} are all multiplexed on HP PUSCH, without UCI dropping

- Case H2: {LP HARQ-ACK, HP CSI} on HP PUSCH

  - Case H2-1: HP CSI consists of two parts

    - {LP HARQ-ACK, HP CSI part 1, HP CSI part 2} are all multiplexed on HP PUSCH

  - Case H2-2: HP CSI consists of single part

    - {LP HARQ-ACK, HP single-part CSI} are all multiplexed on HP PUSCH

- Case L1: {HP HARQ-ACK, LP HARQ-ACK, LP CSI} on LP PUSCH

  - Case L1-1: LP CSI consists of two parts

    - {HP HARQ-ACK, LP HARQ-ACK, LP CSI part 1} are multiplexed on LP PUSCH, by dropping LP CSI part 2

      - If LP PUSCH is not conveying UL-SCH where encoding of the UL-SCH is skipped, then {HP HARQ-ACK, LP HARQ-ACK, LP CSI part 1, LP CSI part 2} can be all multiplexed on the LP PUSCH without UCI dropping

  - Case L1-2: LP CSI consists of single part

    - {HP HARQ-ACK, LP HARQ-ACK, LP single-part CSI} are all multiplexed on LP PUSCH, without UCI dropping

- Case L2: {HP HARQ-ACK, LP CSI} on LP PUSCH

  - Case L2-1: LP CSI consists of two parts

    - {HP HARQ-ACK, LP CSI part 1, LP CSI part 2} are all multiplexed on LP PUSCH

  - Case L2-2: LP CSI consists of single part

    - {HP HARQ-ACK, LP single-part CSI} are all multiplexed on LP PUSCH

- Case L3: {LP HARQ-ACK, LP CSI} on LP PUSCH

  - Case L3-1: LP CSI consists of two parts

    - {LP HARQ-ACK, LP CSI part 1, LP CSI part 2} are all multiplexed on LP PUSCH

  - Case L3-2: LP CSI consists of single part

    - {LP HARQ-ACK, LP single-part CSI} are all multiplexed on LP PUSCH

Proposal #17: Consider at least the following aspects for determining UCI RE mapping rule (order) on PUSCH, according to various combinations of UCI and PUSCH.

- Whether the CSI to be multiplexed on PUSCH consists of two parts or single part.

  - UCI RE mapping rule (order) could be different according to the above.

- Whether the PUSCH for multiplexing of the UCIs is conveying UL-SCH or not.

  - UCI RE mapping rule (order) could be different according to the above.

On top of the above, it needs to consider/discuss how to generate the reserved REs corresponding to 2-bit HARQ-ACK on PUSCH in case when this inter-priority UL multiplexing is enabled. Considering potential missing of the DCI corresponding to HP HARQ-ACK by the UE, the reserved REs are to be generated based on the beta offset configured for HP HARQ-ACK and to be mapped even on LP PUSCH as well as HP PUSCH, even in case when there is no HP HARQ-ACK from UE perspective. For example, for the above Case H2 and Case L3, the reserved REs are generated by the beta offset for HP HARQ-ACK, and mapped on HP PUSCH (in Case H2) or LP PUSCH (in Case L3).

Proposal #18: Consider the following aspect by taking potential missing of the DCI corresponding to HP HARQ-ACK by the UE into account.

- The reserved REs corresponding to 2-bit HARQ-ACK on PUSCH are to be generated based on the beta offset configured for HP HARQ-ACK and to be mapped even on LP PUSCH as well as HP PUSCH, even in case when there is no HP HARQ-ACK from UE perspective.

- Determination of REs for UCI mapping

It was agreed in RAN1#102-e to support separate configuration of beta offset values for multiplexing of different priorities, and whether to support separate configuration even for alpha factor is remained as FFS point. For almost same reason with the beta offset for the purpose of guaranteeing, for example, the reliability of HP UCI (multiplexed on LP PUSCH), separate configuration of alpha factor values for multiplexing of different priorities may need to be supported to guarantee, for example, the reliability of HP PUSCH (with piggybacking of LP UCI).

For example, for each priority (e.g. LP, HP) of PUSCH, beta offset can be configured per each of LP UCI and HP UCI. For another example, for each priority of PUSCH, alpha factor can be configured separately for the case only with LP UCI and for the case with HP UCI, or for LP UCI only case and for HP UCI only case and for the case with both LP UCI and HP UCI. Moreover, for flexible/adaptive control of the (maximum) amount of UCI REs on PUSCH from gNB perspective, it can be considered to indicate alpha factor by DCI similarly with dynamic beta offset.

In RAN1#104-e, it was agreed to support small beta offset value in the range from 0 to 1 for multiplexing of LP HARQ-ACK on HP PUSCH. However, configuring alpha factor for the case of LP HARQ-ACK on HP PUSCH separately from the case of HP HARQ-ACK on HP PUSCH, would be more effective since effect of the alpha factor is more dominant in terms of ensuring HP PUSCH reliability/protection rather than effect of using the small beta offset.

Proposal #19: Consider separate configuration of beta offset as well as alpha factor per each of UCI priority or per UCI priority combination (e.g. for LP and HP, or for LP only case and other cases) for each priority (e.g. LP, HP) of PUSCH, to ensure reliability/protection of HP PUSCH.

In addition, considering the case where LP UCI would be piggybacked on HP PUSCH together with HP UCI, the REs assigned to the LP UCI may not be sufficient to guarantee the performance. For this reason, it may need to consider the maximum UCI coding rate at least for LP UCI on PUSCH as for the UCI on PUCCH. Given that, dropping and/or bundling of the LP UCI on PUSCH could be applied (similarly with LP UCI on PUCCH) in case of exceeding the maximum UCI coding rate.

Proposal #20: Consider the bundling/dropping of LP UCI on PUSCH based on the maximum UCI coding rate as for the case of LP UCI on PUCCH.

Furthermore, regarding UCI multiplexing on NR-U CG PUSCH, currently, one of following two modes can be configured for the UE by the gNB.

- Mode 1

  - When CG PUSCH overlaps with HARQ-ACK PUCCH, CG-UCI and the HARQ-ACK are jointly encoded and mapped on the CG PUSCH.

- Mode 2

  - When CG PUSCH overlaps with HARQ-ACK PUCCH, the CG PUSCH is not transmitted (based on skipping) by the UE.

Considering UCI multiplexing on the above NR-U CG PUSCH with different priority, it may be necessary to consider two aspects: 1) how to determine the priority of CG-UCI (e.g. follow the priority of PUSCH, or follow the priority of HARQ-ACK jointly encoded with the CG-UCI), 2) how to encode the CG-UCI payload (e.g. only allow joint encoding with HARQ-ACK of same priority, or also allow joint encoding with HARQ-ACK of different priority).

Proposal #21: Consider how to determine the priority of CG-UCI and how to encode the CG-UCI payload in case of UCI multiplexing on NR-U CG PUSCH with different priority.

- Simultaneous TX of PUCCH+PUSCH

It was agreed in RAN1#102-e to support simultaneous PUCCH/PUSCH transmissions on different cells. In fact, since this feature had been introduced/specified already in Rel-10 LTE-A as a form of UE capability based on intensive discussions, the overall framework designed in LTE-A in terms of capability, operation procedure and transmission structure (as the following examples) should be the baseline (and starting point) for Rel-17 NR.

- In case with HARQ-ACK and PUSCH in a same slot,

  - HARQ-ACK is on PUCCH, no UCI is on PUSCH.

- In case with (periodic) CSI and PUSCH in a same slot,

  - CSI is on PUCCH, no UCI is on PUSCH.

- In case with HARQ-ACK and CSI and PUSCH in a same slot,

  - HARQ-ACK is on PUCCH, CSI is on PUSCH.

On this simultaneous PUCCH+PUSCH transmission aspect, it was agreed in RAN1#104-e to support RRC configuration per UE for simultaneous PUCCH+PUSCH transmission, but there was an argument on whether this simultaneous PUCCH+PUSCH transmission is applied for any (both same and different) priorities or only for different priorities. Considering UE complexity to apply different behaviour according to priority combination and gNB/UE ambiguity on same or different priorities in case when the UE misses some DCI, the simultaneous PUCCH+PUSCH transmission should be applied for any priority combinations (enabled by single RRC parameter).

Proposal #22: Consider the framework designed in Rel-10 LTE-A as the baseline for supporting simultaneous transmission of PUCCH and PUSCH in Rel-17 NR.

- The simultaneous PUCCH+PUSCH transmission should be applied for any priority combinations (enabled by single RRC parameter) by taking potential UE complexity and gNB/UE ambiguity into account.

- Prioritization of DG/CG PUSCHs with different priority

In the last meeting, it has been clarified that it is already supported Rel-15 prioritization between DG and CG with different priority in Rel-16 maintenance AI. Thus we assume all prioritization in this AI is based on Rel-16 timeline. In Rel-16, the cancelation timeline was made with following aspects.

- Required time gap between high priority (HP) channel indication and starting symbol of the HP channel if the HP channel overlaps with low priority (LP) channel

- Additional offset to the required time gap

- Additional offset to PUSCH preparation time

- Clarification on Rel-16 UE behavior on CG vs. DG PUSCH collision

From Rel-16 URLLC maintenance session and RAN2 discussion, it is almost specified how UE prioritizes one PUSCH among overlapping PUSCHs. In the perspective of PHY layer, it is necessary to verify a PDCCH scheduling timeline if PUSCH scheduled by the PDCCH is overlapping CG PUSCH. If it satisfied timeline and scheduled successfully, UE MAC prioritizes one UL grant, generates MAC PDU for the prioritized UL grant and deprioritizes others.

In this framework, collision between HP CG and LP DG may not be a big issue. Since LP DG may not have critical scheduling timeline and HP CG is semi-static, UE would know both UL grant when UE is scheduled with low priority PUSCH. In this case, UE MAC would prioritizes HP CG easily when it has traffic to transmit, and UE PHY would transmit PUSCH with transport block. Maybe some UL multiplexing rule needs to be refined in that case.

However, collision between LP CG and HP DG could be complicated. In this scenario, HP DG may be scheduled even during PUSCH preparation time of LP CG. It means that, it is required to decode PDCCH and stop LP CG processing after the PDCCH decoding. If skipUplinkTxDynamic is configured, the case become more complicated. In the perspective of UE MAC, this kind of UE behavior cannot be aligned with current UE MAC behavior generating one MAC PDU per a group of overlapping UL grants.

When PUCCH for UCI overlaps LP CG and HP DG, the problem become more complicated. According to RAN1 specification, UCI prioritizes dynamic scheduled UL grant to be piggybacked. However, if MAC doesn’t know the existence of HP DG, UCI may be multiplexed to LP CG so that LP CG cannot be deprioritized unless UE can revert the decision. In principle, it is not allowed to skip PUSCH overlaps with PUCCH especially once MAC generates MAC PDU for the PUSCH.

Therefore, to support the case where low-priority CG-PUSCH collides with high-priority DG-PUSCH, it is necessary for UE to know the existence of high priority DG PUSCH in advance of starting any processing of CG-PUSCH. Therefore, we propose the followings.

Proposal #23: Consider to apply Rel-15 timeline requirements between dynamic grant and configured grant, for PHY prioritization for the case where low-priority CG-PUSCH collides with high-priority DG-PUSCH.

- Separated timeline offset for PUSCH collision

From the discussion in Rel-16, there was a concern to adopt same Rel-16 timeline to PUSCH collision handling. PUSCH processing is actually done by both MAC and PHY since MAC determines which data is transmitted. Meanwhile, PUCCH processing can be done by only PHY layer. If UE is required to cancel a PUSCH on a resource and to transmit another PUSCH on the same resource, it wouldn’t be equivalent to collision with PUCCH at least in the perspective of pipeline exchange between two UL channels. If PHY prioritization between LP DG and HP CG, it can be considered to introduce sufficient timeline and offset for PUSCH collision case to alleviate that difference.

Proposal #24: Consider to introduce new timeline or offset in case of PUSCH collision handling with different priority.

- Potential issue with inter-priority UL multiplexing

As an objective for Rel-17 URLLC WI, it would be discussed how to support intra-UE multiplexing and the prioritization of traffic with different priority as another objective. The main goal of the objective could be to save low priority PUCCH from inter-priority cancelation. Meanwhile, if PUSCH is able to be cancelled, UCI multiplexed on the PUSCH would also be dropped. To make outcomes of both objectives work together, we should consider intra-UE multiplexing case even for the case of PUSCH collision handling. For examples, we should consider sufficient cancelation timeline for low priority PUSCH having UCI or how to preserve/recover the (dropped) UCI on low priority.

Proposal #25: Consider enhanced collision handling between HP PUSCH and LP PUSCH with UCI piggybacking.

- Conclusions

In this contribution, intra-UE multiplexing and prioritization for different priority were discussed, and the followings are proposed.

Proposal #1: Consider to apply RM coding with bit-padding for HP/LP HARQ-ACK of up to 2 bits (in case when the total number of LP and HP HARQ-ACK bits is more than 2), in order to minimize impacts to the specification as well as UE implementation.

Proposal #2: Consider the following UE behaviour for the multiplexing of CSI at least on PUCCH format 3/4.

- In case with HP HARQ-ACK and CSI (without LP HARQ-ACK), the HP HARQ-ACK and CSI part 1 can be separately encoded where CSI part 2 is dropped.

- In case with HP HARQ-ACK, LP HARQ-ACK and CSI, the LP HARQ-ACK and CSI part 1 can be jointly encoded and the HP HARQ-ACK can be solely encoded where CSI part 2 is dropped.

Proposal #3: Consider to support separate encoding for HP HARQ-ACK and LP HARQ-ACK on PUCCH format 2.

- HP (coded) UCI is firstly mapped over distributed REs on the PUCCH resource (to guarantee the reliable HP UCI performance by achieving frequency diversity), then LP UCI is mapped to the remaining REs not occupied by the HP UCI.

Proposal #4: Consider the following aspects for the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH format 0/1 with the total UCI payload size of 2 bits.

- HP UCI bit and LP UCI bit are mapped to MSB and LSB, respectively.

- HP PUCCH format 0/1 resource is selected for the UCI multiplexing.

Proposal #5: Consider additional condition for the processing of inter-priority multiplexing and the latency requirement for HP UCI.

Proposal #6: Consider the overall procedures for the inter-priority multiplexing of UCIs on PUCCH/PUSCH.

- It is desirable to proceed the multiplexing and transmission at least for HP PUCCH/PUSCH (if the timeline requirements among the HP PUCCH/PUSCH are met) even in case when the timeline requirements with LP are not met.

Proposal #7: Consider the following approach as the procedure for the inter-priority multiplexing of UCIs on PUCCH/PUSCH, in order to avoid/reduce dropping of UCI (and loss of PUSCH code rate) and timeline checking complexity.

- The intra-priority multiplexing defined in Rel-16 is applied per each of LP PUCCH/PUSCH and HP PUCCH/PUSCH as the first step.

- As the second step, the inter-priority multiplexing is performed for the outcomes of the first step, specifically between the (intra-priority) multiplexed LP PUCCH/PUSCH and the (intra-priority) multiplexed HP PUCCH/PUSCH.

Proposal #8: Consider RRC configuration for the mechanism to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH or the multiplexing of HARQ-ACK on PUSCH with different priority, with consideration of potential UE complexity and UCI/PUSCH reliability.

Proposal #9: Consider the following LP HARQ-ACK handling for the case of exceeding the maximum UCI coding rate on PUCCH.

- HARQ-ACK bundling for LP HARQ-ACK in spatial domain and/or CBG domain.

- Partial dropping for LP HARQ-ACK according to HARQ-ACK codebook type.

Proposal #10: Consider to reuse the existing maximum UCI coding rate (configured for LP UCI on LP PUCCH) for the LP UCI multiplexed on HP PUCCH.

Proposal #11: Consider to determine HP PUCCH resource set based on the total UCI payload size of HP UCI and LP UCI as done so far.

Proposal #12: Consider the following to determine a PUCCH resource in the HP PUCCH resource set selected based on total UCI payload size.

- In case when at least one HP DL DCI is received by the UE, the PUCCH resource corresponding to the PRI indicated in the last HP DCI is selected.

- In case when LP DL DCI is only received by the UE, the PUCCH resource corresponding to the PRI indicated in the last LP DCI is selected.

Proposal #13: Consider how to generate the HARQ-ACK payload per each of LP and HP for the multiplexing of LP/HP HARQ-ACK on PUCCH (or PUSCH), according to HARQ-ACK codebook type (e.g. Type-1/2/3 codebook).

Proposal #14: Consider to introduce an additional field in the DL/UL HP DCIs for determining the number of LP HARQ-ACK bits multiplexed on PUCCH/PUSCH for both Type-1 and Type-2 codebooks, in order to handle potential ambiguity on the presence of LP HARQ-ACK feedback or the size of LP HARQ-ACK codebook.

Proposal #15: Consider to support Opt 2b for the combinations of {SR PF0 + HARQ-ACK PF0} and {SR PF0 + HARQ-ACK PF1} and {SR PF1 + HARQ-ACK PF0}, to ensure HP SR reliability as well as to keep PUCCH resource overhead.

Proposal #16: Consider to support Opt 2a for the combinations of {SR PF0 + HARQ-ACK PF0} and {SR PF0 + HARQ-ACK PF1} and {SR PF1 + HARQ-ACK PF0}, to guarantee LP HARQ-ACK performance on top of HP SR reliability.

Proposal #17: Consider at least the following aspects for determining UCI RE mapping rule (order) on PUSCH, according to various combinations of UCI and PUSCH.

- Whether the CSI to be multiplexed on PUSCH consists of two parts or single part.

  - UCI RE mapping rule (order) could be different according to the above.

- Whether the PUSCH for multiplexing of the UCIs is conveying UL-SCH or not.

  - UCI RE mapping rule (order) could be different according to the above.

Proposal #18: Consider the following aspect by taking potential missing of the DCI corresponding to HP HARQ-ACK by the UE into account.

- The reserved REs corresponding to 2-bit HARQ-ACK on PUSCH are to be generated based on the beta offset configured for HP HARQ-ACK and to be mapped even on LP PUSCH as well as HP PUSCH, even in case when there is no HP HARQ-ACK from UE perspective.

Proposal #19: Consider separate configuration of beta offset as well as alpha factor per each of UCI priority or per UCI priority combination (e.g. for LP and HP, or for LP only case and other cases) for each priority (e.g. LP, HP) of PUSCH, to ensure reliability/protection of HP PUSCH.

Proposal #20: Consider the bundling/dropping of LP UCI on PUSCH based on the maximum UCI coding rate as for the case of LP UCI on PUCCH.

Proposal #21: Consider how to determine the priority of CG-UCI and how to encode the CG-UCI payload in case of UCI multiplexing on NR-U CG PUSCH with different priority.

Proposal #22: Consider the framework designed in Rel-10 LTE-A as the baseline for supporting simultaneous transmission of PUCCH and PUSCH in Rel-17 NR.

- The simultaneous PUCCH+PUSCH transmission should be applied for any priority combinations (enabled by single RRC parameter) by taking potential UE complexity and gNB/UE ambiguity into account.

Proposal #23: Consider to apply Rel-15 timeline requirements between dynamic grant and configured grant, for PHY prioritization for the case where low-priority CG-PUSCH collides with high-priority DG-PUSCH.

Proposal #24: Consider to introduce new timeline or offset in case of PUSCH collision handling with different priority.

Proposal #25: Consider enhanced collision handling between HP PUSCH and LP PUSCH with UCI piggybacking.

- References

RAN1 chairman’s notes, RAN1#105-e