3GPP TSG RAN WG1 Meeting #106-e			R1-2107449

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.6.1.2

Source: 	LG Electronics

Title: 	Aspects related to the reduced number of Rx branches of RedCap

Document for:	Discussion and decision

- Introduction

As of RAN#90-e meeting, the WI titled “Support of reduced capability NR devices” was approved [1]. The WI objectives are copied below from latest version of the WID [2] for convenience. Related to the reduced number of Rx branches of RedCap, it is noted that as of RAN#91-e meeting, for frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. It was also agreed that a means shall be specified by which the gNB can know the number of Rx branches of the UE.

## 4 Objective ### 4.1 Objective of Core part WI This WI has the following objectives: - Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - … - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - … - Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1] - The existing UE capability framework is used; changes to capability signalling are specified only if necessary. - Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] - Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1] - …

- Discussion

In this contribution, we present our views on the aspects related to the reduced number of Rx branches of RedCap.

  - Early indication in Msg1 and/or Msg3, and MsgA if supported

It was agreed in RAN#91-e meeting to specify the functionality that enables RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and MsgA if supported, including the ability for the early indication to be configurable by the network. The motivations of the early indication of RedCap UEs to the network is to enable the network to apply the coverage recovery techniques and congestion control during the initial access. It can also be used to indicate RedCap UEs to the network via separate initial UL BWP, e.g., when the initial UL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, etc.

The earlier the network identifies the RedCap UEs the better the network can control the RedCap UEs at an earlier stage. The earliest possible indication would be in Msg1. To support early indication of RedCap UEs in Msg1, partitioning of PRACH resources can be considered. However, the main concern on the early RedCap UE indication via PRACH partitioning in Msg1 arises from the limited PRACH resources for partitioning. In NR, the partitioning of PRACH resources have been used for indication of CBRA, CFRA and others (e.g., SI request). Now in Rel-17 RedCap WI, partitioning of PRACH resources to differentiate the RedCap UEs from the non-RedCap UEs are being discussed. In parallel, partitioning of PRACH resources to differentiate the non-RedCap UEs with CE from the non-RedCap UEs without CE is on-going in Rel-17 CE WI.

Given the situation, minimizing the partitioning of PRACH resources is desirable as partitioning of PRACH resources for many different purposes might cause the lack of PRACH resources for a specific purpose. One way to achieve this is to configure separate ROs for RedCap UEs in the initial UL BWP which may be the same as or different from the initial UL BWP for non-RedCap UEs. If it is the same initial UL BWP, then the ROs for RedCap UEs should be separated in time and/or frequency domain within the same initial UL BWP. If it is a different initial UL BWP, then as the PRACH resources for RedCap UEs are already distinct in time/frequency domain from those for non-RedCap UEs, existing PRACH partitioning mechanism specified for non-RedCap UEs can be reused for RedCap UEs. If the partitioning of PRACH resources to differentiate the non-RedCap UEs with CE from the non-RedCap UEs without CE is supported in Rel-17, then the same partitioning mechanism of PRACH resources can be reused to differentiate the RedCap UEs with CE from the RedCap UEs without CE within the separate ROs for RedCap UEs. Or it can be reused to differentiate the RedCap UEs requiring (or requesting) CE (e.g., 1-Rx RedCap UEs or 1-Rx RedCap UEs in NR 4-Rx bands) from the RedCap UEs not requiring (or requesting) the CE (e.g., 2-Rx RedCap UEs).

Proposal 1: Support early indication of RedCap UEs in Msg1 to enable the RedCap UEs to be explicitly identifiable to networks.

Proposal 2: To minimize the partitioning of PRACH resources for early indication of RedCap UEs to networks, support configuring separate ROs for RedCap UEs.

- The separate ROs for RedCap UEs are separated in time and/or frequency domain from the ROs for non-RedCap UEs.

- PRACH partitioning mechanism for non-RedCap UEs, e.g., for differentiating UEs with CE from the UEs without CE if supported, can be reused for RedCap UEs within the separate ROs for RedCap UEs.

  - On the PDCCH blocking rate

Related to DCI formats, it was agreed in the last meeting that DCI format 0_1/1_1 are mandatory as in legacy and DCI format 0_2/1_2 are optionally supported. Modification on existing DCI formats was discussed through an e-mail thread, but no agreement was made. From our perspective, we don’t see there is a critical issue on the PDCCH blocking rate caused by the reduced minimum number of Rx branches, thus there is no need to modify fields of existing DCI formats. Besides, as already discussed through an e-mail thread in the last meeting, most of the existing fields of both DCI format x_1 and x_2 that are not needed for RedCap UEs can already be set to zero. Additional optimization on existing fields does not make much difference in terms of PDCCH blocking rate. Therefore we do not prefer any modifications on the existing DCI format.

If PDCCH blocking rate issue should be addressed, we are open to further discussion to assess the potential impact. As for solutions to mitigate the potential PDCCH blocking issue, we think configuring a separate initial DL BWP and/or CORESET#0 being discussed mainly for offloading purposes can also be considered as a viable solution for this. With the separate initial DL BWP and/or CORESET#0 and with the existing DCI formats without modification, the PDCCH blocking rate can be controlled sufficiently by gNB.

Proposal 3: Modification on fields of existing DCI formats to reduce PDCCH blocking is not considered in Rel-17 RedCap WI.

  - DL coverage recovery

Another aspect related to the reduced number of Rx branches of RedCap is whether to consider the DL coverage recovery or not. We acknowledge that based on our observations during the RedCap SI phase, the performance difference b/w RedCap UEs with 1 Rx branch and non-RedCap UEs is not small (e.g., [2-3 dB] Msg4, [6 dB] Msg2) in some cases (4GHz, and DL PSD 24 dBm/MHz). However, we have already concluded to not recommend the DL coverage recovery for RedCap WI. As there is nothing new since then, there is no reason to reconsider introducing any new mechanism for DL coverage recovery in Rel-17 RedCap WI. It should also be noted that there are existing mechanisms such as TBS scaling, PDSCH repetition, etc. that can be used to alleviate the DL coverage loss.

Proposal 4: New mechanisms for DL coverage recovery are not further discussed in Rel-17 RedCap WI.

- Discussion

In this contribution, we presented our views on the aspects related to the reduced number of Rx branches of RedCap.

Proposal 1: Support early indication of RedCap UEs in Msg1 to enable the RedCap UEs to be explicitly identifiable to networks.

Proposal 2: To minimize the partitioning of PRACH resources for early indication of RedCap UEs to networks, support configuring separate ROs for RedCap UEs.

- The separate ROs for RedCap UEs are separated in time and/or frequency domain from the ROs for non-RedCap UEs.

- PRACH partitioning mechanism for non-RedCap UEs, e.g., for differentiating UEs with CE from the UEs without CE if supported, can be reused for RedCap UEs within the separate ROs for RedCap UEs.

Proposal 3: Modification on fields of existing DCI formats to reduce PDCCH blocking is not considered in Rel-17 RedCap WI.

Proposal 4: New mechanisms for DL coverage recovery are not further discussed in Rel-17 RedCap WI.

- References

RP-202933	New WID on support of reduced capability NR devices, Ericsson

RP-210918	Revised WID on support of reduced capability NR devices, Ericsson