3GPP TSG RAN WG1 #106-e			                 R1-2107474

e-Meeting, August 16th – 27th, 2021

Source: 	ETRI

Title: 	Intra-UE Multiplexing/Prioritization

Agenda Item:	8.3.3 Intra-UE Multiplexing/Prioritization

Document for:	Discussion

- Introduction

In the previous meeting, some important agreements are made, which set guidelines for further discussions. In this contribution, we address our views about details. Many discussion topics are similar to both PUCCH and PUSCH, and we mainly describe our views about PUCCH in the first section, and PUSCH in the next section.

- Discussion

  - Multiplexing on PUCCH

Explicit indication for enabling multiplexing:

Proposal for 2nd round discussion: from the feature lead’s proposal in R1-2106063 For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, at least support RRC configuration for gNB to enable/disable the multiplexing. - FFS whether or not to additionally introduce DCI indication to enable/disable the multiplexing. - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

The scheduling DCI can schedule either HP TB(s) or LP TB(s). There are two alternatives: one alternative is to use PUCCH for the LP TB and the other alternative is to use PUCCH for the HP TB. Regardless of either alternative, the scheduling DCI should indicate to multiplex or prioritize. Otherwise, PUCCH resource should indicate more REs than it is probably used. As a result, the LP UCI has always to less REs if either puncturing or rate matching is applied by the HP UCI. Therefore, we believe that multiplexing should be indicated dynamically. The scheduling DCI can have an additional field to enable this.

When an SPS is activated, DCI is involved as an activating DCI. The activating DCI can decide whether or not multiple HP UCI and LP UCI, but we prefer to have this field in the RRC signalling because CG type 1 PUSCH would have this field in the RRC signalling and it is rather beneficial to have a unified solution to SPS and CG.

Proposal 1: The scheduling DL-DCI has an additional field whether or not to allow multiplex HP UCI and LP UCI, or otherwise by the RRC signalling.

At least for SPS PDSCH, there is no scheduling DCI to produce HARQ-ACK. When HP SPS HARQ-ACK is multiplexed with LP HARQ-ACK from LP DCI, the LP DCI can indicate the final PUCCH resource. Inherently, the PUCCH resource can be chosen in the second PUCCH-Config if the HP UCI and LP UCI are multiplexed.

Proposal 2: The LP DCI determines the final PUCCH resource in at least for the HP SPS case.

Multiplexing rule and order:

In the Rel-16, the multiplex rule is to multiplex per priority and prioritize the HP UCI if exists. In the Rel-17, it may not be directly generalized because some UCIs can be dropped while being multiplexed. For example, the CSI may be dropped if final PUCCH resource may not able to guarantee sufficiently low code rate for the HP UCI. Some or entire CSI (part 2) may also be dropped due to the same reason.

For another example, both HP SR and LP SR can occur in some symbols. If we build a PUCCH resource per priority firstly, then SR is multiplexed with the associated priority. If we confine the multiplexing behavior into HARQ-ACK, then the multiplexing procedure revisit to exclude the LP SR. Otherwise, the UE should check whether positive HP SR has occurred to determine whether LP SR should be further considered or not.

We would propose that the HP UCI follows the Rel-16 rule to determine a PUCCH resource. Then, each LP UCI by the received order of DCI is checked whether to multiplex. The PUCCH resource is updated only when the LP UCI is determined to be multiplexed.

Proposal 3: Multiplex HP UCI, and check to multiplex each LP UCI at earliest order.

Transmission power aspect:

In the Rel-16, the power for PUCCH depends on the payload. Coding scheme is also one of inputs to determine the power for PUCCH. When LP UCI and HP UCI are multiplexed, we need to devise how to count the effective number of bits and how to calculate the offset to compensate the payload from the other priority.

Proposal 4: Further study how to adjust the power of PUCCH for payload from the other priority.

Determining the PUCCH resource set:

Proposal for 3rd round discussion: from the feature lead’s proposal in R1-2106063 For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, down-select from the two options in case the total number of LP and HP HARQ-ACK bits is 2: - Option 1: Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - Option 2: Use the PUCCH resource with more OFDM symbols between the original two PUCCH resources for the HP HARQ-ACK and LP HARQ-ACK. The power control parameters are reused from the PUCCH resource of HP HARQ-ACK.

In Rel-15, the UCI is spread for one or two bits or encoded otherwise, and then mapped to the resource grid. Code rate is configured per format. The payload for UCI determines a resource set, and PRI determines one resource in the chosen resource set. In Rel-17, we have both HP UCI and LP UCI in one channel with different quality (BLER). Thus, we need to describe how to choose a resource set. According to the previous discussion, the common understanding seems that the second PUCCH-config can be used for this purpose. We still think that at least dedicated PUCCH resource can be beneficial to reliably multiplex UCIs of both priorities. This can be considered for 2 bit case and for more than 2 bit case.

Proposal 5: The Option 1 (resource in the second PUCCH-Config) is preferred.

Determining the UCI size for the PUCCH resource:

Proposal for 3rd round discussion: from the feature lead’s proposal in R1-2106063 For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, down-select from the options on how to determine UCI payload size for PUCCH resource determination: - Option 1: UCI payload size = the number of HP UCI bits + the number of LP UCI bits - Option 2: UCI payload size = the number of HP UCI bits + the number of LP UCI bits * scaling factor. - FFS: The scaling factor can be a function of code rate for HP UCI and LP UCI. - FFS: the number of LP UCI bits should be the actual number of LP UCI bits that UE generates based on DCI or RRC configuration or a reference number of LP UCI bits

One of remaining question would be RE mapping on the PUCCH resource. In the current specification, the most promising baseline is to apply the procedure for CSI reporting, where the minimum bandwidth can be calculated from occupied resources.

To determine the UCI payload size, the weight factor can be considered. This factor comes from different code rate per priority, and we can weight HP UCI and LP UCI in terms of code rate. The idea is that each UCI payload is inversely emphasized by own code rate, and their sum which is possibly not an integer is used to determine the resource set. However, in the Rel-16, the code rate is configured per format, it is coupled between the code rate and the resource set.

Proposal 6: The Option2 (considering a scaling factor for UCI size) is considered.

  - Multiplexing on PUSCH

Explicit indication for enabling multiplexing:

Proposal for 2nd round discussion: 105 For multiplexing a HARQ-ACK into a PUSCH with different priorities in R17, at least support RRC configuration for gNB to enable/disable the multiplexing. - FFS whether or not to additionally introduce dynamic mechanism, e.g. DCI indication, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

In the Rel-16, multiplexing UCI is performed by puncturing TB or by rate matching. When two priorities are concerned, the scheduling DCI can indicate to multiplex LP UCI or drop, while HP UCI is being multiplexed. Depending on the scheduling, the amount of REs for TB may or may not be sufficient after UCI multiplexing. Therefore, we believe that multiplexing should be indicated dynamically. The scheduling DCI can have an additional field to enable this. This is a unified approach to both DL-DCI and UL-DCI.

When an configured grant is activated, DCI is involved as an activating DCI for CG type2. The activating DCI can decide whether or not multiple HP UCI and LP UCI, but we prefer to have this field in the RRC signalling because CG type1 would have this field in the RRC signalling and it is rather beneficial to have a unified solution to SPS and CG.

From the previous agreement, there are (up to) four pairs of beta offsets, each of which pairs is indicated by the DCI field. Either one value is for LP HARQ-ACK and the other value is for HP HARQ-ACK, and zero value has been discussed to indicate not to multiplex LP HARQ-ACK. We think that including zero value in the beta offset might not be flexible enough.

In our understanding, the set of beta offsets can be interpreted differently according to the presence of LP HARQ-ACK or not. Based on the same size of beta offset field, the two behavior of multiplexing UCI types or multiplexing LP HARQ-ACK should be distinguished. If one value of beta is zero, then the number of cases for beta offsets are quite limited.

As an example, if a UE determines that no LP HARQ-ACK bits are present, then the beta offset field can be interpreted as for different UCI types such as HARQ-ACK and CSI part1 and part2. If a UE determines that LP HARQ-ACK bits are present, then the beta offset field can be interpreted as for HP HARQ-ACK and LP HARQ-ACK. Since DTX events are unavoidable, we think that additional field should be (re)used to indicate which interpretation is taken. Otherwise, beta offset field should be able to extend to express a number of multiplexing cases. However, if we introduce a field to indicate multiplex/prioritize, then it is simple and similar design with the PUCCH case.

Proposal 7: The scheduling UL-DCI has an additional field whether or not to allow multiplex HP UCI and LP UCI, or otherwise by the RRC signalling.

PUSCH with more than one PUCCHs:

In the Rel-16, the PUSCH repetition type B allow piggybacking only one HARQ-ACK codebook for URLLC scenario. Following this rule, up to one PUCCH can be overlapped. However, also in the Rel-16, the joint codebook can be configured for mTRP scenario. Each HARQ-ACK codebook is concatenated and they form an HARQ-ACK codebook. Following this rule, more than one PUCCH can be overlapped.

If we can introduce an additional rule to build an extended HARQ-ACK codebook, then more than one HARQ-ACK codebook may be transmitted onto PUSCH repetition(s). Similarly, one PUSCH repetition can be conceptually regarded as a PUCCH. In this case, subslot based HARQ-ACK codebooks may be transmitted even in one PUSCH repetition. We would propose to concatenate subslot based HARQ-ACK codebooks if they are multiplexed onto one UL channel. We also note that if the additional rule is adopted, then this rule can be applied to both PUSCH and PUCCH.

Proposal 8: For HARQ-ACK codebook construction, sub-slot based HARQ-ACK codebooks are concatenated, and may be transmitted for PUSCH repetition.

Transmission power aspect:

In the Rel-16, the power for PUCCH in the given format depends on the payload. Coding scheme is also one of inputs to determine the power for PUCCH. When LP UCI and HP UCI are multiplexed, we need to devise how to count the effective number of bits and how to calculate the offset from the other priority.

Proposal 9: Further study how to adjust the power of PUSCH for payload from the other priority.

High priority SR on low priority PUSCH:

We can consider HARQ-ACK multiplexing due to the previous agreement . In our knowledge, discussion about SR on PUSCH does not begin because most companies consider HARQ-ACK multiplexing issues are more urgent. When we consider the multiplexing order, we expect that the PUCCH resources (including SR) per priority are determined firstly. In an alternative, if a UE additionally has a LP data and HP UCI, then the UE should not consider the HP SR on mapping PUSCH. This behavior may result the different PUCCH resource, and at least the UE revisit the calculation of the final resource or the UE can always assume negative SR.

Another alternative just regard a SR as a normal UCI that can be multiplexed on PUSCH. In our understanding, this approach has less specification efforts and simplify the multiplexing procedure.

Proposal 10: Further study the SR multiplexing onto PUSCH.

- Conclusion

Regarding PUCCH transmissions, we would like to suggest the followings.

Proposal 1: The scheduling DL-DCI has an additional field whether or not to allow multiplex HP UCI and LP UCI, or otherwise by the RRC signalling.

Proposal 2: The LP DCI determines the final PUCCH resource in at least for the HP SPS case.

Proposal 3: Multiplex HP UCI, and check to multiplex each LP UCI at earliest order.

Proposal 4: Further study how to adjust the power of PUCCH for payload from the other priority.

Proposal 5: The Option 1 (resource in the second PUCCH-Config) is preferred.

Proposal 6: The Option2 (considering a scaling factor for UCI size) is considered.

Regarding PUSCH transmissions, we would like to suggest the followings.

Proposal 7: The scheduling UL-DCI has an additional field whether or not to allow multiplex HP UCI and LP UCI, or otherwise by the RRC signalling.

Proposal 8: For HARQ-ACK codebook construction, sub-slot based HARQ-ACK codebooks are concatenated, and may be transmitted for PUSCH repetition.

Proposal 9: Further study how to adjust the power of PUSCH for payload from the other priority.

Proposal 10: Further study the SR multiplexing onto PUSCH