3GPP TSG RAN WG1 #106-e		                     R1-2107538

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.4.1

Source:	LG Electronics

Title:	Discussions on timing relationship enhancements in NTN

Document for:	   Discussion and Decision

- Introduction

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac.  How to treat NTA and NTA,offset can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

In RAN1#105-e meeting, the following agreements are captured in the chairman’s note as:

In this contribution, we further discuss the remaining issues for timing relationship enhancements in NTN.

- Discussion

  - Signaling of K_offset in initial access

In the RAN1#103-e meeting, it was agreed K_offset configured in system information and used in initial access is supported by at least cell specific configuration. The first remaining issue is whether the signaling of K_offset is based on the implicit or explicit signaling. For instance, in case of implicit signaling, K_offset can be determined based on common TA, and this option can provide the benefit in terms of signaling overhead reduction. However, coupling K_offset with other information such as common TA may restrict the network flexibility. Moreover, the main purpose of K_offset is to remove the ambiguity between gNB and UE caused by the long round-trip delay in NTN, so mixture of two different functions is not desirable. Lastly, explicit signaling is more cleaner way and more forward compatible compared to implicit signaling. Thus, explicit signaling of K_offset is preferred.

Proposal 1: Support explicit signaling of K_offset.

Agreement:

- For determination of cell-specific K_offset in system information, down-select one option from below:

  - Option 1: Signal one offset value for K_offset

    - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

  - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

    - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

The second issue is determination of cell-specific K_offset in system information between two options captured below:

The K_offset is introduced to cover UE-gNB RTT comprised of feeder link and service link. In our view, the functionality for both options are the same. The one advantage of option 2 may be reducing some signaling overhead by replacing feeder link RTT to common TA. As pointed in [1], the signaling overhead saving in Option 2 over Option 1 is only about 1 bit at the expense of more UE complexity. Also, as mentioned above, implicit signaling for K_offset is not preferred. Thus, signaling one offset value for K_offset (Option 1) seems sufficient.

Proposal 2: For determination of cell-specific K_offset in system information, support signaling one offset value for K_offset (Option 1).

The next issue is whether or not to additionally support beam-specific K_offset in initial access. The cell-specific K_offset signaling has a merit in the overhead perspective. However, in general, the cell size in NTN is much larger than that of TN, therefore differential timing offset would not be negligible between UE in cell center and cell boundary. For example, the maximum satellite beam size can be up to 1000km for LEO which results in up to 3.2 ms maximum differential delay within a satellite beam. Thus, beam-specific K_offset indication would be more suitable in NTN. If the signaling overhead in SIB is really a problem, we may consider beam-group specific K_offset. Compared to beam-specific K_offset, beam-group specific K_offset can effectively reduce the signaling overhead while it can provide finer K_offset granularity. In this beam group specific K_offset signaling, how to make beam-group can be further studied.

Proposal 3: Support beam (group)-specific K_offset signaling in addition to cell-specific K_offset in initial access.

  - Update of K_offset

In the RAN1#104b-e meeting, two options (Option 1: RRC reconfiguration, Option 2: MAC CE) are listed for updating K_offset after initial access. During the discussion of RAN1#105-e meeting, above options were further revised as:

- Option 1: support MAC CE option

- Option 2: support both MAC CE option and RRC reconfiguration option

In our understanding, RRC reconfiguration (Option 2) is targeted for GEO scenario where the both cell-specific and/or UE-specific K_offset can be rarely updated. In case of LEO, frequent update of K_offset is expected, and thus in order to achieve fast timing adjustment, option 2 is more preferable. In both cases, Option 1 and Option 2 can work properly, but, Option 2 has a merit of shorter delay, so Option 1 is preferred.

Proposal 4: Support at least MAC-CE based K_offset update after initial access.

One related issue is when to apply the updated K_offset value. For solving this issue, two options can be considered. One is to apply new K_offset value X slot after the reception of update signaling. The other one is to apply new K_offset value X slot after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception. Among the options, our preference is latter one since gNB can remove some ambiguity whether the UE successfully decode the update signaling or not.

Proposal 5: Apply new K_offset value X slot/symbol after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception.

Another issue is how and when to trigger K_offset update. One possible way can include that update of K_offset is triggered when the estimated/calculated TA value is larger than the configured or most updated K_offset. In Rel-17 NTN, UE autonomous TA estimation/calculation is supported, so UE can initiate/trigger the update of K_offset. In case of NW initiated K_offset triggering, TA reporting by UE can be considered and our detailed view are discussed in our companion contribution [2]. For the TA reporting contents, several options including UE specific TA, Full TA, UE location information and difference between UE-specific K_offset and cell-specific K_offset are listed in the last meeting. In our view, TA information seems enough and the granularity of TA can be further discussed. Between UE specific TA and full TA, there is no big difference, so we prefer reporting UE specific TA if supported.

For the K_offset usage, one FFS point is how to treat additional transmission timings related to fallback DCI formats. Since fall back operation is used for the purpose of robustness, using cell-specific K_offset seems reasonable.

  - Indication of PDSCH-to-HARQ_feedback timing

In RAN1#104 meeting, it was agreed to increase the range of K1 values from (0,…,15) to (0,…,31) for unpaired spectrum. Then, the FFS point is whether there is an impact on the size of K1 field in DL DCI. For non-fallback DCI, gNB pre-configure candidate K1 values via RRC, and then DL DCI is used to indicate actual K1 value. Thus, one solution without increasing the size of K1 field is simply increasing the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31) as below. Note that regarding the parameter, we think simple extension for rel-17 NTN UE (i.e., dl-DataToUL-ACK-r17) can be introduced.

dl-DataToUL-ACK-r17                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)                             OPTIONAL, -- Need M

The other solutions can be increasing the size of K1 field in an explicit or implicit manner. In case of explicit indication, the size of K1 field can be up to 5bits. For the implicit indication, we may consider slot index based as discussed for the HARQ process id identification, but it restricts scheduling flexibility. Therefore, among the listed potential solutions, we prefer to have a simple solution that only increases the range of dl-DataToUL-ACK.

In case of fall-back DCI (i.e., DCI format 1_0), the candidate K1 values are fixed to {1, 2, 3, 4, 5, 6, 7, 8} as in the TS38.213. Thus, increasing K1 range does not provide any benefit in this case. To resolve this issue, we can consider introducing fixed or configurable offset for non-fallback DCI case. For example, we can use minimum K1 value configured for the non-fallback DCI as an offset.

Proposal 6: Do not increase the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- For non-fallback DCI, increase the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31).

- For fallback DCI, consider introducing fixed or configurable offset.

  - RACH procedure triggered by PDCCH order

According to TS 38.213, the UE transmits a PRACH in the selected PRACH occasion for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to the minimum gap of NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec. This minimum gap is depicted in the Figure 1. In NR, UE assumes initial TA = 0 for PRACH transmission.

Figure 1. An illustration of RACH procedure triggered by PDDCH order without timing offset

In Rel-17 NTN, the initial TA for PRACH transmission can be determined based on UE specific TA (estimated by UE) + common TA (if provided). Therefore, as discussed in RAN1#104e meeting, gNB may not know which PRACH occasion was selected by the UE according to the UE’s “next available PRACH occasion”. To alleviate this problem, there are two possible solutions. One is based on the gNB implementation which means gNB may perform additional blind detection for a PDCCH ordered PRACH. The other one is to introduce timing offset which may correspond to UE specific TA + common TA in addition to minimum gap. As shown in Figure 2, the additional timing offset can remove the ambiguity on PDCCH ordered PRACH transmission. Between two solutions, we think introducing additional offset is more cleaner way to resolve this issue.

Figure 2. An illustration of RACH procedure triggered by PDDCH order with timing offset

Proposal 7: For RACH procedure triggered by PDCCH order in Rel-17 NTN, define timing offset in addition to minimum gap, NT,2+ ∆BWPSwitching+∆Delay+Tswitch.

- Conclusion

In this contribution, we discussed on timing relationship enhancements for NTN. Based on the above discussion, we have following proposals:

Proposal 1: Support explicit signaling of K_offset.

Proposal 2: For determination of cell-specific K_offset in system information, support signaling one offset value for K_offset (Option 1).

Proposal 3: Support beam (group)-specific K_offset signaling in addition to cell-specific K_offset in initial access.

Proposal 4: Support MAC-CE based K_offset update after initial access.

Proposal 5: Apply new K_offset value X slot/symbol after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception.

Proposal 6: Do not increase the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- For non-fallback DCI, increase the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31).

- For fallback DCI, consider introducing fixed or configurable offset.

Proposal 7: For RACH procedure triggered by PDCCH order in Rel-17 NTN, define timing offset in addition to minimum gap, NT,2+ ∆BWPSwitching+∆Delay+Tswitch.

Reference

[1] R1-2106325, “Feature lead summary#5 on timing relationship enhancements”, Ericsson

[2] R1-2107539, “Discussions on UL time and frequency synchronization enhancements in NTN”, LG Eletronics