3GPP TSG RAN WG1 Meeting #106-e	R1-2107547

e-Meeting, August 16th– 27th, 2021

Title:	Draft reply LS on PUCCH and PUSCH repetition

Response to:	R4-2107880

Release:	Rel-17

Work Item:	NR_cov_enh-Core

Source:	LG Electronics [RAN1]

To:	RAN4

Contact Person:

Name:

E-mail Address:

Attachments:	None

Overall description

RAN1 would like to provide response on the following question from RAN4 in R4-2103393:

RAN4 also asks RAN1 to provide more information on the scenario when “downlink reception” from UE point of view includes downlink symbols without actual DL transmission from gNB to UE and UE is not assumed to do DL monitoring?

Response:

The understanding of RAN1 for downlink reception of the UE is as follows:

The gNB indicates the UE to monitor a specific time resource in order to transmit the DL, and accordingly, the gNB transmits the actual DL in the corresponding time resource, which means that the UE monitors it. In addition, if the gNB does not transmit DL to a specific time resource, the gNB does not indicate the UE to monitor the corresponding time resource, and accordingly, the gNB does not transmit DL.

On the other hand, the gNB indicates the UE to monitor specific time resource in order to transmit DL in the time resource, but there is a possibility that the UE misses this indication. In such a case, the gNB transmits the actual DL in the corresponding time resource, however the UE may not monitor it. Even if the phase continuity can be guaranteed in UE side, it is hard for the gNB to assume that the phase continuity of the UE is guaranteed. That is, even if the gNB transmits or does not actually transmit DL, the gNB should understand that the UE's phase continuity is not guaranteed for a slot that the UE expects to monitor.

Actions

To RAN4

Actions: RAN1 respectfully asks RAN4 to take the above information into account in their further work.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#106-e,			16th August to 27th August 2021