3GPP TSG RAN WG1 #106-e	R1-2107548

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.8.1.1

Source: 	LG Electronics

Title: 	Discussions on PUSCH repetition type A enhancements

Document for:	Discussion and decision

Introduction

In RAN1#105-e meeting, enhancement of PUSCH repetition type A were discussed including determination rule for the available slots and maximum number of repetitions. Also, several agreements and conclusion were made [1]. From the previous agreement, we continue to discuss on enhancement of PUSCH repetition type A.

Increasing the maximum number of repetitions

In the last meeting, the maximum number of repetitions was discussed and following agreements were made [1].

Agreement: - Down-selection in RAN1#106-e: - Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method, - Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots. Agreement: In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17. - {20, 24, 28}

According to the previous agreement, following two alternatives were identified.

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Main difference among alternatives is whether same number of repetition is applied for both the counting based on physical slots and available slots or not. In FDD case, two cases (i.e., counting based on physical slots and available slots) are identical. In case of Alt.2, if 16 for the counting based on available slots is adopted, it is hard to expect further enhancement of coverage in FDD case. Considering that the coverage enhancement should be provided for both FDD and TDD cases, it is preferred that same number of PUSCH repetition should be applied for both FDD and TDD. Therefore, Alt1 (‘The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.’) should be selected.

Proposal 1: Alt1 (‘The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.’) should be selected.

Also, it is under discussion RRC parameters to be extended for supporting the increased maximum number. In detail, it needs to be determined whether repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions or not.

We think that there will be cases where the UE is located for a long time in an environment that requires coverage enhancement. In this case, it may be necessary to continuously apply a large repetition number to PUSCH transmission, and it seems helpful to support the increased maximum number of the repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Proposal 2: Increase the maximum number of repetitions for pusch-AggregationFactor and repK.

Available slots for the PUSCH repetitions

In RAN1#105-e meeting, determination rule for the available slots was discussed, and it was agreed to discuss select one of the alternatives.

Agreement: Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded) - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - Alt 1-B’ consisting of two steps - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability - Alt 2-A consisting of a single step - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Alt 2-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - FFS timeline for the dynamic signalling - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Main difference between Alt 1 series and Alt 2 series is whether the dynamic signalling for determination of the available slots is considered or not. In case of Alt 1-B and 1-B’, PUSCH transmission can be dropped in a slot due to dynamic signaling even if the slot is determined as available. In this case, it is not hard to achieve all of expected number of repetition. Nevertheless, a benefit of Alt 1-B and Alt 1-B’ is that the overall time duration for PUSCH repetition is kept. On the other hand, in case of Alt 2-A and 2-B, since the availability of a slot is determined dynamically, UL transmission could be postponed. Contrary to the intention of the network, it may be undesirable that the ending time of PUSCH transmission is delayed. In addition, when the determination of the available slot is based on dynamic signaling, the determination of the availability slot between the gNB and the UE is not aligned since there is a possibility that the UE misses dynamic signaling. Therefore, we prefer to adopt Alt 1-B or Alt 1-B’ for determining available slots for PUSCH repetitions.

When the availability of a slot is not determined by dynamic signaling, it should be discussed further which RRC configurations are used to determine the availability. It seems to be enough to determine available slot using semi-static DL symbol and semi-static flexible symbol with SSB transmission, which is aligned behavior for PUCCH repetitions.

Thus, we propose to support Alt 1-B’ with some modifications as follows.

- Step 1: Determine K repetitions based on available slots, where the available slot is the slot satisfying

  - All symbols allocated for PUSCH is not configured to UL symbol or flexible symbol by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - All symbols allocated for PUSCH are not SS/PBCH block symbols

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Proposal 3: Support following procedure of Rel-17 PUSCH repetition Type A.

- Step 1: Determine K repetitions based on available slots, where the available slot is the slot satisfying

  - All symbols allocated for PUSCH is not configured to UL symbol or flexible symbol by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - All symbols allocated for PUSCH are not SS/PBCH block symbols

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Conclusion

In this contribution, we discussed on enhancement of PUSCH repetition type A. From the discussion, we provide following proposals.

Proposal 1: Alt1 (‘The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.’) should be selected.

Proposal 2: Increase the maximum number of repetitions for pusch-AggregationFactor and repK.

Proposal 3: Support following procedure of Rel-17 PUSCH repetition Type A.

- Step 1: Determine K repetitions based on available slots, where the available slot is the slot satisfying

  - All symbols allocated for PUSCH is not configured to UL symbol or flexible symbol by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - All symbols allocated for PUSCH are not SS/PBCH block symbols

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Reference

- RAN1 Chair’s Notes, RAN1 #105-e, e-Meeting, May 10th – 27th, 2021