3GPP TSG RAN WG1 #106-e	R1-2107554

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.10.2

Source: 	LG Electronics

Title: 	Other enhancements for simultaneous operation of IAB-node’s child and parent links

Document for:	Discussion and decision

Introduction

In the last meeting, several agreements were made regarding on other enhancements for simultaneous operation of IAB-node’s child and parent links. [1] In this contribution, we discuss timing modes, CLI enhancement and power control.

Timing modes

Case 6 timing mode

The IAB-MT’s transmission timing is indicated by the parent IAB in terms of timing advance. To support case 6 timing, when indicated, the IAB-MT does not apply the corresponding timing advance, and align the transmission timing with the IAB-DU downlink transmission timing. Therefore, when this is indicated based on timing advance, a lot of timing gaps are required for single indication, which is not possible with the existing TA indication method. In that sense, three alternatives are proposed:

- Introduce additional offset to legacy TA

- Align with own IAB-DU’s downlink transmission,

- Receive common offset from parent and align with IAB-DU transmission timing.

According to the agreement of the last meeting, it was decided to consider four items for case 6 timing mode, so we compared them from that point of view in Table 1.

Table 1. Comparison of alternatives for Case 6 timing

|  | Dependency of DL synchronization scheme at the IAB-DU | Potential additional signaling overhead | Achievable DU Tx / MT Tx alignment error tolerance | Suitability for switching between timing modes |
| --- | --- | --- | --- | --- |
| Alt. 1 Legacy TA loop + offset | None | Additional offset | Depend on granularity of offset | Enabling and disabling of offset |
| Alt. 2 Align to DL Tx | High | None | Minimum | Enabling and disabling of TA |
| Alt. 3 Jointly with IAB-DU Tx timing via common offset from the parent node | Very high (parent node & IAB node) | IAB-MT receiving common offset from parent node | Minimum | Enabling and disabling of TA and common offset |

Alt. 1 requires signalling for an additional offset, and overhead will increase according to the granularity of the offset. Also, considering the existing TA mechanism, at least an offset with finer granularity than the current TA will be needed. In addition, Alt. 3 requires additional signalling because the IAB MT receives a common offset from the parent node. On the other hand, Alt. 2 does not require additional signalling and is expected to have the minimum error tolerance since it is aligned with the DU transmission timing which is aligned among every DU. Therefore it is natural that Alt. 2 is the most preferred.

Proposal 1: For case 6 timing of IAB-MT, Alt. 2 (alignment to own DU downlink transmission timing) is supported.

Case 7 timing mode

Depending on the TA updated in the existing MAC CE, the amount of TA adjustment that can be made by a single indication is about 1/4 symbol regardless of numerology. In addition, since the existing TA command is applied at the beginning of uplink slot n+k+1 after the indication, TA adjustment is effectively performed in units of multiple slots. Therefore, in order to support case 7 timing by exploiting legacy TA, it is not suitable for symbol level alignment because TA adjustment is eventually made in multiple slot units. In addition, realistically, the timing of case 7 can be matched only when TA adjustment is instructed before 2 (n+k+1) slots. Furthermore, since running two TAs leads a large signalling overhead, it would be appropriate to add an offset for legacy TA. According to the agreement of the previous meeting, it was decided to consider three aspects for the case 7 timing, so we compared them from that point of view in Table 2.

Table 2. Comparison of alternatives for Case 7 timing

|  | Potential impact to OTA synch | Signaling overhead | Suitability for switching |
| --- | --- | --- | --- |
| Alt. 1 Legacy TA + offset | Offset signalling needed | Medium | Enabling and disabling of offset |
| Alt. 2 Legacy TA | Multiple TA adjustment timing should be aligned to case 7 timing | Minimum | Maximum suitability |
| Alt. 3 Case 7 specific TA | Additional TA loop needed | Maximum | Enabling and disabling of case 7 specific TA |

Since Case 7 timing is symbol level alignment unlike Case 6 timing, a lower granularity will be sufficient when adding an offset. That is, the overhead of the offset is not large, and if it is based on this, it will be the most flexible because it is possible to instantaneously indicate the timing without the need to indicate the timing in advance. Therefore Alt. 1 is most preferred. Using this offset means that IAB-MT is indicated when applying case 7 timing. That is, the configuration of the offset for case 7 timing is identical to indication of case 7 timing. In addition, since the UL timing boundary for supporting Case 7 timing is applied for group of IAB-MTs, the additional TA offset value can be indicated in MT group specific manner similar with availability indicator in DCI format 2_5.

Proposal 2: For case 7 timing of IAB-MT, Alt. 1 (‘the legacy TA loop plus an offset from the parent node’) is supported.

Proposal 3:  IAB node is indicated when case 7 timing is applied.

Proposal 4: The additional TA offset for case 7 timing is indicated in MT group specific manner (e.g., similar with availability indicator in DCI format 2_5)

CLI

In this section, we discuss about DU-to-DU CLI measurement and report, MT-to-MT CLI measurement and report and CLI enhancement for simultaneous operation.

DU-to-DU CLI measurement and report

The existing CLI framework is a framework between UE-gNBs and cannot be applied to DU-DU measurement and report because there is no report for gNBs. Therefore, for DU-DU measurement and report, RIM should be the starting point of discussion, but it should be specified as a new framework. Also, measurement and report should be discussed together, and it is not desirable to discuss them separately.

MT-to-MT CLI measurement and report

It should be considered in the direction of enhancing the Rel-16 CLI framework. It can be considered to expand the contents of information exchange of CLI. For this, the measurement RS configuration can be considered, however the result of measurement is not desirable since it could be a burden since exchange should occur frequently.

CLI enhancement for simultaneous operation

In the previous meeting, it was agreed to exchange the intended TDD DL-UL configuration. Since this is semi-static and cell-specific information, it has been proposed as an enhancement of CLI, which is totally different for H/S/NA information. The H/S/NA information is for the availability information of soft resources, is dynamically indicated information, and is UE-specific information. Therefore, in order to exchange this, dynamic information exchange should be performed which is a burden, and since it is UE-specific, the amount of information is very likely to be large compared to cell-specific information. More importantly, if information exchange does not occur before the indication, this will be the outdated information. Therefore, information exchange should occur before the indication, which is difficult in reality. That is, the interference management cannot be based on outdated information. Therefore, the exchange of dynamic H/S/NA information is not desirable. On the other hand, it can be considered that H/S/NA information is indicated in long term manner and only availability of S between IAB-DU and IAB-MT is indicated dynamically. In such case, H/S/NA information can be shared between networks.

Proposal 5: H/S/NA information for CLI management purposes can be supported only if H/S/NA information is shared in long term manner.

Power control

- In this section, we discuss about uplink and downlink power control, and CU indicating information to assist power control.

Uplink power control with assistance information

- The scenario for uplink power control according to the multiplexing case is as follows.

Multiplexing case A(MT Tx DU Tx) : If MT/DU is the same panel, power shortage may occur or output power may be limited due to regulation. In such a case, in the downlink, there are signals that the transmission power is not expected to be changed, such as SSB and CSI-RS. In order to protect them, it is necessary to lower the transmission power of the uplink. That is, if the assistance information of the uplink power control is not introduced, the simultaneous transmission IAB may affect coverage by lowering the downlink transmission power to transmit uplink with the indicated power, or may cause ambiguity by transmitting with less power than the indicated uplink power. That is, if the uplink power is lowered without indicating such a situation, the parent DU may not recognize it and misunderstanding may occur. In a word, assistance information to help its MT’s uplink transmit power control is needed.

Multiplexing case C(MT Tx DU Rx) :

In case of the same panel, or even for multi panel case, SI (Self-Interference) may affect DU Rx if isolation is not sufficient. In such a case, since the parent is unaware of this situation, misunderstanding between IAB and parent IAB may occur if the IAB lowers the transmit power of the MT Tx for the DU Rx without any action. Therefore assistance information is required to solve this problem.

- Whatever the form of assistance information is, it should be in a form that indicates the transmit power that the IAB-MT desires. That is, since the IAB-DU cannot know what the current accurate transmission power of the child IAB-MT is due to pathloss, the assistance information should be in the form of an offset of the configured output power of the most recent transmission. Here, since assistance information does not mandate the operation of parent IAB-DU, it is not desirable to spend too many bits to express assistance information. That is, it is preferable to transmit the most compact information, so the desired Tx power is appropriate.

- Since such assistance information does not mandate the operation of the parent IAB-DU, the behavior of the IAB-DU cannot be predicted. Therefore, this information does not need to be shared with the CU. The MT’s power control formula does not need to be changed since the parent IAB DU schedules the transmission power of the MT based on the assistance information, not in reverse order.

Proposal 6: Adopt assistance information for uplink transmission power control.

DL PC with assistance information

It was agreed to introduce desired power adjustment as assistance information of downlink power control. When the parent IAB receives this desired power adjustment from the child IAB-MT, it should indicate to the child IAB-MT about accepting this power adjustment. If not indicated, the IAB-MT cannot know that the transmission power of the gNB is lowered or higher, and this may affect the estimation of pathloss and possibly other behavior. That is, it is necessary to indicate to the IAB-MT the amount of change in transmission power compared to the signal that the IAB-MT expects to be constant power, such as SSB or CSI-RS, which is updated by RRC. However, downlink power control according to the simultaneous operation of the IAB can be performed dynamically, so the parent IAB-DU needs to dynamically inform the IAB-MT about the change in downlink transmit power through MAC-CE or DCI.

Proposal 7: The parent IAB-DU indicates when the desired power adjustment of IAB-MT is applied.

Considering the desired power adjustment for downlink power control according to the multiplexing scenario, it is as follows.

Multiplexing case B(MT Rx DU Rx) : A desired power adjustment may be requested for the purpose of limiting the received power level within a specific range for stability of AGC operation (according to implementation), prevention of quantization distortion, and guarantee of amplifier linearity.

Multiplexing case D(MT Rx DU Tx) : In the downlink of the child link, there are signals such as SSB and CSI-RS that the transmission power is not expected to change. Such signals may effect MT Rx in terms of large power of SI. Therefore, the IAB may request to boost the power level of the signal received from the parent.

Unlike uplink power control, multiplexing case B may be a request to increase or decrease the transmit power of the parent DU, and it may be considered that multiplexing case D is a request to increase the transmit power of the parent DU. Therefore, it is necessary to consider applying assistance information according to the multiplexing scenario. It is natural that the application range of the desired power adjustment for DL power control is applied only to the UE-specific signal, not to the cell-specific signal since cell-specific signals affect the entire cell coverage.

Proposal 8: Downlink power control is applied according to multiplexing scenario.

Proposal 9: Downlink power control by desired power adjustment is applied only for the UE-specific signal/channel.

CU indicating information to assist power control

Instantaneous uplink/downlink power control based on assistance information is necessary, but since it does not mandate operation, it is rather the purpose of eliminating ambiguity than expecting power control. In order to reduce the probability of occurrence of such ambiguity, the CU configuring the maximum allowed power of the uplink/downlink according to the resource can be considered. That is, by changing the transmission power of the uplink/downlink according to the resource for performing the simultaneous operation, it is possible to prevent a decrease in the reception sensitivity of the MT/DU due to the Tx power shortage, Rx power imbalance, and SI in advance.

Proposal 10: The CU configures the maximum allowed transmit power according to the resource type.

Conclusion

- In this contribution, we discuss on specifications of other enhancement for simultaneous operation of IAB. From the discussion, we obtained following proposals:

Proposal 1: For case 6 timing of IAB-MT, Alt. 2 (alignment to own DU downlink transmission timing) is supported.

Proposal 2: For case 7 timing of IAB-MT, Alt. 1 (‘the legacy TA loop plus an offset from the parent node’) is supported.

Proposal 3:  IAB node is indicated when case 7 timing is applied.

Proposal 4: The additional TA offset for case 7 timing is indicated in MT group specific manner (e.g., similar with availability indicator in DCI format 2_5)

Proposal 5: H/S/NA information for CLI management purposes can be supported only if H/S/NA information is shared in long term manner.

Proposal 6: Adopt assistance information for uplink transmission power control.

Proposal 7: The parent IAB-DU indicates when the desired power adjustment of IAB-MT is applied.

Proposal 8: Downlink power control is applied according to multiplexing scenario.

Proposal 9: Downlink power control by desired power adjustment is applied only for the UE-specific signal/channel.

Proposal 10: The CU configures the maximum allowed transmit power according to the resource type.

Reference

- RAN1 chairman’s notes, RAN1 #105-e, e-Meeting, May 10th – 27th, 2021

Agreements in RAN1#105-e

Agreement RAN1 to downselect how the IAB-MT Tx timing is set for Case 6 timing at a given IAB-node: - Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node. - FFS details of the required offset. - Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx. - Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node. Downselection to consider at least the following aspects: - Dependency of DL synchronization schemes at the IAB-DU - Potential additional signaling overhead. - Achievable DU Tx / MT Tx alignment error tolerance. - Suitability for switching between timing modes. Agreement RAN1 to downselect how the IAB-MT Tx timing is set at an IAB-node for Case 7 timing at the parent node: - Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node. - FFS details of the required offset - Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node. - Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node. Downselection to consider at least the following aspects: - Potential impact to OTA synchronization availability for DU Tx at the IAB-node. - Potential additional signaling overhead. - Suitability for switching between timing modes. Agreement An IAB-node is indicated when Case 6 timing is performed at the IAB-node. - FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.). FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node. Agreement Rel-16 CLI coordination signalling (Intended TDD DL-UL Configuration) is extended to support IAB specific UFD patterns. - FFS: Support the exchange of IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors for CLI management purposes. Agreement Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be: - FFS: Desired TX power - FFS: Offset to a baseline PHR - FFS: Desired dynamic range FFS: whether this information is provided to the parent-node, the CU, or both. FFS: whether the MT’s UL TX power control formula needs to be changed Agreement The information to assist DL power allocation of the parent-node is indicated by the IAB-MT to the parent node DU in terms of desired power adjustment. - FFS applicability of assistance information, e.g. per multiplexing scenario, per resource, etc.