3GPP TSG-RAN WG1 Meeting #106-e	R1-2107560

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.8.1.2

Source:	Ericsson

Title:	TB Processing over Multi-Slot PUSCH

# Introduction

In RAN#90e meeting, following objective has been approved for NR coverage enhancement work item in NR Rel-17 for PUSCH [1]:

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

In RAN1#105-e, some agreements were reached regarding Type-A like TDRA, transmission occasion for TBoMS (TOT) , options of TBoMS structure and time units of rate matching. In this contribution, we discuss various aspects of TBoMS with focus on time domain resource allocation, RV determination, rate matching, UCI multiplexing, TBS determination, as well as other issues. Link level simulations are provided to investigate performance of TBoMS under different conditions and RV determination approaches.

# Discussion

Rel-15/16 PUSCH repetition comprises a broad set of mechanisms to determine which time domain resources are used. These include how to determine resources allocated to each repetition, the total number of repetitions, and which slots are omitted and/or which symbols are invalid. Rel-17 TBoMS transmission also requires mechanisms to determine time domain resources. Therefore, one way to reduce complexity in UEs is to construct a multi-slot TB transmission by combining PUSCH repetition time domain resource determination mechanisms with multi-slot transport block construction methods.

- Rel-15/16 PUSCH repetition and TB over multiple slots have great commonality in terms of configuration and signaling of time domain resources as well as invalid symbol patterns.

- Reuse resource determination and signaling of Rel-15/16 PUSCH repetition as much as possible to avoid specifying duplicate functionality.

## Time domain resource determination for TBoMS

It was agreed in RAN1#105e to support only PUSCH repetition Type A like TBoMS. Whether to support TDRA optimization for special slot, e.g. different UL symbols in special slot are used from those in UL slot, needs further study. The use of special slot in TDD according to Type-A TDRA is a working assumption. We will discuss the optimization for special slot and the working assumption in this section.

Agreement: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA. - FFS: details - FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case) Working assumption Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Regarding TDRA optimization for special slot, one proposed optimization is that the allocated symbols in special slot can be different from those in UL slot.  It was claimed that the use of a special slot in TBoMS can maximize resource utilization of UL symbols. therefore the use case mainly considered is the use of contiguous UL symbols in S slot and the following U slot(s). As illustrated in the lower part of Figure 1, a TBoMS containing S and U slots consists of 16 contiguous UL symbols. A straightforward baseline of TBoMS is a TB over two UL slots, e.g. a total of 28 symbols with 3 slots in between. The TBoMS of S and U reduces transmission delay by using the fragmented UL symbols, but doesn’t necessarily improve UL coverage.

Figure 1, TBoMS of S and U vs. TB over two UL slots

It was argued that TBoMS may collide with SRS or HARQ-ACK, which are configured in the last symbols in UL slot, then the optimization for special slot can be considered. If we consider the SRS or HARQ-ACK in UL slot together with the optimization of special slot, the TBoMS can consist of the UL symbols in S and the first several UL symbols in the subsequent UL slot, as shown in the lower figure in Figure 2. This is based on the assumption that PUCCH/SRS is scheduled to be transmitted in UL slot, rather than S slot. Otherwise, the legacy one-slot TB PUSCH can be used by scheduling the transmission of PUCCH/SRS in special slot, as shown in the upper part in Figure 2. With the same number of UL symbols for PUSCH in the two schemes, the TBoMS comprised of UL symbols in special slot doesn’t provide any coverage or latency gain over the legacy TB in one slot.

Figure 2, TBoMS of S and U with SRS/PUCCH in the last UL symbols in UL slot

As the numbers of available UL symbols in S slot and U slot are different, the use of consecutive UL symbols in special slot and the following UL slot belongs to the category of Type-B like TDRA, according to the definition in RAN1#104e agreements. If it is treated as a special case of Type-A like TDRA, e.g. by L>14, issues like DMRS mapping type, DMRS positioning will have a big standard impact.

An alternative approach could be to use a special slot together with all normal uplink slots for all repetitions of a PUSCH, while still leaving some resources available for SRS or A/N after the PUSCH transmission. As shown in [2], the link level performance is essentially the same in this case.

Another issue with using DMRS in one slot to form channel estimates for another slot is that the interference should be the same between the slots for best performance with multi-antenna interference suppression receivers. As discussed in [3], since the special slot can have difference interference characteristics, if a DMRS in a special slot or uplink slot is used to determine interference suppression combining weights for the other type of slot, the losses in the performance of interference suppression could degrade or eliminate the gains from using a small number of symbols in a special slot.

- The net gains and use cases of TBoMS supporting special slot with different number of UL symbols than that in UL slot for the TB should be carefully studied prior to specifying it.

  - Such study should address how SRS and PUCCH can be transmitted as well as the performance of interference suppression when DMRS in a special or normal uplink slot is used for interference suppression in the other type of slot.

  - If specified, and performance gains are targeted for this case, a TB over consecutive UL symbols in special slot and the following UL slot can be based on PUSCH repetition type-B like TDRA.

Without TDRA optimization for special slot, regarding the working assumption on whether to use special slot according to Type-A like TDRA, if the UL symbols in a special slot are determined as time domain resources for a TBoMS, the configured L in TDRA has to be no larger than the number of UL symbols in the special slot. In other words, L is less than 14. Then the transmission of TBoMS will see non-zero gap between consecutive slots, which makes joint channel estimation impossible, except when there is no UL transmission of other channel/signal from the UE during the gap and the UE meets the off power requirement. In addition, it increases the complexity of gNB scheduling, and it is arguable whether such configuration will cause more UL symbols unused in the UL slots than the number of UL symbols used in special slot.

- UL symbols in special slot in TDD are not included for time domain resource determination for TBoMS according to the agreed Type A like time domain resource determination for TBoMS.

## The structure of TBoMS

In RAN1#105-e, a working assumption on transmission occasion for TBoMS (TOT) was agreed. The structure of TBoMS and the time unit for rate-matching still have several options for down-selection.

Working assumption A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission - FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc. - FFS: whether such concept will be specified or not. Agreement: - The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e) - Option 3, if a design based on single RV is adopted. - Option 4, if a design based on different RVs is adopted. - FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc. - The single RV is not constrained to have only the same coded bits in each slot or in each TOT - The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on. Agreement: The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected: - Option a: Rate-matching is performed per slot; - Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT; - Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X. Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

In RAN1#104bis-e and RAN1#105e companies discussed how coded bits of single or multiple RVs are transmitted over time-domain resources of a TBoMS and the possible time units of rate-matching. Option c with rate-matching performed across all slots/TOTs for TBoMS only applies to TBoMS with a single RV. Option a and b support time units of rate-matching being a slot and a TOT respectively. Both options can apply to TBoMS based on single or multiple RVs.

For TBoMS with multiple RVs, all coded bits of the TBoMS are deposited in a circular buffer. The fraction of coded bits of different RVs are mapped in different time-domain resources. As RVs update at slot/TOT borders, one problem is not all systematic bits can be transmitted. Option b and Option a in an example of TBoMS over four slots with TDD UL/DL configuration of DDSUU are illustrated as Option b and a1 respectively in Figure 3. With Option b, the first TOT of two slots carries part of the coded bits from RV0, and Option a1 has RV0 transmitted only in the first slot. It is likely some systematic bits are not transmitted. Simulation results showing the loss of performance from this effect can be found in section 2.6.1. On the other hand, for option b, if the TOT size is large enough, e.g. comprised of many consecutive slots, there would be overlapping encoded bits transmitted in the resources of two RVs. In addition, the use of multiple RVs for the transmission of a TB may deviate the code rate from the expected code rate. The code rate depends on the number of information bits and transmitted coded bits, but TBoMS with different RVs may have variable code rate.

Figure 3, different time units of rate-matching for TBoMS based on multiple RVs

- Options a and b with different RVs perform worse than option c

  - Inappropriate time-domain granularity for RV cycling can cause the problems of partial systematic bits being transmitted and duplicate transmission of encoded bits.

  - Multiple RVs for TBoMS may cause unexpected code rates.

- TBoMS is transmitted using a single RV.

In RAN1#104bis-e and RAN1#105e, another approach of multi-RV TBoMS was proposed, which sees TBoMS as a Type A PUSCH repetition enhancement. This approach has RV cycling across TOTs and NRE for TBS determination equal to the total number of REs available in a single TOT of a TBoMS. For example, in Option b in Figure 3, the TBoMS over four UL slots is equivalent to two repetitions of a two-slot TB, with different RV used for each repetition. This approach has lower code rate than when TBS is determined according to resources in all slots.

According to [4], a variant of the above approach is that bit interleaving is performed per slot, while encoded bits are still continuously selected for the multiple slots of a TOT. As illustrated as Option a2 in Figure 3, Option a2 and b allow the UE to transmit the same encoded bits in a TOT. The difference is whether the interleaving is performed per TOT or per slot. This can also be considered as an enhancement to Type A PUSCH repetition.

For TBoMS based on single RV, the note “The single RV is not constrained to have only the same coded bits in each slot or in each TOT” prevents the TBoMS of single RV from becoming identical repetitions of slot/TOT. The same principle can also be considered for TBoMS with multiple RVs.

- If a TBoMS consists of more than one RV, treating TBoMS as a Type A PUSCH repetition enhancement requires repetition within a TBoMS, which needs further study.

PUSCH repetition improves coverage thanks to a lower code rate by allocating more time domain resources. TB transmission over multiple slots utilizes high power spectrum density by allocating small number of PRBs per slot and spreading resource allocation in time domain. TBoMS without repetition can achieve similar performance as TBoMS with repetition by enlarging the number of slots for a TB or HARQ based retransmission of TBoMS. PDCCH overhead is relatively low for TBoMS compared with one-slot TB. Therefore, TBoMS has similar benefits to repetition, and the need for specifying both TBoMS and repetition should be established before specifying repetition on top of TBoMS.

- The need for repetition of TBoMS is further considered.

- From an interleaving and RV perspective, TBoMS is designed as a new feature, rather than a Type A PUSCH repetitions enhancement.

### TBoMS based on single RV

All the three options on time unit of rate-matching can apply to TBoMS based on a single RV. Options a and b define rate-matching inside a time unit, i.e. a slot or TOT, but don’t restrict how bit selection is done across the time units. Transmission of the same coded bits in each slot or TOT has been excluded with the note “The single RV is not constrained to have only the same coded bits in each slot or in each TOT.” Therefore, for Option a/b with TBoMS based on single RV, we only need to consider where different coded bits are selected across slots/TOTs. Since Option c has one rate matching with continuous coded bits selected across all slots/TOTs for TBoMS, which guarantees systematic bits can be transmitted as much as possible, it is reasonable that continuous bit selection is performed across slots for Option a and across TOTs for Option b. Then the only difference between the three options is the time unit for interleaving. These are illustrated in Figure 4 for TDD configuration of DDSUU and a TBoMS over four slots.

Figure 4, different time units of rate-matching for TBoMS based on single RV

- For TBoMS based on single RV and Option a, b and c, continuous coded bits are selected for all slots/TOTs of TBoMS.

Now we compare the three options in terms of 1) standard impact, 2) TOT size, 3) CB segmentation and 4) cancellation of transmission in a slot.

1) In Rel-15 and Rel-16, one rate matching is performed for a TB if there is no CB segmentation. Otherwise, each CB is rate matched once and the rate matching output sequence lengths of the multiple CBs are equal. Let’s first assume that TBS of a TBoMS is small so that CB segmentation doesn’t happen. The legacy rate matching performed per TB applies to Option c with one rate matching performed across all slots for a TBoMS. But for option a and option b, a TBoMS may be rate matched multiple times if the transmission of a TBoMS spans multiple slots for option a, or multiple TOTs for option b. The legacy method of determination of rate matching output sequence length can’t be used directly for multiple times of rate matching for a TB.

- Option a and Option b have larger standard impact than Option c on determination of rate matching output sequence length.

2) Another problem of option b is how to do rate matching with different TOT sizes, namely the number of UL slots in a TOT, in the transmission of a TBoMS. This problem exists no matter if single RV or multiple RVs are used for a TBoMS. The problem happens in TDD UL/DL configuration with different numbers of consecutive UL slot(s), e.g. DDDSUDDSUU. It was agreed in RAN1#105e that “Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.” For a TBoMS configured with at least three UL slots, the first TOT is one UL slot, while the second TOT spans two consecutive UL slots. With Option b, the different TOT sizes in a TBoMS result in the higher UE complexity of supporting different output lengths of bit selection and interleavers of different sizes. Of course, to avoid dealing with different sizes of TOT, option b can work with a restriction that all TOTs of a TBoMS must have the same size. But this leaves the orphan UL slot unused and is against the principle of improving the usage of UL resources.

Even with the same TOT size in the TDD UL/DL configuration, the problem still happens when the number of slots for a TBoMS can’t be divided by the TOT size. For example, with DDSUU, a TBoMS configured with three slots will be transmitted in the first TOT of two slots and the second TOT of one slot.

- Rate matching with different TOT sizes in the transmission of a TBoMS needs to be considered for option b.

3) In Rel-15 and Rel-16, CB segmentation happens when 1) Ninfo>3824 and R≤1/4, or 2) the quantized intermediate number of information bits  and R>1/4. The number of CBs for a TB is implicitly determined by TBS. CB segmentation is necessary because LDPC’s Base Graph 1 and Base Graph 2 can process a maximum of 8448 and 3824 information bits respectively. The upper limit of the two LDPC base graphs still holds for TBoMS, and therefore CB segmentation is needed for TBoMS in order to reuse LDPC coding.

If TBoMS is used for VoIP or low data rate like 100kbps, the TBS would not reach the threshold for CB segmentation. But in other cases, like 1Mbps data rate or DL heavy TDD configuration, TBS exceeding CB segmentation threshold may happen. To reuse current specification and reduce standardization effort, CB segmentation can be considered for TBoMS.

- CB segmentation is needed for TBoMS in order to reuse Rel-15/16 LDPC coding.

A common problem to Option a and b is how to do rate matching if the number of slots for a TBoMS can’t be divided by the number of CBs. For example, a TBoMS over five slots is segmented into two CBs. As illustrated in Figure 5, with option a, when the time unit of rate matching is a slot, each CB is rate matched three times. With option b, when the time unit of rate matching is a two-slot long TOT, each CB is rate matched two times. In this example, option a and option b respectively use 6 and 8 slots for actual transmission.

Figure 5, the actual TBoMS transmission, if the number of slots for a TBoMS can’t be divided evenly by the number of CBs

- If the number of slots for a TBoMS is not divided evenly by the number of CBs, rate-matching according to time unit of a slot or TOT may lead to the actual transmission of the TBoMS in more slots than configured.

4) Collision handling is another factor to consider when determining the time unit of rate matching. In NR Rel-15 and Rel-16, UE may drop PUSCH repetition in a slot if collision happens, caused by e.g. Cancellation Indication (CI), intra-UE higher priority transmission, or transmission of PUCCH repetitions. If the omission of transmission in the overlapping slot is reused for TBoMS, the larger the interleaving time unit is, the more robust the TBoMS is to the omission of transmission in one of its multiple slots. Simulation results for omission of transmissions in a slot can be found in section 2.6.2.

- A larger interleaving time unit makes the TBoMS more robust to the omission of transmission in one of the multiple slots.

In a summary, compared with option c, option a and option b have larger standard impact to support multiple times of rate matching per TB, may lead to different number of slots used for the actual transmission of the TBoMS than the configured number of slots, and suffer more from the cancellation of transmission in a slot. In addition, option b has to deal with the issue of rate matching over different TOT sizes for a TBoMS.

- Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS, if CB segmentation doesn't happen. Otherwise every CB is rate matched once.

## UCI multiplexing in TBoMS

When UCI transmission overlaps in time with PUSCH, UCI is multiplexed on PUSCH if the timeline check passes. In Rel-17, the transmission of HARQ-ACK or CSI report may overlap with a multi-slot TB. In the past RAN1 meetings, it was proposed that UCI can be multiplexed in one or more slots of a TBoMS and some issues were also raised.

Let’s consider UCI multiplexing in one slot first. Similar to UCI multiplexing in the first nominal repetition of PUSCH repetition Type B, if UCI is scheduled to be transmitted in other slots than the first one of the TBoMS, and the time unit of rate matching is larger than a slot, the complexity of PUSCH rate matching, e.g. bit interleaving, increases. However, in such case puncturing UCI into one of the multiple slots of a TBoMS doesn’t increase in UE complexity of rate matching.

If UCI can be multiplexed in multiple slots of a TBoMS, one method is that the total UCI bits are spread over multiple slots, occupying less resources in a slot than legacy UCI multiplexed in PUSCH of one slot. The other method is repetitions of UCI in multiple slots. This can improve UCI coverage and can be considered especially when there is no UL-SCH for multi-slot TB. It needs study on how UCI multiplexing in multiple slots can work for TBoMS with different number of UL symbols in each slot, namely the optimization for special slot, for example, the number of modulated UCI symbols varies in each slot.

- If UCI is multiplexed in one slot of a TBoMS by rate matching, and the time unit of rate matching is larger than a slot, UCI multiplexing by rate matching around PUSCH in slots other than the first one of the TBoMS increases UE complexity.

- If UCI is multiplexed in one slot of a TBoMS by puncturing, it doesn’t increase UE complexity of rate matching.

- It needs study on how UCI multiplexing in multiple slots can work for TBoMS with different number of UL symbols in UL slot and special slot.

- If UCI multiplexing in TBoMS is supported, HARQ-ACK can be multiplexed in any overlapping slot by puncturing, and CSI or HARQ-ACK can be repeated in all slots of a TBoMS.

## TBS determination

In RAN1#105e, number of slots allocated for TBoMS, xOverhead and NInfo for TBS determination were agreed with the following agreements. But the definition of K was not decided.

Agreement: Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC. - FFS: details. Working assumption: Agreement: For TBS determination of TBoMS: - NohPRB is configured by xOverhead and represents the overhead per slot. - NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated. Note: xOverhead configuration is as per Rel-15/16. Agreement: The following approach is used to calculate NInfo for TBoMS: - Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1. - FFS: the definition of K. L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA FFS: impacts and further details if repetitions of TBoMS is supported. FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Rel-15/16 PUSCH repetition allows multi-slot transmission of a single transport block, including where different redundancy versions of the PUSCH are transmitted in different slots. For TB over multiple slots, the multi-slot transport block transmission differs from Rel-15/16 PUSCH transmission in that the transport block size is determined based on multiple slots, whereas Rel-15/16 repetition’s TB size is calculated assuming the entire transport block is carried in each slot.

Option 1 for the definition of K in the last agreement above is that K equals the number of slots in a TBoMS. This applies to TBoMS based on single or multiple RVs. While in option 2, to determine K is to set it equal to the number of slots in a TOT. This option targets TBoMS based on multiple RVs, which update at TOT borders. Some problems about option 2 need to be considered first. Can TOTs of different sizes be allowed in a TBoMS? How is K configured, by explicit configuration in addition to the number of slots for a TBoMS or implicit determined according to TDD UL/DL configuration?

Simulation results in section 2.6.1.2 show the performance of the two options of K for TBoMS based on multiple RVs with time unit of rate matching as TOT. The TBS with the two options of K are the same. Option 1: K = the number of slots of TBoMS outperforms option 2: K = the number of TOTs, because a larger NInfo allows a lower MCS index, which improves the performance (at least with the parameters simulated).

- NInfo for TBoMS should be based on the number of REs across all slots of the TBoMS, no matter if the TBoMS is based on single RV or multiple RVs. Namely, K= the number of slots for the TBoMS.

### Non-consecutive physical slots

It was agreed in RAN1#104bis-e that non-consecutive physical slots for UL transmission can be used to transmit TBoMS at least for unpaired spectrum, but it is FFS for paired spectrum. Support of non-consecutive physical slots for transmission of TBoMS in FDD can enable flexible UL scheduling, for example by scheduling SRS or other UL transmission from the same UE or other UEs between the multiple slots for TBoMS. It doesn’t need more standardization efforts than TDD to support non-consecutive physical slots for FDD. In order to keep alignment between TDD and FDD and allow for flexible UL scheduling, we have the proposal below.

- Non-consecutive physical slots can be supported for TBoMS for paired spectrum.

### Handling of unavailable slots

In addition to Rel-15 PUSCH repetition based on physical slots, Rel-17 will support enhanced PUSCH repetition on the basis of available slots. The two approaches can be referred to when considering TBoMS.

In NR operation of Rel-15 Type A PUSCH repetition, there are cases where a UE should not transmit in a slot. For example, Rel-15 UEs drop the PUSCH repetition in the DL slot configured by TDD UL/DL configuration. Likewise, if the number of physical slots for a TBoMS is configured, TBS determination of TBoMS can be based on TDD UL/DL configuration. Configuring the number of physical slots for a TBoMS has the advantage that the total time span of the TBoMS is fixed, but the TBS is not explicitly indicated, which depends on TDD configuration. The allowed maximum number of physical slots for TBoMS needs to take TDD configuration into consideration.

- Unless the number of slots for TBoMS is highly restricted (e.g. to at most 2), a large number of physical slots is needed (e.g. 16 for 4-slot TBoMS with 4:1 DL:UL ratio), if the number of physical slots is configured for TBoMS.

Regarding configuring the number of available slots for a TBoMS, the advantages are the configured value can be much smaller than the one based on physical slots and TBS is directly indicated by the number of available slots. The UE needs to determine the physical slots mapping to available slots prior to UL transmission.

- If TBoMS with more than 2 slots is to be supported, TBoMS configuration uses the number of available slots, otherwise physical slots are used.

- When the number of symbols in each slot is the same for TBoMS,

  - If the number of physical slots is configured, use TDD UL/DL configuration for TBS determination

  - If the number of available slots is configured, TBS determination is according to the number of available slots.

## Other issues

The following issues are discussed in this section.

- DMRS, MCS, number of layers

- Unit of retransmission

- Power control

Issues in this section intertwine with above-mentioned issues. To facilitate progress, we have the proposal:

- RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached.

### DMRS, MCS, and number of layers

It was agreed in RAN1#104e that the same number of PRBs per symbol is allocated across slots for TBoMS transmission. A similar decision is needed on whether the same DMRS configuration, MCS index, and number of layers is used across multiple slots of a TB. The need for any new DMRS configuration, should be discussed in the Agenda 8.8.1.3 for cross-slot channel estimation in the NR Rel-17 Coverage enhancement WI rather than the RAN1 TBoMS session to avoid duplicate discussion.

### Unit of retransmission

NR Rel-15/16 supports both CBG-based and TB-based retransmission. CBG-based or TB-based transmission is configured by RRC. In NR Rel-15/16, one or multiple CBs can form a CBG, if code-block-group (CBG) based transmission is enabled by RRC. CBG is the unit of retransmission in order to avoid sending the successfully decoded CBGs. The number of CBGs for a TB is no larger than the number of CBs determined by TBS, so that each CBG contains at least one CB.

As mentioned in section 2.2.1, CB segmentation is needed for TBoMS in order to reuse Rel-15/16 LDPC coding. As a coverage enhancement scheme, the data rate for a cell edge UE is smaller than that for cell center UE. Accordingly, TBS and number of CBs in the TB are also smaller. In NR operation, the number of CBGs in the TB can be no larger than number of CBs in the TB. With a small number of CBs/CBGs, the benefit of reduced payload by CBG-based retransmission decreases for TBoMS.

It was proposed that slot-based retransmission can be considered for TBoMS, namely UE can be indicated with the index of one or multiple slots of the TBoMS for retransmission. This is similar to CBG-based transmission, where the UE is indicated which specific CBG is to be retransmitted. However, for TBoMS, one CBG may not fit exactly into one or multiple slots. As illustrated in section 2.2.1, one of the multiple CBs for a TBoMS may not fit into the integer number of slots, so is the case for a CBG which contains one or multiple CBs. Therefore slot-based retransmission is different from and can’t reuse directly CBG-based transmission. The benefit and standard impact of slot-based retransmission for TBoMS needs study.

### Power control

In Rel-15/16, the term “transmission occasion” is used in different aspects independently, e.g. RV cycling and UL transmission power determination. It is the same for the two aspects for one-slot TB, namely the scheduled UL symbols in a slot, but it can have different lengths in the two aspects for TBoMS.

In NR Rel-15/16, UE determines PUSCH transmission power for each slot. A decision is needed on if transmission occasion of power determination for a TBoMS is one or all slots of a TB. If the transmission occasion is to reuse the allocated symbols in a slot, it is backward compatible and needs less standardization effort. One issue is how to calculate BPRE in each slot. If Type-B like TBoMS is supported, it is complex for UE to calculate the number of information bits in each slot, except for an average BPRE calculated across multiple slots. If the transmission occasion for TBoMS power determination is all slots of the TB, a certain transmission power is used across the multi-slot transmission. Possible issues include how to handle the updated pathloss estimation and group-common TPC command amid UL transmission.

## Performance evaluation of TB processing over multi-slot PUSCH

In NR Rel-15 and 16, transmission of a TB is determined by RE resources within a number of PRBs and a number of at most 14 OFDM symbols. To reach a certain UL data rate, usually multiple PRBs in a slot are allocated for a TB transmission. However, increasing resources in frequency domain will reduce the power density of the signals transmitted in each PRB, thus making the channel estimation accuracy worse, given the limitation of the total power of UE can have. In order to increase power spectrum density, one possible solution is to extend a TB that is in a small number of PRBs across multiple slots.

- The feasible use cases of TB over multiple slots are the low data rate services, such as VoIP and low rate data.

### Performance of TBoMS with single or multiple RVs

In this section, we compare the simulation results of different RV schemes, including one RV continuous rate matching across all slots of the TBoMS, multiple RVs cycling at the slot border or TOT border for TBoMS in TDD and FDD mode. Other simulation parameters can be found in section 5.

#### FDD mode

Figure 6-a shows the performance of TBoMS over four UL slots. The scheme with one RV continuous rate matching has about 0.5dB performance gain over multiple RVs cycling across slots. In both schemes, TBS is determined by all slots of the TBoMS, and the same MCS index is used. The reason for the performance gap is that part of the coded bits transmitted in slot 1 and slot 3 are overlapping in the multi-RV scheme, as shown in Figure 6-b.

Figure 6-a, one RV vs. RV cycling for a TBoMS of 4 slots

Figure 6-b, rate matching of coded bits for different RV schemes

#### TDD mode

We compare the results of TBoMS of eight UL slots in TDD mode with four different schemes. In addition to the two schemes simulated in FDD, time unit of rate matching as TOT, which consists of two UL slots, is considered for TBoMS with multiple RVs, together with the two options of K to determine NInfo. The TBS of the four schemes is exactly the same. In each UL slot, 2 PRBs are allocated. Figure 7-a shows the performance of four schemes, with details listed in Table 2-1.

Table 2-1: The procedures of four schemes

|  | NInfo is determined by | RV | Interleaving |
| --- | --- | --- | --- |
| Scheme 1 | 8 UL slots | One RV (RV 0) | all TBoMS |
| Scheme 2 | 8 UL slots | RVs cycling per TOT | per TOT |
| Scheme 3 | 2 UL slots(One TOT) | RVs cycling per TOT | per TOT |
| Scheme 4 | 8 UL slots | RVs cycling per slot | per slot |

From Fig 7-a, it can be observed that one RV with continuous rate matching has the best performance. In Scheme 2, part of the coded bits in TOT 1 and TOT 3 are overlapping, as is illustrated in Fig 7-b, leading to 0.5 dB loss compared with Scheme 1.

To align the TB size of Scheme 3 with other schemes, Scheme 3 uses an MCS index with higher target code rate and higher modulation order. 16QAM is used in Scheme 3, rather than QPSK as in other schemes,  which causes performance loss.

For Scheme 4, the number of slots for a TBoMS exceeds the number of RVs, therefore, the latter four slots are exactly the same as the former four slots. With TBS determined by eight slots, only half of systematic bits are transmitted in slot 1 and slot 5 with RV0, as illustrated in Figure 7-b. This leads to decoding failure with BLER equal to 1.

Figure 7-a, one RV vs. multiple RVs for a TBoMS of 8 UL slots

Figure 7-b, bit selection of coded bits for different RV schemes

- For TBoMS based on multiple RVs, calculating NInfo based on all slots of TBoMS outperforms that based on the size of rate matching time unit, due to lower MCS index. However, it is at the risk of decoding failure if the time unit of rate matching is much smaller than the number of slots for the TBoMS.

### Performance of dropping a slot for TBoMS with different time units of interleaving

In this section, we compare the performance of different time units of interleaving, including slot, TOT and TBoMS, when transmission in one slot is cancelled. The TBoMS is based on one RV with continuous bit selection across all slots. It contains 4 UL slots and 2 PRB in one slot. MCS 3 is used. Then in each UL slot, 576 coded bits can be transmitted. Dynamic signaling, such as a Cancelation Indicator, indicates the transmission in the first slot is to be dropped. The results of different time units of interleaving are shown in Figure 8.

Figure 8, transmission in the first slot of the TBoMS is dropped.

After the LDPC coding and rate matching, if the interleaving time unit is per TBoMS or per TOT, only half of all systematic bits are lost. However, all systematic bits are dropped in the case of interleaving per slot, which has worst performance. Different parity bits are transmitted in the cases of interleaving per slot and interleaving per TOT, which explains the 1 dB gain of interleaving per TBoMS against interleaving per TOT.

- For TBoMS with a single RV, when coded bits are continuously selected for all slots, the larger time unit of interleaving is more robust and provides better time diversity against cancellation transmission in a slot.

# Summary

In this contribution, we considered time domain resource allocation, RV determination, rate matching, UCI multiplexing, TBS determination and other issues.

Based on the discussion above, we have following proposals.

Proposals:

- Reuse resource determination and signaling of Rel-15/16 PUSCH repetition as much as possible to avoid specifying duplicate functionality.

- The net gains and use cases of TBoMS supporting special slot with different number of UL symbols than that in UL slot for the TB should be carefully studied prior to specifying it.

  - Such study should address how SRS and PUCCH can be transmitted as well as the performance of interference suppression when DMRS in a special or normal uplink slot is used for interference suppression in the other type of slot.

  - If specified, and performance gains are targeted for this case, a TB over consecutive UL symbols in special slot and the following UL slot can be based on PUSCH repetition type-B like TDRA.

- UL symbols in special slot in TDD are not included for time domain resource determination for TBoMS according to the agreed Type A like time domain resource determination for TBoMS.

- TBoMS is transmitted using a single RV.

- The need for repetition of TBoMS is further considered.

- From an interleaving and RV perspective, TBoMS is designed as a new feature, rather than a Type A PUSCH repetitions enhancement.

- For TBoMS based on single RV and Option a, b and c, continuous coded bits are selected for all slots/TOTs of TBoMS.

- CB segmentation is needed for TBoMS in order to reuse Rel-15/16 LDPC coding.

- Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS, if CB segmentation doesn't happen. Otherwise every CB is rate matched once.

- If UCI multiplexing in TBoMS is supported, HARQ-ACK can be multiplexed in any overlapping slot by puncturing, and CSI or HARQ-ACK can be repeated in all slots of a TBoMS.

- NInfo for TBoMS should be based on the number of REs across all slots of the TBoMS, no matter if the TBoMS is based on single RV or multiple RVs. Namely, K= the number of slots for the TBoMS.

- Non-consecutive physical slots can be supported for TBoMS for paired spectrum.

- If TBoMS with more than 2 slots is to be supported, TBoMS configuration uses the number of available slots, otherwise physical slots are used.

- When the number of symbols in each slot is the same for TBoMS,

  - If the number of physical slots is configured, use TDD UL/DL configuration for TBS determination

  - If the number of available slots is configured, TBS determination is according to the number of available slots.

- RAN1 is to discuss issues of DMRS, MCS, number of layers, unit of retransmission and power control after agreements of time unit for rate matching are reached.

# References

RP-211566, “Revised WID on NR coverage enhancements”, China Telecom, 3GPP TSG RAN meeting #92e, June 14th – 18th, 2021.

R1-2107561, "Joint Channel Estimation for PUSCH", Ericsson, 3GPP TSG RAN WG1 #106-e, August 16th – August 27th, 2021

R1-2107564, "Other Coverage Enhancements for PUSCH", Ericsson, 3GPP TSG RAN WG1 #106-e, August 16th – August 27th, 2021

R1-2104686, “TB Processing over multi-slot PUSCH”, Qualcomm, 3GPP TSG RAN WG1 #105-e, May 19th – May 27th, 2021

# Appendix

Table 1: Basic setup for TBoMS

| System | - Carrier frequency 700MHz/4GHz - 15 kHz SCS/30 kHz SCS - FDD/TDD - Waveform: DFT-s-OFDM - TDD pattern: DDSUU |
| --- | --- |
| UE speed | - 3 km/h |
| MCS table | - Table 1 for PUSCH with transform precoding (q=2) |
| DMRS configuration | - Type 1, 2 DMRS symbols |
| PUSCH duration | - 14 symbols |
| PRB num | - 2 PRBs in one slot |
| Channel | - TDL-C (NLoS), 300ns delay spread, medium correlation |
| Antennas | - 1T2R for FDD/1T4R for TDD |
| Function | - Disable HARQ and Link adaptation |
| Time domain Resource allocation | - TBoMS: Type-A like TDRA |