3GPP TSG RAN WG1 #106-e                                                              	       R1-2107571

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.1

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH

Document for: 	Discussion/Decision

# Introduction

According to NR MIMO WID RP-193133, the following is the scope of work in this AI.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

In this contribution we provide our views on the features that should be considered to improve reliability for PUSCH and PUCCH channels using multi-TRP/multi-panel framework.

# PDCCH

## Overlapping candidates

Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, for the purpose of BD counting and interpretation of a detected DCI, select one option among the following in RAN1#105-e: - Option 1: The individual candidate is not counted for monitoring - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Option 2: The candidate in a higher SS set ID is not counted for monitoring - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - Option 3: The candidate associated with SS set(s) with lower priority is not counted for monitoring, where for two linked SS sets, the priority is according to one of the two SS sets with a lower SS set ID - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - FFS: Whether a max limit on number of such overlaps is needed. Additional specification support may be introduced for the purpose of resolving ambiguity (if any) for interpretation of the detected DCI. For example, - Distinguished by different RNTIs defined for the linked candidate versus the individual candidate - Distinguished by aggregation level restrictions that can be expected by the UE in the case of overlap

Figure 1: If a UE always prioritizes a linked candidate (option 1), then PDSCH scheduled via individual PDCCH candidate may be limited by unnecessary latency that gNB has to consider

A reason to allow overlap of individual and linked candidate (from two different SS sets) is because individual and linked candidates are not expected to be in the same SS set that inherently provides BD/CCE counting benefits due to overlap. Therefore restriction on such overlap is detrimental to PDCCH scheduling as it reduces available BD at the UE.

Rel-17 design of not allowing individual and linked candidates in the same SS set relies on allowing such overlap. This is the reason it was clarified in a note and agreed in RAN1#104.

Agreement#104 When two SS sets are linked for PDCCH repetition, they do not contain individual PDCCH candidates. - Note 1: For configuration of individual PDCCH candidates, a different SS set can be configured by network. - Note 2: When one of the linked PDCCH candidates uses the same set of CCEs as an individual PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, Rel. 15 rule is followed wrt not counting an additional BD.

Also, based on channel conditions at the UE, a gNB may find that it is beneficial to use an individual candidate and not a linked candidate for a particular PDCCH transmission (which can also benefit from latency) as shown above. If a UE always prioritizes a linked candidate then there could be unnecessary latency penalty.

Proposal-1: Support Rel-15 mechanism of prioritizing a candidate based on SS set-ID in case of candidate overlap with the same set of CCEs, associated with the same DCI size, scrambling, and CORESET (options 2 or 3). It is not desirable to place restrictions on the number of such allowed overlaps.

## Candidate dropping

Agreement For PDCCH repetition with two linked candidates, if due to Rel. 15/16 procedures, one of the linked candidates is not monitored (is dropped), select one option from Options 1 and 2 in RAN1#105-e: - Option 1: UE still monitors the linked candidate that is not dropped and interprets the DCI based on Rel. 17 PDCCH rules (wrt reference PDCCH candidate) - QC - Option 2: Even the candidate that is not dropped is not monitored (Both linked candidates are dropped if at least one of them is dropped) - FFS: Which of the following Rel. 15/16 rules are applicable for this purpose: - Case 1: Overlap with SSB - Case 2: Overlap with rate matching resources: RateMatchPattern, lte-CRS-ToMatchAround, or LTE-CRS-PatternList-r16, availableRB-SetPerCell-r16 - Case 3: Due to TDD DL/UL related conflicts: Overlap with semi-static / dynamic UL symbols or overlap with PRACH - Case 4: QCL-TypeD prioritization rule among CORESETs result in one of the linked candidates not being monitored - Case 5: Overbooking results in one of the linked candidates not being monitored - Case 6: Overlap with reserved PRB(s) and OFDM symbol(s) indicated by DCI format 2_1 where UE may assume no transmission intended for the UE - Other cases are not precluded - FFS: Whether there is an impact to BD count

Rel-17 PDCCH repetition is based on the premise that individual and linked candidates will not be present in the same SS-set. gNB is expected to transmit both linked PDCCH candidates. Therefore there is no motivation to specify UE behavior where UE monitors a linked candidate and drops the other one. Option 2 is simple to specify and sufficent. A side benefit of option 2 is also that it means the options 2/3 in the previous agreement is equivalent. This option also has the least specification impact.

Proposal-2: If one of the linked candidates is not monitored (is dropped), both linked candidates are dropped.

## BD count

Agreement (Issue#2) For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support - UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates - Candidate values: 2, 3. - FFS: Default behaviour - FFS: Whether one of the candidate values imply that UE supports soft combining - FFS: Whether additional candidate values are supported (e.g. non-integer numbers) - FFS: RRC configuration based on reported UE capability

The simplest solution is to specify a single BD number for the two PDCCH candidates that are linked. Given that Rel-17 design does not allow the gNB to mix linked and individual candidates in the same SS set, we don’t see the motivation for the UE to report whether or not it supports soft combining.

Proposal-3: For the number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, better to support one number (and the same one number for a given UE). No need for UE reporting of soft-combining capability.

## Single CORESET repetition

Agreement When DL DCI is transmitted via PDCCH repetition, for PUCCH resource determination for HARQ-Ack when the corresponding PUCCH resource set has a size larger than eight, starting CCE index and number of CCEs in the CORESET of one of the linked PDCCH candidates is applied, and option 2 is supported - Option 2: The one with the lowest SS set ID is applied. - FFS: Support of Option 2 does not mean PDCCH repetition based on two linked search space set within one CORESET is supported

We believe single-TRP PDCCH repetition should be supported by reusing the mTRP repetition framework. This can be beneficial to improve PDCCH reliability beyond AL16 while re-using the hardware in UEs supporting multi-TRP PDCCH repetition. The specifications can naturally support 2 linked SS sets from the same CORESET. There is no need to add restrictions.

Proposal-4: There is no need to add restrictions to disallow single TRP PDCCH repetitions

## Type B time reference

Agreement If a PDSCH with mapping Type B is scheduled by a DCI in PDCCH candidates that are linked for repetition - For the purpose of the earliest time that the PDSCH can be scheduled as well as for the purpose of the reference symbol for SLIV (when UE is configured with ReferenceofSLIV-ForDCIFormat1_2, and when receiving the PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0), a reference candidate is used. Select one among the following: - Alt1: The candidate that starts later in time - Alt3: The candidate that starts earlier in time - FFS: How to define d1,1 for PDSCH processing time in this case

According to Rel-15, a UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the scheduling PDCCH is received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation. In order to maintain the same restriction in Rel-17 for both the scheduling PDCCHs (due to repetition) we support Alt-1 that uses the PDCCH candidate that starts later in time as the reference for Type B PDSCH.

Proposal-5: Support using the PDCCH candidate that starts later in time as a reference candidate for a linked PDCCH scheduling a Type B PDSCH

## QCL Type D for multi-panel UE

Agreement For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs - FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs - Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.

In order to support PDCCH repetition from two TRPs to be received at a multi-panel UE on the same symbol, it is beneficial to extend the PDCCH prioritization introduced in Rel-15 from a single beam to 2 beams. A straightforward way is to identify the first and the second priority CORESETs using Rel-15 prioritization rules as a means to select 2 CORESETs for PDCCH monitoring in case of time overlap. This, however, introduces another layer of complexity for gNB in terms of association between SS-set indices and CORESETs.

Proposal-6: For PDCCH repetition, support identifying first and second priority CORESETs using existing prioritization rules that are overlapping in the same symbol

Another question is whether the same solution should/could be used for multi-DCI mTRP operation. In our view, for mDCI operation, the Rel-15 PDCCH prioritization technique can be easily extended by specifying the prioritization operation within the set of CORESETs associated with the same value of CORESETPoolIndex.

Proposal-7: For multi-DCI mTRP operation, extend the Rel-15 PDCCH prioritization technique by specifying the prioritization operation within the set of CORESETs associated with the same value of CORESETPoolIndex

## Multi-DCI

It is not clear whether/how to support PDCCH repetition for multi-DCI based multi-TRP operation. We think for the case of multi-DCI multi-TRP, PDCCH repetition could be allowed within the same CORESETPoolIndex value (this avoids mixing of PDCCH repetition with out of order scheduling).

Proposal-8: For multi-DCI multi-TRP, PDCCH repetition could be allowed within the same CORESETPoolIndex value.

## Inter-slot repetition

In general, given per-slot BD/CCE limit at the UE and limited CCE resources at the gNB, intra-slot PDCCH repetitions adversely affect blocking probability as the fraction of UEs requiring mTRP repetition grows as shown in Figure 3.

If PDCCH repetitions in limited to intra-slot only, then a UE is blocked if any of the TRPs experiences PDCCH blocking (see Figure 1). If both intra-slot and inter-slot repetition is allowed, then a gNB can transmit rep-1 (from TRP-1) in slot-1 and rep-2 (from TRP-2) in slot-2 (or vice-versa) and these combinations are not possible in intra-slot only repetition (see Figure 2). This helps to reduce PDCCH blocking but of course it comes at a cost of increased scheduling latency (which may be acceptable in FR2 due to small slot-duration). The reduction of PDCCH blocking enabled by allowing such inter-slot repetition is shown in Figure 4. Note that for inter-slot PDCCH repetition, the required BD count is divided between 2 slots instead of 1 slot.

In the following simulations, we assumed a load of 10 UEs, the fraction of UEs receiving PDCCH repetition is a parameter, total number of CCEs = 16 from each TRP and total number of BD candidates at the UE to be 12 which is quite typical in FR2 (max is 20).

| Figure 2: Intra-slot repetition only | Figure 3: Intra + inter slot repetition |
| --- | --- |

| Figure 4: PDCCH blocking probability is increased due to PDCCH repetition | Figure 5: PDCCH blocking probability is reduced by inter-slot repetition (scheduling 2 slots together) |
| --- | --- |

Proposal-9: Support inter-slot mTRP PDCCH repetition that allows joint scheduling of PDCCH across multiple slots at the gNB to reduce blocking probability (co-existing with intra-slot repetition)

# PUSCH

## Power Headroom Reporting

Agreement (104b-e) For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting For further study in future meetings (105-e): For PHR reporting related to M-TRP PUSCH repetition, study following aspects related to option 4, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. - FFS1: How the PHRs are calculated for reporting (actual PHR or virtual PHR) - FFS2: How the PHRs are calculated for reporting for other CCs if the multi-cell PHR MAC CE is applied. - FFS3: Required changes to triggering conditions including the required higher layer parameters (e.g.,’phr-PeriodicTimer’, ‘phr-ProhibitTimer’, ‘phr-Tx-PowerFactorChange’ as TRP specific). - FFS4: Report P-MPR and MPE per TRP within the same MAC-CE extension. Note: Down-selection between Options 1-5 will be based on this study as well as the trade-off between benefit versus UE complexity.

According to TS38.321, PHR triggering timers are specific to a MAC entity and a PHR can be triggered if at least in one serving cell the pathloss has changed more than phr-Tx-PowerFactorChange dB. It is also noted that the path loss variation for one cell is assessed by the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last reporting of PHR on the pathloss reference in use at that time.

Option 2 and Option 4 as agreed above is suited for TRP-specific PHR reporting, however, the existing philosophy of PHR triggering, determination and reporting in RAN2 is cell specific and not TRP-specific. In view of the above, we think it is beneficial to inform RAN2.

Figure 6. Multiple entry PHR MAC CE (left) and Single entry PHR MAC CE (right)

TS38.213 introduces the concept of ‘virtual’ in PHR. It states that “if a UE is configured with a SCG and if phr-ModeOtherCG for a cell group (CG) indicates 'virtual' then, for power headroom reports transmitted on the CG, the UE computes PH assuming that the UE does not transmit PUSCH/PUCCH on any serving cell of the other CG”. Further, in TS 38.321, the ‘V’ field in multiply entry PHR MAC CE can be used to indicate if the PH value is based on a real transmission or a reference format, as shown in Fig. 1. In TS 38.331, it states “phr-ModeOtherCG  indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. If the UE is configured with only one cell group (no DC), it ignores the field.” For TRP specific PHR reporting, further discussion is needed regarding PHR calculation with multiple cell groups– specifically a decision on the assumption on the transmission status of the other cell group (reference format based or no transmission). However, the  case of single cell group/single-entry PHR reporting is quite simple and we can focus on this case..

For example, the UE is configured with only one CC in one cell group (CG), where single entry PHR MAC CE is used, as shown in Fig. 1 (right). PHR is triggered to be reported in multi-TRP PUSCH repetitions in CC1 (in CG1) in slot #n towards TRP1 and slot #n+1 towards TRP2, as shown in Fig. 2. According to Option-4 in the previous agreements, two PHRs (PHR1 and PHR2 corresponding to the PUSCH repetitions towards TRP1 and TRP2 respectively) should be included in both PUSCH repetitions in two single entry PHR MAC CE. Determination of PHR1 and PHR2 can be based on actual PUSCH transmission.

Figure 2. PHR reporting in a mTRP operation scenario

Proposal-10: For single entry PHR reporting related to M-TRP PUSCH repetition, use option-4 to compute the PHRs based on actual PUSCH transmission  Inform RAN2 of the decision.

Multi-entry PHR/multiple cell-group case:

Next, we would like to highlight an issue in the multiple entry/multi-CC PHR scenarios. For example, if the UE is configured with CC1 in CG1 and CC2 in CG2, PUSCH repetitions are scheduled to be transmitted in slot #n towards TRP1 and slot #n+1 towards TRP2 in CC1, and the PHR is triggered to be transmitted in slot #n in CC1 (as shown in Fig. 3), then RAN1 should decide what should be the PUSCH transmission assumption for slot #n+1 on CC2 in order to compute PHR for CC1 in slot #n.

Figure 3. PHR reporting in a mTRP operation scenario with multiple CCs

## Configured grant operation

Agreement(104be) For type 1 or type 2 CG based multi-TRP PUSCH repetition, - Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ - For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'. - For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI. - FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1 - FFS3: Details on RV mapping. - FFS4: Possible transmission occasion for initial transmission - FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'

In Rel-16, for a PUSCH corresponding to a CG Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value “0” in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [TS38.212]. The association between PTRS ports and DMRS ports are predefined due to semi static nature of RRC configuration. We think the same philosophy (default relation) can be used for CG PUSCH transmission towards both TRPs.

In terms of RV mapping, the same philosophy as agreed for DG can be reused. The configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs); with a possibility of configuring an offset for the starting redundancy version corresponding to the second TRP.

Proposal-11: For CG operation, the configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs) with a possibility of configuring an offset.

## Default P0, alpha, PL-RS, and closed loop index

Agreement For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP. Select one from the following in RAN1 #106-e meeting, - Alt.1 - The first P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the first SRS resource set. - The second P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the second SRS resource set. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.2 - The first set of values {the first value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.3 - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the first set of values {the first value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the first SRS resource set and closed-loop index l = 0} is used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControlassociated with the second SRS resource set and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise} is used for TRP2. - Otherwise, the first set of values {the first value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id=0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

According to the previous agreement, if the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes a SRI field, we have the following agreement.

Agreement For indicating per-TRP OLPC set in DCI format 0_1/0_2, if two SRI fields present in the DCI, - Use the existing field (1 bit) for OLPC set indication and a second p0-PUSCH-SetList-r16. - if value of the field equals to ‘0’, the UE determine value of P0 from SRI-PUSCH-PowerControl with a sri-PUSCH-PowerControlId value mapped to the SRI field value corresponding to each TRP. - if value of the field equals to ‘1’, the UE determine value of P0 from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value corresponding to each TRP. Note: the values can be different for different TRP.

Alt. 2 allows mTRP operation without configuring sri-PUSCH-PowerControl , which can be a more general solution. If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE,  Alt. 2 can be used.

Proposal-12: For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP, support Alt. 2.

## Open loop power control

Agreement For indicating per-TRP OLPC set in DCI format 0_1/0_2, if two SRI fields present in the DCI, - Use the existing field (1 bit) for OLPC set indication and a second p0-PUSCH-SetList-r16. - if value of the field equals to ‘0’, the UE determine value of P0 from SRI-PUSCH-PowerControl with a sri-PUSCH-PowerControlId value mapped to the SRI field value corresponding to each TRP. - if value of the field equals to ‘1’, the UE determine value of P0 from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value corresponding to each TRP.

In current specification for single-TRP based transmission, the open-loop power control (OLPC) parameter set indication field is 1 bit when SRI field is present in the DCI, and it can be either 1 or 2 bits when the SRI field is not present in the DCI. In particular, if the SRI field does not exist in the DCI, a 1-bit OLPC field can indicate two OLPC values, i.e., 1) a first ‘P0-PUSCH-AlphaSet’ in ‘p0-AlphaSets’, and 2) a first value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’value, a 2-bit OLPC field can indicate three OLPC values, i.e., 1) a first ‘P0-PUSCH-AlphaSet’ in ‘p0-AlphaSets’, 2) a first value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’value, and 3) a second value in ‘P0-PUSCH-Set-r16’ with the lowest ‘p0-PUSCH-SetID’ value.

In last 105e meeting, the mTRP enhancement of OLPC field design for the case that two SRI fields present in the DCI has been made. We can use a similar approach for the case where there is no SRI field presenting in the DCI. Particularly, we prefer to use the existing 2-bit field for OLPC parameter indication.

Proposal-13: For indicating per-TRP OLPC set in DCI format 0_1/0_2, if no SRI field presents in the DCI,

- Use the existing field (1 or 2 bits) for OLPC set indication and a second p0-PUSCH-SetList-r16.

  - the field is 1-bit if olpc-ParameterSetForDCI-Format0-1 indicates integer 1

  - the field is 2-bit if olpc-ParameterSetForDCI-Format0-1 indicates integer 2

  - If P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 are provided to the UE and the DCI format includes an open-loop power control parameter set indication field

    - if value of the field equals to ‘0’ or ‘00’, this would depend on the choice of ‘default P0, alpha, PL-RS, and closed loop index is defined per TRP’, i.e., Alt1, Alt2, and Alt3.

      - If Alt 2 is supported (see Proposal-3), then the UE determine value of P0 from a first and second P0-PUSCH-AlphaSet in p0-AlphaSets corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘1’ or ‘01’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘10’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘11’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

  - else, the UE determine value of P0 from a first P0-PUSCH-AlphaSet for both TRP1 and TRP2.

## Dynamic switching field for CB/NCB

Agreement For the new field in the DCI for dynamic switching, support Alt.1 (modified). Alt.1 - Support 2 bits with the following combinations. Codepoint; SRS resource set(s); SRI (for both CB and NCB)/TPMI (CB only) field(s). 00; s-TRP mode with 1st SRS resource set (TRP1); 1st SRI/TPMI field (2nd field is unused). 01; s-TRP mode with 2nd SRS resource set (TRP2); 1st SRI/TPMI field (2nd field is unused). 10; m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set; Both 1st and 2nd SRI/TPMI fields. 11; m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: FFS 2nd SRI/TPMI field: FFS; Both 1st and 2nd SRI/TPMI fields. - The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set. - For codebook and non-codebook usage, respectively

In 105e meeting, a 2-bit field in DCI is agreed for single-TRP/multi-TRP dynamic switching. For the codepoint ‘11’, we prefer to associate the 1st and the 2nd SRI/TPMI field with the 1st and the 2nd SRS resource set, respectively.

Proposal-14: For the codepoint ‘11’ of the single-TRP/multi-TRP dynamic switching field, associate the 1st and the 2nd SRI/TPMI field with the 1st and the 2nd SRS resource set, respectively.

## PT-RS DMRS association

Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

In current specification, the PTRS-DMRS association is according to Table 7.3.1.1.2-25 and Table 7.3.1.1.2-26 for one and two PTRS ports, respectively with a maximum of 2 bits DCI field size. We slightly prefer that for MaxRank > 2, the PTRS-DMRS association field size can be kept at 2 bits while reducing the resolution of PTRS-DMRS indication with MSB and LSB associated to the 2 TRPs.

Proposal-15:  For UL PTRS port 0 indication (1 PTRS port) and for port 0 and port 1 indication (2 PTRS ports) only values of 0, 1 can be indicated per TRP (reduced resolution) with MSB and LSB associated with the 2 TRPs respectively.

## SP-CSI

Agreement For SP-CSI report on mTRP PUSCH repetition Type A and B activated by a DCI, further study the use of a similar mechanism to A-CSI multiplexing on M-TRP PUSCH without a TB, which includes the following, - When SP-CSI multiplexed on m-TRP PUSCH, SP-CSI multiplexed on the two repetitions associated with the two TRPs, and the number of repetitions is always assumed to be 2, regardless of the value indicated. - Reuse similar conditions (e.g. UCIs other than the A-CSI are not multiplexed, same number for first actual repetitions, the content of the CSI is the same) to support SP-CSI multiplexing on m-TRP PUSCH as defined in A-CSI multiplexing on M-TRP PUSCH.

In the current specification when activation DCI is present, SP-CSI is transmitted on the first actual repetition which is expected to be the same length as a nominal repetition. We propose to use the same principle as PUSCH multiplexing case such that SP-CSI is transmitted in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition.

Proposal-16: For the case of multi- TRP PUSCH type B repetition with no scheduled data, support SP-CSI transmission in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition.

# Conclusions

PDCCH

Proposal-1: Support Rel-15 mechanism of prioritizing a candidate based on SS set-ID in case of candidate overlap with the same set of CCEs, associated with the same DCI size, scrambling, and CORESET (options 2 or 3). It is not desirable to place restrictions on the number of such allowed overlaps.

Proposal-2: If one of the linked candidates is not monitored (is dropped), both linked candidates are dropped.

Proposal-3: For the number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, better to support one number (and the same one number for a given UE). No need for UE reporting of soft-combining capability.

Proposal-4: There is no need to add restrictions to disallow single TRP PDCCH repetitions

Proposal-5: Support using the PDCCH candidate that starts later in time as a reference candidate for a linked PDCCH scheduling a Type B PDSCH

Proposal-6: For PDCCH repetition, support identifying first and second priority CORESETs using existing prioritization rules that are overlapping in the same symbol

Proposal-7: For multi-DCI mTRP operation, extend the Rel-15 PDCCH prioritization technique by specifying the prioritization operation within the set of CORESETs associated with the same value of CORESETPoolIndex

Proposal-8: For multi-DCI multi-TRP, PDCCH repetition could be allowed within the same CORESETPoolIndex value.

Proposal-9: Support inter-slot mTRP PDCCH repetition that allows joint scheduling of PDCCH across multiple slots at the gNB to reduce blocking probability (co-existing with intra-slot repetition)

PUSCH

Proposal-10: For single entry PHR reporting related to M-TRP PUSCH repetition, use option-4 to compute the PHRs based on actual PUSCH transmission  Inform RAN2 of the decision.

Proposal-11: For CG operation, the configured RV sequence can be applied separately for the two sets of PUSCH repetitions (towards different TRPs) with a possibility of configuring an offset.

Proposal-12: For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP, support Alt. 2.

Proposal-13:

For indicating per-TRP OLPC set in DCI format 0_1/0_2, if no SRI field presents in the DCI,

- Use the existing field (1 or 2 bits) for OLPC set indication and a second p0-PUSCH-SetList-r16.

  - the field is 1-bit if olpc-ParameterSetForDCI-Format0-1 indicates integer 1

  - the field is 2-bit if olpc-ParameterSetForDCI-Format0-1 indicates integer 2

  - If P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 are provided to the UE and the DCI format includes an open-loop power control parameter set indication field

    - if value of the field equals to ‘0’ or ‘00’, this would depend on the choice of ‘default P0, alpha, PL-RS, and closed loop index is defined per TRP’, i.e., Alt1, Alt2, and Alt3.

      - If Alt 2 is supported (see Proposal-3), then the UE determine value of P0 from a first and second P0-PUSCH-AlphaSet in p0-AlphaSets corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘1’ or ‘01’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘10’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

    - if value of the field equals to ‘11’, the UE determine value of P0 from a first value in P0-PUSCH-Set-r16_list1 and P0-PUSCH-Set-r16_list2 corresponding to the first and second TRP, respectively.

else, the UE determine value of P0 from a first P0-PUSCH-AlphaSet for both TRP1 and TRP2.

Proposal-14: For the codepoint ‘11’ of the single-TRP/multi-TRP dynamic switching field, associate the 1st and the 2nd SRI/TPMI field with the 1st and the 2nd SRS resource set, respectively.

Proposal-15:  For UL PTRS port 0 indication (1 PTRS port) and for port 0 and port 1 indication (2 PTRS ports) only values of 0, 1 can be indicated per TRP (reduced resolution) with MSB and LSB associated with the 2 TRPs respectively.

Proposal-16: For the case of multi- TRP PUSCH type B repetition with no scheduled data, support SP-CSI transmission in the first actual repetition for first beam and the first actual repetition for the second beam and such PUSCH length is expected to be equal to a nominal repetition.

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86