3GPP TSG RAN WG1 #106-e                                                              	        R1-2107572

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.2

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for inter-cell operation

Document for: 	Discussion/Decision

# Introduction

The feMIMO WID has been revised in the last RAN plenary to clarify the scope of this AI:

- Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

# PCI for CSI-RS

Agreement - For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC. - Applicable at least for non-cross carrier QCL indication - FFS: Cross carrier scheduling QCL indication - RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP - Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e Conclusion Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

For the purposes of CSI measurement (other than L1-RSRP and TRS) it is beneficial to be able to associate a configured NZP-CSI-RS-ResourceSet with a TCI-State associated with a SSB-Index from a non-serving cell PCID. A single additional PCI per CC is sufficient because we do not expect CSI-RS for CSI to be transmitted from TRPs not involved in PDCCH/PDSCH transmission.

Proposal-1: A single additional PCI per CC is sufficient when the target RS is CSI-RS for CSI.

# PCI association with CORESETPoolIndex

Agreement For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e - Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex - Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex - Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

A QCL source from a non-serving PCI need not be coupled with multi-DCI mTRP configuration. It is simply a QCL assumption that can be applicable to DPS or single-DCI mTRP operation as well. With this motivation Alt3 can be used.

Proposal-2: Associate a non-serving PCI with TCI states for PDSCH/PDCCH via QCL relationship without association with CORESETPoolIndex

# Rate matching

In terms of rate-matching, the simplest approach is to replicate the current rate-matching behavior of UE based on ssb-PositionsInBurst and half-frame index for the non-serving cell. In other words, UE should be provided PositionsInBurst and half-frame index corresponding to a non-serving cell and it can perform PDSCH rate-matching based on the SSB patterns in both the serving and the non-serving cell for all assigned PDSCH.

Proposal-3: Support indication of ssb-PositionsInBurst and half-frame index associated with the non-serving cell to the UE

Proposal-4: UE performs PDSCH rate-matching based on the union of ssb-PositionsInBurst and half-frame index associated with the serving cell and the non-serving cell.

# Uplink spatial relation info and PL-RS

RANP clarified that Rel-17 inter-cell mTRP is based on Rel-15/16 beam-management. It is, therefore, natural to associate a SSB Index with a non-serving cell PCID for following purposes:

- Measurements for PUCCH to the other cell: PUCCH-PathlossReferenceRS, PUCCH-SpatialRelationInfo

- Measurements for PUSCH to the other cell: PUSCH-PathlossReferenceRS

- Measurements for SRS to the other cell: pathlossReferenceRS, SRS-SpatialRelationInfo

Proposal-5: Support indication of ss-PBCH-BlockPower associated with the non-serving cell to the UE

Proposal-6: Support configuration of SSB with non-serving PCID as QCL source RS for SRS, PUCCH, and PUSCH transmission

# Association of non-serving PCI with TCI state

Association of a non-serving PCI with a TCI state can be done in multiple different ways. We think that RAN1 should decide the conditions of associating a non-serving PCI and TCI state. As an example:

- 1 additional non serving cell PCI is supported per CC to be associated with one or more TCI states that are activated for CSI-RS for CSI/PDSCH/PDCCH

- 1 additional PCI is supported per CC to be associated with one or more activated PUCCH-spatialRelationInfo or SRS-spatialRelationInfo for PUSCH

- Source RS for non-serving cell PCI is SSB

- Target RS for non-serving cell PCI is CSI-RS, DMRS for PDCCH/PDSCH, PL-RS (PUCCH, PUSCH)

The actual design for association of a non-serving PCI with TCI state can be left to RAN2.

Proposal-7: Association of non-serving PCID with TCI state can be left to RAN2. RAN1 can provide the following information to RAN2 – a single non-serving PCI associated to activated TCI states for CSI-RS for CSI/PDSCH/PDCCH, a single non-serving PCI associated to activated TCI states for PUCCH-spatialRelationInfo or SRS-spatialRelationInfo for PUSCH, source RS for non-serving cell PCI is SSB and target RS for non-serving cell PCI is CSI-RS, DMRS for PDCCH/PDSCH, PL-RS (PUCCH, PUSCH)

# Conclusions

Proposal-1: A single additional PCI per CC is sufficient when the target RS is CSI-RS for CSI.

Proposal-2: Associate a non-serving PCI with TCI states for PDSCH/PDCCH via QCL relationship without association with CORESETPoolIndex

Proposal-3: Support indication of ssb-PositionsInBurst and half-frame index associated with the non-serving cell to the UE

Proposal-4: UE performs PDSCH rate-matching based on the union of ssb-PositionsInBurst and half-frame index associated with the serving cell and the non-serving cell.

Proposal-5: Support indication of ss-PBCH-BlockPower associated with the non-serving cell to the UE

Proposal-6: Support configuration of SSB with non-serving PCID as QCL source RS for SRS, PUCCH, and PUSCH transmission

Proposal-7: Association of non-serving PCID with TCI state can be left to RAN2. RAN1 can provide the following information to RAN2 – a single non-serving PCI associated to activated TCI states for CSI-RS for CSI/PDSCH/PDCCH, a single non-serving PCI associated to activated TCI states for PUCCH-spatialRelationInfo or SRS-spatialRelationInfo for PUSCH, source RS for non-serving cell PCI is SSB and target RS for non-serving cell PCI is CSI-RS, DMRS for PDCCH/PDSCH, PL-RS (PUCCH, PUSCH)

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86