3GPP TSG RAN WG1 #106-e			R1- 2107633

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.1.3 Joint channel estimation for PUSCH

Source: 	Sierra Wireless

Title: 	Design Considerations for Joint channel estimation for PUSCH

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for the following coverage enhancement objectives in the WID [2]:

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

The following Agreements have been made:

Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots

    - It’s subject to UE capability

For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

FFS the units the time domain window (e.g. repetitions, slots, and/or symbols)

FFS : association between the potential use case(s) and units of the time window

FFS: single or multiple time domain windows

FFS: relation with UE capability

FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

FFS whether or not to further consider impacting of timing advance

For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

For the time domain window for joint channel estimation, down select on the following two options:

Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

PUSCH repetition type A

PUSCH repetition type B, if agreed

TBoMS, if agreed

Different TB, if agreed

Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

PUSCH repetition type A

PUSCH repetition type B, if agreed

TBoMS, if agreed

Different TB, if agreed

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

FFS: additional specification enhancements on top of that defined to support repetition Type A

Only for single layer transmissions

Subject to UE capability

FFS: Over back-to-back PUSCH transmissions with different TBs

Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

Draft LS to RAN4 is approved, with final LS in R1-2106212.

- For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, down select one of the following alternatives for the time domain window.

- Alt 1: All the repetitions are covered by one single time domain window

  - The start of the window is the first PUSCH transmission

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum

  - FFS: frequency hopping and precoder cycling

- Alt 2: All the repetitions are covered by one or multiple time domain windows

  - For the start of each window,

    - The start of the first window is the first PUSCH transmission.

    - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum

  - For the length of each window,

    - FFS Each window consists of at least two adjacent physical slots for UL transmission.

    - The length of each window is no longer than the maximum duration.

    - FFS: how to determine the length of each window

    - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum.

  - FFS: frequency hopping and precoder cycling

- Other alternatives are not precluded.

# Joint Channel Estimation (JCE) LLS Results

## FDD

To evaluate performance of joint channel estimation (JCE), LLS were conducted for the eMBB case with a TBS of 888 bits and 8 repeats with JCE lengths of 1, 2, 4, and 8 slots (note: full simulation assumptions for this paper can be found in appendix A):

Figure 1: LLS result for eMBB with residual CFO = ~U[-30,30Hz]

As seen from the above results, for the eMBB case, JCE provides about 1.5 dB coverage gain. Also seen from above, the degree of gain is diminishing as the length of JCE is increased which is partly because of residual CFO effects and partly because JCE with 4 slots already provides a good channel estimation at this SNR.

For the FDD eMBB scenario, JCE can provide ~1.5 dB of coverage gain.

The following LLS results are for the VoIP case with a TBS of 320 bits and 32 repeats:

Figure 2: LLS result for VoIP with residual CFO = ~U[-30,30Hz]

As seen from the above results, for the VoIP case, JCE provides about 3.5 dB coverage gain. The increased gain is because VoIP can operate at a lower SNR where JCE is more effective.

For the FDD VoIP scenario, JCE can provide ~3.5 dB of coverage gain.

## TDD

Below are the LLS results for the eMBB scenario (TBS=888, 8 repeats) with JCE across TDD frames:

Figure 4: JCE across TDD frames ‘DDDSU’ for eMBB

As seen from the above results, there is a gain of ~1.1 dB when doing JCE over 4 slots (i.e. over 4 TDD frames) vs 1 slot (no JCE). When using JCE over 8 slots or 40ms, performance is worse than 4 slots because the residual CFO of [-30,+30]Hz is causing too much phase rotation to average over such a long interval.

For the TDD DDDSU eMBB scenario, JCE across non-continuous slots can provide ~1.1 dB coverage gain.

The following figures shows the LLS results for VoIP scenario (TBS=320, 8 repeats) for JCE across TDD frames:

Figure 5: JCE across TDD frames ‘DDDSU’ for VoIP

As seen from the above results, there is a large coverage gain of ~2.2 dB using 4 non-continuous slots for JCE vs 1 slot (no JCE).

For the TDD DDDSU VoIP scenario, JCE across non-continuous slots can provide ~2.2 dB coverage gain.

# Use cases

The following potential use cases have been identified with only use case 5 as FFS:

Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot  - agreed supported

  - Use case 2: non-back-to-back PUSCH transmissions within one slot – agreed not supported

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots. - agreed supported

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots. – working assumption supported

  - Use case 5: PUSCH transmissions across non-consecutive slots - FFS

Use case 4: non-back-to-back PUSCH transmissions across consecutive slots

We found no technical issues with the working assumption and thus propose it be confirmed:

Confirm WA:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Use Case 5 Considerations:

Heavy DL:UL TDD ratios are common in real networks. Assuming TDD patterns of DDDSU and DDDDDSUU (from TR 38.830 “Study on NR coverage enhancements”), if the PUSCH transmission time is longer than 2 slots, the gNB will schedule the “PUSCH transmissions across non-consecutive slots”. For coverage limited cases, this use case will be a VERY common occurrence for TDD and, as shown in the LLS section, JCE for TDD provides good coverage gain. Thus, from a RAN1 perspective, we should strive to support this use case as it tends to have the greatest need for coverage enhancement.

In TDD bands, the gNB will often be required to schedule “PUSCH transmissions across non-consecutive slots”

Based on the latest [3] RAN4 LS response, JCE may be possible in the TDD case if the UE does not need to receive nor monitor the DL during the gaps:

“The “downlink reception” means downlink symbols with actual DL transmission from gNB to UE and/or DL monitoring with the assumption that UE is receiving information.”

Although precluding reception between “PUSCH transmissions across non-consecutive slots” will be quite limiting, the coverage gain to support JCE in TDD is quite large and will, in certain scenarios, be practical.  However, the RAN4 LS [3] also states that the gap between PUSCH transmissions needs to be less than 13 symbols, which precludes most TDD systems:

“RAN4 has continued discussing the un-scheduled gap consisting of unscheduled symbols between two PUCCH repetitions or PUSCH transmissions and reached a conclusion that it is feasible for UE to maintain phase continuity when the gap is 13 symbols or less.”

But ultimately RAN4 indicated in the LS that it needs more time to study JCE in TDD:

“Phase discontinuity tolerance LLS is ongoing in RAN4 study and conditions of whether the phase continuity can be maintained in TDD case that has downlink reception in-between the PUSCH or PUCCH repetition could be revisited in future meeting with consideration of phase discontinuity tolerance.  RAN4 is also still checking whether there are any optional UE antenna configurations where a UE could overcome this problem and still gain from using the feature"

Wait for RAN4 to complete LLS to determine if JCE in TDD case can be supported (i.e. Case 5 PUSCH transmissions across non-consecutive slots)

# JCE over Multiple TBs

The following issue is open for discussion:

FFS: Over back-to-back PUSCH transmissions with different TB

FFS: Over non-back-to-back PUSCH transmissions with different TBs

Given the high efficacy of JCE (e.g. up to 3.5 dB gain), JCE should be supported whenever possible. Since it will be common for gNB to schedule TBs “back-to-back”, unless there is a good technical reason, this use case should be supported.

JCE should be supported when gNB schedules PUSCH transmissions with different TB

# JCE Time Domain Window (TDW)

The following agreement was made last meeting WRT to the defined time domain window:

For the time domain window for joint channel estimation, down select on the following two options:

Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

PUSCH repetition type A

PUSCH repetition type B, if agreed

TBoMS, if agreed

Different TB, if agreed

Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

PUSCH repetition type A

PUSCH repetition type B, if agreed

TBoMS, if agreed

Different TB, if agreed

The above choice can wait until other details of the JCE TDW are made:

Wait to decide on the units for the JCE time domain window until other details of the time domain window are finalized

The following agreement was made last meeting WRT to the defined time domain window:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, down select one of the following alternatives for the time domain window.

- Alt 1: All the repetitions are covered by one single time domain window

  - The start of the window is the first PUSCH transmission

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum

  - FFS: frequency hopping and precoder cycling

- Alt 2: All the repetitions are covered by one or multiple time domain windows

  - For the start of each window,

    - The start of the first window is the first PUSCH transmission.

    - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum

  - For the length of each window,

    - FFS Each window consists of at least two adjacent physical slots for UL transmission.

    - The length of each window is no longer than the maximum duration.

    - FFS: how to determine the length of each window

    - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum.

  - FFS: frequency hopping and precoder cycling

- Other alternatives are not precluded.

Alt 2 has many issues and should be avoided unless there are strong technical issues raised with Alt 1. Alt 1 is the simplest option for specification and UE implementation. The only issue with Alt 1 is that with longer repeats (e.g. 32 slots) in the TDD case (if supported), the UE may have to maintain phase continuity for a long period (e.g. 160ms for TDD DDDSU pattern) even though the gNB may only be combining up to 4 slots. The solution to this is to allow the UE to break phase continuity every X TDD cycles, where X can be RRC configured and can nominally be 1 (every TDD cycle or non-consecutive slot). This additional complexity is only needed if case 5 is supported which looks doubtful at this time given RAN4’s response.

Select Alt 1: All the repetitions are covered by one single time domain window

The start of the window is the first PUSCH transmission

# Signalling

The following agreement was made last meeting:

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

With the following FFS being a key open issue for discussion:

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

If dynamic signalling or DCI signalling is supported, the following could be signaled:

Whether gNB will use JCE or not

The choice of time domain window duration (e.g. an index into an RRC configured table of time window duration)

- The mechanism and algorithm to choose which time domain durations to signal can be left up to gNB implementation, but it is assumed the gNB would not signal a duration which the UE does not support.  The exact signalling can be agreed upon later when more agreements WRT the time domain window are made but deciding whether some dynamic indication is used now, would be good.

The UL Grant can at least signal, either implicitly or explicitly, whether the JCE and the phase continuity time domain window is enabled or disabled.

# Conclusions

- For the FDD eMBB scenario, JCE can provide ~1.5 dB of coverage gain.

For the FDD VoIP scenario, JCE can provide ~3.5 dB of coverage gain.

For the TDD DDDSU eMBB scenario, JCE across non-continuous slots can provide ~1.1 dB coverage gain.

For the TDD DDDSU VoIP scenario, JCE across non-continuous slots can provide ~2.2 dB coverage gain.

- Confirm WA:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

In TDD bands, the gNB will often be required to schedule “PUSCH transmissions across non-consecutive slots”

Wait for RAN4 to complete LLS to determine if JCE in TDD case can be supported (i.e. Case 5 PUSCH transmissions across non-consecutive slots)

JCE should be supported when gNB schedules PUSCH transmissions with different TB

Wait to decide on the units for the JCE time domain window until other details of the time domain window are finalized

Select Alt 1: All the repetitions are covered by one single time domain window

The start of the window is the first PUSCH transmission

The UL Grant can at least signal, either implicitly or explicitly, whether the JCE and the phase continuity time domain window is enabled or disabled.

# References

R1-2007931 PUSCH Baseline coverage performance, Sierra Wireless

RP-202928 China Telecom, “New WID on NR coverage enhancements”

R1-2106423	Reply LS on PUCCH and PUSCH repetition	RAN4, Qualcomm

# Appendix A: LLS Simulation Assumptions:

| Parameter | Value |
| --- | --- |
| UE Speed | 3km/h |
| Bandwidth Part | 20 MHz/100 MHz |
| Configuration | FDD / TDD |
| Carrier frequency | 700 MHz / 4 GHz |
| RX Antenna ports | 2 |
| Channel model | TDL-C 300ns |
| Number of PRBs | FDD-4, TDD-8 |
| Carrier Frequency Offset | ~U[-30,30] |
| Channel Estimation | Practical |
| Frequency Hopping | As specified |
| Frequency Averaging Window | 3 PRBs |
| Time Averaging Window | As specified |
| Hop Size | 10 MHz (where applicable) |
| Hop Pattern | As specified |
| DMRS Configuration | 2 DMRS per slot |
| Transform Pre-coding | Yes |
| SCS | FDD 15 kHz TDD 30 kHz |
| Modulation | QPSK |
| TBS | FDD eMBB 888 bits VoIP 320 bits TDD eMBB -2220 bits |
| Repeats/slots | 8, 32 |
| HARQ | Disabled |
| iBLER Target | 10% |