3GPP TSG RAN WG1 Meeting #106-e	R1-2107670

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	Further considerations and simulations for Rel-17 PS codebook design

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss CSI enhancements for Rel-17 to provide supplementary information for companion paper [1]. More simulation results and details are discussed as following:

- Simulation results for the design of Rel-17 PS CB with Rank 2

- Simulation results and design principles for Rel-17 PS CB with Rank 3/4

Additional simulation Results for Rel-17 PS CB Design with Rank 2

As discussed in [1], configured values of K1, Mv and K0 can be the same at least up to rank 2 for the simplicity, but same or different FD basis subset, port selection and non-zero coefficient selection across two layers may lead to different performance and CSI report overhead and some analysis/simulation results are given in [1]. In this part, we will give additional simulation results and observations for above issues.

-	Additional simulation results of port and FD basis selection for rank 2

On selecting the port and FD basis subset for Rel-17 PS CB fixed rank 2, the performance-overhead trade-offs of four alternatives are compared as following. And the simulation results are provided in Figure 1, and the relevant simulation assumptions and parameters are enlisted in Table A-1 in Appendix A.

•	port selection is layer-common and FD basis subset selection is layer-common

•	port selection is layer- common and FD basis subset selection is layer- specific

•	port selection is layer- specific and FD basis subset selection is layer- common

•	port selection is layer- specific and FD basis subset selection is layer- specific

- 50MHz and  PCSI-RS=32                             (b) 50MHz and  PCSI-RS=8

Figure 1 Comparison of the performance with different schemes for port and FD basis selection

According to Figure 1, when port selection or FD basis subset selection is layer-common or layer-specific，layer-specific selection performs slightly better than layer-common selection, since independent basis selection will provide more degree of freedom.

Observation 1: Considering additional simulation cases, e.g. 50MHz with PCSI-RS=32 and 50MHz with PCSI-RS=8, similar observations/preference can be found that for port selection and FD basis subset selection for rank 2, the performance of layer-specific selection is similar or marginally better than layer-common selection in low overhead regime.

-	Additional simulation results of non-zero coefficient selection for rank 2

For non-zero coefficient selection, besides the simulation and analysis in [1], we also provides additional simulation as following, which compare the layer-common and layer specific non-zero coefficients selection. The simulation results are provided in Figure 2 and the relevant simulation assumptions and parameters are enlisted in Table A-1 in Appendix A.

Figure 2 Comparison of the performance with different schemes with 50MHz and  PCSI-RS=8

From Figure 2, it can be observed that for non-zero coefficient selection for rank 2, layer-specific selection can achieve average gain up to 12% in low overhead regime. To sum, we have the following observations:

Observation 2: Considering additional simulation cases, e.g. 50MHz with PCSI-RS=8, similar observations/preference can be found that layer-specific selection performs better than layer-common,   especially at low overhead regime, until performance gain of Rel-17 PS codebook saturates.

Simulation results and design principles for Rel-17 PS CB with Rank 3/4

Simulation results for Rel-17 PS CB with Rank 3/4

In last meeting, Rank 2 is supported for Rel-17 PS CB and some simulation results in [1] show that compared Rel-16 PS CB with Rank 3/4, Rel-17 PS CB with Rank3/4 can provide better performance under SU-MIMO up to Rank 4 with 32 ports CSI-RS and IRC receiver. In this part, we evaluate the performance of Rel-17 PS CB with rank 3/4 in more scenarios, e.g. dynamic SU/MU-MIMO at high RU.

Dynamic SU/MU-MIMO up to Rank 4

Figure 3 Comparison of the performance with R17 and R16 under Dynamic SU/MU MIMO up to Rank 4

Figure 3 gives the performance of R16 enhanced Type II port selection codebook and Rel-17 PS CB enhancement up to Rank 4 with SRS error and calibration error. In the simulation, R of R16 PS CB equals to 2 and R of R17 PS CB equals to 4.

It can be observed that compared with R16, R17 can provide more than 10% performance gain for mean UPT at both high and low CSI feedback overhead if the same reporting overhead is assumed. By utilizing angle and delay reciprocity, free bases selection and non-DFT-based channel decomposition especially in frequency domain can be achieved at gNB sides for R17 PS CB, which could provide significant performance gain. Furthermore, R17 PS CB can achieve finer precoding granularity for PMI (larger R, numberOfPMISubbandsPerCQISubband) so as to provide better performance.

Observation 3: For dynamic SU/MU-MIMO up to Rank 4, more than 10% performance gain can be observed for R17 across all CSI overhead regimes when RU=70%.

Design principles for Rel-17 PS CB with Rank 3/4

-	Design of port and/or FD basis subset selection for Rank 3/4

The design of port selection and FD basis subset selection for Rank 2 of Rel-17 PS CB is discussed in [1], and layer-common design is proposed for both port selection and FD basis subset selection., Under the principle that Rank 3/4 reuses the design of Rank 1/2 as much as possible, following design can be considered for Rel-17 PS CB with Rank 3/4 for both port selection and FD basis subset selection.

Figure 4. Different options for port selection and FD basis subset selection

In above options, based on the design of port and FD basis subset selection for Rank 2, the port selection and FD basis selection keep layer common for layer 1 and layer 2, which is similar for layer 1 and layer 2 under Rank 2. And the main differences between above options are summarized as following.

- Option 1: Port selection and/or FD basis selection is layer-common between layer 3 and layer 4, but is layer-specific with the subset selected for layer 1 and layer 2.

- Option 2: Port selection and/or FD basis selection is layer-common between layer 3 and layer 4, and is also layer-common with the subset selected for layer 1 and layer 2.

Based on above analysis, we have the following proposal for the port selection and FD basis selectin of Rel-17 PS CB with Rank 3 and Rank 4.

Proposal 1: For Rel-17 PS codebook design for rank 3/4

- Keeping CSI feedback overhead comparable to rank 2, e.g. by adjusting compression coefficients or configuration parameters

- Reusing rank 1/2 CB design as much as possible, e.g. layer-specific non-zero coefficient selection in W2

- Study whether port selection and FD basis selection are layer common across all layers or partially common

-	Linear combination coefficients quantization for R17 PS CB for Rank 3/4

For Massive MIMO, multiple spatial layers can transmit to one UE simultaneously to improve the performance. If the number of total layers does not exceed four, only one CW can be scheduled according to NR specification from the UE perspective. However, layers within one CW can give rise to significantly variant SNRs among layers.

For TDD system, the power difference between layers can be obtained by gNB based on full reciprocity between UL channel and DL channel. But for FDD system, there has only partial reciprocity between UL channel and DL channel, gNB can’t obtain the power difference between different layers. In addition, Linear combination coefficients are normalized per layer in R15 Type II/PS TypeII and R16 eType II/ PS eTypeII. Based on this normalization per layer, the difference between layers are missed and gNB can’t obtain the difference between layers based on the CSI feedback by UE, shown as Figure 5.

Figure 5 Power difference between layers based on R15/R16 Type II CB CSI

If linear combination coefficients can be normalized cross layers for R17 PS CB, the difference between layers can be retained and feedback to gNB, which is shown as Figure 6. Similar to the port selection, FD basis selection and non-zero coefficients selection for Rank 3/4, under the principle that Rank 3/4 reuses the design of Rank 1/2 as much as possible, we can have the following design for the linear combination coefficients quantization for Rank 3/4, in which the coefficients of layer 1 and layer 2 are quantitated separately, and taking layer 1 as reference point, the coefficients can be normalized cross layer 3 and layer 4. For example, we have linear combination coefficientsC1={c11,c21,…,cK01},C2=c12,c22,…,cK02, C3={c13,c23,…,cK03} and C4={c14,c24,…,cK04} and eigenvalues γ1, γ2, γ3 and γ4 for layer 1~4 respectively. And ci1, cj2, cp3, and cq4 are coefficient with maximum power for C1, C2, C3 and C4 respectively, of which ci1 is the largest coefficient. When deriving PMI and associated CQI reporting, coefficients in C1, C2, C3 and C4 are normalized by ci1, cj2, cp3, and cq4  respectively. However, in order to provide supplementary information, the amplitude of  γ3 and γ4 are normalized by the amplitude of  γ1 and then are quantized by M bits, e.g. M=3. Based on the quantization of amplitude of γ3 and γ4 feedback by UE, the power difference between first two layers, layer 3 and layer 4 can be obtained by gNB, and gNB can do some power allocation between layers to improve the performance.

Figure 6 Power difference based on normalization cross layers

Proposal 2: For Rel-17 PS CB enhancement with Rank 3 and Rank 4, coefficient normalization can be considered to be reported cross subsets of layers to equalize power allocation for more efficient high rank transmission.

Conclusions

This contribution provides our views on further considerations and simulations for Rel-17 PS  codebook design based on angle and delay reciprocity. In summary, the following proposals and observations are made.

Proposal 1: For Rel-17 PS codebook design for rank 3/4

- Keeping CSI feedback overhead comparable to rank 2, e.g. by adjusting compression coefficients or configuration parameters

- Reusing rank 1/2 CB design as much as possible, e.g. layer-specific non-zero coefficient selection in W2

- Study whether port selection and FD basis selection are layer common across all layers or partially common

Proposal 2: For Rel-17 PS CB enhancement with Rank 3 and Rank 4, coefficient normalization can be considered to be reported cross subsets of layers to equalize power allocation for more efficient high rank transmission.

Observation 1: Considering additional simulation cases, e.g. 50MHz with PCSI-RS=32 and 50MHz with PCSI-RS=8, similar observations/preference can be found that for port selection and FD basis subset selection for rank 2, the performance of layer-specific selection is similar or marginally better than layer-common selection in low overhead regime.

Observation 2: Considering additional simulation cases, e.g. 50MHz with PCSI-RS=8, similar observations/preference can be found that layer-specific selection performs better than layer-common,   especially at low overhead regime, until performance gain of Rel-17 PS codebook saturates.

Observation 3: For dynamic SU/MU-MIMO up to Rank 4, more than 10% performance gain can be observed for R17 across all CSI overhead regimes when RU=70%.

- References

3GPP R1-2106469, Discussion on CSI Enhancements for Rel-17, Huawei, HiSilicon, RAN1#106-e, e-Meeting, August 16th – 27th, 2021.

- Appendix A.

Table A-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 [21] |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with fixed Rank 2 SU-MIMO with fixed Rank 4 |
| CSI feedback |  | Feedback assumption: - CSI feedback periodicity (full CSI feedback) : 5ms - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| SRS Configuration |  | - SRS periodicity with 10ms - Comb: 2 - Number of OFDM symbols: 2 |
| SRS Error Model |  | SRS error Modelling in Table A.1-2 in 36.897. Δ=9dB and detailed derivation of Δ can be found in R1-144943[20]. |
| Calibration error model at gNB |  | HUL=HUL⋅E=HUL⋅a1ejθ1⋯0⋮⋱⋮0⋯aNejθN amplitude error (expressed in decibels) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation for simulation bandwidth , respectively |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI reporting overhead |