3GPP TSG RAN WG1 Meeting #106-e	R1-2107676

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.15.3

Source:	Huawei, HiSilicon

Title:	Other aspects to support IoT in NTN

Document for:	Discussion and Decision

Introduction

In RAN#92, a WI on NB-IoT/eTMC support for NTN was approved. The objective related to RAN1 of this WI is as follows [1]:

Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763:

-	UE pre-compensation including ephemeris format (orbital / Position -Velocity)

-	UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris

-	Timing advance formula (granularity of the timing advance may be different)

-	Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state

Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any.

Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763:

-	Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments.

-	Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects

-	DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB.

-	GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission

Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763:

-	Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763

-	UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD

-	Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

In this contribution, we discuss UL time/frequency synchronization for long connections for IoT in NTN.

Discussion

Validity timers for long connections

The main characteristics of IoT devices include half duplex transmission mode, large number of repetitions for DL/UL transmission and system information acquisition only in RRC idle mode. In the context of IoT over NTN, some enhancements should be introduced to address the large timing and frequency drift introduced by the satellite motion. In our companion paper [2], the UL time/frequency synchronization enhancements for sporadic short transmission are discussed by introducing a single validity timer for UL synchronization.  In this section, potential enhancements are discussed for long connections.

Validity for GNSS

It was agreed that for sporadic short transmission, the UE acquires GNSS position fix before accessing the network and does not need to re-acquire a GNSS position fix for the transmission of the packets.  However, for long connections, UE in RRC-connected depending on UE mobility may need a new GNSS position fix in order to accommodate the accumulated time and frequency errors. While, for stationary UE, there is no need to have another GNSS position fix for long connections. However, moving UEs may need another GNSS position fix for long connections. Therefore, a UE-specific validity timer for GNSS position fix can be introduced. The following two options can be considered:

- Option 1: Validity timer for GNSS is configured by the eNB without UE assistance information

- Option 2: Validity timer for GNSS is recommended by the UE and configured by the eNB

A UE shall use the previous GNSS acquired position for UL time/frequency pre-compensation until the timer expires. If the UE does not enter RRC_IDLE mode by the time of expiration, a GNSS position fix should be rescheduled through the GNSS measurement window. Alternatively, RLF can be triggered by the UE.

Observation 1: For long connections, the validity of a GNSS position fix is UE specific and is dependent on UE speed.

Proposal 1: The validity timer for GNSS (if needed) is recommended by the UE and configured by the eNB.

Validity for satellite ephemeris and common TA

For UL synchronization with long connections, the need of validity timer for satellite ephemeris was discussed in the last meeting. As discussed in [2], a single validity timer for common TA and ephemeris can be indicated by the NW, and radio link failure will be triggered when the timer is expired. The analysis also holds for long connections

Proposal 2: A single validity timer for UL synchronization can be configured for long connections.

Closed loop corrections for long connections

To reduce the radio link failure caused by the outdated common TA or the satellite ephemeris, we discuss the potential enhancement for UL synchronization in long connections by using validity timer based close loop correction for ephemeris/common TA parameters.

Figure 1. Timer based close loop ephemeris indication.

As shown in Figure 1, a UE reports its propagation validity for ephemeris in the initial access. Then, a combined or separate timer will be triggered based on validity of ephemeris and common TA parameters, respectively. Before the timers expire, a corresponding close-loop correction is expected to be indicated by the NW. If UE receives the close-loop correction information, the timer will be restarted. Otherwise, radio link failure will be triggered directly or after specific times of retransmissions.

Proposal 3: A combined or separate timer for common TA and ephemeris can be applied for closed loop corrections of ephemeris/common TA parameters for long connection mode to reduce radio link failure.

NTN SIB for common TA and ephemeris

Different from terrestrial NW, some system information in NTN for UL time/frequency synchronization, such as common TA and ephemeris, may change frequently. When an IoT terminal stays in connected mode for a long time, these parameters will become out of date and radio link failure may happen. Beside the closed loop corrections, another scheme to reduce radio link failure for both short and long connection duration is introducing NTN related SIB for IoT NTN. IoT devices can read this NTN related SIB whilst in the connected mode. As IoT devices operate in half duplex mode,  periodical windows for reading the NTN related SIB are needed as shown in Figure 2.

Figure 2. NTN SIB reading during long connection

Apart from introducing an NTN SIB, common TA and ephemeris can also be scheduled in a semi-persistent way. The scheduling period of common TA and ephemeris can be different because the UE may propagate ephemeris for a longer time duration compared to common TA. According to our companion paper in [3], ephemeris can be predicted for 30s with only 11m absolute position error. The UE may only need to acquire common TA during the connected mode.

Proposal 4: For long connections, an NB-IoT UE in RRC_CONNECTED mode may reacquire common TA and satellite ephemeris carried in NTN SIB.

Conclusions

In this contribution, other considerations for UL time/frequency synchronization and power saving to support IoT in NTN are discussed. The following observations and proposals are presented:

Observation 1: For long connections, the validity of a GNSS position fix is UE specific and is dependent on UE speed.

Proposal 1: The validity timer for GNSS (if needed) is recommended by the UE and configured by the eNB.

Proposal 2: A single validity timer for UL synchronization can be configured for long connections.

Proposal 3: A combined or separate timer for common TA and ephemeris can be applied for closed loop corrections of ephemeris/common TA parameters for long connection mode to reduce radio link failure.

Proposal 4: For long connections, an NB-IoT UE in RRC_CONNECTED mode may reacquire common TA and satellite ephemeris carried in NTN SIB.

- References

RP-211601, WID on NB-IoT/eMTC support for NTN.

R1-2106485, “Discussion on time and frequency synchronization enhancement for IoT in NTN”, Huawei, HiSilicon

R1-2106483, “Discussion on UL time and frequency synchronization enhancement for NTN”, Huawei, HiSilicon