3GPP TSG RAN WG1 #106-e	R1-2107701

e-Meeting, August 16th – 27th, 2021

Title:	Draft Reply LS on Synchronous Operation between Uu and SL in TDD Band

Response to: 	R4-2108114

Release:	Release 17

Work Item:	NR_SL_enh-Core

Source:	RAN1, Apple

To:	RAN4

Cc:

Contact Person:

Name:	   Chunxuan Ye

Email Address:  chunxuan_ye@apple.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# Overall description

RAN1 thanks RAN4 for the LS on synchronous operation between Uu and SL in TDD band. The two options were considered in RAN4.

Option 1: To follow the Rel-16 agreement to align SL transmission timing with DL timing. Option 2: To reconsider SL transmission timing to align with UL timing to mitigate the interference between Uu and SL, i.e. - For sidelink transmissions, - SL transmission timing is aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, including TDM coexistence within the same carrier or different carriers. - Otherwise, SL transmission timing is aligned with Downlink timing.

Regarding the fesability of Option 2 from RAN1 perspecive, RAN1 would like to clarify that for a mode 1 UE in RRC idle or inactive state or a mode 2 UE, the uplink timing may not be maintained at the UE, and hence Option 2 is infeasible. For a mode 1 UE in RRC connected state, the uplink timing is maintained at the UE, and hence Option 2 is feasible. In other words, Option 2 is feasible for mode 1 UE in RRC connected state.

# Actions

To RAN WG4:

ACTION:  RAN1 kindly asks RAN4 to take above information into account for their future works.

# Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #106b-e		  11 -- 19 October, 2021		    eMeeting

TSG RAN WG1 Meeting #107-e		  11 -- 19 November, 2021		  eMeeting