3GPP TSG RAN WG1 #106-e			R1-2107707

e-Meeting, August 16th – 27th, 2021

Agenda item:	5.2

Source:	Apple Inc.

Title:	Discussion on physical layer aspects of small data transmission

Document for:	Discussion/Decision

# Introduction

NR small data enhancement WI was started in RAN2#112-e meeting, an LS [1] was sent to RAN1 to request RAN1’s inputs on the following questions.

- Configuration of the coreset and search space for monitoring the PDCCH addressed to the C-RNTI after successful completion of the RACH procedure during RA-SDT.

- Configuration of association between the type 1 CG resource(s) for CG-SDT and SSB(s).

And in RAN1#104bis-e meeting, the following agreement had been made for CG-SDT [2].

Agreement:

- CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling.

  - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration.

  - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any.

The agreements made in RAN1#105e meeting are showing below[3].

Agreement:

- The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined.

- The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec.

  - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission

  - The ordering of the SSB can reuse from the SSB-to-RO mapping

  - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible

- FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived

- FFS any limitation on the combination of the parameters for CG resources

In this contribution, we discuss the SDT remaining issues from the physical layer perspective.

# Discussion

For CG-SDT, it was agreed in RAN104bis-e meeting that CG resources per CG configuration are associated with a set of SSB(s) by explicit signalling, in addition, it was agreed in RAN1#105-e meeting that SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined. The PUSCH resource is clarified further it refers to a transmission occasion and a DMRS resource used for PUSCH transmission. With this clarification, there is no FDMed and TDMed PUSCH resources within the CG configuration. Similar as MsgA PUSCH ordering in two-step RACH, the CG-PUSCH resource indexing can be determined in the following order

- First,  in increasing the order of  DMRS port index, i.e., antennaPort in ConfiguredGrantConfig

- Second,  in the ascending order of DMRS sequence index, i.e., dmrs-SeqInitialization in ConfiguredGrantConfig

Another parameter related to the SSB to CG-PUSCH resource mapping is the association period, this parameter can be configured explicitly by the network, the association period can be multiple of configured grant periodicity. One example of SSB to PUSCH resource association is showing in Figure 1.

Figure 1: SSB to PUSCH resource mapping

For the  indication of mapping ratio between SSB and CG-PUSCH resource, e.g., one-to-one, multiple-to-one, one-to-multiple mapping, the mapping ratio could be implicitly derived based on the total available PUSCH resources in the association period and the number of SSB, which is similar as the preamble to MsgA PUSCH mapping ratio determination in two-step RACH. Another way is the network configures the mapping ratio directly. To save the signalling overhead, the implicit derivation of mapping ratio is preferred.

Proposal 1:  SSB-to-PUSCH association period is configured by the network explicitly. The SSB-to-PUSCH  mapping ratio is derived implicitly.

According to [1], for CG-SDC RAN2 agreed that A SS-RSRP threshold is configured for SSB selection. If the measured SS-RSRP above the threshold, UE would use the configured CG resource for UL data transmission. With this agreement, the UE would transmit the small data only when UE is closer to the gNB, otherwise normal PRACH procedure will be applied. If the repetition is supported for CG-SDT, SS-RSRP threshold can be setting lower, and more users could get the benefits of small data transmission without switching into RRC_CONNECTED mode. The cost is more CG-SDT resources are reserved. However, the configuration flexibility can be left to gNB, gNB decides whether repetition is configured according to the scheduling strategy. The standard impact to support CG-SDT repetition is limited, the parameter in existing configured grant configuration, i.e., repK, can be reused.

Proposal 2: Time domain repetition can be supported for CG-SDT.

An LS in [4] was sent to RAN1 to inform RAN2’s progress on SDT. The related RAN2 agreements are showing below.

Agreement: For RA-SDT - CFRA is not supported for RA-SDT For CG-SDT - Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed - CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm - RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission. RAN2 assumes that existing mechanism can be used.

As for RAN2’s questions, our considerations are shared in below.

- Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed

- Answer: the working assumption from RAN2 can be confirmed, the UE-specific search space can be configured before UE leave the RRC_CONNECTED state to RRC_INACTVIE state.

- CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm

According to our understanding, the CG-SDT resource here includes both UL and DL resource, which can be configured in both DL and UL initial BWP or separate SDT BWP.

- RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission.  RAN2 assumes that existing mechanism can be used

- For the subsequent transmission, NR-U defined feedback mechanism for UL HARQ could be the starting point for SDT feedback.

# Summary

In this contribution, we discuss the small data transmission impacts on physical layer aspects and the issues raised by RAN2, and have the following proposals:

Proposal 1:  SSB-to-PUSCH association period is configured by the network explicitly. The SSB-to-PUSCH  mapping ratio is derived implicitly.

Proposal 2: Time domain repetition can be supported for CG-SDT.

# References

- R1-2100025, “LS on physical layer aspects of small data transmission”, RAN2, ZTE

- R1-2104151, “Report of RAN1#104bis-e meeting”, ETSI MCC

- R1-2106402, “Report of RAN1#105-e meeting”, ETSI MCC

- R1-2106405, “Reply LS to RAN1 on physical layer aspects of small data transmission”, RAN2