3GPP TSG RAN WG1 #106-e		R1-2107713

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	7.2.5

Source:	Apple

Title:	Discussions on HARQ-ACK timing for sub-slot-based HARQ-ACK feedback in Rel-16

Document for:	Discussion/Decision

# Introduction

For HARQ-ACK, the PUCCH for HARQ-ACK is transmitted in UL slot n+k, where k is indicated in UL DCI, and n is determined based on PDSCH. When DL and UL have different numerology, two different interpretations existed in the history of RAN1 discussions. This was discussed in RAN1#104b-e [1] and RAN1#105-e [2], and it was concluded that the two different interpretations can exist in Rel-15, but for Rel-16, a working assumption was made to adopt interpretation 2 below.

Conclusion: (RAN1#104b-e)

For HARQ-ACK timing in Rel-15, in case UL SCS is larger than DL SCS, there are two different interpretations:

-       Interpretation 1: k = 0 corresponds to the last UL slot that overlaps with the PDSCH

-       Interpretation 2: k = 0 corresponds to the last UL slot that overlaps with the DL slot for the PDSCH

Further discuss this issue for Rel-16 in future meetings.

Working Assumption (RAN1#105-3)

For HARQ-ACK timing in Rel-16 with slot-based HARQ-ACK feedback, in case UL SCS is larger than DL SCS, k = 0 corresponds to the last UL slot that overlaps with the DL slot for the PDSCH.

- Further discuss the HARQ-ACK timing for sub-slot-based HARQ-ACK feedback

- FFS specification impact

What remains open is the HARQ-ACK timing for sub-slot-based HARQ-ACK feedback. In this contribution, we discuss the issue and propose that we also adopt interpretation 2 in this case.

# Discussion

For slot-based HARQ-ACK feedback, assuming an example as shown in Fig. 1 where DL uses 15 kHz SCS and UL uses 30 kHz SCS, interpretation 2 means that k=0 corresponds to UL slot 7, in contract to UL slot 6 with interpretation 1 k=0. The main reasons for adopting interpretation 2 in RAN1#105 working assumption are that it is aligned with earlier RAN1 agreement and it is also aligned with the Type-1 HARQ-ACK codebook construction in TS 38.213 (the pseudo code for Type-1 CB does not always work with interpretation 1).

Figure 1 Example of slot-based HARQ-ACK timing

For sub-slot-based HARQ-ACK feedback, the issue is similar, and one DL slot can overlap with multiple UL sub-slots. There are also two alternatives (corresponding to the two interpretations for slot-based HARQ-ACK):

- Alt 1: k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH

- Alt 2: k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH

Alt 1 is aligned with the following RAN1#97 agreement, while Alt 2 is aligned with the working assumption that we just made for slot-based HARQ-ACK timing in RAN1#105-e.

Agreements:

For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH.

- Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association.

- FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth.

- Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot.

The question now is whether we should follow the RAN1#97 agreement, or we should align it with slot-based HARQ-ACK timing considering the discussions.

Comparing Alt 1 vs Alt 2, the drawback of Alt 2 is that the HARQ-ACK latency may be slightly larger in some cases, while sub-slot HARQ-ACK feedback has been introduced exactly to reduce the HARQ-ACK latency. Figure 2 shows an example case where Alt 2 might result in larger latency. In this example, both DL and UL SCS is 15kH, and there are 7 sub-slots in a UL slot. With interpretation 1, k=0 corresponds to UL sub-slot 0, but the earliest time where the UE can provide HARQ-ACK feedback based on PDSCH processing time capability 2 (3 symbols plus 1 symbol for overlapping PDCCH) is sub-slot 3 (2nd symbol in sub-slot 3 considering TA). On the other hand, with interpretation 2, k=0 corresponds to UL sub-slot 6. This means that in the worst case, Alt 2 results in 5 more symbols (~0.36 ms) in HARQ-ACK latency.

Figure 2 Alt 1 vs Alt 2 for sub-slot-based HARQ-ACK timing

If the DL SCS is 30kHz, the additional HARQ-ACK latency for Alt 2 in the worst case is roughly 4 symbols (~0.14 ms).

Note that the worst case occurs only if PDSCH is transmitted at the beginning of the slot, and the earliest symbol for HARQ-ACK is an UL symbol. In most cases, there is either no difference or minimum difference between the two alternatives. E.g. for the case of 2 sub-slots in a slot and same SCS, there is no difference between Alt 1 and Alt 2.

The question is whether the additional delay for Alt 2 is considered significant enough. HARQ-ACK latency only matters when there is enough delay budget for HARQ retransmissions, which means the latency bound has to be on the order of a few ms. The gNB processing time typically considered should be longer than the UE processing time due to a large number of UEs the gNB needs to process and schedule. Considering these factors, the additional HARQ-ACK delay for Alt 2 should not be the major component of the overall delay.

As a short summary,

- Pros of Alt 2 comparing to Alt 1

  - Consistent with the working assumption for slot-based HARQ-ACK feedback

  - The principle of Type-1 HARQ-ACK codebook construction for slot-based HARQ-ACK can be reused for sub-slot-based HARQ-ACK. Note that Type-1 HARQ-ACK codebook for sub-slot-based HARQ-ACK is currently being discussed in Rel-17.

  - This reduces the implementation complexity for both UE and gNB

- Cons of Alt 2 comparing to Alt 1

  - Additional latency for HARQ-ACK feedback in some cases

    - However, the additional latency is more noticeable only under certain conditions (e.g. when PDSCH is transmitted at the beginning of the slot with 2-symbol duration, and the UL occupies about half of the same slot).

    - In most cases, the additional latency is either zero or minimal.

    - Even in the worst case, the additional latency should not be the major component of the overall delay.

Based on the analysis, we propose that Alt 2 is also adopted for sub-slot-based HARQ-ACK feedback timing in Rel-16.

Proposal 1: For the sub-slot-based HARQ-ACK timing in Rel-16, adopt Alt 2, i.e., k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH.

# Conclusion

Proposal 1: For the sub-slot-based HARQ-ACK timing in Rel-16, adopt Alt 2, i.e., k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH.

# References

- R1-2104105, Summary of email discussion [104b-e-NR-7.1CRs-04] on the correction for HARQ-ACK timing, Moderator (Apple Inc.), RAN1#104b-e, April 2021.

- R1-2106301, Summary of email discussion [105-e-NR-7.1CRs-13] on the correction for HARQ-ACK timing in Rel-16, Moderator (Apple Inc.), RAN1#105-e, May 2021.