3GPP TSG- RAN WG1 Meeting #106-e                                                 R1-2107718

# e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.1

Source:	Apple Inc.

Title:	On Beam Management Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #105 meeting, the following agreements and conclusion on beam management enhancement have been achieved.

Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. - Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability - FFS: Application time and whether additional offset is needed for the application time in case of cross carrier beam indication and common TCI state ID update across a set of configured CCs if CCs have different SCSs - FFS: Whether inter-cell beam switching needs higher X/Y values than intra-cell - FFS: Whether application time can be indicated/determined dynamically for different scenarios, e.g. cross CC, inter-cell, inter-panel without reverting previous RAN1 agreements Agreement For M=N=1, on Rel-17 unified TCI, for separate DL/UL TCI, one instance of beam indication using DCI formats 1_1/1_2 (with and without DL assignment) can be used as follows: - One TCI field codepoint represents a pair of DL TCI state and UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state and UL TCI state. - One TCI field codepoint represents only a DL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state, and keeps the current UL TCI state. - One TCI field codepoint represents only an UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding UL TCI state, and keeps the current DL TCI state. FFS: the cases of M or N>1 Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, focus study (including down-selection) and, if needed, specification effort on the following options: - Group 1: Aim for at most one solution for Group 1 in Rel-17 to address issue 6 - Opt 1-A. UE-initiated beam selection/activation based on beam measurement and/or reporting (without beam indication or activation from NW) - Opt 1-B. Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request) - Opt 1-C. Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency - Group 2: Aim for at most one solution for Group 2 in Rel-17 to address issue 6 - Opt 2-A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking - Opt 2-B: Latency reduction for MAC CE based PL-RS activation - Opt 2-C: One-shot timing update for TCI state update Agreement On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2 - The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability - FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration - FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS - Note: PL-RS is assumed to be periodic Agreement On Rel.17 unified TCI framework, - Any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) - Note: This does not imply that all such DL RSs necessarily share a same TCI state - The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH - FFS: Whether some SRS resources or resource sets for BM can be configured as a target signal/channel of a Rel-17 UL TCI (hence the Rel-17 UL TCI state pool) - Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL/UL TCI (this issue is still TBD) Agreement On Rel.17 unified TCI framework, discuss and decide by RAN1#106-e (August 2021) - Whether each of the following DL RSs can share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC - CSI-RS resources for CSI - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’) - CSI-RS for tracking - DMRS(s) associated with non-UE-dedicated reception on PDSCH and all/subset of CORESETs - Whether some SRS resources or resource sets for BM can share the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC Agreement On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), discuss and down-select by RAN1#106-e (August 2021) between the following two alternatives: - Alt1. Rel-15/16 TCI state update signaling/configuration mechanism(s) are reused to update/configure the Rel-17 TCI state - Alt2. Rel-17 TCI state update signaling/configuration mechanism(s) are used, e.g. with Rel-17 MAC-CE/DCI-based beam indication for Rel-17 joint/separate TCI Note: The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH Note: For some channels/signals, only one of the above two alternatives may apply (to be discussed). Agreement On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - Support at least K=4, where K is defined as the number of beams associated at least with non-serving cell(s) reported in a single CSI reporting instance - The maximum value of supported K is a UE capability - K is configured by NW based on the UE capability - FFS: The support of K=8 and 16 - For K>4, the maximum number of beams associated with one cell is 4 - FFS: Support L1-based event-driven reporting based on Rel-16 SCell BFR framework or analogous to L3-based event-driven reporting, including the definition of L1-based event, if needed Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies Agreement On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, decide by RAN1#106-e whether to support the following RS types as measurement RS or not: - CSI-RS for mobility/RRM associated with a non-serving cell - CSI-RS for BM associated with a non-serving cell - CSI-RS for tracking associated with a non-serving cell Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies Note: An RS is associated with a non-serving cell means that it is either configured for a non-serving cell or configured for a serving cell but is QCLed with a non-serving cell SSB Working Assumption On Rel.17 beam indication enhancements for L1/L2-centric inter-cell mobility, support the following: - Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) - FFS (to be decided in RAN1#106-e): Whether this also applies to PDSCH/PUSCH associated with UE-dedicated CORESETs only or additional target channels (e.g. UE-dedicated PDCCH/PUCCH) - FFS: Whether the above is supported only for joint TCI, or both joint TCI and separate DL/UL TCI (including that, if separate DL/UL TCI is supported, the DL TCI and UL TCI associated with a same cell) - FFS: Whether to support activation of TCI states for more than one cells simultaneously - FFS: Whether down-selection between MAC-CE only based and MAC-CE+DCI-based beam indication scheme is necessary - The DL QCL and UL spatial relation rules already agreed for intra-cell scenario - FFS: The use of SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference for UE-dedicated PDSCH - FFS (to be decided in RAN1#106-e): Whether this also applies to UE-dedicated PDCCH - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel - FFS (to be decided in RAN1#106-e): Whether SSB associated with a physical cell ID different from that of the serving cell can also be used as a direct QCL reference (source RS) for UE-dedicated PDCCH/PDSCH Agreement On Rel.17 unified TCI framework, for common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of configured CCs/BWPs - The source RS determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for a target CC can be configured in the target CC or other CC - For intra-band CA, the following configurations can be supported without additional QCL rules: - One source RS across CCs can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs - One source RS per CC can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs, and the CC-specific source RSs are further associated with a same QCL-TypeD RS - “A set of configured CCs/BWPs” includes all the BWPs in the set of configured CCs Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC Conclusion On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling - There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based) Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, - For each of PUSCH and PUCCH, the setting of (P0, alpha, closed loop index) can be associated with UL or (if applicable) joint TCI state per BWP. - In this case, multiple settings are configured. Each setting can be associated with at least one TCI state, and, for a given TCI state, only one setting for PUSCH and only one setting for PUCCH can be associated at a time. - (Working Assumption) In this case, for each of the PUSCH and PUCCH, each of the activated UL or (if applicable) joint TCI states is associated with one of the settings. - If not associated, for each of the PUSCH and PUCCH, the setting(s) of (P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or (if applicable) joint TCI states - FFS: If the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state. - FFS: (to be decided in RAN1#106-e) whether to configure the same setting of (P0, alpha, closed loop index) per TCI state across channels and apply a channel dependent component, or configure a channel dependent setting of (P0, alpha, closed loop index) per TCI state Agreement On Rel-17 unified TCI, in RAN1#106-e, for M>1 and/or N>1: - Identify and agree on use cases - Decide whether to support M>1 and/or N>1, and if so, the maximum value of M and/or N - If supported, identify feasible candidate schemes for beam indication signaling mechanism (including TCI state activation) Note: - Previous agreement in RAN1#104b-e that remaining unused DCI fields and codepoints are reserved in R17 are not to be reverted. - The use case of simultaneous UL transmission from multiple UE panels are not to be considered in Rel-17 as it is out of scope Conclusion On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling - There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based) For future meetings: Continue to study necessary enhancements to optimize transmission from UEs with different number of max number of UL MIMO layers per panel entity

In this contribution, we provide some discussion on remining issues on beam management enhancement.

# Issue 1: Unified TCI framework

For unified TCI indication framework, there are the following open issues:

- Remaining details for the downlink TCI state: source signal, target signal and QCL types

- Remaining details for the uplink TCI state: source signal, target signal and power control parameters

## Downlink TCI state

In Rel-15, the QCL rule is defined as shown in Figure 1.

Figure 1: QCL rule defined in Rel-15

The unified TCI framework should follow the QCL rule defined in Rel-15 with regard to UE implementation complexity. As it was agreed, the TCI state can be applied for both PDCCH and PDSCH. The source reference signal and QCL type indication for PDCCH and PDSCH are the same based on current QCL rule. For periodic CSI-RS, although it is dedicatedly configured, it can be shared by multiple UEs from network perspective. Thus, it is not necessary to change the beam for periodic CSI-RS. For aperiodic CSI-RS, gNB can use DCI to update its beam, which should be fast enough. However, there may be a default CSI-RS beam and PDSCH beam collision if the unified TCI cannot be applied for aperiodic CSI-RS. For semi-persistent CSI-RS, gNB can use MAC CE to update its beam, which is also with a low latency.

Proposal 1: For unified TCI framework, the downlink TCI indicated by MAC CE and/or DCI format 0_1/0_2 should be applied for at least aperiodic CSI-RS in addition to PDCCH and PDSCH to avoid default CSI-RS and PDSCH beam mismatch, where the source reference signal should be periodic TRS.

In addition, it is agreed that the downlink TCI is applied at least for “dedicated PDCCH/PDSCH”. However, before UE detecting the PDCCH, there is no way to identify whether the PDCCH is dedicated or not. Currently, there are several types of SSs defined:

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI.

Dedicated PDCCH can be transmitted by Type3-PDCCH CSS and USS. Broadcast/groupcast signals can be transmitted by all types of CSS. All signals can be tied to a single CORESET. Thus, it is difficult to distinguish the common PDCCH and dedicated PDCCH. One simple way is to apply the unified TCI to all kinds of signals.

Proposal 2: For unified TCI framework, the downlink TCI indicated by MAC CE and/or DCI format 0_1/0_2 should be applied for all types of PDCCH and PDSCH.

## Uplink TCI state

In RAN1 #105, the following agreements on power control parameter configuration are agreed. There are still some remining issues on the details for power control parameter configuration.

Agreement On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state. - Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability - Note: The term “beam misalignment” is for discussion purpose only - Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2 - The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state - The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability - FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration - FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS - Note: PL-RS is assumed to be periodic Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, - For each of PUSCH and PUCCH, the setting of (P0, alpha, closed loop index) can be associated with UL or (if applicable) joint TCI state per BWP. - In this case, multiple settings are configured. Each setting can be associated with at least one TCI state, and, for a given TCI state, only one setting for PUSCH and only one setting for PUCCH can be associated at a time. - (Working Assumption) In this case, for each of the PUSCH and PUCCH, each of the activated UL or (if applicable) joint TCI states is associated with one of the settings. - If not associated, for each of the PUSCH and PUCCH, the setting(s) of (P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or (if applicable) joint TCI states - FFS: If the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state. - FFS: (to be decided in RAN1#106-e) whether to configure the same setting of (P0, alpha, closed loop index) per TCI state across channels and apply a channel dependent component, or configure a channel dependent setting of (P0, alpha, closed loop index) per TCI state

The first issue is the definition of “beam mis-alignment”. Since it is related to UE capability and would have no impact on other issues, it is better to decide it in UE capability session.

Proposal 3: Definition of “beam mis-alignment” shall be discussed in UE feature session.

The second issue is the details for PL-RS. In Rel-15, the L1-RSRP is measured based on SSB and CSI-RS for BM. Since higher-layer filtered RSRP should be measured based on L1-RSRP, it is better that the RS for PL-RS measurement should be the RS for L1-RSRP measurement. Meanwhile, 2-port CSI-RS has not been widely used since Rel-15, which would bring in additional UE complexity. It is better to preclude 2-port CSI-RS.

Proposal 4: The PL-RS should be SSB or periodic 1-port CSI-RS for BM.

The third issue is on the association between unified TCI and other power control parameters. There is a working assumption that each TCI is associated with one of the settings. However, with regard to different performance requirement for URLLC and eMBB, it is better that one TCI can be associated with 2 power control parameter sets, where the set can be dynamically selected by DCI.

Proposal 5: With regard to different performance requirement for URLLC and eMBB, support one unified UL/joint TCI state to be associated with 2 sets of power control parameters, i.e. P0, alpha and closed-loop process index, and support to select the power control parameter set by DCI.

## TCI state pool configuration

In RAN1 #105, the following is agreed for TCI state pool configuration.

Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

The working assumption is quite helpful to reduce RRC overhead. Since the unified TCI is applied for CCs within a band, the CCs/BWPs that can share TCI states should be the CCs that within a band at least. Thus, the working assumption should be configured by changing “[configured] CCs/BWPs” into “CCs/BWPs at least within a band”.

Proposal 6: Confirm the working assumption on TCI state pool configuration below with modification

Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs at least within a band: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

# Issue 2: L1/L2 Centric Inter-Cell Mobility

## Scenarios

The RRC parameters update should depend on the scenarios to be supported. With regard to different backhaul mode, there can be the following 3 modes on RRC parameters update on inter-cell mobility.

- Mode 1: UE maintains all L1/L2/L3 parameters without serving cell change

- Mode 2: UE maintains all L2/L3 parameters and updates L1 parameters based on a BWP switching without serving cell change

- Mode 3: UE updates all L1/L2/L3 parameters with serving cell change

Figure 2: Mode 1

Figure 3: Mode 2

Figure 4: Mode 3

In RAN #92, serving cell change (Scenario 2) is down-scoped. Without serving cell change (Scenario 1), only Mode 1 and Mode 2 can be considered in Rel-17.

Proposal 7: For L1/L2 centric inter-cell mobility scenario 1, RAN1 can consider the following modes

- Mode 1: UE maintains all L1/L2/L3 parameters without serving cell change

- Mode 2: UE maintains all L2/L3 parameters and updates L1 parameters based on a BWP switching without serving cell change

## Measurement and tracking

To support L1/L2 centric mobility, there can be the 3 types of cells:

- Type 1 Cell: Serving cell: The cell that UE is communicating with.

- Type 2 Cell: Candidate cell: The cell that UE may potentially switch to, which can be considered as a cell with a L3 measurement result above a threshold

- Type 3 Cell: Other non-serving cell: The cell that UE may not switch to, which can be considered as a cell with a L3 measurement result below a threshold

For different cells, UE measurement and tracking behavior could be different. Table 1 illustrates potential UE behavior for different cells.

Table 1: Potential UE behavior on measurement and tracking for different types of cell

|  | Serving cell | Candidate cell | Other non-serving cell |
| --- | --- | --- | --- |
| L1 measurement | Yes | Yes | No |
| L3 measurement | Yes | Yes | Yes |
| Fine Time/Frequency tracking | Yes | Optional | No |
| Beam tracking | Yes | Optional | No |

As the status candidate cell and other non-serving cell may be switched quickly, to avoid RRC reconfiguration, gNB can provide CSI-reportConfig related configuration for all the non-serving cells by RRC and use MAC CE to dynamically activate or deactivate CSI-reportConfig for some non-serving cells based on some rough L3 measurement results. The cell with an active CSI-reportConfig can be considered as a candidate cell.

Proposal 8: UE is only required to perform L1 measurement on candidate cell instead of all non-serving cells

- A candidate cell can be defined as a cell that UE may potentially switch to, which is with a L3 measurement result above a threshold

Proposal 9: Support MAC CE based dynamic activation/deactivation for a CSI-reportConfig corresponding to a non-serving cell

## Control signaling

To support inter-cell operation, another problem is about how to indicate the beam based on neighbor cell RS. To support such functionality, gNB should be able to provide the TCI indication based on SSB from a neighbor cell. Thus, the TCI can be divided into several groups, where each group is associated with a physical cell ID. As shown in Figure 5, TCI state x and y should belong to different TCI state groups, and after switching to TCI state y from TCI state x, UE is assumed to connect to the target gNB. Moreover, if target gNB uses different higher layer parameters, such higher layer parameters can be configured per TCI state group, and after TCI switching, gNB does not need to trigger additional RRC reconfiguration procedure, but UE can apply such RRC configurations associated with the new TCI state group automatically.

Figure 5: TCI configuration to support inter-cell mobility

Proposal 10: To facilitate L1/L2 centric inter-cell mobility, the TCI states can be divided into N groups, where each group is associated with one PCI.

## Event based beam report

The event-based report is widely used for normal handover procedure. For L1/L2 centric mobility, such event-based operation can still be used to facilitate the mobility management, where UE can report some beam information based on MAC CE. For example, when the RSRP from the neighbor cell beam is above the RSRP from current active beam plus an offset, UE can trigger such MAC CE report. Further, UE can also report such MAC CE when some uplink resources for PUSCH are available. Compared to L1 beam report, this can provide good flexibility for UE to report the beam quality.

Proposal 11: Support MAC CE based beam reporting to facilitate L1/L2 centric inter-cell mobility, which can be triggered by a particular event.

- The event could be the L1-RSRP measured from a neighbor cell SSB is above best L1-RSRP measured for current serving cell SSBs plus an offset

## TA measurement

With regard to different propagation delay for different links toward different cells, some TA adjustment should be required. To measure the TA, one possible way is to introduce 4-step RA procedure for L1/L2 inter-cell mobility, which is similar to handover, but it would increase the overall latency quite a lot. Another possible way is to use PDCCH ordered PRACH. Then before beam indication, gNB can measure the TA for the target cell and provide TA indication together with TCI indication. Thus, it is necessary to support PDCCH ordered neighbor cell PRACH for TA measurement.

Proposal 12: Support PDCCH ordered neighbor cell PRACH for TA measurement for L1/L2 centric inter-cell mobility.

## Backoff Mechanism

Currently the TCI update can be based on DCI or MAC CE, where an ACK/NACK can be reported. However, compared to traditional handover, the possibility of miss detection ratio for L1 ACK/NACK could be much higher. Thus, it is necessary to consider backoff mechanisms with regard to ACK/NACK miss detection. There can be the following cases:

- Case 1: ACK is detected as NACK/DTX. In this case, UE may start to communicate with the target cell, while network still tries to communicate with UE from the source cell

- Case 2: NACK/DTX is detected as ACK. In this case, UE may still maintain the communication with the source cell, while network starts to communicate with UE from the target cell.

Some backoff mechanisms needs to be studied with regard to the 2 cases above.

Proposal 13: Study backoff mechanism with regard to incorrect detection of ACK/NACK with the following cases:

- Case 1: ACK is detected as NACK/DTX. In this case, UE may start to communicate with the target cell, while network still tries to communicate with UE from the source cell

- Case 2: NACK/DTX is detected as ACK. In this case, UE may still maintain the communication with the source cell, while network starts to communicate with UE from the target cell.

# Issue 3: TCI update signaling

In RAN1 #103, it was agreed that DCI format 1_1 and 1_2 can be used for unified TCI indication, which can update the TCI for all CCs in a band.

## Action delay

Since the TCI update is based on a two-stage signalling, i.e. MAC CE+DCI. One open issue is how to interpret the TCI in DCI if the action delay for MAC CE happens in between the DCI and action time for the DCI as shown in Figure 6. To reduce the beam indication latency, it is better to select the TCI based on the latest MAC CE before the action time for the DCI.

Figure 6: Potential ambiguity for MAC CE + DCI based TCI indication

Proposal 14: The TCI in DCI should be selected based on the latest MAC CE before the action time for the DCI.

## ACK for DCI

Currently UE reports HARQ-ACK for the PDSCH triggered by DCI format 1_1/1_2. But when UE reports NACK, gNB cannot identify whether this NACK is because UE failed to decode PDSCH or PDCCH. Thus some ACK mechanisms for the TCI update DCI need to be introduced.

Usually, all the UE action regarding the DCI can be considered as ACK for the DCI. If the DCI triggered aperiodic SRS transmission and UE transmits the SRS, gNB can assume the DCI is received regardless of whether UE reports an ACK or NACK. In addition, for the downlink slots corresponding to an HARQ codebook, it is not necessary to trigger multiple TCI update signaling. So, one additional bit can be added in the HARQ codebook to indicate whether UE receives an TCI update signaling for the corresponding downlink slots as shown in Figure 7.

Figure 7: ACK mechanism for TCI update based on Type1 HARQ codebook, where the 5th bit indicates whether UE receives a TCI update or not

Proposal 15: UE shall expect gNB updates TCI at most once in downlink slots corresponding to a HARQ codebook.

- Introduce one additional bit in HARQ codebook to indicate whether UE receives a TCI update in the corresponding downlink slots

- If the DCI to update TCI triggers aperiodic SRS, the additional bit is not reported, where whether UE transmits SRS can be used to determine the HARQ-ACK for the DCI

## Extension to Multi-TRP Operation

To support multi-TRP operation, 2 TCI states need to be indicated. With regard to different backhaul scenario, there can be the following options:

- Option 1: One DCI is used to indicate a TCI for a TRP

  - The target TRP is the same as the one with the DCI transmission

- Option 2: One DCI is used to indicate 2 TCIs for 2 TRPs

  - One TCI code-point in DCI can be mapped to 2 TCI states in MAC CE

Option 1 can be used for non-ideal backhaul scenario, which is similar to multi-DCI operation, and option 2 could be beneficial for ideal backhaul scenario, which is with least of overhead. Therefore, both option 1 and option 2 can be supported with regard to different backhaul scenario.

Proposal 16: Support to extend unified TCI to multi-TRP operation based on the following options:

- Option 1: One DCI is used to indicate a TCI for a TRP

  - The target TRP is the same as the one with the DCI transmission

- Option 2: One DCI is used to indicate 2 TCIs for 2 TRPs

  - One TCI code-point in DCI can be mapped to 2 TCI states in MAC CE

## Band specific BFR based on unified TCI framework

The unified TCI has been agreed to be applied at least for all CCs within a band. However current BFR framework is still per CC basis. Such CC specific BFD and BFR would waste UE power quite a lot. Based on the principle of unified TCI state that a common beam should be applied for all CCs within a band, it is better to support band specific BFR compared to legacy CC specific BFR.

Proposal 17: Support band specific BFR based on unified TCI framework with regard to the principle of unified TCI framework that a common beam should be applied for all CCs within a band.

## Extension of TCI update for a band group

It was agreed that the TCI update should be applied for all CCs within a band. However, some bands may share the same antennas, and such bands should be based on a common beam as well. As the bands related aspects should be based on different implementation schemes, and it should depend on some RF related designs, it is better to send an LS to RAN4 about the definition of band group.

Proposal 18: Consider to extend the TCI update for a band(s) group

- The definition of band(s) group can be up to RAN4 and send an LS to RAN4

## QCL assumption during CBRA

For connected mode UE, after transmitting PRACH for CBRA procedure, UE can start to monitor the CORESETs associated with ra-SearchSpace based on the beam associated with PRACH. UE can monitor the Msg4 based on the beam associated with PRACH after it transmits Msg3. After decoding Msg4, UE can reset the beam to be based on the beam indicated by TCI. But there is no decoding latency reserved, so it may be possible that beam mismatch could happen within a time window after Msg3 and after Msg4. Further such behavior is not fully aligned with the principle for unified TCI state, where a common beam is applied for control and data channel in a band. Figure 8 illustrates legacy QCL assumption during CBRA.

Figure 8: Legacy QCL assumption during CBRA procedure

Proposal 19: Study to enhance the QCL assumption during and after CBRA procedure for connected mode UE with unified TCI framework to avoid potential beam mismatch and to maintain a common beam for a band.

## Beam indication signaling for inter-cell mobility

In RAN1 #105, the following working assumption is achieved for beam indication for inter-cell mobility.

Working Assumption On Rel.17 beam indication enhancements for L1/L2-centric inter-cell mobility, support the following: - Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) - FFS (to be decided in RAN1#106-e): Whether this also applies to PDSCH/PUSCH associated with UE-dedicated CORESETs only or additional target channels (e.g. UE-dedicated PDCCH/PUCCH) - FFS: Whether the above is supported only for joint TCI, or both joint TCI and separate DL/UL TCI (including that, if separate DL/UL TCI is supported, the DL TCI and UL TCI associated with a same cell) - FFS: Whether to support activation of TCI states for more than one cells simultaneously - FFS: Whether down-selection between MAC-CE only based and MAC-CE+DCI-based beam indication scheme is necessary - The DL QCL and UL spatial relation rules already agreed for intra-cell scenario - FFS: The use of SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference for UE-dedicated PDSCH - FFS (to be decided in RAN1#106-e): Whether this also applies to UE-dedicated PDCCH - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel - FFS (to be decided in RAN1#106-e): Whether SSB associated with a physical cell ID different from that of the serving cell can also be used as a direct QCL reference (source RS) for UE-dedicated PDCCH/PDSCH

The first open issue is the target channel. Since single active TCI state is the minimal UE capability widely used in current network, the inter-cell mobility should also support UE with capability of single active TCI state. Then to restrict the beam indication for some channel would break such kind of UE capability. Meanwhile, there is no concept like “UE-dedicated CORESET”. Since the non-serving cell SSB information is provided, UE can receive PDCCH in any CORESET with the TCI associated with the non-serving cell RSs. Thus, the indicated TCI should be applied for all data and control channels.

Proposal 20: For inter-cell mobility, the indicated unified TCI should be applied for all data and control channels.

# Issue 4: Fast UL Panel Selection

## UE initiated panel selection

In RAN1 #104b, the following agreement on UE initiated panel selection was reached.

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/reporting, down select and/or modify from the following candidates: - Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW - FFS: How to inform through CSI/beam reporting framework - FFS: Detailed design of the correspondence including the conveyed information - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16) - Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID - Note: The association between the new panel ID and the panel entity is determined by the UE - Opt1-3: No additional specification support - The duration in which the above panel entity reference is valid and the respective setting are FFS Note: “panel entity” is only used for discussion purpose

In beam reporting, UE can report the transmission process index associated with a DL RS. Then if the DL RS is configured in active TCI, UE can switch to the corresponding panel entity. Compared to Opt1-2, Opt1-1 could be more specific, and it allows UE to change the panel entity for a SSB/CSI-RS in different beam reporting instance. One additional aspect is that if gNB does not trigger beam report, UE may not be able to change the panel. Thus Opt1-1 cannot work without UE initiated beam report. In addition, If the UE panels are symmetric, where number of antenna ports for UE panels are the same, it is not necessary to define transparent UE panel selection.

Proposal 21: For UE initiated panel activation and selection, if UE initialted beam report and UE capability on maximum number of antenna ports/layers per panel entity are supported, Opt1-1 should be supported; Otherwise, Opt1-3 is supported.

## UE panel related information report

In RAN1 #104b, the following agreement on UE panel related information report was reached.

Agreement On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following: - UE reporting of panel-specific information as a UE capability, for example: - Information related to the total number of DL/UL panel entities - Information related to the number of (max) antenna ports/layers per panel entity - Information related to the maximum number of resources per panel entity for SRS BM - Information related to panel selection delay - Information related to panel activation delay - UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling - UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling) - UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status - Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation - Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed

As it has been agreed, different panels could be with different number of antenna ports, EIRP and number of beams. Thus, such information should be reported. In addition, the panel activation and selection delay could be important for TCI update. Since the panel activation/deactivation status could be different and there could be some intermediate state between activation and deactivation status for a panel, it is better to report the minimal delay dynamically. So, it is better to report such information as well.

Proposal 22: Support to report the following information as UE capability:

- Information related to the total number of DL/UL panel entities

- Information related to the number of (max) antenna ports/layers per panel entity

- Information related to the maximum number of resources per panel entity for SRS BM

Proposal 23: Support to report minimal activation/selection delay for a panel based on L1 or L2 signaling.

## Control signaling for panel selection

In RAN1 #104b, the following agreement on control signaling for panel selection for PUSCH was reached.

Agreement On Rel.17 enhancements for MPUE, for codebook based UL transmission, decide by August RAN1 meeting whether to support CB-based SRS resources with different numbers of ports - FFS details (e.g. per resource or per resource set) - Note: the above is not for Rel-16 full power transmission but for Rel-17 panel-specific UL transmission - FFS: non-codebook based UL transmission for MPUE - FFS whether existing BWP switch based mechanism (discussed previously in Rel-16 power saving WI) can serve such purpose

The first issue is to define the association between SRS and panel entity. As it was agreed in multi-TRP agenda, different SRS resource sets can be associated with different TRP, it is better to use the same approach for panel selection that each SRS resource set is associated with a panel entity. For single-TRP operation, only 1 SRS resource set can be considered to be a valid SRS resource set, which should be associated with the same panel as the PUSCH. In addition, since the beam/panel association for a SRS can be dynamically changed by DCI, it is better that the number of antenna ports for SRS can be dynamically changed.

Proposal 24: For SRS for CB/NCB, each SRS resource set is associated with a panel entity, and for single-TRP operation, only 1 SRS resource set can be valid for SRI indication.

- The valid SRS resource set is the one associated with the same panel as the TCI indicated for PUSCH

Proposal 25: Support to dynamically update the number of ports for SRS by TCI update signaling

# Issue 5: MPE Handling

As a result of maximum power emission (MPE), the maximum power reduction (MPR) for each panel could be different. As shown in Figure 9, sometimes due to physical panel separation, the beam in panel 2 may be still worse than panel 1, although additional P-MPR should be included for beams in panel 1. While sometimes, the beams in panel 2 may be better, where the RSRP difference between beams in panel 2 and panel 1 could be smaller than the MPR offset. So, in general, such P-MPR related information could help gNB to select the best beam.

Figure 9: Potential MPR difference for different UE panels due to MPE issue

In Rel-15/Rel-16, some solutions on MPE have been defined. One is to introduce a duty cycle. However, a clear uplink performance degradation can be observed with this method. Another is to report the P-MPR. but the problem is that gNB still does not know whether such P-MPR should be applied to candidate beams or not, and only P-MPR and L1-RSRP cannot help gNB to identify the best beam.

No matter whether there is a MPE issue or not, the best metric for beam selection should still be receiving power. Table 2 illustrates the system level simulation results for different beam selection schemes. The baseline is beam selection without panel switching, where UE would continue to use the beam with power back-off. The second scheme is to select the beam based on panel specific P-MPR, where the beam is selected based on L1-RSRP minus P-MPR. The third scheme is based on beam specific PHR, which takes PHR into account. The last scheme is based on UL Rx power, where it is assumed that the UE reports something that can help gNB to calculate the Rx power for a corresponding scheduled bandwidth. It can be observed that the UL Rx power based beam selection should still be the best scheme. The detailed simulation assumption is shown in Table A-1 in appendix.

Table 2: System level simulation results for different UL beam selection schemes

|  | average cell SE (bps/Hz) | average cell SE (%) | 5% CDF SE (bps/Hz) | 5% CDF SE (%) |
| --- | --- | --- | --- | --- |
| beam selection without panel switching (UE level P-MPR) | 3.6012 | 100.00% | 0.0855 | 100.00% |
| beam selection with panel specific P-MPR (DL-RSRP – P-MPR) | 4.1719 | 115.85% | 0.1243 | 145.36% |
| beam selection with beam specific PHR (If PHR < 0, PHR is used as metric; otherwise, DL-RSRP is used as metric) | 4.1584 | 115.47% | 0.1341 | 156.86% |
| beam selection based on UL Rx power (Actual UL-RSRP based beam selection) | 4.2253 | 117.33% | 0.139 | 162.57% |
|  | 50% CDF SE (bps/Hz) | 50% CDF SE (%) | 95% CDF SE (bps/Hz) | 95% CDF SE (%) |
| beam selection without panel switching (UE level P-MPR) | 0.3202 | 100.00% | 0.6542 | 100.00% |
| beam selection with panel specific P-MPR (DL-RSRP – P-MPR) | 0.3645 | 113.82% | 0.734 | 112.20% |
| beam selection with beam specific PHR (If PHR < 0, PHR is used as metric; otherwise, DL-RSRP is used as metric) | 0.3995 | 124.76% | 0.7175 | 109.68% |
| beam selection based on UL Rx power (Actual UL-RSRP based beam selection) | 0.407 | 127.09% | 0.7317 | 111.86% |

The UL receiving power for a resource element should be calculated as follows:

PRx=PTx-10log10(NSC)-PL

Where NSC indicates the total number of scheduled subcarriers, PL indicates the pathloss including antenna gain, and PTx denotes the transmission power, which can be obtained as follows:

PTx=min⁡{P0+10log10NRB×2u+α×PL+f+∆, Pcmax-PMPR}

Where P0 and α are configured by gNB, NRB denotes the number of scheduled RBs, u is the subcarrier spacing scaling factor, f is the closed-loop power control factors and ∆ refers to the MCS compensation factor.

Thus, the receiving power could be calculated as follows:

PRx=Pcmax-PMPR-10log10NSC-PL,                       if PTx≥Pcmax-PMPRP0+10log102uNSCRB+α-1PL+f+∆,  if PTx<Pcmax-PMPR

Where NSCRB denotes number of subcarriers per RB.

It can be observed that the receiving power per resource element should be determined based on the number of scheduled RBs, P-MPR, Pcmax, pathloss, closed-loop power control factor as well as MCS compensation factor. Then the beam reporting can report something for gNB to calculate the receiving power per resource element. A starting point could be to report virtual PHR, Pcmax, P-MPR as well as L1-RSRP, where the virtual PHR can be calculated as follows:

PH=Pcmax-PMPR-{P0+α×PL+f+∆}

Where for fast beam measurement and reporting, PL can be calculated based on the L1-RSRP as well as the transmission power for the measured SSB/CSI-RS.

Usually, the UL and DL beam should be the same. But with regard to MPE issue, UE may apply some power back-off for some UL beams. Such power back-off may not change the UL beam selection decision if the UE locates in the cell center. However, if the UE is in power limit status, some impact from the power back-off would be observed as shown in Figure 10. Thus, the best UL beam could be different for different scheduled bandwidth if some power back-off needs to be applied due to MPE issue. For smaller bandwidth, the best UL beam could be the same as the DL beam, but for larger bandwidth, UE may choose a different beam for uplink transmission. For unified TCI framework, gNB can provide a bandwidth range for the TCI state, and the TCI can only be applicable for the uplink channel withing the bandwidth range.

Figure 10: Uplink beam indication with regard to MPE impact

In RAN1 #104b, the following agreement on MPE was reached.

Agreement On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options: - Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported. - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - FFS: how to determine the virtual PHR or the modified version. - Opt 1D. {Rel.16 P-MPR based (beam/panel-level)} - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting FFS: If differential report is supported when multiple UL beams are reported in the same report

As shown in the simulation results, the best metric for UL beam selection with regard to MPE is the uplink RSRP. However the UL RSRP depends on the power control parameters and scheduling bandwidth. Thus the UE report should provide enough information for gNB to calculate the UL RSRP. As discussed above, based on PHR and DL L1-RSRP, it is possible for gNB to calculate the UL RSRP for different scheduling decisions.

Proposal 26: With regard to MPE impact, support beam report enhancement to facilitate UL receiving power based beam selection.

- Support Opt 1A + Opt 2A, where UE reports Pcmax with P-MPR included, virtual PHR and L1-RSRP for SSB/CSI-RS in a beam reporting instance

- The virtual PHR should be calculated based on the measured L1-RSRP

# Issue 6: Beam tracking latency reduction

In RAN1 #105, the following agreement on beam tracking latency reduction was reached.

Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, focus study (including down-selection) and, if needed, specification effort on the following options: - Group 1: Aim for at most one solution for Group 1 in Rel-17 to address issue 6 - Opt 1-A. UE-initiated beam selection/activation based on beam measurement and/or reporting (without beam indication or activation from NW) - Opt 1-B. Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request) - Opt 1-C. Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency - Group 2: Aim for at most one solution for Group 2 in Rel-17 to address issue 6 - Opt 2-A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking - Opt 2-B: Latency reduction for MAC CE based PL-RS activation - Opt 2-C: One-shot timing update for TCI state update

In Group 1, Opt 1-B is covered by Opt 2-A in Group 2. Opt 1-A and Opt 1-C are used for different purposes. Opt 1-A is used to reduce beam indication overhead and beam update latency, while Opt 1-C is used for joint P2/P3 to reduce the beam refinement latency. To trigger multiple CSI reports can achieve the same functionality as Opt 1-C. Compared to Opt 1-C, Opt 1-A can help to reduce system overhead and latency quite a lot. Thus for Group 1, Opt 1-A is preferred. The starting point for Opt 1-A could be CBRA procedure, and after CBRA, UE can update beam for both control and data channel to be based on the associated SSB for PRACH. In addition, the normal beam report can also be considered, but since the beam report is transmitted by PUCCH or UCI in PUSCH, some acknowledgement mechanism needs to be considered.

Proposal 27: For advanced beam refinement/tracking Group 1, support Opt 1-A (UE-initiated beam selection/activation based on beam measurement and/or reporting (without beam indication or activation from NW))

- For Opt 1-A, after CBRA procedure, UE can update the beam for both control and data channel to be based on the associated SSB for PRACH

For Group 2, among the 3 options, Opt 2-A should be the most important. For Opt 2-B, there would be no way to reduce the latency for PL-RS activation unless new metric, e.g. L1-RSRP, can be introduced. Opt 2-C is not the most critical issue for TCI update. Currently for unknown TCI indication, additional delay for UE beam tracking is needed. For example, UE may need to track the SSB 8 times to identify the best UE beam, which could lead to at least 160ms additional delay. Therefore, to reduce the TCI indication delay, one possible way is to trigger aperiodic CSI-RS for beam tracking and time/frequency offset tracking based on the TCI switching signaling.

In addition, since SSB can also be used for time/frequency offset tracking, the actual delay for the TCI update should depend on the first SSB/TRS instance, where the action delay for the TCI update should be X slots after the first SSB/TRS, where X is the processing delay for the SSB/TRS reception.

Proposal 28: For advanced beam refinement/tracking Group 2, support Opt 2-A (Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking)

- Support to trigger aperiodic CSI-RS resources for beam/time/frequency offset tracking by the MAC CE or DCI for TCI indication

# Conclusion

In this contribution, we discussed some remaining issues on beam management enhancement. Based on the discussion, the following proposals have been achieved.

Issue 1

Proposal 1: For unified TCI framework, the downlink TCI indicated by MAC CE and/or DCI format 0_1/0_2 should be applied for at least aperiodic CSI-RS in addition to PDCCH and PDSCH to avoid default CSI-RS and PDSCH beam mismatch, where the source reference signal should be periodic TRS.

Proposal 2: For unified TCI framework, the downlink TCI indicated by MAC CE and/or DCI format 0_1/0_2 should be applied for all types of PDCCH and PDSCH.

Proposal 3: Definition of “beam mis-alignment” shall be discussed in UE feature session.

Proposal 4: The PL-RS should be SSB or periodic 1-port CSI-RS for BM.

Proposal 5: With regard to different performance requirement for URLLC and eMBB, support one unified UL/joint TCI state to be associated with 2 sets of power control parameters, i.e. P0, alpha and closed-loop process index, and support to select the power control parameter set by DCI.

Proposal 6: Confirm the working assumption on TCI state pool configuration below with modification

Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs at least within a band: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

Issue 2

Proposal 7: For L1/L2 centric inter-cell mobility scenario 1, RAN1 can consider the following modes

- Mode 1: UE maintains all L1/L2/L3 parameters without serving cell change

- Mode 2: UE maintains all L2/L3 parameters and updates L1 parameters based on a BWP switching without serving cell change

Proposal 8: UE is only required to perform L1 measurement on candidate cell instead of all non-serving cells

- A candidate cell can be defined as a cell that UE may potentially switch to, which is with a L3 measurement result above a threshold

Proposal 9: Support MAC CE based dynamic activation/deactivation for a CSI-reportConfig corresponding to a non-serving cell

Proposal 10: To facilitate L1/L2 centric inter-cell mobility, the TCI states can be divided into N groups, where each group is associated with one PCI.

Proposal 11: Support MAC CE based beam reporting to facilitate L1/L2 centric inter-cell mobility, which can be triggered by a particular event.

- The event could be the L1-RSRP measured from a neighbor cell SSB is above best L1-RSRP measured for current serving cell SSBs plus an offset

Proposal 12: Support PDCCH ordered neighbor cell PRACH for TA measurement for L1/L2 centric inter-cell mobility.

Proposal 13: Study backoff mechanism with regard to incorrect detection of ACK/NACK with the following cases:

- Case 1: ACK is detected as NACK/DTX. In this case, UE may start to communicate with the target cell, while network still tries to communicate with UE from the source cell

- Case 2: NACK/DTX is detected as ACK. In this case, UE may still maintain the communication with the source cell, while network starts to communicate with UE from the target cell.

Issue 3

Proposal 14: The TCI in DCI should be selected based on the latest MAC CE before the action time for the DCI.

Proposal 15: UE shall expect gNB updates TCI at most once in downlink slots corresponding to a HARQ codebook.

- Introduce one additional bit in HARQ codebook to indicate whether UE receives a TCI update in the corresponding downlink slots

- If the DCI to update TCI triggers aperiodic SRS, the additional bit is not reported, where whether UE transmits SRS can be used to determine the HARQ-ACK for the DCI

Proposal 16: Support to extend unified TCI to multi-TRP operation based on the following options:

- Option 1: One DCI is used to indicate a TCI for a TRP

  - The target TRP is the same as the one with the DCI transmission

- Option 2: One DCI is used to indicate 2 TCIs for 2 TRPs

  - One TCI code-point in DCI can be mapped to 2 TCI states in MAC CE

Proposal 17: Support band specific BFR based on unified TCI framework with regard to the principle of unified TCI framework that a common beam should be applied for all CCs within a band.

Proposal 18: Consider to extend the TCI update for a band(s) group

- The definition of band(s) group can be up to RAN4 and send an LS to RAN4

Proposal 19: Study to enhance the QCL assumption during and after CBRA procedure for connected mode UE with unified TCI framework to avoid potential beam mismatch and to maintain a common beam for a band.

Proposal 20: For inter-cell mobility, the indicated unified TCI should be applied for all data and control channels.

Issue 4

Proposal 21: For UE initiated panel activation and selection, if UE initialted beam report and UE capability on maximum number of antenna ports/layers per panel entity are supported, Opt1-1 should be supported; Otherwise, Opt1-3 is supported.

Proposal 22: Support to report the following information as UE capability:

- Information related to the total number of DL/UL panel entities

- Information related to the number of (max) antenna ports/layers per panel entity

- Information related to the maximum number of resources per panel entity for SRS BM

Proposal 23: Support to report minimal activation/selection delay for a panel based on L1 or L2 signaling.

Proposal 24: For SRS for CB/NCB, each SRS resource set is associated with a panel entity, and for single-TRP operation, only 1 SRS resource set can be valid for SRI indication.

- The valid SRS resource set is the one associated with the same panel as the TCI indicated for PUSCH

Proposal 25: Support to dynamically update the number of ports for SRS by TCI update signaling

Issue 5

Proposal 27: With regard to MPE impact, support beam report enhancement to facilitate UL receiving power based beam selection.

- Support Opt 1A + Opt 2A, where UE reports Pcmax with P-MPR included, virtual PHR and L1-RSRP for SSB/CSI-RS in a beam reporting instance

- The virtual PHR should be calculated based on the measured L1-RSRP

Issue 6

Proposal 27: For advanced beam refinement/tracking Group 1, support Opt 1-A (UE-initiated beam selection/activation based on beam measurement and/or reporting (without beam indication or activation from NW))

- For Opt 1-A, after CBRA procedure, UE can update the beam for both control and data channel to be based on the associated SSB for PRACH

Proposal 28: For advanced beam refinement/tracking Group 2, support Opt 2-A (Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking)

- Support to trigger aperiodic CSI-RS resources for beam/time/frequency offset tracking by the MAC CE or DCI for TCI indication

# Appendix – Simulation Assumption

Table A-1: Simulation Assumption for Beam Management for issue 5

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban Macro |
| Number of BS | 21 |
| Number of UE | 210 |
| gNB antenna architecture (M, N, P, Q) | (4, 8, 2, 2) |
| UE antenna architecture (M, N, P, Q, Mg, Ng) | (1, 4, 2, 2, 1, 3) |
| UE speed | 3 km/h for indoor and 30km/h for outdoor |
| Indoor UE percentage | 80% |
| System bandwidth | 80 MHz |
| Handover margin | 0 dB |
| Carrier frequency | 30 GHz |
| gNB noise figure | 7 dB |
| UE maximum Tx power | 23 dBm |
| UL power control parameters | P0 = -90dBm, alpha = 0.9 |
| Subcarrier spacing | 120 kHz |
| Blockage modeling | Randomly selecting 1 panel to be blocked with additional 10dB pathloss |
| Power back-off | P-MPR = 10dB for the blocked panel |
| UE and panel orientation | Vertical but random in azimuth |
| Traffic model | Full buffer |
| MIMO mode | SU-MIMO |
| Transmission scheme | Codebook based |
| Scheduler | PF |