3GPP TSG RAN WG1 #106-e	                                         	      R1-2107723

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.3

Source:	Apple Inc.

Title:	Views on Rel-17 SRS enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding SRS enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In the last RAN1 meeting RAN1#104be, the following agreement is reached [2]

Agreement For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - N_symbol = 8, R = {1, 2, 4, 8} - N_symbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1 Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - [4T8R: N_max = 2] - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed R1-2103878 FL summary #2 on SRS enhancements Moderator (ZTE) Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the RBs in the RBs is , where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition Working Assumption For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs Agreement - Up to 4 “t” values can be configured per SRS resource set. R1-2103928 FL summary #3 on SRS enhancements Moderator (ZTE) Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1: is an integer value - Alt 2: is an integer value with minimum value 4 - Alt 3: is a multiple of 4 - Alt 4: Round to a multiple of 4 in case of Alt 1 or Alt 2 Agreement On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2 Agreement For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e) - Alt 1: Generate length- ZC sequence - Alt 2: Truncate from legacy length- sequence according to the location of RPFS SRS Agreement For antenna switching, support one of the following - Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set - Note: the two SP-SRS resource sets are not activated at the same time. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed - FFS whether configurations on SRS repetitions have impact - FFS relevant UE capability design

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement

# AP-SRS triggering flexibility enhancement

In the last meeting, we agreed to introduce new DCI field to dynamically indicate the slot offset for AP-SRS. We  also agreed to allow DCI format 0_1 and 0_2 to trigger AP-SRS without AP-CSI and without data, following proposal address the remaining issue

Proposal 1 When DCI format 0_1 and 0_2 is used to trigger AP-SRS without data and without CSI, do not

re-purpose unused fields in DCI format 0_1 and 0_2

In the last meeting, companies also discussed that when single DCI triggers multiple AP-SRS resource sets, if some AP-SRS resource sets collide in the same symbol, whether we should introduce any specific collision handling

Proposal 2 When single DCI triggers multiple AP-SRS resource sets, if some AP-SRS resource sets collide in the same symbol, regarding collision handling

- Do not introduce special collision handling between different AP-SRS resource sets triggered by the same DCI

- If any collision handling is specified, it is UE optional feature

# SRS resource set with multiple usage

In Rel-15/16, SRS-ResourceSet can only be configured with 4 possible “usage” among {beamManagement, codebook, nonCodebook, antennaSwitching}, and, for each SRS-ResourceSet, only one usage can be configured.

For example, when SRS-ResourceSet is configured with “usage” of “antennaSwitching”, it is intended to be used for reciprocity-based DL channel acquisition. However, the same SRS-ResourceSet may also be allowed to be used for antenna selection to be combined with other use cases, such as, codebook based or non-codebook based PUSCH operation. Therefore, we have the following proposal

Proposal 3 In Rel-17, support to use SRS-ResourceSet configured with usage = “antennaSwitching” for codebook based PUSCH scheduling

- This feature is RRC explicitly configured

- This feature is optional with capability signalling for the UE

- FFS: when multiple SRS-ResourceSet are configured with usage = “antennaSwitching”

- FFS: when SRS-ResourceSet configured with usage = “antennaSwitching” contains more than 2 SRS-Resource

For the dynamic antenna switching SRS, we have the following proposal

Proposal 4 In Rel-17, consider to allow UE to indicate the preferred antenna switching configuration in terms of nTmR

# SRS capacity and coverage enhancement

In the last meeting, we agreed on three solutions to enhance the SRS capability and coverage, we have the following proposing to address some of the remaining issues

Proposal 5 For RB-level partial frequency sounding (RPFS),

- For the number of SRS PRB, i.e. 1PFmSRS,BSRS , only support the number of SRS PRB supported by the current specification without RPFS

- For the SRS sequence length, do not support truncation, only support the SRS sequence length supported by the current specification

Proposal 6 For SRS with Combo 8, support maximum number of  cyclic shift of 6

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement with the following proposals

Proposal 1 When DCI format 0_1 and 0_2 is used to trigger AP-SRS without data and without CSI, do not

re-purpose unused fields in DCI format 0_1 and 0_2

Proposal 2 When single DCI triggers multiple AP-SRS resource sets, if some AP-SRS resource sets collide in the same symbol, regarding collision handling

- Do not introduce special collision handling between different AP-SRS resource sets triggered by the same DCI

- If any collision handling is specified, it is UE optional feature

Proposal 3 In Rel-17, support to use SRS-ResourceSet configured with usage = “antennaSwitching” for codebook based PUSCH scheduling

- This feature is RRC explicitly configured

- This feature is optional with capability signalling for the UE

- FFS: when multiple SRS-ResourceSet are configured with usage = “antennaSwitching”

- FFS: when SRS-ResourceSet configured with usage = “antennaSwitching” contains more than 2 SRS-Resource

Proposal 4 In Rel-17, consider to allow UE to indicate the preferred antenna switching configuration in terms of nTmR

Proposal 5For RB-level partial frequency sounding (RPFS),

- For the number of SRS PRB, i.e. 1PFmSRS,BSRS , only support the number of SRS PRB supported by the current specification without RPFS

- For the SRS sequence length, do not support truncation, only support the SRS sequence length supported by the current specification

Proposal 6 For SRS with Combo 8, support maximum number of  cyclic shift of 6

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #104b, e-Meeting, January 25th – February 5th, 2021