3GPP TSG RAN WG1 #106-e			R1-2107736

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.4.1

Source:	Apple

Title:	On Timing Relationship Enhancements for NR NTN

Document for:	Discussion/Decision

# Introduction

One of the RAN1-lead objectives in the NTN WID [1] is timing relationship enhancements for NTN. The timing relationship enhancement is related to introducing an additional time offset to compensate the large propagation delay in NTN. A list of topics regarding timing relationship enhancements was discussed [2]. The following agreements have been made in RAN1 #105-e meeting [3].

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot , where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  NTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of  and K_mac.  How to treat  and  can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

In this contribution, we provide our views on the topics of time offset in initial access, time offset after initial access, MAC CE activation time, RAR window starting time, K1 range extension and timing relationship enhancement for beam failure recovery.

# Discussion

## Time offset in initial access

It was agreed [5] that a cell specific Koffset is configured in system information and is used in initial access. It was agreed [4] that this cell specific Koffset is used for all timing relationships that require Koffset enhancement, if UE is not provided with Koffset value other than one signaled in system information.

On the cell specific Koffset configured in system information, two options of signaling were identified in RAN1 #104b-e meeting [4]. Option 1 is to signal a single offset value of Koffset, which covers the largest round-trip time (RTT) of service link and the RTT between satellite and timing reference point.

Option 2 is to signal a first offset value Koffset1 to cover the RTT between satellite and timing reference point, and a second offset value Koffset2 to cover the largest RTT of service link. The final Koffset is equal to the sum of Koffset1 and Koffset2. Here, the first offset value Koffset1 may be signaled or may be determined by common TA broadcasted by network. The benefit of Option 2 is that if Koffset1 is determined by the broadcasted common TA, then only Koffset2 needs to be signaled, which has a smaller value range than Koffset and hence has a reduced payload size from Koffset.

The signaling of RTT between satellite and timing reference point is still in discussion under AI 8.4.2. Although a common TA broadcast is supported, whether or not to broadcast common TA drift rate and its high order derivatives is still open. If the common TA drift rate and its high order derivatives are broadcasted, then Koffset1 may be maintained by UE, based on broadcasted common TA parameters. This is not desirable since it is not guaranteed that the same Koffset1 is maintained by all UEs. Furthermore, the specification impact and UE complexity of Option 2 is higher than Option 1.

On the other hand, the signaling overhead saving from Koffset to Koffset2 may not be significant. It is likely the range of Koffset is no more than two times of the range of Koffset2. This implies the signaling saving is only about 1 bit. The 1 bit saving in higher layer signaling is negligible. Hence, we prefer Option 1 of Koffset signaling, i.e., a single offset value of Koffset, due to its specification simplicity and non-ambiguous value on the cell specific Koffset.

Proposal 1: Support to signal a single offset value for cell specific Koffset.

## Time offset after initial access

The update of Koffset after initial access is supported [5]. Two signaling options of updating Koffset were identified [3]: RRC reconfiguration or MAC CE, where at least one option is supported.

In our view, both options are able to reliably deliver the updated Koffset value. MAC CE has the benefit of lower latency and smaller signaling overhead, while RRC reconfiguration has less specification impact.

Depending on how often the Koffset needs to be updated, both RRC reconfiguration and MAC CE can be supported. For example, when Koffset is updated frequently (e.g., in LEO scenario), MAC CE is suitable due to its lower latency. When Koffset is updated infrequently (e.g., in GEO scenario), RRC reconfiguration can be used.

Proposal 2: For signaling of updating Koffset after initial access, both RRC reconfiguration and MAC CE are supported.

Based on the signaling of updating Koffset after initial access, the updated Koffset is likely to be UE specific. The value of UE specific Koffset is supposed to be larger than UE’s full timing advance (TA) in order to achieve casual scheduling.

The UE’s full TA is composed of UE specific TA, broadcasted common TA, as well as TA command received from gNB. The UE specific TA depends on the distance between UE and serving satellite, which is calculated by UE based on its GNSS location and serving satellite ephemeris. Note that gNB is unable to autonomously derive UE specific TA, without the knowledge of UE’s GNSS location. If UE does not report the information about UE specific TA to gNB, then gNB is unable to know whether and how to update UE specific Koffset.

In the RAN2 LS [6], it was mentioned that UE may report information about the UE specific TA pre-compensation. The exact content and frequency of UE reporting of information about the UE specific TA pre-compensation are based on RAN1’s input.

Several options of the exact content of UE reporting to gNB were discussed [2]. These includes UE specific TA, full TA, UE location, difference between UE specific Koffset and cell specific Koffset.

In our view, the main purpose of UE reporting to gNB is to facilitate gNB’s derivation of UE specific Koffset for uplink scheduling adaptation. Since Koffset is assumed to have granularity of slots, the UE’s reporting does not need to have a smaller granularity. This could save the signaling overhead of UE reporting. In this sense, it is preferred for UE to report suggested UE specific Koffset directly.

The signaling overhead could be further reduced if differential value is reported. Specifically, UE may compare current UE specific Koffset (or cell specific Koffset, if it is not updated) with its full TA. If the difference between them is beyond a certain range, then UE knows the necessity of updating UE specific Koffset. Otherwise, there is no need for UE to update UE specific Koffset. Instead of reporting a preferred UE specific Koffset, UE could report the difference between its preferred UE specific Koffset and the current UE specific Koffset. This could further reduce the signaling overhead of UE reporting.

Proposal 3: Support UE reports the preferred differentiated UE specific Koffset for the purpose of uplink scheduling adaptation.

Regarding the UE reporting frequency, we think at least the event triggered approach should be supported. Basically, a UE reports to gNB only if it detects the necessity of updating UE specific Koffset. This could largely save the signaling of UE reporting. The transmission of differentiated UE specific Koffset could be via high layer signaling.

There is a proposal of periodic UE reporting. Consider a LEO satellite with speed of 7.56 km/s. It will take about 20 seconds to make 1 ms TA difference. If UE reports the differentiated UE specific Koffset with granularity of 1 slot, then the reporting periodicity is as large as 20 seconds for subcarrier spacing of 15 kHz. It may not be beneficial to support UE’s periodic reporting with such a large periodicity.

Proposal 4: Support at least event-triggered UE reporting for the purpose of uplink scheduling adaptation.

The usage of cell specific Koffset has been discussed, in the context of updating Koffset after initial access. It was agreed [3] that the Koffset value signaled in system information is always used for initial access procedure when UE is in RRC Idle state, e.g., RAR/fallbackRAR grant scheduled PUSCH, Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, etc.

It is open whether the cell specific Koffset value is used in determining the transmission timing of HARQ-ACK on PUCCH to Msg4/MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI. In our view, if the CRC of DCI format 1_0 is scrambled by C-RNTI, then there is no ambiguity between gNB and UE on the updated value of Koffset. Hence, there is no need to apply cell specific Koffset in this case.

Proposal 5: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timing of HARQ-ACK on PUCCH to Msg4/MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

It is open whether the cell specific Koffset value is always used in determining the uplink transmission timing scheduled by fallback DCI formats. The main motivation of using only cell specific Koffset value for fallback DCI is to avoid the ambiguity of Koffset, during the Koffset value update progress. This guarantees UE is always reachable. However, if we consider high layer signaling of Koffset update (e.g., MAC CE or RRC reconfiguration), the activation time is fixed and ambiguity does not exist. Hence, we prefer to apply the Koffset value not signaled in system information for the transmission timings related to fallback DCI formats, due to its efficiency.

Proposal 6: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timings scheduled by fallback DCI formats.

## MAC CE activation time

It was agreed [5] that the activation timing for downlink configuration from a MAC CE command depends on a scheduling offset Kmac, which is different from Koffset. It was agreed [4] that Kmac value can be provided by network. The MAC CE activation time based on Kmac was agreed [3], where Kmac is assumed to have the unit of PUCCH slot. The detailed signaling of Kmac needs to be specified.

It was agreed [3] that Kmac is used to estimate the UE-gNB RTT, which is set as the ra-ResponseWindow offset or msgB-ResponseWindow offset. This implies Kmac value needs to be known before UE’s initial access. Hence, it is natural that Kmac is carried in system information.

On the other hand, the Kmac used to determine MAC CE activation time is in the unit of PUCCH slot. Note the numerology of PUCCH may be UE dependent. In other words, the Kmac carried in system information may not be aligned with numerology of UE specific PUCCH.

Regarding the signaling of Kmac, one way is the Kmac carried in system information is in absolute time (e.g., ms). Another way is the Kmac carried in system information is in the unit of a given subcarrier spacing in a given frequency range. For example, Kmac is in the unit of slots for the subcarrier spacing of 15 kHz if the carrier frequency is below 7.125 GHz and Kmac is in the unit of slots for the subcarrier spacing of 120 kHz if the carrier frequency is above 7.125 GHz.

Proposal 7: The scheduling offset Kmac is carried in system information, in the unit of milli-seconds or in the unit of slots for a given subcarrier spacing in a frequency range.

## RAR window starting time

It was agreed [3] that the RAR window offset for both 2-step RACH and 4-step RACH is equal to an estimate of UE-gNB RTT, which is the sum of UE’s TA and Kmac. The UE’s TA is based on the formula TTA= NTA+NTA,UE-specific +NTA, common+NTA,offset× TC. It is open how to determine NTA and NTA,offset in the formula.

In our view, the NTA should be set to 0 in estimating the UE-gNB RTT, which is aligned with PRACH transmissions. Since there is no TA command available when UE estimate the RAR window offset, it is natural to assume NTA as 0. The value of NTA,offset could be broadcasted (e.g., servingCellConfigCommon) or could be a default value for a given frequency range (cf. [7]).

Proposal 8: In estimating the UE-gNB RTT for RAR window offset, set NTA to 0 and set NTA,  offset based on the signaled value or default value for a given frequency range.

## K1 range extension

It was agreed [5] that the value range of K1 is extended to 31. It is open whether this value range extension has impact on the size of PDSCH-to-HARQ_feedback timing indicator field in DCI. In our view, this field size should remain unchanged in DCI. In DCI formats 1_1 or 1_2, this field size is upper bounded by 3 bits, where the actual field size depends on the number of entries in a high layer configured table. The similar upper bound could be used for extended K1 values. Only the value range of the entries in the configured table needs to be updated.

In DCI format 1_0, the field size is fixed to 3 bits to indicate the value of K1 from 1 to 8 slots. It is worth checking whether K1 range indicated by DCI format 1_0 needs to be extended beyond 8 slots. No matter whether the K1 range extension for DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field size should be remained to 3 bits.

Proposal 9: The K1 range extension does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

## Timing relationship enhancement for beam failure recovery

In NR beam failure recovery procedure [8], a UE sends beam failure recovery request to gNB to indicate PCell of PSCell beam failure and provide new beam index. The beam failure recovery request is sent via PRACH in slot n. Then the UE monitors PDCCH in dedicated search space configured for receiving beam failure recovery response from gNB, starting from slot n+4. If the UE does not receive beam failure recovery response within a configured time window, then it re-sends the beam failure recovery request or fallbacks to contention based PRACH based beam failure recovery.

TS 38.213 Section 6 For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in Clause 8.1. For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4 within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

It is not crystal clear whether the “slot n” or “slot n+4” in the specifications refers to uplink slot or downlink slot. Different interpretations may result in large difference on the beam failure recovery response window starting time, due to the large TA in NTN.

One interpretation is that the beam failure recover request is sent via PRACH in uplink slot n and UE monitors PDCCH for receiving beam failure recovery response starting from downlink slot n+4. Considering the case where downlink and uplink frame timings are not aligned at gNB. The beam failure recovery request has to reach gNB before it can send back beam failure recovery response. Hence, the additional Kmac needs to be added to enhance the time relationship on the beam failure recovery response window offset. This is similar to RAR window starting time, where Kmac is used to capture the RTT between timing reference point and gNB. Specifically, we should clarify that UE monitors PDCCH starting from the first slot after uplink slot n+4+Kmac. With the enhancement of beam failure recovery response window offset, it is unnecessarily to enlarge the existing beam failure recovery response window size.

Proposal 10: RAN1 to enhance the timing relationship on the beam failure recovery response window offset, by introducing Kmac.

Furthermore, in NR terrestrial network, when a UE receives a first PDCCH in the dedicated search space set configured for beam failure recovery, it resets its PUCCH transmission beam based on the newly identified beam after 28 symbols after the last symbol of the PDCCH. This also implies the time from which gNB adjusts its reception beam for the PUCCH.

TS 38.213 Section 6 For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS- C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

In NTN, due to the large propagation delay, the time from which gNB adjusts its reception beam for the PUCCH (with new beam) needs to be clarified. This is illustrated in Figure 1.

Figure 1: Timing relationship of PUCCH with new beam

Hence, it is necessary to enhance the timing relationship on PUCCH transmission/reception beam. This is a typical downlink to uplink timing interaction problem. Like other timing relationship enhancements (e.g., transmission timing of HARQ-ACK on PUCCH), we could introduce Koffset to enhance the timing relationship on PUCCH. Specifically, the PUCCH with new beam is transmitted after K'offset+28 symbols after the uplink symbol corresponding to the last downlink symbol of PDCCH reception with beam failure recovery response, where K'offset is Koffset converted to symbols. This is shown in Figure 2.

Figure 2: Illustration of introducing Koffset to enhance PUCCH transmission time

Proposal 11: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

# Conclusion

In this contribution, we provided our views on timing relationship enhancement for NTN. Our proposals are as follows:

Proposal 1: Support to signal a single offset value for cell specific Koffset.

Proposal 2: For signaling of updating Koffset after initial access, both RRC reconfiguration and MAC CE are supported.

Proposal 3: Support UE reports the preferred differentiated UE specific Koffset for the purpose of uplink scheduling adaptation.

Proposal 4: Support at least event-triggered UE reporting for the purpose of uplink scheduling adaptation.

Proposal 5: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timing of HARQ-ACK on PUCCH to Msg4/MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 6: If a UE is provided with a Koffset value beyond system information, then that Koffset value is used for the transmission timings scheduled by fallback DCI formats.

Proposal 7: The scheduling offset Kmac is carried in system information, in the unit of milli-seconds or in the unit of slots for a given subcarrier spacing in a frequency range.

Proposal 8: In estimating the UE-gNB RTT for RAR window offset, set NTA to 0 and set NTA,  offset based on the signaled value or default value for a given frequency range.

Proposal 9: The K1 range extension does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

Proposal 10: RAN1 to enhance the timing relationship on the beam failure recovery response window offset, by introducing Kmac.

Proposal 11: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

# References

- RP-211557, “Solutions for NR to support non-terrestrial networks (NTN),” Mar. 2021.

- R1-2106325, “Feature lead summary #5 on timing relationship enhancements,” May 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #105-e Meeting, May 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104b-e Meeting, Apr. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- R1-2104230, “LS on TA pre-compensation,” May 2021.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, v17.2.0, July 2021.

- 3GPP TS 38.213, NR; Physical layer procedures for control, v16.5.0, Mar. 2021.