3GPP TSG RAN WG1 #106-e	R1- 2107742

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.5.3

Source:	Apple Inc.

Title:	Accuracy enhancements for DL-AoD positioning technique

Document for:	Discussion, Decision

# Introduction

In 3GPP RAN#90-e, a work item for Rel-17 NR positioning enhancements was approved and was updated in RAN#91-e [1]. One of the objectives of Rel-17 positioning WI, is to specify solutions to enhance positioning accuracy, as it can be found below [1]:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

In this contribution, we present our views on potential techniques and required signalling to enhance NR positioning accuracy for angle-based positioning techniques. More specifically, in this paper we discuss DL-AoD positioning technique and in our companion contribution [2], we consider solutions to enhance accuracy for UL-AoA positioning technique.

# Accuracy Enhancements for DL-AoD Positioning Technique

## Enhancements on measurements and reports associated to the first arrival path

In RAN1#104e couple of options was agreed, which were further discussed in RAN1#104b-e and RAN1#105-e, as part of enhancements to measure and report information related to the first arriving path. As a result, it was agreed in the previous meeting that [3]:

Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path

- FFS: Details of measurement and reporting of PRS RSRP of the first path

Although some details, like how the “first path” is selected/identified, what additional assistance data is required at UE, etc, are still under further discussions. Measurements and reports associated to the first arrival path is useful if the first arrival path is detected. In case the LoS path goes through a sever blockage, such that the first detected path by UE is indeed a NLoS path, measurements associated to the first detected path will not result to enhanced accuracy. Thus, as mentioned in our companion paper [2], we think the path-specific PRS-RSRP measurement should be performed within a configured time window. The concept of time window for positioning is already defined in Rel-16 specification. For example, in 38.214 Sec. 5.1.6.5, the UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS. Similarly, a “search window” is defined for UL-RTOA technique, which “indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Rreference Time” (38.455, Sec. 9.2.26). This concept can be extended for PRS-RSRP measurements at a UE, where UE is assisted by LMF about a timing window per TRP, wherein the LOS path for a PRS reception from that TRP is expected to arrive. The configuration of such window needs some priori knowledge of UE’s location, similar to the configuration of nr-DL-PRS-ExpectedRSTD. Once the window is indicated by LMF, UE may be further indicated to perform PRS-RSRP measurement only based on paths that lie within the window, i.e., all paths that arrive out of the window duration will not contribute into PRS-RSRP measurements.

Proposal 1: For DL-AoD technique, support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

- Alternatively, or additionally, for DL-AoD technique, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

To improve the accuracy of DL-AoD, RAN1 made couple of agreements in RAN1#104e, including to increase number of PRS-RSRP measurements per TRP, for the same PRS resource or different PRS resources.

Agreement: For UE-assisted DL AOD, select one of the following options for reporting of RSRP measurements per TRP

- Option 1: Up to 8 measurements in a measurement report (as in release 16)

- Option 2: Up to 8 measurements in a measurement report, for the same Rx beam index

- Option 3: Up to N>=8 measurements

  - Note: Multiple measurements corresponding to different Rx Beam index may be  reported for a given PRS resource.

  - FFS: value for N.

The above discussion was continued in RAN1#104b-e and RAN1#105-e, although no agreement was made. From 38.214 Sec. 5.1.6.5, in Rel-16 and subject to UE capability, UE can be configured to measure and report, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with a TRP. If UE performs, e.g. 8 PRS-RSRP measurements per PRS resource, although the PRS-RSRP associated to that PRS resource becomes more reliable, but it does not necessarily resolve/identify whether or not this PRS is received through a NLOS. On the other hand, although in theory increasing number of PRS-RSRP measurements per TRP for different PRS resources, should help to identify NLOS impact, but the improvement is not quantified. Thus, we make the following proposal:

Proposal 2: For reporting of RSRP measurements per TRP, subject to UE capability, support Option 1, i.e. up to 8 measurements in a measurement report, as in release 16.

## Further assistance information for DL-AoD

In order to eliminate Pos-SRS reception from NLOS paths, it was agreed in RAN1# 104e that LMF indicates to TRP an expected AoA value, plus an uncertainty range associated with the expected AoA. RAN1 had a similar discussion in 104b-e for the dual angle-based positioning technique in UL, i.e. DL-AoD. The following agreement is made in RAN1#104b-e [4]:

Agreement: For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options:

- Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

- Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP]

- Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

- Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced.

- FFS: details of signaling

The matter was further discussed in RAN1#105-e, although no additional agreement was made. In our view, option 2 needs additional side information about UE location to be available at LMF which in general is not feasible. On the other hand, Option 1 is simply dual of previous RAN1 agreement in UL-AoA for DL-AoD. The usage of Option 1 is justified, given that such side assistance information will help UE to eliminate measurements on PRS beams received on NLOS. Thus we propose:

Proposal 3: For DL-AoD technique, support DL-AoD/ZoD assistance information (expected and uncertainty window), signaled from LMF to the UE for each TRP measurement.

# Conclusions

In this contribution, we provided our thoughts on potential techniques to enhance the positioning accuracy for the NR DL-AoD positioning technique. Based on views shared in this paper, we make the following proposals:

Proposal 1: For DL-AoD technique, support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

- Alternatively, or additionally, for DL-AoD technique, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

Proposal 2: For reporting of RSRP measurements per TRP, subject to UE capability, support Option 1, i.e. up to 8 measurements in a measurement report, as in release 16.

Proposal 3: For DL-AoD technique, support DL-AoD/ZoD assistance information (expected and uncertainty window), signaled from LMF to the UE for each TRP measurement.

# References

- RP-210903, “New WID on NR Positioning Enhancements”, March 2021

- R1- 2107741, Accuracy enhancements for UL-AoA positioning technique, Apple Inc.

- Chairman’s Notes RAN1#105-e final

- Chairman’s Notes RAN1#104b-e final